| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E473 |
| --- | --- |

| Title: | Verification of JCTVC-E335: Scans for coefficient level coding with CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Frank Bossen 3240 Hillview Ave Palo Alto, CA 94304 USA | Tel: Email: | +1 650 496 4742 bossen@docomolabs-usa.com |
| Source: | DOCOMO USA Labs |  |  |

_____________________________

# Abstract

This contribution partially confirms the results presented in contribution JCTVC-E335 on scans for coding coefficient levels with CABAC. Two parts are described in E335 and only the first part titled “Unified scan patterns” is addressed in this contribution.

# Introduction

Software implementing JCTVC-E335 on top of HM 2.0 basis was provided by Qualcomm. Modifications include:

- ContextTables.h: changing the values of the arrays INIT_ONE_FLAG and INIT_ABS_FLAG defining initial probabilities to be used with CABAC. The size of the tables is unchanged, suggesting that the number of contexts is the same as for HM 2.0. The values of the arrays are non-trivial (i.e., not systematic 50/50 probability initialization). It is unclear how these values were derived (same applies to values in HM 2.0).

- TComTrQuant.cpp: changing the RDOQ function to reflect the proposed change in the scan order

- TypeDef.h: adding control macros

- TDecSbac.cpp: changing the parsing of coefficient data to reflect the proposed change in the scan order

- TEncSbac.cpp: changing the coding of coefficient data to reflect the proposed change in the scan order

Overall the code seems to be somewhat simplified with respect to HM 2.0, as the case for coding a 4x4 residual block is not treated separately.

# Results

Coding results using common conditions (High-Efficiency settings) are summarized in the table below.

|  | Intra |  |  | Random access |  |  | Low-delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | Y | U | V | Y | U | V |
| A | 0.2 | 0.0 | 0.0 | 0.3 | -0.3 | -0.1 | --- | --- | --- |
| B | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 | 0.1 | -0.6 | -0.6 |
| C | -0.1 | 0.0 | 0.0 | 0.1 | -0.2 | -0.1 | 0.0 | -0.5 | -0.5 |
| D | -0.1 | -0.1 | -0.1 | 0.0 | -0.5 | -0.3 | -0.1 | -1.5 | -1.5 |
| E | 0.1 | 0.1 | 0.1 | --- | --- | --- | -0.1 | -1.4 | -0.2 |
| All | 0.0 | 0.0 | 0.0 | 0.1 | -0.3 | -0.1 | 0.0 | -1.0 | -0.7 |
| Enc T | 97% |  |  | 100% |  |  | 100% |  |  |
| Dec T | 98% |  |  | 99% |  |  | 99% |  |  |

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).