| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E475 |
| --- | --- |

| Title: | Cross Verification of Combination of JCTVC-E046 and JCTVC-E323 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jizheng Xu | Email: | jzxu@microsoft.com |
| Source: | Microsoft |  |  |

_____________________________

# Abstract

This document reports cross-verification results for the combined scheme of JCTVC-E046 and JCTVC-E323.

# Introduction

We got the source code of the combined scheme of JCTVC-E046 and JCTVC-E323 from Qualcomm and independently compiled and ran the code with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

The objective results we got match theirs. The encoding/decoding times compared with the anchors’ are also similar.

# Results

The summary of the results are shown in the following tables. The detail results can be found in the file JCTVC-E475.xls.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.0 |
| Class B | -0.1 | 0.1 | 0.1 |
| Class C | -0.2 | 0.1 | 0.1 |
| Class D | 0.0 | 0.1 | 0.1 |
| Class E | -0.1 | 0.1 | 0.1 |
| All | -0.1 | 0.1 | 0.1 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 93% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | 0.1 | 0.0 |
| Class B | -0.7 | 0.1 | 0.1 |
| Class C | -0.5 | 0.2 | 0.2 |
| Class D | -0.2 | 0.1 | 0.1 |
| Class E |  |  |  |
| All | -0.5 | 0.1 | 0.1 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 89% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.9 | -0.1 | 0.0 |
| Class C | -0.8 | 0.2 | 0.1 |
| Class D | -0.3 | -0.3 | -0.1 |
| Class E | -1.3 | 0.1 | 0.0 |
| All | -0.8 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 90% |  |  |