| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E478 |
| --- | --- |

| Title: | BoG report on extending LCEC to larger block sizes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco M. Karczewitz, Qualcomm M. Haque, Sony | Tel: Email: Tel: Email: Tel: Email: | +47 92414082 arild.fuldseth@cisco.com martak@qualcomm.com +1-408-352-4099 munsi.haque@am.sony.com |
| Source: | Cisco, Qualcomm, Sony |  |  |

_____________________________

# Abstract

The purpose of this BoG group was to reach a common understanding of the difference between contributions JCTVC-E319 (Sony) and JCTVC-E383 (Qualcomm) with respect to extending LCEC to block sizes larger than 8x8.

# Companies participating

Qualcomm, Sony, Nokia, Telnet, Panasonic, Cisco

# Summary of results

Both JCTVC-E319 and JCTVC-E383 contributions propose to extend the number of transform coefficients encoded by LCEC from 8x8 to 16x16 and 32x32.  The main differences between the two contributions are the following:

JCTVC-E319 (Sony)

- Extension to 32x32 increases table sizes by at least 30 KB and 20 KB in the encoder and decoder respectively.

- Coding gains of 2.4%, 1.5%, and 0.5% for intra, random access, and low delay, respectively are reported.

JCTVC-E383 (Qualcomm)

- This contribution reduces the total table size of LCEC by

    - Removing/reducing some of the major tables used for coding of 8x8 blocks.

    - Extending the coefficient coding to 16x16 and 32x32 blocks without adding any new major tables.

- Coding gains of 2.9%, 1.4% and 1.0% for intra, random access, and low delay, respectively are reported.

# Recommendations

Based on the results above, the BoG recommends adopting JCTVC-E383 in the HM.