| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E479 |
| --- | --- |

| Title: | Cross-verification of Yonsei university’s proposal on coefficient coding table improvement in LCEC (JCTVC-E446) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jianle Chen Sunil Lee JeongHoon Park | Email: | jianle.chen@samsung.com sunil.lee@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This document reports the cross-verification results of Yonsei university’s proposal on coefficient coding table improvement in LCEC (JCTVC-E446). The test results provided in JCTVC-E446 were confirmed. The source code was also investigated and it was confirmed that it matched to the description of the proposal.

# Test Results

The HM2.0-based implementation of the method proposed in JCTVC-E446 was provided by the proponent. As a result of the code-level investigation, it was verified that the implementation of the algorithm is consistent with the description of the proposal [1]. Software was compiled on 64-bit Linux and executed on Linux cluster. Simulation tests were conducted under common test condition [2]. The test results summarized in Table 1 shows that the results are matched with the BD-rate provided by proponent. More detailed information of experimental results can be found in the accompanying Excel sheets.

Table 1. Cross-verified performance of the method proposed in JCTVC-E446

| Config. Class | Intra LoCo |  |  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9 | -0.3 | -0.4 | -0.8 | 0.7 | 0.7 |  |  |  |
| Class B | -0.6 | -0.6 | -0.5 | -0.4 | 0.1 | 0.0 | -0.1 | 0.1 | 1.2 |
| Class C | -0.4 | -0.5 | -0.5 | -0.2 | -0.1 | -0.2 | -0.1 | 0.7 | 0.4 |
| Class D | -0.4 | -0.4 | -0.5 | -0.3 | 0.0 | -0.3 | 0.0 | 0.5 | 0.5 |
| Class E | -0.5 | -0.0 | -0.2 |  |  |  | 0.1 | 0.8 | 2.1 |
| Average | -0.6 | -0.4 | -0.4 | -0.4 | 0.2 | 0.0 | 0.0 | 0.8 | 1.0 |

# Conclusion

The further investigation of this proposal, possibly in the next CE on the LCEC improvement, is recommended.

# References

[1] Seongwan Kim, Jaeho Lee, and Sangyoun Lee, “Coefficient coding table improvement in LCEC,” JCTVC-E446, Geneva, Mar. 2011.

[2] Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, Daegu, Korea, Jan. 2011.