| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E481 WG11 Number: m20194 |
| --- | --- |

| Title: | BoG report of CE9: MV Coding and Skip/Merge operations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | B. Bross J. Jung Y.-W. Huang Y. H. Tan Il-Koo Kim T. Sugio M. Zhou T.K. Tan E. Francois Kimihiko Kazui Wei-Jung Chien S. Sekiguchi S. Park Wade Wan | Email: | benjamin.bross@hhi.fraunhofer.de joelb.jung@orange-ftgroup.com yuwen.huang@mediatek.com yhtan@i2r.a-star.edu.sg ilkoo.kim@samsung.com sugio.toshiyasu@jp.panasonic.com zhou@ti.com ttk@pacific.net.sg edouard.francois@crf.canon.fr kazui.kimihiko@jp.fujitsu.com wchien@qualcomm.com Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp seungwook.park@lge.com wwan@broadcom.com |
| Source: | Fraunhofer HHI, Orange Labs, MediaTek, I2R, Samsung, Panasonic , TI, NTT DOCOMO, Canon, Fujitsu, Qualcomm, Mitsubishi, LGE, Broadcom. |  |  |

_____________________________

# Abstract

This contribution summarizes the activities of BoG on motion vector coding and skip/merge operations in CE9.

# Attendees

Edouard Francois, Guillaume Laroche, Patrice Onno (Canon)

Benjamin. Bross (Fraunhofer HHI)

Kimihiko Kazui (Fujitsu)

Yih Han Tan (I2R)

S. Park, J. Park (LGE)

Shun-ichi Sekiguchi (Mitsubishi)

Yu-Wen Huang, Xun Guo (Mediatek)

T.K. Tan, Y. Suzuki, A. Fujibayashi  (NTT DOCOMO)

Joel Jung, Gordon Clare (Orange FT)

T. Sugio (Panasonic)

Wei-Jung Chien, Muhammed Coban (Qualcomm)

Woo-Jin Han, Il-Koo Kim, Jianle Chen (Samsung)

M. Zhou (TI)

W. Wan (Broadcom)

# Introduction

In the Core Experiment 9 on motion vector coding and skip/merge operations, the following two schemes were identified to be the best performing [1][2].

| Experiment | Inter | MERGE | SKIP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Candidates | Candidates | Candidates | ref_idx | inter_pred_idc |  |
| M31 | D125 S2.1 (1 T MVP*) S2.3 D274 (3') / S2.4’ | D125 S2.4’ | D125 S2.1 (1 T MVP*) S2.3 D274 (3') / S2.4’ | D274 (2) |  | HM2.0 |
| T | D125 S2.1 (1 T MVP*) S2.3 D274 (3') / S2.4’ | D125 S2.1(1 T MVP*) D274 (3') / S2.4’ | D125 S2.1(1 T MVP*) D274 (3') / S2.4’ | derived |  | derived |

The following two schemes were identified to represent simplifications:

| Experiment | Inter | MERGE | SKIP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Candidates | Candidates | Candidates | ref_idx | inter_pred_idc |  |
| L | HM2.0 | HM2.0 | D051 (MERGE reduced candidate set) | derived |  | derived |
| F | D055-c | HM2.0 | D055-c | HM2.0 |  | HM2.0 |

# Interaction between temporal motion vector and memory compression

It was raised the concern that changing the temporal motion vector predictor (TMVP) is closely related to motion vector memory compression (MVMC) issues.

It was suggested to test the best performing simplified proposals with the TMVP used in HM2.0. This is desired because changing the TMVP would affect motion vector memory compression experiments. Due to issues with the current MVMC implementation in HM2.0, reliable conclusions on a TMVP using the current MVMC cannot be drawn. Thus, keeping the HM2.0 TMVP is desired to have, together with a MVMC fix, a clean anchor for further experiments. A MVMC fix has already been adopted at this meeting. This fix will be integrated in the final tool combination suggestion.

The interaction between the TMVP and MVMC schemes can be summarized as follows:

| MVMC scheme (MV kept) | MV Prediction scheme |  |
| --- | --- | --- |
| Top left | D125 S2.1 (1 T MVP*) | Experiment M01 from CE9 |
| Bottom right | HM2.0 collocated | JCTVC-E307 / JCTVC-E092 / … |
| Center | HM2.0 collocated | JCTVC-E059 |

# Tests on simplifications

The following tests were performed to investigate the influence of the simplifications on the best performing schemes.

| Experiment | Inter | MERGE | SKIP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Candidates | Candidates | Candidates | ref_idx | inter_pred_idc |
| M31* | D125 S2.1 (1 T MVP*) S2.3 D274 (3') / S2.4’ | D125 S2.1 (1 T MVP*) D274 (3') /S2.4’ | D125 S2.1 (1 T MVP*) S2.3 D274 (3') /S2.4’ | D274 (2) | HM2.0 |
| M31+F | D125 S2.1 (1 T MVP*) S2.3 +F D274 (3')/S2.4’ | D125 S2.4’ | D125 S2.1 (1 T MVP*) S2.3 + F D274 (3') / S2.4’ | D274 (2) | HM2.0 |
| M31* + F | D125 S2.1 (1 T MVP*) S2.3 + F D274 (3') / S2.4’ | D125 S2.1(1 T MVP*) D274 (3')/ S2.4’ | D125 S2.1 (1 T MVP*) S2.3 + F D274 (3')/ S2.4’ | D274 (2) | HM2.0 |
| T+F | D125 S2.1 (1 T MVP*) S2.3 + F D274 (3')/ S2.4’ | D125 S2.1(1 T MVP*) D274 (3') / S2.4’ | D125 S2.1(1 T MVP*) D274 (3') / S2.4’ | derived | derived |
| T+L+F | D125 S2.1 (1 T MVP*) S2.3 + F D274 (3')/ S2.4’ +F | D125 S2.1(1 T MVP*) D274 (3')/S2.4’ | D125 S2.1(1 T MVP*) D274 (3') / S2.4’ + L simplification | derived | derived |

| Experiment | Software provided by | Tester 1 | Tester 2 |
| --- | --- | --- | --- |
| M31* | Tester | MediaTek | HHI |
| M31+F | TI | TI | HHI |
| M31*+F | TI | TI | MediaTek |
| T+F | HHI | Samsung | I2R |
| T+L+F | I2R | Panasonic | I2R |

# Unification of M31 and T with simplifications

Some participants tried to unify M31 and T further by adding JCTVC-D274 (2) to the TMVP of merge. The remaining difference is now whether AMVP or MERGE is used for skip mode.

| Experiment | Inter | MERGE | SKIP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Candidates | Candidates | Candidates | ref_idx | inter_pred_idc |
| MERGE Skip (T* + F) | D125 S2.1 (1 T MVP*) S2.3 +F D274 (3')/ S2.4’ | D125 S2.1(1 T MVP*) D274 (3') / S2.4’ T MVP D274 (2) | D125 S2.1(1 T MVP*) D274 (3') / S2.4’ | derived T MVP D274 (2) | derived |
| AMVP Skip (M31** + F) | D125 S2.1 (1 T MVP*) S2.3 + F D274 (3') / S2.4’ | D125 S2.1(1 T MVP*) D274 (3')/ S2.4’ T MVP D274 (2) | D125 S2.1 (1 T MVP*) S2.3 + F D274 (3')/ S2.4’ | D274 (2) | HM2.0 |

It was also tested how keeping the HM2.0 TMVP performs.

| Experiment | Inter | MERGE | SKIP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Candidates | Candidates | Candidates | ref_idx | inter_pred_idc |
| MERGE Skip HM2.0 TMVP | S2.3+F D274 (3')/ S2.4’ | D274 (3') / S2.4’ T MVP D274 (2) | D274 (3') / S2.4’ | derived T MVP D274 (2) | derived |
| AMVP Skip HM2.0 TMVP | S2.3+F D274 (3') / S2.4’ | D274 (3')/ S2.4’ T MVP D274 (2) | S2.3 + F D274 (3')/ S2.4’ | D274 (2) | HM2.0 |

| Experiment | Software provided by | Tester 1 | Tester 2 |
| --- | --- | --- | --- |
| MERGE Skip | Panasonic | HHI | MediaTek |
| AMVP Skip | Panasonic | Panasonic | MediaTek |
| MERGE Skip HM2.0 TMVP | Panasonic | HHI | MediaTek |
| AMVP Skip HM2.0 TMVP | Panasonic | Panasonic | MediaTek |

# Results

Average BD-rates [%] and encoding runtimes [%] (Tester1 / Tester2)

| Experiment | RA HE |  | RA LC |  | LD HE |  | LD LC |  | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M31 | -1.3 | 103 | -1.8 | 104 | -1.5 | 105 | -2.5 | 107 | -1.8 |
| M31* | -1.3 | 104/103 | -1.8 | 104/102 | -1.5 | 105/104 | -2.5 | 107/105 | -1.8 |
| T | -1.4 | 100 | -2.0 | 99 | -1.9 | 102 | -1.1 | 103 | -1.6 |
| M31+F | -1.3 | /102 | -1.8 | /102 | -1.4 | /103 | -2.5 | /104 | -1.8 |
| M31*+F | -1.3 | /103 | -1.8 | /104 | -1.5 | /105 | -2.5 | /107 | -1.8 |
| T+F | -1.4 | /100 | -2.0 | / 99 | -1.9 | /101 | -1.1 | /102 | -1.6 |
| T+L+F | -1.3 | /100 | -1.9 | /100 | -1.7 | /102 | -1.0 | /103 | -1.5 |
| MERGE Skip | -1.4 | 97 | -1.9 | 96 | -2.1 | 98 | -1.6 | 99 | -1.8 |
| AMVP Skip | -1.3 | 103 | -1.7 | 103 | -1.6 | 105 | -2.6 | 106 | -1.8 |
| MERGE Skip HM2.0 TMVP | -0.6 | 99 | -1.0 | 98 | -0.9 | 101 | -0.3 | 102 | -0.7 |
| AMVP Skip HM2.0 TMVP | -0.6 | 102 | -0.9 | 103 | -0.8 | 104 | -1.6 | 105 | -1.0 |

# Conclusions

- Adoption of F simplification

- Adoption of MERGE Skip configuration

# Configuration details

Details of the proposed configuration are provided in the following sections.

## D125 S2.3 / D095 - Modified spatial MVP derivation process

A priority-based scheme is applied for deriving each spatial MVP candidate. In the spatial MVP derivation process of HM 2.0, only the MV with the same reference list and the same reference picture index is considered as an available spatial MVP candidate. In the proposed priority-based scheme, a spatial MVP candidate can be derived from different list and different reference picture. Figure 8-1 shows an example of a B-slice with single reference picture per list (picture j for list 0 and picture l for list 1). The current list is list 0, and the current reference picture is picture j. The dotted blue arrow denotes the list 0 MV of the neighboring block, and the dotted red arrow denotes the list 1 MV of the neighboring block. The numbers denote the priorities of the two MVs. When the list 0 MV is available, it is used as the spatial MVP candidate. Otherwise, when the list 1 MV is available, it is scaled to the current reference picture as the solid red arrow and then used as the spatial MVP candidate.

Figure 8-1 Example of deriving a spatial MVP candidate for a B-slice with single reference picture per list.

Figure 8-2 shows another example of a B-slice with two reference pictures per list (pictures j and i for list 0, pictures m and j for list 1). The current list is list 0, and the current reference picture is j. The dotted purple arrow, the dotted red arrow, the dotted green arrow, and the dotted blue arrow denote the possible list 0 MV for picture j, the possible list 0 MV for picture i, the possible list 1 MV for picture m, and the possible list 1 MV for picture j, respectively. The solid arrows denote the scaled MVs of the four possible MVs to the current reference picture, j. The scaling factors are based on relative temporal distances. The numbers denote the priorities of scaled MVs for deriving the spatial MVP candidate. The general rule is stated as follows. There is one (list 0) or two MVs (list 0 and list 1) in a neighboring block. The MV of the neighboring block with the reference picture same as the current reference picture is used as the spatial MVP candidate. When the two MVs of the neighboring block both have the reference picture same as the current reference picture, the one with the list same as the current list is used as the spatial MVP candidate. When the two MVs of the neighboring block both have reference pictures different from the current reference picture, the one with the list same as the current list is scaled and then used as the spatial MVP candidate.

Figure 8-2 Example of deriving a spatial MVP candidate for a B-slice with multiple reference pictures per list.

## F / D055 - Reduced search in left/top spatial MVP candidate detection

As shown in the right part of Figure 8-3, left/top MVP candidate calculation is simplified compared to the current HM 2.0 method shown in the left part of the figure. For the search of left predictor, instead of scanning all the left neighboring PUs from bottom to top, only up to two neighboring PUs, Am+1,Am, are used. Similarly, For the search of top predictor, only up to three neighboring PUs, Bn+1,Bn ,B-1 are used.

Figure 8-3 HM 2.0 spatial candidate scanning (left) and the simplified scaning process (right)

## D274 (3') / D125 S2.4' - Modified TMVP derivation process

One temporal motion vector predictor (mvL0Col or mvL1Col) is selected according to referred direction and added to mvpListX. The flowchart of modified derivation process of temporal motion vector predictors is shown in Figure 8-4. Non-highlighted blocks are same as original HM2.0 process, and highlighted blocks are newly added in this modification.

Figure 8-4 flowchart of modified derivation process for temporal motion vector predictor

## D125 S2.1 (1 T MVP*) - Modified TMVP candidate position

The temporal MVP candidate, TBR, is located below right to the temporal collocated PU as shown in Figure 8-5. If this candidate is not available, the centered collocated candidate from the HM 2.0 is used.

The AMVP candidate set becomes:

- Left predictor

- Top predictor

- TBR

Figure 8-5 The temporal candidate at the above left position of the below right PU

## T MVP D274 (2) - Modified TMVP refidx derivation for merge

In HM2.0, the reference index for the TMVP used in MERGE mode is always assumed to be 0.

The reference index for the MERGE mode TMVP follows the majority of reference indices of neighboring prediction units A, B and C (Figure 8-5). If prediction unit C is not available, prediction units D or E is used. The TMVP is scaled accordingly.

Figure 8-5 Neighboring prediction units to decide reference indices for skip mode

## Merge Skip

In HM2.0, the skip mode uses the AMVP method to derive motion information that is slice dependent. For P slices, a motion vector predictor (MVP) index is signaled for list0 prediction. For B slices, a MVP index is signaled for list0 prediction and a MVP index is signaled for list1 prediction. The reference index is always set to 0, the motion vector(s) is(are) equal to the MVP(s) and no residual is transmitted.

If the skip mode uses the MERGE method derive motion information, a merge index is signaled. The motion information is fully derived from the neighboring block indexed by the merge index and no residual is transmitted either.

# Concerns to be further studied

Following points are suggested to be further studied in either AGH or CE.

- Unification of the temporal candidates (H or Collocated): suggestion to check one position only

- Study of spatial MVP scaling for small (8x8, 4x4) blocks

- Study of the use of temporal MVP for small (16x16, 8x8, 4x4) blocks

- Skip mode with no fixed reference index

Encourage to study unified solutions for candidate positions for AMVP and Merge

# Working Draft Text

## Syntax

### MERGE skip

In 7.3.7	Prediction unit syntax:

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| ... |  |

## Decoding Process

### D125 S2.3 / D096 and F / D055

##### Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A,or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-6 – Spatial motion vector neighbours

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of nA two sample locations be (xAk, yAk), with k = 0 .. nA k = 0 .. 1, specifies sample locations with xAk = xP – 1, yA0 = yP and yAk-1 = yAk - MinPuSize xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere, nA = nPSH / MinPuSize]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xAnA, yAnA ) to ( xA0, yA0 ) where yAk-1 = yAk - MinPuSize ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- If availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] and ListA is set equal to LY.

- Otherwise, if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise, if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- If availableFlagLXA is equal to 1, the following applies.

- If RefPicOrderCnt( currPic , refIdxA, ListA ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXA = mvLXA	(8-XX)

- Otherwise, mvLXA is derived as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-XX)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-XX)

mvLXA = ClipMv( ( DistScaleFactor * mvLXA + 128 ) >> 8 )	(8-XX)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxA, ListA ) )	(8-XX)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-XX)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of nB three sample location (xBk, yBk), with k = -1,0 .. nB 0,1,2, specifies sample locations with yBk = yP – 1, xB-1 = xP - MinPuSize, xB0 = xP , and xBk-1 = xBk - MinPuSize xB0 = xP + nPSW, xB1 = xB0 MinPuSize , and xB2 = xP - MinPuSize. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere, nB = nPSW / MinPuSize]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB1 ) to ( xB2, yB2 ) where xBk-1 = xBk - MinPuSize xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1 and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- If availableFlagLXB is equal to 0, for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ](with Y = !X) is equal to 1 and RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ] and ListB is set equal to LY.

- Otherwise, if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise, if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- If availableFlagLXB is equal to 1, the following applies.

- If RefPicOrderCnt( currPic , refIdxB, ListB ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXB = mvLXB	(8-XX)

- Otherwise, mvLXB is derived as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-XX)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-XX)

mvLXB = ClipMv( ( DistScaleFactor * mvLXB + 128 ) >> 8 )	(8-XX)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxB, ListB ) )	(8-XX)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-XX)

- If availableFlagLXB is equal to 1 and mvLXB is identical to mvLXA, availableFlagLXB is set equal to 0.

### D274 (3') / D125 S2.4' and D125 S2.1 (1 T MVP*)

##### Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb =  xP +  nPSW  	(8-XX)

yPRb =  yP +  nPSH   	(8-XX)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb + 2 ) >> 4 ) << 4, ( yPRb + 2 ) >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

- Centrol luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1  	(8-XX)

yPCtr = ( yP + ( nPSH >> 1 ) – 1   	(8-XX)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr + 2 ) >> 4 ) << 4, ( yPCtr + 2 ) >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following applies.

    - The following asignments are made with X being 0 or 1.

      - RefIdxColLX = RefIdxLX[ xPCol ][ yPCol ]

      - ColDistX = abs(RefPicOrderCnt( colPic, RefIdxColLX, LX ) - PicOrderCnt( colPic ))

      - If PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than PicOrderCnt( currPic ), the variable MvXCross is derived as follows.

        - MvXCross = 1

      - Otherwise (PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than or equal to PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than or equal to PicOrderCnt( currPic )), the variable MvXCross is derived as follows.

        - MvXCross = 0

    - If  one of the following conditions is true, the motion vector mvCol is set equal to MvL1[ xPCol ][ yPCol ], the reference index refIdxCol is set equal to RefIdxColL1 and ListCol is set equal to L1.

      - Mv0Cross is equal to 0 and Mv1Cross is equal to 1

      - Mv0Cross is equal to Mv1Cross and reference index list is equal to L1 ColDist1 is less than ColDist0

    - Otherwise, the motion vector mvCol is set equal to MvL0[ xPCol ][ yPCol ], the reference index refIdxCol is set equal to RefIdxColL0 and ListCol is set equal to L0.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, L0 ListCol) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxCol refIdxLX, LX ),

mvLXCol = mvCol	(8-XX)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-XX)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-XX)

mvLXCol = ClipMv( ( DistScaleFactor * mvCol + 128 ) >> 8 )	(8-XX)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, L0 ListCol ) )	(8-XX)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxCol refIdxLX, LX ) )	(8-XX)

### T MVP D274 (2)

##### Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N and the prediction list utilization flags predFlagL0N and predFlagL1N with N being replaced by A, B, C or D.

- The derivation process of reference indices for temporal merging candidate in subclause x.x.x.x.x is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.
The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX refIdxLXCol (with X being 0 or 1, respectively) equal to 0 as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-1)

predFlagLXCol = availableFlagLXCol	(8-2)

- The merging candidate list, mergeCandList, is constructed of which elements are given as specified order:

  - A, if availableFlagA is equal to 1

  - B, if availableFlagB is equal to 1

  - Col, if availableFlagCol is equal to 1

  - C, if availableFlagC is equal to 1

  - D, if availableFlagD is equal to 1

- If several merging candidates have the motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- If the number of elements NumMergeCand within the mergeCandList is equal to 1, mergeIdx is set equal to 0, otherwise, mergeIdx is set equal to merge_idx[ xP][ yP ].

- The following assignments are made with N being the candidate at position mergeIdx in the merging candidate list mergeCandList ( N = mergeCandList[ mergeIdx ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-3)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-4)

refIdxLX = refIdxLXN	(8-5)

predFlagLX = predFlagLXN	(8-6)

##### Derivation process of reference indices for temporal merging candidate

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

Outputs of this process are (with X being replaced by 0 or 1)

- the reference indices refIdxLX of the current prediction units,

Figure x-x – Spatial neighbours, whcich can be used in deriving reference indices for temporal merging candidates,  (informative) illustrates the position of the spatial neighbours A, B, C, D and E relative to the current prediction unit.

For the derivation of refIdxLXN, with N being A, B and C the following applies.

If the following conditions is true, refIdxLXA is assigned to refIdxLX[ xP-1, yP ].

  - The prediction unit covering luma location ( xP-1, yP ) is available and PredMode is not MODE_INTRA.

Otherwise, refIdxLXA is assigned to -1.

If the following conditions is true, refIdxLXB is assigned to refIdxLX[ xP, yP-1 ].

  - The prediction unit covering luma location ( xP, yP-1 ) is available and PredMode is not MODE_INTRA.

Otherwise, refIdxLXB is assigned to -1.

If the following conditions is true, refIdxLXC is assigned to refIdxLX[ xP+nPSW, yP-1 ].

  - The prediction unit covering luma location ( xP+nPSW, yP-1 ) is available and PredMode is not MODE_INTRA.

Else if the following conditions is true, refIdxLXC is assigned to refIdxLX[  xP-1, yP+nPSH ].

  - The prediction unit covering luma location ( xP-1, yP+nPSH ) is available and PredMode is not MODE_INTRA.

Else if the following conditions is true, refIdxLXC is assigned to refIdxLX[  xP-1, yP-1 ].

  - The prediction unit covering luma location ( xP-1, yP-1 ) is available and PredMode is not MODE_INTRA.

Otherwise, refIdxLXC is assigned to -1.

The variables refIdxLX is derived as follows.

- If refIdxLXA is equal to refIdxLXB and  refIdxLXB is equal to refIdxLXC, the variables refIdxLX is specified by

  - If  refIdxLXA is equal to -1 then refIdxLX = 0

  - Otherwise refIdxLX = refIdxLXA

- Else if refIdxLXA is equal to refIdxLXB, the variables refIdxLX is specified by

  - If  refIdxLXA is equal to -1 then refIdxLX = refIdxLXC

  - Otherwise refIdxLX = refIdxLXA

- Else if refIdxLXB is equal to refIdxLXC, the variables refIdxLX is specified by

  - If  refIdxLXB is equal to -1 then refIdxLX = refIdxLXA

  - Otherwise refIdxLX = refIdxLXB

- Else if refIdxLXA is equal to refIdxLXC, the variables refIdxLX is specified by

  - If  refIdxLXA is equal to -1 then refIdxLX = refIdxLXB

  - Otherwise refIdxLX = refIdxLXA

- Else if refIdxLXA is equal to -1, the variables refIdxLX is specified by

  - refIdxLX = min( refIdxLXB, refIdxLXC )

- Else if refIdxLXB is equal to -1, the variables refIdxLX is specified by

  - refIdxLX = Min( refIdxLXA, refIdxLXC )

- Else if refIdxLXC is equal to -1, the variables refIdxLX is specified by

  - refIdxLX = Min( refIdxLXA, refIdxLXB )

- Otherwise, the variables refIdxLX is specified by

  - refIdxLX = Min( Min( refIdxLXA, refIdxLXB ), refIdxLXC )

### MERGE skip

##### Derivation process for motion vector components and reference indices

Input to this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xB, yB ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- luma motion vectors mvL0 and mvL1 and chroma motion vectors mvCL0 and mvCL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Let ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture where xP = xC + xB and yP = yC + yB.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1 as well as PredFlagL0 and PredFlagL1, the following applies.

-	If PredMode is equal to MODE_SKIP, the derivation process for luma motion vectors for and slice_type is equal to P, the derivation process for luma motion vectors for skipped prediction units in P slices in subclause 8.4.2.1.1 is invoked with the luma location ( xP, yP ), variables nPSW and , nPSH as inputs and the output being the luma motion vector mvL0 and reference index refIdxL0, and the prediction list utilization flag predFlagL0 is set equal to 1. Both components of mvL1 are set to 0, refIdxL1 is set equal to -1 and predFlagL1 is set equal to 0.

-	Otherwise, if PredMode is equal to MODE_SKIP and slice_type is equal to B, the derivation process for luma motion vectors for skipped prediction units in B slices in subclause 8.4.2.1.2 merge mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and , nPSH and the partition index PartIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1.

- If PredMode is equal to MODE_SKIP and slice_type is equal to P, the derivation process for luma motion vectors for skipped prediction units in P slices in subclause  is invoked with the luma location ( xP, yP ), variables nPSW and , nPSH as inputs and the output being the luma motion vector mvL0 and reference index refIdxL0, and the prediction list utilization flag predFlagL0 is set equal to 1. Both components of mvL1 are set to 0, refIdxL1 is set equal to -1 and predFlagL1 is set equal to 0.

- Otherwise, if PredMode is equal to MODE_SKIP and slice_type is equal to B, the derivation process for luma motion vectors for skipped prediction units in B slices in subclause  merge mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and nPSH and the partition index PartIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1.

- Otherwise, if PredMode is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1,, the derivation process for luma motion vectors for merge mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and nPSH and the partition index PartIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1.

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX and in Pred_LX and in the syntax elements ref_idx_lX and mvd_lX, the following applies.

- The variables LcToLx, refIdxLX and predFlagLX are derived as follows.

- If inter_pred_flag[ xP ][ yP ] is equal to Pred_LC and PredLCToPredLx[ ref_idx_lc[ xP ][ yP ] ] is equal to Pred_LX,

refIdxLX = RefIdxLCToRefIdxLx[ ref_idx_lc[ xP ][ yP ] ]	(8-7)					predFlagLX = 1		(8-8)					mvd_lX[ xP ][ yP ][ 0 ] = mvd_lc[ xP ][ yP ][ 0 ]	(8-9)
					mvd_lX[ xP ][ yP ][ 1 ] = mvd_lc[ xP ][ yP ][ 1 ]	(8-10)
					mvp_idx_lX[ xP ][ yP ][ 0 ] = mvp_idx_lc[ xP ][ yP ]	(8-11)
					LcToLx = LX		(8-12)

- Otherwise, if inter_pred_flag[ xP ][ yP ] is equal to Pred_LX or Pred_BI,

refIdxLX = ref_idx_lX[ xP ][ yP ]		(8-13)
					predFlagLX = 1		(8-14)

- Otherwise, the variables refIdxLX and predFlagLX are specified by

refIdxLX = -1		(8-15)
					predFlagLX = 0		(8-16)

- The variable mvdLX is derived as follows.

mvdLX[ 0 ] = mvd_lX[ xP ][ yP ][ 0 ]		(8-17)
				mvdLX[ 1 ] = mvd_lX[ xP ][ yP ][ 1 ]		(8-18)

- When predFlagLX is equal to 1, the variable mvpLX is derived as follows.

- The derivation process for luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and nPSH and refIdxLX as the inputs and the output being mvpLX.

- When predFlagLX is equal to 1, the luma motion vector mvLX is derived as

mvLX[ 0 ] = mvpLX[ 0 ] + mvdLX[ 0 ]	(8-19)
				mvLX[ 1 ] = mvpLX[ 1 ] + mvdLX[ 1 ]	(8-20)

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in subclause Error: Reference source not found is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

##### Derivation process for luma motion vectors for skipped prediction units in P slices

This process is invoked when PredMode is equal to MODE_SKIP and slice_type is equal to P.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

Outputs of this process are

- the motion vector mvL0,

- the reference index refIdxL0.

The reference index refIdxL0 for a skipped prediction unit is derived as

refIdxL0 = 0		(8-21)

For the derivation of the motion vector mvL0 of a MODE_SKIP in P, the process specified in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and nPSH and refIdxL0 as inputs and the output is assigned to mvL0, since the predictor is equal to the actual motion vector.

##### Derivation process for luma motion vectors for skipped prediction units in B slices

This process is invoked when PredMode is equal to MODE_SKIP and slice_type is equal to B.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

Outputs of this process are

- the reference indices refIdxL0, refIdxL1,

- the motion vectors mvL0 and mvL1,

- the prediction list utilization flags, predFlagL0 and predFlagL1.

For X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX, and in Pred_LX and in the syntax elements ref_idx_lX and mvd_lX, the following applies.

The variables refIdxLX and predFlagLX are derived as follows.

- If inter_pred_flag[ xP ][ yP ] is equal to Pred_LX or to Pred_BI, the variables refIdxLX and predFlagLX are specified by

refIdxLX = 0		(8-22)

predFlagLX = 1	(8-23)

- Otherwise, the variables refIdxLX and predFlagLX are specified by

refIdxLX = -1	(8-24)

predFlagLX = 0	(8-25)

The variable mvLX is derived as follows.

- If predFlagLX is equal to 1, the derivation process for luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH and refIdxLX as the inputs and the output is directly assigned to mvLX, since the predictor is equal to the actual motion vector.

- Otherwise (predFlagLX is equal to 0), both components of mvLX are set equal to 0.

# References

- Joel Jung, Benjamin Bross, Peisong Chen, Woo-Jin Han, “Description of Core Experiment 9 (CE9): MV Coding and Skip/Merge operations”, JCTVC-D609, Daegu, January, 2011.

- Joel Jung, Benjamin Bross, Peisong Chen, Woo-Jin Han, “CE9: Summary report of core experiment 9 on MV Coding and Skip/Merge operations”, JCTVC-E029, Geneva, March, 2011.