| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E483 WG11 Number: m20199 |
| --- | --- |

| Title: | BoG on fine granularity slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Shawmin Lei Yu-Wen Huang Qiu Shen | Email: | rickard sjoberg@ericsson com shawmin.lei@mediatek.com yuwen.huang@mediatek.com shenqiu@huawei.com |
| Source: | BoG |  |  |

_____________________________

Ed note: The change-bars in this document capture the following decisions from JCTVC-E_Notes_d6.doc that were made by the JCT-VC group during the presentation of the original report:

- The CE had recommended support of fine granularity slices (harmonizing E298, E260, E043) without restricting slice boundaries to be at LCU boundaries. The BoG produced a proposed syntax for this, which supported establishing a slice boundary at any SCU boundary – signaling an LCU address, a flag specifying whether a sub-LCU address will be sent within the LCU for the slice, and an SCU address within the LCU. It was suggested by a participant that it may be desiralbe to be able to disable the refinement granularity signaling at the sequence level

- A participant suggested limiting the slice granularity to 16x16 or larger. Decision: Agreed.

- The initial BoG proposal did not support 32x32 granularity with 64x64 LCUs. It was agreed to support this. For signaling, we will send the deepening of granularity (relative to LCU size) at the PPS level, using 2 bits. In the slice header, we send first_slice_flag and, if zero, send the necessary number of bits as a u(v) to identify the start position at the specified granularity. Decision: Agreed.

# Background

This is a report from the BoG on unification of fine granularity slice proposals JCTVC-E298 by Huawei, JCTVC-E260 by Ericsson and JCTVC-E043 by MediaTek. All three documents propose to define slices as a sequence of CU blocks instead of largest CU blocks, i.e. removing the restriction to align slices on LCU boundaries. This enables fine control over slice sizes at an average BDR penalty of less than 0.1 percent for 1500 byte slices.

However, the syntax was slightly different in the three proposals and a proponent's break-out group was requested by the JCT-VC group in order to unify the syntax.

Syntax changes for the unification are attached. The changes relative to WD2 are highlighted in yellow.

# Syntax and semantics

## Picture parameter set

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| rbsp_trailing_bits( ) |  |
| } |  |

slice_granularity indicates the slice granularity within a picture. The value of slice_granularity shall not be larger than Min(Log2MaxCUSize-4, log2_diff_max_min_coding_block_size). The variable SliceGranularity is set to the value of (slice_granularity << 1).

## Slice header

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_slice_in_pic_flag | 2 | u(1) |
| if(first_slice_in_pic_flag == 0) { |  |  |
| slice_address | 2 | u(v) |
| } |  |  |
| } |  |  |

first_slice_in_pic_flag indicates whether the slice is the first slice of the picture. If first_slice_in_pic_flag is equal to 1, the variables SliceAddress and LCUAddress are both set to 0 and the decoding starts with the first LCU in the picture.

slice_address specifies the address in slice granularity resolution in which the slice starts and shall be represented by (Ceil(Log2(NumLCUsInPicture))+SliceGranularity) bits in the bitstream where NumLCUsInPicture is the number of LCU:s in a picture.

The variable LCUAddress is set to slice_address>>SliceGranularity and represents the LCU part of the slice address in raster scan order. The variable GranularityAddress is set to slice_address - (LCUAddress << SliceGranularity) and represents the sub-LCU part of the slice address expressed in z-scan order.

The variable SliceAddress is then set to (LCUAddress << ( log2_diff_max_min_coding_block_size << 1)) + (GranularityAddress << ((log2_diff_max_min_coding_block_size << 1) - SliceGranularity) and the slice decoding starts with the largest coding unit possible at the slice starting coordinate.

  - Slice data

| slice_data( ) { | Descriptor |
| --- | --- |
| CurrTbAddr = LCUAddress |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = -1 |  |
| do { |  |
| xCU = HorLumaLocation( CurrTbAddr ) |  |
| yCU = VerLumaLocation( CurrTbAddr ) |  |
| IsCuQpDeltaCoded = 0 |  |
| moreDataFlag = coding_tree( xCU, yCU, Log2TbSize ) |  |
| CurrTbAddr = NextTbAddress( CurrTbAddr ) |  |
| } while( moreDataFlag ) |  |
| } |  |

## Coding tree structure

| coding_tree( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && log2CUSize > Log2MinCUSize && cuAddress(x0 ,y0) >= SliceAddress ) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) { |  |
| cuDepth = Log2MaxCUSize – log2CUSize |  |
| if( cuDepth <= alf_cu_control_max_depth ) |  |
| if( cuDepth == alf_cu_control_max_depth ¦¦ |  |
| split_coding_unit_flag[ x0 ][ y0 ] == 0 ) |  |
| AlfCuFlagIdx++ |  |
| } |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( cuAddress(x1,y0) > SliceAddress ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CUSize – 1 ) |  |
| if(cuAddress(x0,y1) > SliceAddress && moreDataFlag && x1 < PicWidthInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CUSize − 1 ) |  |
| if(cuAddress(x1,y1) > SliceAddress && moreDataFlag && y1 < PicHeightInSamplesL ) { |  |
| moreDataFlag = coding_tree( x0, y1, log2CUSize − 1 ) |  |
| if( moreDataFlag && x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CUSize − 1 ) |  |
| } else { |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| coding_unit( x0, y0, log2CUSize ) |  |
| if( !entropy_coding_mode_flag ) |  |
| moreDataFlag = more_rbsp_data( ) |  |
| else { |  |
| if( get_bit_left( ) > 0 ) |  |
| moreDataFlag = 1 |  |
| else { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } |  |
| } |  |
| } |  |
| return moreDataFlag |  |
| } |  |