| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E487 WG11 Number: m20203 |
| --- | --- |

| Title: | CE8 Subtest 5: Cross-check of eBrisk+TI's proposal (JCTVC-E492) with vertical length-5 filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents Samsung’s cross-check results for eBrisk and TI’s joint proposal JCTVC-E492 [1] to CE8 Subtest 5, on using filters with vertical length-5 shapes. The shapes include 7×5 clipped diamond (same as in JCTVC-E060 [2]) and 19×5 cross (cross shape was proposed in JCTVC-D385 [3] and JCTVC-E342 [4]), both with 12 coefficients. RD performance match exactly, and the reduction in decoding time is also confirmed.

# Cross-check results

Summary of the results are shown in the table below and more detailed results can be found in the accompanying Excel sheet. The simulations were run in the 64-bit Linux cluster environment.

|  | CE8.5 results by eBrisk and TI ( JCTVC-E492 [1] ) |  |  |  | Cross-check results by Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | IO | RA | LD |  | IO | RA | LD |
| Class A |  | 0.5 | 0.4 |  |  | 0.5 | 0.4 |  |
| Class B |  | 0.2 | 0.3 | 0.5 |  | 0.2 | 0.3 | 0.5 |
| Class C |  | 0.1 | 0.0 | 0.0 |  | 0.1 | 0.0 | 0.0 |
| Class D |  | 0.1 | 0.1 | 0.0 |  | 0.1 | 0.1 | 0.0 |
| Class E |  | 0.5 |  | 0.8 |  | 0.5 |  | 0.8 |
| BD-rate |  | 0.3 | 0.2 | 0.3 |  | 0.3 | 0.2 | 0.3 |
| Enc. Time |  | 99% | 100% | 100% |  | 112% | 101% | 103% |
| Dec. time |  | 88% | 95% | 91% |  | 91% | 88% | 95% |
| Vert. taps |  | 7 |  |  |  | 7 |  |  |
| # of coeff. |  | 12 |  |  |  | 12 |  |  |

# Findings and recommendations

- The decision of which filter shape to use is performed first, followed by CU-based on/off decision. The computation of the initial filter coefficients can be performed in parallel.

# References

[1]	“CE8 Subtest 5: Results on a Combined Adaptive Loop Filtering Algorithm Using Two Filter Shapes,” JCTVC-E492, Geneva Switzerland, March 2011.

[2]	“CE8 Subtest 5: Luma ALF with reduced vertical filter size,” JCTVC-E060, Geneva Switzerland, March 2011.

[3]	“Adaptive Loop Filtering Using Multiple Filter,” JCTVC-D385, Deagu Korea, March 2011.

[4]	“CE8 Subtest 5: Adaptive Loop Filtering Using Two Filter Shapes,” JCTVC-E342, Geneva Switzerland, March 2011.

# Patent rights declaration(s)

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).