| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E488 |
| --- | --- |

| Title: | BoG report on padding of unavailable reference samples for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg (Ericsson) Changcai Lai (HiSilicon) Keiichi Chono (NEC) Viktor Wahadaniah (Panasonic) | Email: | rickard.sjoberg@ericsson com laichangcai@huawei.com chono@ct.jp.nec.com viktor.wahadaniah@sg.panasonic.com |
| Source: | Ericsson, HiSilicon, NEC Corporation, Panasonic Corporation |  |  |

_____________________________

# Introduction

Proposals JCTVC-E203 (Panasonic), JCTVC-E283 (HiSilicon), JCTVC-D386 (Ericsson) and JCTVC-D086 (NEC, Panasonic) are all related to the handling of unavailable reference samples for intra prediction in HEVC. The JCT-VC group requested a proponent's break-out group to identify a single unified proposal from these four input documents.

Two BoG meetings were held and attended by proponents of the four related input documents. A joint proposal was agreed upon.

# Joint proposal

The joint proposal on the handling of unavailable reference samples for intra prediction is conceptually very simple.

Firstly, intra prediction reference samples are marked as available or unavailable for prediction. Reasons for a sample to be marked as unavailable for intra prediction include:

- the sample is outside the picture;

- the sample is outside the slice;

- the sample is not intra coded and constrained intra prediction is enabled.

If all reference samples are unavailable, we set all prediction pixels to 128. Otherwise, we use either sample repetition or lightweight sample interpolation ((q + r + 1) >> 1) to make all reference samples available for prediction. That way, the directional intra prediction modes operate in the same way regardless of picture boundaries, slice borders and constrained intra conditions.

Proposed working draft text is included below using the following text conventions:

- Removed text from WD2 (JCTVC-D503_r1) is shown with strikethrough.

- Newly added text is shown in red.

- Editorial notes or suggestions to WD editors for interoperability are shown in red italics with yellow highlight.

6.4.x	Derivation process for the availability of treeblock addresses

[Ed.: It is expected that several subclauses would be appropriate to be placed between 6.4.1 and 6.4.x.]

[Ed.: The term “treeblock” and related/derived terms are temporary and shall be replaced.]

[Ed.: This subclause only lists the conditions for marking an LCU/treeblock as “not available”. Other parts of clause 6 related to neighboring relationships are assumed to be similar to AVC.]

Input to this process is a treeblock address tbAddr.

Output of this process is the availability of the treeblock tbAddr.

NOTE – The meaning of availability is determined when this process is invoked.

The treeblock is marked as available, unless one of the following conditions is true in which case the treeblock shall be marked as not available:

- tbAddr < 0

- tbAddr > CurrTbAddr

- the treeblock with address tbAddr belongs to a different slice than the treeblock with address CurrTbAddr

…

8.3.3.1	Intra sample prediction

…

The nS*4+1 neighbouring samples p[ x, y ] that are constructed luma samples prior to the deblocking filter process, with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y=-1, are derived as follows.

- The luma location ( xBN, yBN ) is specified by

xBN = xB + x 	(8-1)

yBN = yB + y 	(8-2)

- The derivation process for neighbouring locations in subclause XXX is invoked for sample location with ( xBN, yBN ) as input and the treeblock address tbAddrN as output.

- Each sample p[ x, y ] with x = -1, y= -1..nS*2-1  and x = 0..nS*2-1, y = -1 is derived as follows

  - If any of the following condition is true, the sample p[ x, y ] is marked as “not available for intra prediction”

- tbAddrN is not available

- the coding unit covering ( xBN, yBN ) is not coded as intra mode and constrained_intra_pred_flag is equal to 1

  - Otherwise, the sample p[ x, y ] is marked as “available for intra prediction” and the sample at the location ( xBN, yBN ) inside the treeblock tbAddrN is assigned to p[ x, y ]

When any samples p[ x, -1 ], with x = 0..nS-1 are marked as “not available for intra prediction,” samples p[ x, -1 ], with x = 0..nS-1 are marked as “not available for intra prediction.”

When any samples p[ -1, y ], with y = 0..nS-1 are marked as “not available for intra prediction,” samples p[ -1, y ], with y = 0.. nS-1 are marked as “not available for intra prediction.”

When any samples p[ x, -1 ], with x = nS..nS*2-1 are marked as “not available for intra prediction” and the sample p[ nS-1, -1 ] is marked as “available for intra prediction” the sample value of p[ nS-1, -1 ] is substituted for samples p[ x, -1 ], with x = nS..nS*2-1 and samples p[ x, -1 ], with x = nS..nS*2-1 are marked as “available for intra prediction”.

When any samples p[ -1, y ], with y = nS..nS*2-1 are marked as “not available for intra prediction” and the sample p[ -1, nS-1 ] is marked as “available for intra prediction” the sample value of p[ -1, nS-1 ] is substituted for samples p[ -1, y ], with y = nS..nS*2-1 and samples p[ -1, y ], with y = nS..nS*2-1 are marked as “available for intra prediction”.

Otherwise, if all samples p[ x, y ] are marked as “not available for intra prediction,” the following applies.

p[ x, y ] = 1 << ( ( BitDepthY + increased_bit_depth_luma ) – 1 )	(8-16)

When at least one sample p[ x, y ] with x = -1, y= -1..nS*2-1 and x = 0..nS*2-1, y = -1 is marked as “not available for intra prediction,” the reference sample substitution process for intra sample prediction in subclause 8.3.3.1.1 is invoked with the samples p[ x, y ] with x = -1, y= -1..nS*2-1 and x = 0..nS*2-1, y = -1 as input and the modified samples p[ x, y ] with x = -1, y= -1..nS*2-1 and x = 0..nS*2-1, y = -1 as output.

[Ed.: If necessary, specifying p[ x, y ] as both input and output can be changed to specifying p[ x, y ] as input and p'[ x, y ] as output (subsequent subclauses needs to be changed accordingly).]

[Ed.: After invoking subclause 8.3.3.1.1, all reference samples p[ x, y ] are marked as “available for intra prediction”.]

Table 8-1 specifies the value for intraPredMode and the associated names.

- Table 8-1 – Specification of intraPredMode and associated names

| intraPredMode | Name of intraPredMode |
| --- | --- |
| 0 | Intra_Vertical |
| 1 | Intra_Horizontal |
| 2 | Intra_DC |
| Otherwise (3..33) | Intra_Angular |

Depending on intraPredMode, the following ordered steps apply:

- IfWhen cIdx is equal to 0, filtering process of neighbouring samples specified in Error: Reference source not found8.3.3.1.2 is invoked with the sample array p and the prediction size nS as the inputs and the output is a (2*nS+1)x(2*nS+1) filtered sample array pF.

- One of the intra prediction modes specified in subclause Error: Reference source not found to Error: Reference source not found8.3.3.1.3 to 8.3.3.1.6 is invoked with the sample array pF and the prediction size nS as the inputs and the output is the predicted sample array predSamples.

[Ed.: The above steps are phrased differently from AVC and may need some improvement.]

8.3.3.1.1		Reference sample substitution process for intra sample prediction

Inputs to this process are the reference samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 for intra sample prediction.

Outputs of this process are the modified reference samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 for intra sample prediction.

The values of the samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are modified as follows:

- If all samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are marked as “not available for intra prediction,” the value ( 1 << ( ( BitDepthY + increased_bit_depth_luma ) -1 ) is substituted for the values of all samples p[ x, y ].

- Otherwise (at least one but not all samples p[ x, y ] are marked as “not available for intra prediction”), the following steps are performed for each sample p[ x0, y0 ] with x0 = -1, y0 = -1..nS*2-1 and x0 = 0..nS*2-1, y0 = -1 marked as "not available for intra prediction":

- Let the variables q and r be initially set to -1.

- The variable q is modified as follows:

  - If x0 is equal to -1, a reference sample p[ x, y ] marked as “available for intra prediction” is identified by searching sequentially among p[ x, y ] starting from x = -1, y = y0+1 to x = -1, y = nS*2-1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to q.

  - Otherwise (x0 is not equal to -1), a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = x0-1, y = -1 to x = -1, y = -1. When a sample marked as “available for intra prediction” is not found among p[ x, y ] with x = x0-1..-1, y = -1, the search is continued among p[ x, y ] sequentially starting from x = -1, y = 0 to x = -1, y = nS*2-1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to q.

- The variable r is modified as follows:

  - If x0 is equal to -1, a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = -1, y = y0-1 to x = -1, y = -1. When a sample marked as “available for intra prediction” is not found among p[ x, y ] with x = -1, y = y0-1..-1, the search is continued among p[ x, y ] sequentially starting from x = 0, y = -1 to x = nS*2-1, y = -1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to r.

  - Otherwise (x0 is not equal to -1), a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = x0+1, y = -1 to x = nS*2-1, y = -1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to r.

- The value of the sample p[ x0, y0 ] is modified as follows:

  - If q is not equal to -1 and r is not equal to -1, the value ( ( q + r + 1 ) >> 1 ) is substituted for the value of p[ x0, y0 ].

  - Otherwise, if q is not equal to -1, the value of q is substituted for the value of p[ x0, y0 ].

  - Otherwise (q is equal to -1 and r is not equal to -1), the value of r is substituted for the value of p[ x0, y0 ].

All samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are marked as “available for intra prediction.”

8.3.3.1.18.3.3.1.2		Filtering process of neighbouring samples

…

8.3.3.1.28.3.3.1.3		Specification of Intra_Vertical prediction mode

…

This mode shall be used only when the samples p[ x, -1 ] with x = 0..nS-1 are marked as “available for intra prediction”.

…

8.3.3.1.38.3.3.1.4		Specification of Intra_Horizontal prediction mode

…

This mode shall be used only when the samples p[ -1, y ] with y = 0..nS-1 are marked as “available for intra prediction”.

…

8.3.3.1.48.3.3.1.5		Specification of Intra_DC prediction mode

…

8.3.3.1.58.3.3.1.6		Specification of Intra_Angular prediction mode

…

# Conclusion

The BoG participants propose to adopt the unified solution into HM 3.0.

# Related Contributions

- H. Aoki, K. Chono, X. Jing, V. Wahadaniah, C.S. Lim, S.M.T. Naing, “Constrained intra prediction for reducing visual artifacts caused by lossy decoder-side memory compression,” JCT-VC document JCTVC-D086, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Daegu, KR, January 2011.

- R. Sjöberg, J. Samuelsson, “Constrained Intra source code implementation,” JCT-VC document JCTVC-D386, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Daegu, KR, January 2011.

- V. Wahadaniah, C.S. Lim, S.M.T. Naing, “Constrained intra prediction scheme for flexible-sized prediction units in HEVC,” JCT-VC document JCTVC-E203, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, CH, March 2011.

- Y. Lin, C. Lai, J. Zheng, L. Liu, “Constrained CE4 Subset2: Report of Intra Coding Improvements for Slice Boundary Blocks,” JCT-VC document JCTVC-E203, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, CH, March 2011.

# Patent rights declaration(s)

Ericsson, HiSilicon, NEC Corporation and Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).