| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E494 |
| --- | --- |

| Title: | BoG report on the draft text for the combination of proposals JCTVC-E227, JCTVC-E338 and JCTVC-E344 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Break out Group report |  |  |
| Author(s) or Contact(s) | J. Sole, C. Auyeung, H. Sasai | Email: | joels@qualcomm.com Cheung.Auyeung@am.sony.com sasai.hisao@jp.panasonic.com |
| Source: | Panasonic, Sony, Qualcomm |  |  |

_____________________________

# Abstract

This document describes the combination of JCTVC-E227, JCTVC-E338, and JCTVC-E344 proposals. The combination achieves separation of last flag coding and significance flag coding while reducing the number of contexts in CABAC. The contexts for significance map are reduced by 10% and the contexts for the last significant coefficient by 21% compared to HM2.0.

- Introduction

This document describes the combination of JCTVC-E227 [1], JCTVC-E338 [2], and JCTVC-E344 [3]. The combination achieves separation of last flag coding and significance flag coding while reducing the number of contexts in CABAC. The elements taken from each proposal are the following:

- JCTVC-E227: Significance flag contexts of n1 case

  - Context extension for 8x8 not included

- JCTVC-E338: Parallelization of the significance map coding by explicit signaling of the last significant coefficient

  - Additional contexts for significance map for low frequency positions not included

- JCTVC-E344: Contexts for the last significant coefficient signaling

- Draft Text

The location of the last nonzero coefficient is read as described below. The contexts for the last position depend on the row/column number. The context is derived using the following table:

g_uiCtxXY[ 31 ] =

{0, 1, 2, 3, 3, 4, 4, 5, 5, 5, 5, 6, 6, 6, 6, 7, 7, 7, 7, 8, 8, 8, 8, 9, 9, 9, 9, 10, 10, 10, 10};

Each block size has its own set of contexts. X and Y do not share any context. Furthermore, luma and chroma do not share any context.

1. Parse location x (truncated unary code)

1.1 Set x=0

1.2 For x=0, 1, ..., BlockWidth-2

1.2.1 Decode LAST using context number OffsetBlockSize + g_uiCtxXY[x]

1.2.2 If ( LAST==0 )  x++

1.2.3 Otherwise, go to 1.3

1.3 If ( LAST==0 ) x++

2. Parse location y (truncated unary code)

2.1 Set y=0

2.2 For y=0, 1,..., BlockWidth-2

2.2.1 Decode LAST using context number OffsetBlockSize + g_uiCtxXY[y]

2.2.2 If ( LAST=0 )  y++

2.2.3 Otherwise, go to 2.3

2.3 If ( LAST=0 )  y++

3. If ( ScanType = Vertical ) swap(x,y)

4. Set (x,y) coefficient as significant

5. For coefficient (0,0) up to (but not including) (x,y) in scan order, decode significance of coefficient (as described below)

Description of the significance context generation as presented in the JCTVC-E227 case n1, but without the 8x8 extension:

5. Decode significance of coefficient

5.1 If block size is less than 16x16 for luma, or chroma

5.1.1 Use context depending on coefficient position which is same as current HM2.0 (but missing in WD)

5.2 Otherwise

5.2.1 If coefficient_position = (0,0), (0,1), (1,0), (1,1), use context depending on the position and block size

5.2.2 Otherwise, use context which is derived by current HM method (but missing in WD), not depending on block size

- Number of contexts

The number of used contexts and the percentage of used contexts with respect to the anchor are shown in table 1 and table 2, respectively.

Table 1: Number of used contexts

|  | Proposal |  |  |  | ANCHOR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | sig | last | coeff | total | sig | last | coeff | total |
| Intra only | 96 | 82 | 120 | 333 | 107 | 104 | 120 | 366 |
| Random access | 96 | 82 | 120 | 376 | 107 | 104 | 120 | 409 |
| Low Delay | 96 | 82 | 120 | 376 | 107 | 104 | 120 | 409 |

Table 2: Percentage of used contexts with respect to HM2.0

| increase number of context | increase sig % | increase last % | increase coeff % | increase total % |
| --- | --- | --- | --- | --- |
| Intra only | -10.3% | -21.2% | 0.0% | -9.0% |
| Random access | -10.3% | -21.2% | 0.0% | -8.1% |
| Low Delay | -10.3% | -21.2% | 0.0% | -8.1% |

- References

- H. Sasai, T. Nishi, "CE11: Context size reduction for the significance map", JCTVC-E227, 5th Meeting, Geneva, CH, March 2011.

- J. Sole, R. Joshi, M. Karczewicz, “CE11: Parallel Context Processing for the significance map in high coding efficiency”, JCTVC-E338, 5th Meeting, Geneva, CH, March 2011.

- C. Auyeung, W. Liu, “Context reduction of the last transform position in JCTVC-D262 for CE11.1”, JCTVC-E344, 5th Meeting, Geneva, CH, March 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).