| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E497 |
| --- | --- |

| Title: | BOG report on complexity assessment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | BOG report |  |  |
| Author(s) or Contact(s): | Daniele Alfonso STMicroelectronics Via Tolomeo 1 Cornaredo, 20010 MI, Italy | Tel: +39.02.93519755 Email: daniele.alfonso@st.com |  |
|  | Xing Wen Hong Kong University of Science & Technology, Clear Water Bay, Kowloon, Hong Kong | Tel: 86-13709280020 Email: wxxab@ust.hk |  |
| Source: | Complexity Assessment AHG |  |  |

_____________________________

# Abstract

This contribution summarizes the results of Break-Out Group discussion on Complexity Assessment at the 5th JCT-VC meeting.

# Introduction

The BOG on Complexity Assessment met on Monday March 21st between 11 AM and 1 PM, with the following goals:

- Discuss how to harmonize the different complexity assessment methodologies in a common method shared among the various groups.

- Identify criteria to determine the parallelization and hardware implementability of the key hardware modules.

According to an attendance sheet, the following people participated to the discussion:

- Chakib Nouira, Telnet

- Daniele Alfonso, STMicroelectronics (Complexity Assessment AHG chair)

- David Flynn, BBC R&D

- Dzung Hoang, Zenverge

- Elena Ashina, Samsung

- Jianle Chen, Samsung

- Kenji Kondo, Sony

- Xing Wen, HKUST (Complexity Assessment AHG vice-chair)

Starting from contribution JCTVC-E086 [1], a revised version of the Three-Level Complexity Assessment (TLCA) has been drafted, as reported in the next section of the document.

It has been remarked that defining a complexity assessment method that is universally valid for all the possible kinds of contributions is extremely difficult and perhaps not even needed, therefore rather than a set of fixed rules that everybody shall follow, the proposed TLCA is intended to be a set of suggested rules that AHG chairs and CE coordinators should consider, picking the suggestions that are more appropriate for the proposals under consideration.

# Three Level Complexity Assessment

The new proposed TLCA has the following levels:

- Level 1: analytical description

- Level 2: software and algorithmic measures

- Level 3: hardware measures

## Level 1: analytical description

As a first level, it is recommended that proponents should provide the following information when presenting a new technical contribution to JCT-VC:

- Exhaustive textual description of proposed tool(s), possibly including flowcharts and system block schemes to allow better understanding.

- Basic complexity assessment, to be analytically computed and reported e.g. in an Excel sheet, including the following information:

  - Elementary operations count analysis

  - Storage requirements analysis

  - Bandwidth requirement analysis

- Number of processing stages, to understand pipelining possibilities. In particular, the critical path should be considered, i.e. the maximum number of consecutive processing stages that are needed to produce an output.

- Data dependency analysis, to understand parallelization possibility. It has been remarked that data dependency should be examined within the proposed tool itself and also between the proposed tool and other existing tools in HEVC.

- It has been noted that current CE 7 (core experiment on alternative transforms) and CE 10 (core experiment on spatial transforms) made some remarkable complexity analysis for this meeting, therefore the reader is addressed to the respective CE reports to find exemplary complexity assessment reports [2][3].

## Level 2: software and algorithmic measures

As a second level, it is recommended that proponents should provide the following information when presenting a new technical contribution to JCT-VC:

- HM runtime

  - Values printed by the HM software shall be considered and reported as per JCT-VC common practices.

  - It has been remarked that instead of reporting average numbers only, the proponents should consider, highlight and explain worst case results.

  - For the sake of consistency of results, the simulations shall be executed on the same machine. This applies at least for the decoder, because the current HM encoder reportedly required about 48 CPU days to produce all the anchors and that could likely be an excessive time if the simulations are run sequentially.

  - The AHG can offer a software to benchmark variance of results within clusters of machines, in order to assess the precision of the measures.

  - Report the environment, such as Operating System, 32 or 64 bits executables used, CPU model, and especially the compiler used and its version.

  - It has been remarked that HM runtime measures are not sufficient for a reliable assessment of HEVC complexity, therefore the runtime results must be taken “with a grain of salt”.

- Profiling

  - The BOG agreed that some internal understanding of HM complexity is mostly relevant, although no consensus has been reached on how to perform it.

  - The AHG on Complexity Assessment will conduct further investigation on this topic and will report the results at next meetings.

- Memory

  - Report and explain any significant memory usage difference with respect to reference HM version.

  - The memory bandwidth can be measured with software tool developed by NEC for the HM. It is recommended to integrate NEC’s tool in HM main branch (disabled by default) so that any proponent can use it.

  - Average, best and worst cases should be considered.

  - Peak memory bandwidth can be reported at various coding levels, from sequence level to coding unit level.

  - Slice types should be considered, as I, P and B slices can have much different bandwidth.

## Level 3: hardware measures

Hardware design results are the third and last level of complexity assessment.

On this point the BOG did not reach consensus about what hardware measures should be reported. It has been agreed that throughput and latency are a first priority. According to some BOG participants, other useful numbers are clock frequency, technology, silicon area and power consumption.

# Conclusions

The Complexity Assessment AHG would like to continue refining the proposed three-level complexity assessment, coordinating with other AHGs and CEs.

# References

- X. Wen, O. C. Au, M. Mathur, “Summary of Complexity Assessment for CEs and Three-Level Assessment method”, JCTVC-E086.

- R. Cohen, C. Yeo, R. Joshi, F. Fernandes, “CE7: Summary report of core experiment on alternative transforms”, JCTVC-E027.

- P. Topiwala, M. Budagavi, R. Joshi, A. Fuldseth, I. Kim, “CE10: Summary report for CE on core transform design”, JCTVC-E030.

- D. Alfonso, “Proposals for video coding complexity assessment”, JCTVC-B020, 2nd JCT-VC meeting, July 2010, Geneva, CH.