| 7 Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E501_r1 WG11 Number: m20233 |
| --- | --- |

| Title: | BoG report on subjective viewing test for deblocking filter proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin1 Kenneth Andersson1 Keiichi Chono2 In Suk Chong3 Matthias Narroschke4 Byengwoo Jeon5 David Flynn6 Teruhiko Suzuki7 | Email: | andrey.norkin@ericsson.com |
| Source: | Ericsson1, NEC2, Qualcomm3, Panasonic4, SKKU5, BBC6, Sony7 |  |  |

_____________________________

# Introduction

This contribution is a report of the break-out group on informal subjective viewing for the deblocking filtering that was held during the Geneva meeting on March 19, 2011. The subjective viewing was conducted according to the mandate of core experiment 12 on deblocking filtering D612 [1]. The goal of informal subjective viewing test was to determine how the deblocking filter proposals affect the subjective quality.

# Test procedure

There were twelve proposals tested (including combinations of proposals). The proposals were split into two groups of six proposals in each group. Each group of proposals was evaluated by a group of three-four experts at one viewing test. The identities of the proposals and the anchor were hidden (the test subjects saw A and B instead).

The following six sequences have been used in the test.

Sequence 1: BQMall, low-delay low-complexity, QP 32
Sequence 2: BQMall, random-access high-efficiency, QP 37 
Sequence 3: Kimono, low-delay low-complexity, QP 32
Sequence 4: Kimono, random-access high-efficiency, QP 37 
Sequence 5: Vidyo3, low-delay low-complexity, QP 32
Sequence 6: Vidyo3, low-delay high-efficiency, QP 37

The sequences were encoded according to common test conditions D600 [2].

The test subjects were given score sheets with 3 grades: “A is better”, “Similar”, “B is better”.

The test procedure was as follows. The proposals were compared against the anchor. The order of the anchor and the proponents was randomized for each pair of sequences. The pair of the proponent and anchor was shown twice (ABAB). After watching a pair of anchor and proposal twice, test subjects were given 6 second to score the results. The sequences were shown in the following order:

Sequence 1 was shown for proposal C1.

Sequence 1 was shown for proposal C2.

…

Sequence 1 was shown for proposal C6.

Sequence 2 was shown for proposal C1.

Sequence 2 was shown for proposal C2.

…

Sequence 2 was shown for proposal C6

Sequence 3 was shown for proposal C1.

etc.

The length of one test was 36 minutes. The total number of test subjects was ten in test 1 and ten in test 2. The first several test sessions were conducted by Keiichi Chono (NEC) who also checked that the randomization scripts worked properly. The last session was conducted by a group of proponents.

After conducting the test, the results have been counted by proponents and an independent cross-verifier. Then the identities were revealed and linked to the results.

# Test software

The test was conducted with the test program Qualiview provided by the MPEG Test coordinator. The test was performed according to DSCQS configuration. A script for automatic generation of the test (randomization of proposals) based on DSCQS configuration was written by Kenneth Andersson (Ericsson).

# Tested proposals

The following proposals and combinations of proposals have been tested in the subjective test.

C1 E181 Sony Item3 (line-wise decisions + parallelization of filtering).

C2 E224 Panasonic (parallel decisions).

C3 E079 Combination of Ericsson + MediaTek Chroma

C4 E251 Panasonic

C5 E079 MediaTek Chroma only

C6 E144 Microsoft

C7 E417_v1 Combined 2

C8 E417_v2 Combined 3

C9 E417_v2 SKKU

C10 E276 Ericsson

C11 E079 MediaTek Luma_and_Chroma

C12 E181 Sony Item 2	(parallelization of filtering)

# Problems during subjective test

During the test the following problems were noted:

- The test software crashed occasionally.

- There were occasional flashes sometimes between clips.

- One viewer (David Flynn) reported that:

- The test system randomly re-scaled video clips causing video artifacts in the form of removed columns/rows of pixels. The viewer recognized this as a well-known computer graphics artifact. This was verified after the test was already done.

- The test system randomly dropped frames, causing motion artifacts.

- I.e., In an A-B-A-B test, sometimes A1 was in error and A2 not.

- In order to conduct the test in a timely manner, the random order of clips were kept between tests such that e.g. the first proponent in the test was always shown first in all tests. Ideally, the order should be random between the tests.

# Test results

The number of votes “Better than Anchor”, “Similar”, “Worse than the anchor” have been counted and the excel sheet with the results is attached to the file. The mean values and confidence intervals have been calculated and are also included into the Excel sheet. For most of the proposals confidence intervals overlap with the anchor and each other.

In order to process the results, votes for the “proposal better than the anchor” were counted as 1, similar to the anchor was assigned value 0 and worse than the anchor was assigned value -1. Then, an average of the vote results for each proposal and a confidence interval of 95% have been calculated. The mean values with confidence intervals are presented graphically in Figure 1.

Figure 1. Mean values obtained in the subjective viewing with confidence intervals.

# Conclusions

The reported test software problems can affect the outcome of the test. We should aim at correcting these problems for the following meetings if possible.

The scores did not reveal any subjective problems for the deblocking schemes E181 Sony Item 2 and E224 Panasonic of Subset 2 (that have been accepted during this meeting). Confidence intervals of most of the proposals and combinations of the proposals in Subset 1 overlap with the anchor and each other. Proposal E144 has higher scores with a non-overlapped confidence interval with the anchor.

# References

[1]	A. Norkin, B. Jeon, M. Narroschke et al., “Description of CE12: deblocking filtering,” JCTVC-D612, Daegu, January 2011.

[2]	F. Bossen, “Common test conditions and software reference configurations,” JCTVC-D600, Daegu, January 2011.