| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E502 |
| --- | --- |

| Title: | Scalable enhancement requirements for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-3470 aluthra@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11

MPEG2011/ m19968

March 2011, Geneva, Switzerland

| Source | Motorola Mobility |
| --- | --- |
| Purpose | Information and Discussion |
| Title | Scalable enhancement requirements for HEVC and AVC |
| Author | Ajay Luthra |

Abstract

A brief overview of various scalability features that have been standardized in the past as well as being currently researched is presented. Looking forward, the environments of digital video distribution will change exponentially fast. The so called “convergence” of computer networks and platforms, digital TV networks and platforms, and telephone networks and platforms, which has been so widely discussed but had remained illusive for more than 15 years, is now (finally) beginning at large commercial deployment scales. Therefore, instead of looking only in the rear-view mirror and having a tendency to insert scalability tools in HEVC as a band-aid or patch work , it will be very valuable to also look at new digital video distribution environments, understand and develop clear requirements associated with them and then provide appropriate solutions. The main purpose of this document is to help in initiating a more comprehensive discussion with the expectation that a clearer and more comprehensive understanding of the needs for the scalability features will emerge. It is expected that this will help significantly in providing the most appropriate solutions. It will also help in understanding if new and different scalability tools need to be introduced in AVC as well.

- Background

Scalable video coding approaches have been investigated for more than 20 years to satisfy various needs associated with the distribution of video in heterogeneous environment where either the network does not have fixed quality of service (QoS) or the clients/receivers have widely varying processing power and display capabilities or both. The commercial use of these methods has not been significant in comparison to single-layer compression techniques. This has primarily been because the environments for the video distribution for commercial application have so far not been very heterogeneous.  Cable, Satellite and xDSL network have largely operated under fixed QoS settings. The distribution of video over the internet with no guarantee of service has been mainly a hobby, until recently. The so called “convergence” of computing networks and platforms with digital TV networks and platforms, so widely discussed for more than 15 years, did not happen. The use of scalability has been limited to narrow areas of video conferencing or university research. However, all the signs indicate that this is going to change in a big way. Exponentially increasing numbers of smart phones (Droid, Atrix, iPhone etc) and tablets (Xoom, iPad etc.) have populated the marketplace with the devices that have wide range of video decoding and display capabilities. They now share the digital video space with Set Top Boxes and Televisions. On the networking side, streaming video is finally coming out of the hobby domain to the commercial deployment domain. Although, the author is not aware of a scientifically conclusive study, it is largely believed that around 20% of internet capacity today in many countries is taken by streaming and/or downloading video. It is also believed that this is going to further increase significantly to as high as 80+%. Furthermore, the commercial deployment of digital video services is not expected to be constrained to wired networks. Increasingly, wireless networks (Cellular, WiFi etc.) are getting used for the distribution of the digital video. Thus, going forward, the network side of the video distribution is also expected to become significantly more heterogeneous and the environments of digital video distribution will change exponentially fast.

Therefore, instead of looking only in the rear-view mirror and having a tendency to insert scalability tools in HEVC as a band aid or patch work , it will be very valuable to also look at new digital video distribution environments, understand and develop clear requirements associated with them, and then provide appropriate solutions. Even though some scalability tools were not commercially successful in the past they may be required looking forward. In that case we should identify those and see what further improvements can be made. Furthermore, there will indeed be requirements for new tools. We should identify them as well.

In this document we discuss various scalability needs that have been identified in the past as well as what may be needed going forward. The main purpose of this document is to help in initiating a more comprehensive discussion with expectations that collection of converged opinions may point to facts (although this is not always true, it is perhaps the best that can be expected at this stage). It is expected that as a clearer understanding of requirements and priorities are developed, good solutions will emerge. It is also expected that this discussion will not only help in providing good solutions in HEVC but also in AVC as well. Deeper discussions on what solutions to provide, which scalability is better etc are not included here on purpose.

- Scalability, Why and What

  - Scalability vs. Transcoding

There have been two main schools of thoughts with regard to what is a better technical approach to satisfy the needs of various heterogeneous environments for digital video distribution. One school of thought pushed for transcoding-based solutions and another pushed for scalability based solutions. Today, many of the solutions (as an example, the interested reader may want to do internet search on Motorola Mover, MCUBE and/or MCU3E) being deployed favor transcoding because of ease of use, availability of hardware solutions and economics of those solutions. For example, video is transcoded from 720p60 resolution coded (using AVC or MPEG-2) at 5+Mbps to HVGA (480x320)p30 at around 1 Mbps to send to a smart phone, etc. However, going forward, if good scalable solutions are provided, they are expected to provide simpler and more economical solutions in many circumstances.

To start the discussion, let us take a look at various scalability solutions that may be needed. The following is a list (perhaps not a complete list) of the various scalability tools:

- Temporal Scalability

- Spatial Resolution Scalability

- Quantization Noise (sometimes also called SNR) Scalability

- Sharpness/Frequency Scalability

- Robustness Scalability

- Fine Granular Scalability

- View Scalability

- Frame Compatible Scalability for 3D TV

- Bit Depth Scalability

- Chroma Scalability

- Picture Aspect Ratio Scalability

- Interlace-to-Progressive Scalability

- Others?

  - Usage and Needs

    - Temporal Scalability

In a way, temporal scalability is one of the oldest kinds of scalability with roots in the analog world where the receivers were told to increase the frame rate from 24 fps to 30 fps via 3:2 pull down hints. Instead of sending an extra bit stream, enhancement only included flags embedded in the bit stream to signal which fields to repeat in going from 24 fps to 30 fps. It is a degenerate case of temporally scalability and may not be accepted universally as temporal scalability, but it was a clever and simple way to interface the world of 24 fps with the world of 30 fps. Another type of temporal scalability included the methods of splitting the bit stream such certain frames can be dropped or included to adjust the frame rate. This is a simple and effective approach to achieve a varying frame rate, but it limits how well the frame rate can be controlled and many times produces video that is not smooth temporally.

What is needed: The new receivers used in digital video distribution sustain varied frame rates: around 20 fps, 24 fps, 30 (25) fps, 60 (50) fps, 120 (100) fps, and so on. There is a clear need to support rates ranging from 20 to 60 fps. For higher frame rates (e.g. 120 fps), it is not expected that content will be sent at these rates in most of the networks commercially deployed today or to be deployed in the near future. In this case, the conversion will happen at the receiving end. The question arises as to what hints and supports can be provided in the bit stream (or in the enhancement layer) to help a receiver do a good job of conversion. There is an overlap of some of this with the Interlace-to-Progressive Scalability discussed below in 2.2.12.

    - Spatial Scalability

One of the early attempts made to use spatial scalability commercially was during initial HDTV development phase where the HDTV resolution was built as a spatial enhancement of SDTV resolution. Due to its poor coding efficiency and high complexity at that time, this approach lost to the simpler and more efficient single layer HDTV schemes. However, this prevented easy migration from SDTV to HDTV distribution and it perhaps caused a delay in the deployment of HDTV. Looking forward, one of the goals of HEVC is to address the needs of 1080p60 and higher resolutions. To avoid a delay in deployment of the distribution of video with higher resolutions, it will be advantageous to provide an easier migration path. In addition, in a mobile environment the picture sizes that are becoming popular are HVGA (480x320), VGA (640x480) and HD (1280x720).

What is needed: In addition to 1:2 resolution scalability (e.g. QVGA to VGA), there are commercial needs where the lower and higher resolutions are not related by factor of 2. For example, a factor of 1:1.5 (2:3) is also very broadly needed - HVGA (480x320) to VGA (640x480), or VGA to 720p , or WVGA to 720p  resolution scalability. In some of these circumstances, the picture aspect ratios at the higher and lower resolutions are not the same, which result in ratios of horizontal and vertical resolutions at the base and enhancement layers that are not the same. This can be addressed either by using black/gray bars or Picture Aspect Ratio Scalability (see 2.2.11)

    - Quantization / SNR Scalability

In this scalability, the scalable layers are created such that by using bits from the enhancement layers the distortions (e.g. blocking noise) due to quantization are reduced as the higher layer(s) is (are) used. Today, with AVC based coding typically HVGA is used with bit rates in the range of about 0.5 to 1.0 Mbps. For VGA, the range is around 1 to 2 Mbps and for 720p30 the range is around 2 to 4 Mbps.

What is needed: It is desirable to be able to scale the bit rates by a factor of 2 (in comparison to the lower end of the range). Generally, a wider range of variation is provided by changing the spatial resolution and/or temporal rate. A comprehensive solution may include a combination of this scalability with Spatial, Temporal and/or Sharpness (see 2.2.4) scalabilities.

    - Sharpness Scalability

In this scalability, the picture resolution is kept the same across the scalable layers but as more bits are added from the enhancement layers, edges in the pictures become sharper. In comparison, Quantization scalability tends to provide pictures with blocking noise at lower rate. It can also be argued that Spatial Scalability provides similar capability where lower resolution pictures can be scaled up to provide larger picture-sizes and the sharpness of those pictures can be improved by using enhancement layers. However, there are distinct possibilities that simpler other solutions than spatial scalability may be developed and it will be good to leave the door open at this stage.

What is needed: The needs are similar to that of Quantization Scalability - to be able to scale the bit rates by a factor of 2 (in comparison to the lower end of the range). Generally, a wider range of variation is provided by changing the spatial resolution and/or temporal rate. A comprehensive solution may include a combination of this scalability with Spatial, Temporal and/or Quantization Scalability.

    - Robustness Scalability

It is becoming increasingly important to satisfy the needs of distributing digital video over networks that do not guarantee QoS, as these are being more widely used. Today, one of the most popular ways is to use TCP-based protocols for live streaming over HTTP. However, this may not be an optimal way of the distributing video over such networks. Another way is to provide redundant information so that a receiver can recover from the errors.

What is needed: It is desirable to explore the design of a joint video and channel layer based schemes where the use of enhancement layers can provide a much simpler and efficient solutions than the ones currently used.

    - Fine Granular Scalability

In this scalability, the encoder has much better control over bit rates used for the distribution of the compressed video. It can adapt the compressed video finely to the time-varying nature of the network. The first formal attempt to standardize Fine Granularity scalability was done in MPEG-4. However, as is the case many times, the first attempt was not very successful from a commercial deployment point of view. Some of the reasons for this were loss of coding efficiency, non-availability of the hardware and the use of networks with fixed bandwidth for commercial distribution of video. It is beginning to change fast. Networks with varying bandwidth/capacity are increasingly used for the distribution of video. The processing power has also increased significantly, so that more complex and efficient algorithms can now be deployed.

What is needed: Looking forward, as more and more digital video distribution is expected to happen over networks that have a wide range of variability in bandwidth, it will very useful to re-examine the needs the possible new and more efficient solutions. The variation in the bandwidth can be a function of time (e.g. less bandwidth in the evening when more users are sharing the same network), network (e.g. wired and wireless network) and location (e.g. distance from the transmitter).

    - View Scalability

View scalability provides the capability of add depth perception to the TV by sending additional views in enhancement layers. The first formal attempt to standardize the view scalability was done in 1994-95 in the MPEG-2 standard where MVP (Multi-View Profile) was developed for this purpose. However, this was not commercially successful primarily because it was too ahead of its time. The Multi-View Coding (MVC) extension of AVC has recently seen much better commercial success with adoption in Blu-Ray. Initial trials are also being done for distribution of MVC-based 3D TV over so called broadcasting networks.

What is needed: At least one more view enhancement layer in HEVC should be added to provide stereoscopic 3DTV. There is a strong desire in the industry to reduce the bandwidth associated with additional view(s). Depth based 3DTV distribution may help in reducing the bandwidth required for sending additional views. Providing the capability to efficiently compress and send depth maps is expected to be helpful in achieving these goals. However, it may also be timely to think about other approaches. For example, artificial 2D to 3D conversion techniques have become significantly efficient in past few years. So, adding the capability in the syntax to provide hints (as opposed to sending the views or depth maps) to a receiver to generate the second view can provide an extremely efficient way to deploy 3DTV.

    - Frame Compatible Stereoscopic View Scalability

One of the drawbacks of MVC has been that it is not compatible with Half Resolution per eye based distribution of stereoscopic 3D TV. MPEG has recently developed requirements in this regard (see N11681).

What is needed: To explore if a more unified solution can be provided to migrate from 2D to 3D TV

    - Bit Depth Scalability

Digital video for consumer applications uses 8 bits per luma and chroma samples. Increasingly, consumer displays are having the capabilities to display higher bit depths (e.g. 10 bits). However, it is not clear if the human visual system can see the difference in 8 or 10 bits video (except for very small range of specially designed pictures with slowly changing luma) after it has gone through high compression and/or due to wide range of Gamma factors. The first formal attempt to standardize bit depth scalability was done in mid 90s when MPEG-2 part 8 (ISO/IEC 13818-8) focused on high fidelity (4:2:0 10-bits) video. The main focus was more on contribution quality video for studio (production/post-production) applications. The need to have 4:2:2 chroma resolution became more dominating requirement and focus was moved from 10-bit depth to 4:2:2 8-bits video coding. More recently, requirements of bit depth scalability have also been discussed in AVC.

What is needed: It needs to be established if Bit Depth Scalability provides some benefits to certain applications and, if yes, then develop specific requirements.

    - Chroma Scalability

Digital video for consumer applications uses 4:2:0 chroma sub-sampling. Increasingly, consumer displays are having the capabilities that can display higher chroma resolution (e.g. 4:4:4). Question here is whether it is needed to distribute video with 4:4:4 resolution. On the contribution side, due to multiple generations of encoding and decoding and editing requirements, the need for codec simplicity is much stronger than scalability. The GOP structure used in contribution grade video is quite different than that used for distribution to video. So, chroma scalable version provides no help.

What is needed: It needs to be established if Chroma Scalability provides some benefits to certain applications and, if yes, then develop specific requirements.

    - Picture Aspect Ratio Scalability

This is also one of the oldest kinds of scalability with roots in the analog world where 16:9 aspect ratio of HDTV was obtained by sending side panels as additional layers on top of the pictures with 4:3 aspect ratios. However, due to the complexity associated with achieving that task via analog means and avoiding the problem of seeing seams at the stitching lines and simplicity of single layer digital HDTV, these approaches were dropped. However, even today a vast majority of digital TV suffer from the problem of not satisfactorily showing pictures with 4:3 aspect ratios on displays with 16:9 aspect ratios or vice-versa (people either look too fat or too skinny or are shown with heads or toes not visible on the screen). Seeing pictures with black/gray bars has also not been enthusiastically accepted by consumer community. With an increase in the diversity of displays (e.g.VGA, WVGA, SD, QHD, HD) in mobile environments, this problem is expected to get worse.

What is needed: See if time has come to revisit the possibility of providing Picture Aspect Ratio Scalability.

    - Interlace-to-Progressive Scalability

This is also one of the oldest kinds of scalability with roots in early advanced digital TV standard development activities. A love and hate relationship with interlaced scanned video has been going on for more than 20 years. However, just because some do not like interlaced scanned video, it has not stopped others from capturing video in this format. Today, a vast amount of video (SD and HD) is still captured in interlaced format. However, the displays are all becoming progressive. Therefore, there has been a need to not only compress interlaced scanned video but also a need to convert it to progressive video for display. There appears to be three ways to do it: (i) convert video from interlaced to progressive at the encoding side, (ii) let the receiver/display do interlace to progressive video conversion by providing it hints (by sending extra side bits in enhancement layers), or (iii) let receiver/display do its own motion estimation and the conversion. Each one has its own pluses and minuses. Converting video from interlaced to progressive at the encoding end (e.g. from 1080i60 fields to 1080p60 frames) may increase the bandwidth requirements significantly. Converting video at the encoding end by lowering the temporal rate (e.g. from 1080i60 fields to 1080p30 frames) may cause unacceptable motion rendering artifacts and may force the receivers to do frame rate up conversion anyway. Letting the receivers do the full conversion puts large burden on them and that may not be feasible in many applications (including mobile video where battery consumption is also an important factor). Providing hints to a receiver on how to convert the video can be a middle ground but it requires additional bits to be sent.

What is needed: An investigation is needed to determine if providing hints to do interlace to progressive conversion provides good middle ground.

    - Others – to be investigated.

- Recommendations

Keeping in mind that digital video distribution environments are morphing at exponentially fast pace, the needs of scalability for future digital video systems should be developed. Instead of looking only in the rear-view mirror and having the tendency to insert scalability tools in HEVC as a patch work, it will be very valuable to also look at the needs of the new environments, understand and develop clear requirements associated with them and then provide cleaner solutions that will satisfy those needs.  This may also help in extending AVC for these new environments.

It will also be important to keep in mind that just because certain scalabilities were not commercially successful in the past, it does not mean they will not be needed in the future. It could be that some of the ideas were ahead of their time and some could not be economically implemented 10-15 years back. The cost and acceptable complexity parameters have changed significantly in last several years.

A better understanding of the requirements of new digital video distribution environments will not only help in providing good solutions in HEVC but also help in possibly extending AVC as well.

- Acknowledgement

Author is thankful to Dr. David Baylon for his very valuable editorial help.

- Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).