| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E505 |
| --- | --- |

| Title: | CE8 Subtest 2: Adaptation between Block-based and Region-based Filter Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | I. S. Chong1 M. Karczewicz1 Ching-Yeh Chen2 Chih-Ming Fu2 Chia-Yang Tsai2 Yu-Wen Huang2 Shawmin Lei2 Tomoo Yamakage3 Takeshi Chujoh3 Takashi Watanabe3 | Emails: | ichong@qualcomm.com martak@qualcomm.com chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com tomoo.yamakage@toshiba.co.jp takeshi.chujoh@toshiba.co.jp takashi39.watanabe@toshiba.co.jp |
| Source: | Qualcomm1 , MediaTek Inc.2, and TOSHIBA Corporation3 |  |  |

_____________________________

# Abstract

This contribution reported the results of merging CE8 Subtest 2: Block based adaptive loop filter (ALF) (JCTVC-E323) and CE8 Subtest 2: adaptation between pixel-based and region-based filter selection (JCTVC-E046). In comparison with the JCTVC-D600 anchor, the proposed method reportedly achieved 0.1%, 0.5%, and 0.8% bit rate reductions for HE-AI, HE-RA, and HE-LD, respectively, with unchanged encoding time and 6-9% decoding time reduction. And this contribution is cross checked by JCTVC-E475.

- Introduction

In CE8 Subset2, two block based ALF were proposed, which are CE8 Subtest 2: Block based adaptive loop filter (ALF) (JCTVC-E323) and CE8 Subtest 2: adaptation between pixel-based and region-based filter selection (JCTVC-E046). In this proposal those two proposals are merged to have a better block based ALF.

The rest of this document is organized as follows. The proposed pixel classification will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Algorithm description

In this proposal, adaptive loop filter is switching pixel classification in picture level among two ways. First it is 4x4 block based metric with activity and direction information (see JCTVC-E323 for more details). For block based ALF, Laplacian based activity metrics of 4x4 blocks are averaged, and each 4x4 block has one Laplacian activity value.

Additionally directional information is computed using 1D lapalcian activity metric. First vertical/horizontal activity for each block is computed as follows.

Vertical activity = sumi,j| (R(i,j)<<1) –R(i-1,j)-R(i+1,j)| with i,j =0…3

Horizontal activity = sumi,j| (R(i,j)<<1) –R(i,j-1)-R(i,j+1)| with i,j=0…3

Then directional information for each block is decided as follows.

If Vertical activity > threshold*Horizontal activity, Direction = 1

Else if Horizontal activity > threshold* Vertical activity, Direction = 2

Else Direction = 0

Note that Vertical/Horizontal activity computation can be done during Laplacian activity computation, because Laplancian activity is the sum of Vertical and Horizontal activity.

With Direction information, new Activity metric for each block is derived as follows.

Activity Metric = 3*Laplacian Activity + Direction

Two groups with successive Activity Metric can be merged. After merging, one filter is applied to one group.

The other classification methods are region-based method as in JCTVC-E046. In the region-based method, a picture is always divided into 16 roughly-equal-size regions where each region is aligned with LCU boundaries, as shown in Figure 1. Each region is assigned with an index, and two regions with successive indices can be merged. After region merging, one filter is applied to one region.

The supported filter sizes of the region/block-based method are the same as the anchor. As for the encoding flow, only one extra encoding pass is added for the region-based ALF before the original multi-pass pixel-based ALF encoding algorithm in the anchor. Please note that the extra encoding pass can be processed in parallel with the original group merging process of the block-based method. After the original group merging process, the current picture can choose to use the block-based ALF or the region-based ALF for the following refinement process including filter redesign with CU on/off control and filter tap decision. Compared to the block-based ALF, the region-based ALF can save computations of block adaptation, so the average decoder complexity can be reduced when the two methods are switched. Moreover, the region-based ALF also significantly reduces the number of filter switches in a picture, which means less switching power is consumed. The proposed syntax is shown in Table 1, and the modified part is highlighted with yellow background. The only syntax change is to add one flag in the slice header is used to select between the block-based ALF and region-based ALF.

Figure 1  16 regions in a 416x240 picture where each square denotes a 64x64 LCU

Table 1  Proposed syntax modification

| alf_param( ) { | C | Descriptor |
| --- | --- | --- |
| alf_flag | 2 | u(1)¦ae(v) |
| if( alf_flag ) { |  |  |
| region_adaptation_flag | 2 | u(1)¦ae(v) |
| … // original ALF parameters for luma |  |  |
| alf_chroma_idc | 2 | u(2)¦ae(v) |
| … // original ALF parameters for chroma and luma CU on/off control |  |  |
| } |  |  |
| } |  |  |

Unlike the FUs proposed in JCTVC-D119 where the FU size can be changed picture by picture, the regions for adapting filters in this proposal are fixed as long as the picture size is determined.

# Simulation results

As requested in JCTVC-D608 , the high efficiency (HE) test configurations in JCTVC-D600  were tested. Experimental results are shown below. The proposed block based ALF with new metric can achieve 0.1%, 0.5%, and 0.9% bit rate reductions for HE-AI, HE-RA, and HE-LD, respectively. The encoding time is almost unchanged. The decoding time is reduced by 5-7%. Generally speaking, the proposed method can achieve better compression performance with less decoding time than the anchor. And this contribution is cross checked by JCTVC-E475.

|  | Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A | -0.1 | 0.0 |  | 0.0 |
| Class B | -0.1 | 0.1 |  | 0.1 |
| Class C | -0.2 | 0.1 |  | 0.1 |
| Class D | 0.0 | 0.1 |  | 0.1 |
| Class E | -0.1 | 0.1 |  | 0.1 |
| All | -0.1 | 0.1 |  | 0.1 |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 94% |  |  |  |
|  |  | Random access |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A | -0.5 | 0.1 |  | 0.0 |
| Class B | -0.7 | 0.1 |  | 0.1 |
| Class C | -0.5 | 0.2 |  | 0.2 |
| Class D | -0.2 | 0.1 |  | 0.1 |
| Class E |  |  |  |  |
| All | -0.5 | 0.1 |  | 0.1 |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 91% |  |  |  |
|  | Low delay |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A |  |  |  |  |
| Class B | -0.9 | -0.1 |  | 0.0 |
| Class C | -0.8 | 0.2 |  | 0.1 |
| Class D | -0.3 | -0.3 |  | -0.1 |
| Class E | -1.3 | 0.1 |  | 0.0 |
| All | -0.8 | 0.0 |  | 0.0 |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 93% |  |  |  |

# Conclusions

In CE8 Subtest 2, Qualcomm tested an switching block/region based adaptive loop filter with a novel activity metric which combines Laplacian metric with directional information. It is reported that bit rate reductions are 0.1%, 0.5%, and 0.8% for HE-AI, HE-RA, and HE-LD, respectively. The encoding time is about the same as the JCTVC-D600 anchor, and the decoding time is reduced by 6-9%. It is proposed to adopt this tool into HM and enable it for HE configurations.

# Patent rights declarations

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I. S. Chong, T. Yamakage, T. Chujoh, and T. Watanabe, “CE8 Subtest2: A Joint Proposal on Improving the Adaptive Loop Filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba,” Document of Joint Collaborative Team on Video Coding, JCTVC-D119, Jan. 2011.

[2] T. Yamakage, Y.-W. Huang, I. S. Chong, T. Ikai, M. Narroschke, W. L. Lai, A. Tabatabai, M. Budagavi, and F. Kossentini, “Description of CE8: Non-deblocking Loop Filtering” Document of Joint Collaborative Team on Video Coding, JCTVC-D608, Jan. 2011.

[3] F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.

[4] T. Ikai and Y. Yasugi, “Region-based adaptive loop filter using two-dimensional feature,” Document of Joint Collaborative Team on Video Coding, JCTVC-D116, Jan. 2011.