# INTERNATIONAL ORGANISATION FOR STANDARDISATION

# ORGANISATION INTERNATIONALE DE NORMALISATION

# ISO/IEC JTC1/SC29/WG11

# CODING OF MOVING PICTURES AND AUDIO

# ISO/IEC JTC1/SC29/WG11

# MPEG2010/M18385

# Oct 2010, Guangzhou, China

| Source | Samsung Electronics |
| --- | --- |
| Status | Proposal |
| Title | Annotation and Attributes for Media Compositions |
| Author | Sungryeul Rhyu, Seoyoung Hwang, Nhut Nguyen |

Abstract

This contribution addresses pending issues of Evaluation Experiment #1, Composition of Media Presentation (CMP), resulted from the DASH ad-hoc meeting at Paris in September 2010 [1].

Specifically this contribution describes proposed extensions to DASH baseline MPD to support compositions of media presentations.

References

[1] ISO/IEC JTC1/SC29/WG11/M18009, Ad-hoc meeting report of Dynamic Adaptive Streaming over HTTP in Paris, France, September 2010.

Media Composition

[Definition] A media composition is a combination (grouping) of media components in a Period for presentation to the user.

A client player can use media composition specified in the MPD or make its own media compositions for presentation. Media compositions in the MPD are indicated by MediaComposition element grouped together by MediaCompositionGroup element. A media composition (MediaComposition) is a combination of Representations to present to the user. A media composition group (MediaCompositionGroup) has multiple media compositions. During presentation time a client player can choose a media composition in a MediaCompositionGroup with user’s interaction or automatically without user interaction. Information in attributes of MediaComposition and MediaCompositionGroup element are provided for media composition selection by a client player.

A Period has zero or one MediaCompositionGroup element. A MediaCompositionGroup has one or more MediaComposition elements. A MediaComposition has an @ID attribute and zero or more RepresentationInfo and GroupInfo elements. A RepresentationInfo refers the @ID attribute of a Representation in the same Period with @refID attribute. A GroupInfo refers one or more Representations having same @group value in the same Period with the @refID attribute.

In the following example, there are 5 Representations. ID #1 is an English audio, ID #2 is a German audio and ID #3 to #5 are video Representations for various bitrates. By referencing ID #1 of Representation in RepresentationInfo, the media composition can have English audio Representation as its subset. By referencing group value #10 in GroupInfo, the MediaComposition can have video Representations group 10 as its subset.

Example of MediaCompositionGroup:

<MPD> <Period> <MediaCompositionGroup> <MediaComposition ID=”1”> <RepresentationInfo refID=”1”/> <GroupInfo refID=”10”/> </MediaComposition> <MediaComposition ID=”2”> <RepresentationInfo refID=”2”/> <GroupInfo refID=”10”/> </MediaComposition> </MediaCompositionGroup> <Representation ID=”1” lang=”en”>…</Representation> <Representation ID=”2” lang=”de”>…</Representation> <Representation ID=”3” Group=”10”>…</Representation> <Representation ID=”4” Group=”10”>…</Representation> <Representation ID=”5” Group=”10”>…</Representation> </Period> </MPD>

Annotation for MediaCompositionGroup

Attributes are provided to annotate characteristics of the MediaCompositionGroup. There are human readable descriptions and machine readable descriptions. Human readable descriptions can be used when client player let user choose a media composition among possible selections. Machine readable attributes can be used when client player automatically chooses a media composition among possible selections for presentation. Criteria for selection can be based on user preferences and device capabilities but it is solely an implementation matter.

Attributes for MediaCompositionGroup

A MediaComposition is consisted of Representations and Groups. Thus a MediaComposition has all attributes derived from referring Representations and Groups. However, it is unclear which attribute is representative for a MediaComposition. For example, suppose a MediaComposition whose value of lang for an audio Representation is English but value of lang for a subtitle Representation is Korean. Client player may need other criteria to decide one of languages as the representative attribute. Attributes for MediaComposition is proposed to provide clear annotation for MediaComposition.

A MediaComposition has @description, @lang, @minBandwidth, @maxBandwidth, @mimeType attributes.

@description has description which is mainly purposed to be read by user.

@lang describes representative language for the MediaComposition.

@minBandwidth and @maxBandwidth are range from minimum to maximum bandwidth this MediaComposition requires to be played. Range has comma separated two strings whereas first parameter is minimum bandwidth and second parameter is maximum bandwidth.

@mimeType is representative mimeType for the MediaComposition.

Example of attributes of MediaComposition:

The attribute @description is human readable and have enough information to distinguish this MediaComposition from others.

The attributes @lang, @bandwidth, and @mimeType are representative for those in each Representation.

<MPD> <Period> <MediaCompositionGroup> <MediaComposition ID=”1” description="Korean subtitle" lang="ko" minBandwidth="128000” maxBandwidth=”512000" mimeType="video/3gpp; codecs=mp4v.20.9, mp4a.E1, mp4t"> <RepresentationInfo refID=”1”/> <GroupInfo refID=”10”/> <RepresentationInfo refID=”8”/> </MediaComposition> </MediaCompositionGroup> … <Representation ID=”1” lang="en" mimeType="video/3gpp; codecs=mp4a.E1">…</Representation> <Representation ID=”2” lang="de" mimeType="video/3gpp; codecs=mp4a.E1">…</Representation> <Representation ID=”3” Group=”10” lang="en" bandwidth="512000" mimeType="video/3gpp; codecs=mp4v.20.9”>…</Representation> <Representation ID=”4” Group=”10” lang="en" bandwidth="256000" mimeType="video/3gpp; codecs=mp4v.20.9”>…</Representation> <Representation ID=”5” Group=”10” lang="en" bandwidth="128000" mimeType="video/3gpp; codecs=mp4v.20.9”>…</Representation> <Representation ID=”6” lang="en" bandwidth="8000" mimeType="subtitle; codecs=mp4t”>…</Representation> <Representation ID=”7” lang="de" bandwidth="8000" mimeType="subtitle; codecs=mp4t”>…</Representation> <Representation ID=”8” lang="ko" bandwidth="8000" mimeType="subtitle; codecs=mp4t”>…</Representation> </Period> </MPD>

Structural approach in consideration of functionality and optimization

Position of MediaCompositionGroup

Multiple media composition can be provided in every Period. If a selected media composition is available in next Period, client player can select the same media composition by its ID.

Hierarchical structure for media compositions

Many media compositions can be very similar. For example, there are multiple media compositions where video and audio are same but the subtitle for each composition is different. To reduce redundancy reference to other compositions by their ID is allowed in the MPD.

A MediaComposition has MediaCompositionInfo element. A MediaCompositionInfo has the @refID attribute to facilitate media composition referencing.

Example of MediaCompositionGroup with reference IDs:

<MPD> <Period> <MediaCompositionGroup> <MediaComposition ID=”1”> <GroupInfo refID=”1”/> //Video <GroupInfo refID="2"/> //Audio </MediaComposition> <MediaComposition ID=”2”> <MediaCompositionInfo refID=”1”/> <RepresentationInfo refID="10"/> //English subtitle </MediaComposition> <MediaComposition ID=”3”> <MediaCompositionInfo refID=”1”/> <RepresentationInfo refID="11"/> //Korean subtitle </MediaComposition> </MediaCompositionGroup> ... </Period> </MPD>

Commonality of Representations from the same group

Representations from the same group share many representation attributes and are distinguished from each other by a few attributes. For example, multiple Representations with same group can have same @width, @height, @lang, @mimeType, @startWithRAP, @qualityRanking, ContentProtection, TrickMode, SegmentInfo values but different @bandwidth value. The common attributes of Represenatations belonging to the same group can be listed in GroupInfo of MediaComposition to convey the commonality of Representations.

Example of commonality:

<MPD> <Period> <MediaCompositionGroup> <MediaComposition ID=”1”> <RepresentationInfo refID=”1”/> <GroupInfo refID=”10” lang="en" mimeType="video/3gpp; codecs=mp4v.20.9, mp4a.E1"/> </MediaComposition> </MediaCompositionGroup> … <Representation ID=”1”>…</Representation> <Representation ID=”2” Group=”10” bandwidth="128000">…</Representation> <Representation ID=”3” Group=”10” bandwidth="256000">…</Representation> <Representation ID=”4” Group=”10” bandwidth="512000">…</Representation> </Period> </MPD>

- Content Protection Signaling

This chapter is the response for action items from the DASH ad-hoc meeting at Paris in September 2010 [1]: improve and enhance DRM proposals

  - ContentProtection and SegmentEncryptionInformation

<Representation> element in the MPD can contain multiple <ContentProtection> elements. Each ContentProtection element has the following attributes and element to provide the information associated with the specific protection scheme.

- @schemeIdUri provides an absolute URL to identify the scheme employed for content protection.

- @ licenseInfos contains the information to specify location for the acquisition of the associated license. If a EmbededLicense element is not included as a child element of ContentProtection, then @licenseInfos shall be included.

- @downloadableDRMInfo provides the information which specifies the location securely to install and manage DRM system identified by @schemeIdUri.

- <EmbededLicense> element allows associated licenses to be embedded in the MPD.

<SegmentEncryptionInformation> has the following attributes and element.

- @algorithmId specifies the identifier of the encryption algorithm used to encrypt Samples of all Segments.

- <SampleEncryptionInfo> element provides additional encryption parameter per Segment. There are three attributes @initialVector, @sourceURL and @segRef.

  - @initialVector is an initialization vector(IV) used by encryption mechanism.

  - @sourceURL specifies the location of the additional encryption parameters such as initialization vector per Sample.

  - @segRef is a reference to a segment and shall correspond to a @segNum attribute contained in the <URL> child element of <SegmentInfo> element.

Example of ContentProtection and SegmentEncryptionInfo:

<MPD> <Period> <Representation ID=“1” Group =“10” > <ContentProtection schemeIdUri=”OMA” licenseInfos=“ ” downlodableDRMInfo=“ ”/> <ContentProtection schemeIdUri=”Marlin” downloadableDRMInfo=“ „> <EmbededLicense>MarlinLicenseHere</EmbededLicense> </ContentProtection> <SegmentEncryptionInfo algorithmID=“0x01”> <SampleEncryptionInfo initialVector=”0x1111” sourceURL=“1sampleinfos“ segRef=“1”/> <SampleEncryptionInfo initialVector=”0x1211” sourceURL=“2sampleinfos“ segRef=”2”/> </SegmentEncryptionInfo> <SegmentInfo duration="PT10S" baseURL="rep1/"> <InitialisationSegmentURL sourceURL="seg-init.3gp"/> <Url sourceURL="seg-1.3gp“ segNum=“1”/> <Url sourceURL="seg-2.3gp" segNum=”2”/> </SegmentInfo> </Representation> </Period> </MPD>

  - Content Protection Signaling at the level of MediaComposition

It is proposed that a MediaComposition can have multiple <ContentProtection> elements. Thus, each content protection scheme can correspond to a subset of Representations. Upon the reception of the MPD, a client player may select one of protection schemes in MediaComposition and consistently use selected one when access encrypted contents in Representations which GroupInfo and RepresentationInfo refer. This reduces redundancy when specifies all available protection schemes for every Representation.

The following example is the case when two protection systems are provided for the MediaComposition. Two groups of Representations are referenced with group value #1 for video with various bitrates and #2 for audio. Once the client chooses one of protection systems and obtain the corresponding licenses, it can consistently access any of video tracks with various bitrates and audio tracks referred by the MediaComposition.

Example of Content Protection Signaling in MediaComposition:

<MPD> <Period> <MediaCompositionGroups> <MediaComposition ID=”1”> <ContentProtection schemeIdUri=”A” licenseInfos=“ ” downlodableDRMInfo=“ ”/> <ContentProtection schemeIdUri=“B” downloadableDRMInfo=””/> <EmbededLicense>…</EmbededLicense> </ContentProtection> <GroupInfo refID=”1”/> //Video <GroupInfo refID="2"/> //Audio </MediaComposition> </MediaCompositionGroups> … </Period> </MPD>

Proposed extensions on Baseline MPD

|  | Period |  |  |  |  |  |  | E | 1…N | M | Provides the information of a Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MediaCompositionGroup |  |  |  |  |  | E | 0,N | O |  |
|  |  |  | MediaComposition |  |  |  |  | E | 1…N | M |  |
|  |  |  |  | ID |  |  |  | A | 1 | M |  |
|  |  |  |  | description |  |  |  | A | 0,1 | O |  |
|  |  |  |  | lang |  |  |  | A | 0,1 | O |  |
|  |  |  |  | minBandwidth |  |  |  | A | 0,1 | O |  |
|  |  |  |  | maxBandwidth |  |  |  | A | 0,1 | O |  |
|  |  |  |  | mimeType |  |  |  | A | 0,1 | O |  |
|  |  |  |  | ContentProtection |  |  |  | E | 0…N | O |  |
|  |  |  |  |  |  |  | schemeIdUri | A | 1 | M |  |
|  |  |  |  |  |  |  | licenseInfos | A | 0,1 | O |  |
|  |  |  |  |  |  |  | downloadableDRMInfo | A | 0,1 | O |  |
|  |  |  |  |  |  |  | EmbeddedLicense | E | 0,1 | O |  |
|  |  |  |  | RepresentationInfo |  |  |  | E | 0,N | O |  |
|  |  |  |  |  | refID |  |  | A | 1 | M |  |
|  |  |  |  | GroupInfo |  |  |  | E | 0,N | O |  |
|  |  |  |  |  |  | refID |  | A | 1 | M |  |
|  |  |  |  |  |  | bandwidth |  | A | 0,1 | O | If exist, same attribute will be inherited all Representations of same group value. |
|  |  |  |  |  |  | width |  | A | 0,1 | O | If exist, same attribute will be inherited all Representations of same group value. |
|  |  |  |  |  |  | height |  | A | 0,1 | O | If exist, same attribute will be inherited all Representations of same group value. |
|  |  |  |  |  |  | lang |  | A | 0,1 | O | If exist, same attribute will be inherited all Representations of same group value. |
|  |  |  |  |  |  | mimeType |  | A | 0,1 | O | If exist, same attribute will be inherited all Representations of same group value. |
|  |  |  |  |  |  | startWithRap |  | A | 0,1 | O | If exist, same attribute will be inherited all Representations of same group value. |
|  |  |  |  |  |  | qualityRanking |  | A | 0,1 | O | If exist, same attribute will be inherited all Representations of same group value. |
|  |  |  |  | MediaCompositionInfo |  |  |  | E | 0,N | O |  |
|  |  |  |  |  | refID |  |  | A | 1 | M |  |
|  |  | Representation |  |  |  |  |  | E | 1..N | M | This element contains a description of a Representation. |