INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11

MPEG2010/ m18420

October 2010, Guangzhou, China

| Source | Nokia |
| --- | --- |
| Status | Report |
| Title | 3DV EE4 results on “Pantomime” sequences |
| Author | Dmytro Rusanovskyy, Miska M. Hannuksela |

# Introduction

This document presents the experimental results of EE4 (coding experiments) C2&C3 configuration for “Pantomime” sequence with a new depth map, provided by Nagoya University. The experiments were conducted according to the description of exploration experiment [1] and with unified MVC configuration file, provided by Sehoon from 22.Sep.2010. Coding of texture and depth data were coded separately with JMVC 8.0 with several QP settings. View synthesis was done with VSRS 3.5.

Objective quality evaluation was done with PSNR calculation and reference synthesized views (synthesized from uncompressed data) were utilized as an anchor. The best QP combination for texture and depth data was decided on the best PSNR of the synthesized view.

# Experimental Results

During the exploration experiment we utilized a most recent depth map data provided by Nagoya University. Experimental setup with conducted according with description of exploration experiment [1] which implied 300 coded frames.

## MVC and View Synthesis configurations

MVC coding for texture and depth data was performed with unified configuration file, provided by Sehoon from 22.Sep.2010.

The VSRS 3.5 was utilized for view synthesis. The best for the “Pantomime” synthesis tools configuration was estimated with PSNR calculation for the reference synthesized view 40 against the original view 40. VSRS with following configuration settings was found to provide the best performance, 35.89dB against the original texture data. . The full VSRS configuration settings are provided in Appendix.

Boundary Noise Removal	1

SynthesisMode		0

ViewBlending			0

Achieved results confirmed an optimal VSRS configuration for Pantomime sequence reported earlier by NTT in [2] for old EE4 settings

## Experimental Results for 2 View Case (C2)

Table 1 presents total MVD bit-rate for two views (39,41) and related depth maps. Allowing 5% difference, several quantization configurations of MVD can be preselected as close to the target points. Please see related operational points marked in color.

Table 1 Resulting total bit rate of MVD for Pantomime sequence, EE4 C2 coding conditions.

| Total bit-rates (kbit/s), MVD C2 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP texture |  |  |  |  |  |  |  |  |  |  |  |
| QPs depth | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 |
| 30 | 3716 | 3239 | 2882 | 2579 | 2363 | 2168 | 2029 | 1920 | 1852 | 1829 | 1820 | 1817 |
| 32 | 3443 | 2967 | 2609 | 2307 | 2091 | 1895 | 1756 | 1647 | 1579 | 1556 | 1547 | 1544 |
| 34 | 3229 | 2753 | 2395 | 2092 | 1877 | 1681 | 1542 | 1433 | 1365 | 1342 | 1333 | 1330 |
| 36 | 3066 | 2589 | 2232 | 1929 | 1713 | 1518 | 1379 | 1269 | 1202 | 1179 | 1169 | 1167 |
| 38 | 2946 | 2469 | 2112 | 1809 | 1593 | 1398 | 1259 | 1150 | 1082 | 1059 | 1050 | 1047 |
| 40 | 2836 | 2360 | 2002 | 1700 | 1484 | 1288 | 1149 | 1040 | 972 | 949 | 940 | 937 |
| 42 | 2739 | 2262 | 1905 | 1602 | 1386 | 1191 | 1052 | 942 | 875 | 852 | 842 | 840 |
| 44 | 2668 | 2192 | 1834 | 1532 | 1316 | 1121 | 981 | 872 | 804 | 781 | 772 | 770 |
| 46 | 2620 | 2144 | 1786 | 1484 | 1268 | 1072 | 933 | 824 | 756 | 733 | 724 | 721 |
| 48 | 2604 | 2128 | 1770 | 1468 | 1252 | 1057 | 918 | 808 | 741 | 717 | 708 | 706 |
| 50 | 2598 | 2122 | 1764 | 1462 | 1246 | 1050 | 911 | 802 | 734 | 711 | 702 | 700 |
| 52 | 2600 | 2123 | 1766 | 1463 | 1247 | 1052 | 913 | 803 | 736 | 712 | 703 | 701 |

Considering the results of RD performance, we chose the combination of QP values for texture and depth map MVC coding which achieve highest PSNR for synthesized view at the target bit-rate. The results of this selection are presented in the Table 2.

Table 2 Optimal MVD (C2) coding configurations for target bit-rates (total for all views).

| Total, bit rate, kbit/s |  | View 40 PSNR_Y, dB | Texture QP | Depth QP | Target met, % |
| --- | --- | --- | --- | --- | --- |
| 2000 | 2002.18 | 37.24 | 34 | 40 | 100.1 |
| 1250 | 1267.69 | 34.98 | 38 | 46 | 101.42 |
| 1000 | 981.42 | 32.89 | 42 | 44 | 98.14 |
| 750 | 735.62 | 30.55 | 46 | 50 | 98.08 |

## Experimental Results for 3 Views Case (C3)

Table 3 presents total bit-rate of MVD coding of three views (37, 39 and 41) and related depth maps. Allowing 5% difference to a target bit-rate, we retrieve several candidate quantization configurations of MVD. Please see related operational points marked in color.

Considering the results of RD performance, we chose the combination of QP values for texture and depth map MVC coding which achieve highest PSNR for synthesized view at the target bit-rate. The results of this selection are presented in the Table 4.

Table 3 Resulting total bit rate of MVD for Pantomime sequence, EE4 C3 coding conditions.

| Total bit-rates (kbit/s), MVD C3 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP texture |  |  |  |  |  |  |  |  |  |  |  |
| QPs depth | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 |
| 30 | 5145 | 4498 | 4016 | 3625 | 3339 | 3071 | 2881 | 2745 | 2654 | 2626 | 2615 | 2614 |
| 32 | 4762 | 4114 | 3632 | 3242 | 2956 | 2688 | 2498 | 2361 | 2270 | 2242 | 2231 | 2231 |
| 34 | 4457 | 3809 | 3327 | 2937 | 2651 | 2383 | 2193 | 2056 | 1965 | 1938 | 1927 | 1926 |
| 36 | 4217 | 3569 | 3087 | 2697 | 2411 | 2143 | 1953 | 1816 | 1725 | 1698 | 1687 | 1686 |
| 38 | 4035 | 3388 | 2906 | 2515 | 2229 | 1961 | 1771 | 1635 | 1544 | 1516 | 1505 | 1504 |
| 40 | 3872 | 3224 | 2743 | 2352 | 2066 | 1798 | 1608 | 1471 | 1381 | 1353 | 1342 | 1341 |
| 42 | 3721 | 3073 | 2591 | 2201 | 1914 | 1647 | 1457 | 1320 | 1229 | 1201 | 1190 | 1190 |
| 44 | 3622 | 2975 | 2493 | 2102 | 1816 | 1548 | 1358 | 1222 | 1131 | 1103 | 1092 | 1091 |
| 46 | 3549 | 2901 | 2419 | 2029 | 1742 | 1475 | 1285 | 1148 | 1057 | 1029 | 1018 | 1018 |
| 48 | 3525 | 2877 | 2395 | 2005 | 1718 | 1451 | 1261 | 1124 | 1033 | 1005 | 994 | 994 |
| 50 | 3517 | 2869 | 2387 | 1997 | 1710 | 1443 | 1253 | 1116 | 1025 | 997 | 986 | 986 |
| 52 | 3514 | 2866 | 2384 | 1994 | 1707 | 1439 | 1250 | 1113 | 1022 | 994 | 983 | 982 |

Table 4 Optimal MVD (C3) coding configurations for target bit-rates (total for all views).

| Total Bit rate, kbit/s |  | Average PSNR_Y, dB | Texture QP | Depth QP | Target met, % |
| --- | --- | --- | --- | --- | --- |
| 2500 | 2492.90 | 36.00 | 34 | 42 | 99.72 |
| 1750 | 1742.41 | 33.77 | 38 | 46 | 99.57 |
| 1350 | 1284.73 | 31.87 | 42 | 46 | 95.17 |
| 1000 | 1033.04 | 29.73 | 46 | 48 | 103.30 |

# Conclusions

This document presents the experimental results of EE4 C2&C3 coding experiments for “Pantomime“ sequence with a new depth map, provided by Nagoya University. The experiments were conducted according to the description of exploration experiment [1] and unified MVC configuration file from 22.Sep.2010. Texture and depth data were coded separately with JMVC 8.0 with several QP settings. View synthesis was performed with VSRS 3.5.

Objective quality evaluation was done with PSNR calculation and a reference synthesized view (synthesized from uncompressed data) were utilized as an anchor. The best QP combination for texture and depth data was decided on the best PSNR of the synthesized view. VSRS and MVC configuration files utilized during experiments are provided in this contribution.

# References

[1] 3DV/FTV AHG, “Description of Exploration Experiments in 3D Video Coding”, N11477, Geneva, Switzerland, July 2010.

[2] S.Shimizu, H.Kimata, “3DV/FTV EE4 results on Pantomime sequence”, M17191, Kyoto, Japan, Jan.2010.