| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E701 WG11 Number: m20236 |
| --- | --- |

| Title: | Description of Core Experiment CE1: Motion Data Storage Reduction |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Tool Experiment Report |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 92130 ISSY LES MOULINEAUX France | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com |
|  | P. Onno Canon Research Centre France Rue de la Touche Lambert 35512 CESSON-SEVIGNE France | Tel: Email: | +33 2 87 68 92 patrice.onno@crf.canon.fr |
| Source: | CE coordinators |  |  |

___________________________

# Introduction

This Core Experiment relates to Motion Data Storage Reduction. At the 5th JCTVC meeting some input documents were proposed to modify the MV storage reduction tool in combination with the temporal MV predictor. No adoption has been done since the current MV Coding and Skip/Merge operations studied in a CE were interacting significantly.

The goal of this Core Experiment is to test the different Motion Data Storage Reduction proposals submitted in the 5th JCTVC meeting in the HM3.0.

# Participants List

| Participant | Person, Email address | P | C |
| --- | --- | --- | --- |
| Canon | P. Onno (patrice.onno@crf.canon.fr) G. Laroche (guillaume.laroche@crf.canon.fr) | x |  |
| ETRI | S.-C. Lim (sclim@etri.re.kr) H. Y. Kim (hykim5@etri.re.kr) | x |  |
| HHI | B. Bross (benjamin.bross@hhi.fraunhofer.de) |  | x |
| LGE | S. Park (seungwook.park@lge.com) J. Park (jy.park@lge.com) |  |  |
| JVC Kenwood | S. Fukushima (fukushima.shigeru@jk-holdings.com) H. Takehara (takehara.hideki@jk-holdings.com) M. Ueda (ueda.motoharu@jk-holdings.com) |  |  |
| Microsoft | J.-Z. Xu (jzxu@microsoft.com) | x |  |
| MediaTek | Y.-W. Huang (yuwen.huang@mediatek.com) J.-L. Lin (jl.lin@mediatek.com) Y.-W. Chen (yiwen.chen@mediatek.com) X. Guo (xun.guo@mediatek.com) | x | x |
| Orange Labs | J. Jung (joelb.jung@orange-ftgroup.com) |  | x |
| Panasonic | T. Sugio (sugio.toshiyasu@jp.panasonic.com) |  |  |
| Samsung | I.-K. Kim (ilkoo.kim@samsung.com) |  |  |
| Toshiba | A.Tanizawa (akiyuki.tanizawa@toshiba.co.jp) T. Shiodera (taichiro.shiodera@toshiba.co.jp) |  |  |

Table 1: participant list.

# Motion Data Storage Reduction

## Related Contributions

The descriptions of the experiments performed in the CE are described in this section. Some commonalities between the different contributions can be mentioned here:

- Selection of the representative motion data: JCTVC-E059, JCTVC-E092, JCTVC-E142, JCTVC-E147, JCTVC-E211, JCTVC-E221, JCTVC-E307.

- Finite-bit representation of motion data: JCTVC-E142, JCTVC-E221.

JCTVC-E059 [LGE] Modifications of temporal mv memory compression and temporal mv predictor

In this contribution, three modifications of temporal motion data compression scheme are proposed.

- Position change of a representative motion data in a pre-defined region.

- Searching scheme for getting efficient motion data when it is not available.

- position change of temporal predictor and its synergy effect with motion data storage reduction.

In this CE, the best position of representative motion data, its searching method and the best combination with various temporal predictors under a circumstance of different motion data storage reduction ratio will be searched.

JCTVC-E092 [MediaTek] Motion Vector Decimation for Temporal Prediction

This contribution proposes a motion vector (MV) decimation method for storing the MV data at lower resolution. Specifically, the MVs from the bottom right 4x4 block in a MV compression unit are used as the MVs of the whole unit instead of the top left one in HM2.0. Moreover, the reference indices of the bottom right block are also used for the whole unit to avoid the mismatch between MVs and reference indices.

JCTVC-E096 [JVC Kenwood] Partition size based selection for motion vector compression

In this proposal, the candidates to be stored are center four 4x4 blocks in the 16x16 block. The prediction partition size is compared in z-scanning order. The motion vector of the block which partition size is the largest is selected.

JCTVC-E142 [ETRI] Dynamic range restriction of temporal motion vector

This contribution presents a dynamic range restriction of temporal motion vector in order to reduce the memory size and the memory access bandwidth. The method restricts the value of temporal motion vectors into such a range that can be fit into a predefined fixed bit-width, which can be signaled in sequence level. It is recommended to evaluate this method with variation in bit-widths, e.g., 8-bits and 10-bits, to find the best trade-off between storage reduction factor and coding loss.

JCTVC-E147 [USTC, Microsoft] On motion information compression

This contribution proposes:

1.            To reduce the memory requirement, compress reference frame indexes and modes.

2.            Use the mv of the last block instead of the first block in the region.

3.            Scan from the last block for the first available block in the region.

JCTVC-E211 [Toshiba] Modified motion vector memory compression

This contribution proposes a modified motion vector memory compression (MMC) method which includes two parts.

- The reference index memory compression which reduces the reference index memory to one sixteen in done in the same manner as motion vector memory compression.

- Representative motion vector derivation which uses MV of bottom-right 4x4 block as a representative MV instead of upper-left 4x4 block in compressed 16x16 block.

JCTVC-E221 [Canon] On memory compression for motion vector prediction

3 proposed methods were described in this contribution:

- Block position: Change the top left position by the bottom right position for the summarization in order to gives more diversity compared to the spatial predictors for small CUs and to match the center of the collocated predictor for large CUs.

- MV components clipping: each component is clipped from 12 to 8 bits and according to the spatial distance between the current frame and its nearest reference frame the least significant bit (which represents the ¼-pel, ½-pel) in order to increase the range value of the motion vector.

- Scaling: in order to avoid reference frame and/or list indexes memory access, the motion vector are scaled.

JCTVC-E231 [Panasonic] Modified motion vector compression method

In this contribution, a modified motion vector compression method was proposed. It was proposed that a representative motion vector was searched among 4x4 blocks according to encoding order. And when referring as a co-located partition, the representative motion vector was scaled by using reference index of each 4x4 block and reference index correspond to it. Experimental results reportedly showed 0.6% BR saving for HE and LC on average in the RA scenarios, and 0.6% BR saving for HE and 0.7% BR saving for LC on average relative to the HM2.0.

JCTVC-E307 [Samsung] Improved motion vector decimation

This contribution proposes modified motion vector selection to be stored in motion vector decimation process. The proposed method selects the right-bottom motion vector instead of the left-top motion vector as the motion vector to be stored in the motion vector buffer.

## Experiment Descriptions

### Category A: Selection of the representative motion data

Several proposals select different MV candidate for motion data storage reduction. This first subset deals with looking at the best MV representative already tested in the different proposals listed above, in combination with the temporal MV predictor location. Related contributions are JCTVC-E059, JCTVC-E092, JCTVC-E142, JCTVC-E147, JCTVC-E211, JCTVC-E221, and JCTVC-E307.

| Experiment | MVSR (position) | Prediction (position) | Tester | Checker |
| --- | --- | --- | --- | --- |
| Anchor | HM3.0 Top-left | HM3.0 (H or T) | - | - |
| A.1 | Bottom-right | HM3.0 (H or T) | Canon | Orange Labs |
| A.2 | Center | HM3.0 (H or T) |  |  |
| A.3 | HM3.0 Top-left | Collocated |  |  |
| A.4 | Bottom-right | Collocated | Canon |  |
| A.5 | Center | Collocated |  |  |
| A.6 | Largest size | HM3.0 (H or T) | JVC KENWOOD |  |

Table 2: Summary of the experiments for category A.

### Category B: Finite-bit representation of motion data

Different methods to reduce the motion data storage have been presented. The goal of this test is to evaluate the performance of each of the reduction method related to motion vectors.

| Experiments | Finite-bit MV representation |  | Reference scaling (E221) | Tester | Checker |
| --- | --- | --- | --- | --- | --- |
|  | bits per MV component (E142) | LSBs removed (E221) |  |  |  |
| Anchor |  | 0 (fixed) | X | - | - |
| B.1 | 10 | 0 (fixed) | X | ETRI | Canon |
| B.2 |  |  | 0 (Ref only) | Canon | ETRI |
| B.3 |  |  | 1 (One single list) | Canon | ETRI |
| B.4 |  | 1 (adaptive) | X | Canon | ETRI |
| B.5 |  |  | 0 (Ref only) | Canon | ETRI |
| B.6 |  |  | 1 (One single list) | Canon | ETRI |
| B.7 | 8 | 0 (fixed) | X | ETRI | Canon |
| B.8 |  |  | 0 (Ref only) | Canon | ETRI |
| B.9 |  |  | 1 (One single list) | Canon | ETRI |
| B.10 |  | 1 (adaptive) | X | Canon | ETRI |
| B.11 |  |  | 0 (Ref only) | Canon | ETRI |
| B.12 |  |  | 1 (One single list) | Canon | ETRI |

Table 3: Summary of the experiments for category B.

### Combination of the best solutions

The best results obtained in Category A and in Category B will be combined to find the best compromise in term of coding efficiency and compression ratio.

### (Optional) Impact of the spatial sampling factor variations

The impact of the block summarization with the new proposed tools could be study with the new proposed tools.

# Test Sequences / Test Points / SW Configuration

Simulation results will be generated for sequences and coding conditions specified in the document JCTVC-E700. Latest version of the HM (HM 3.0), available on April 11th will be used for evaluation.

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-Rate).

Report the compression factor of all the MV data (MV fields, Indexes and mode).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration)

- Decoding time (compared to the default configuration)

# Time-line

| T1 | 2011-April 11 ??? | Final CE description and description of proposals HM3.0 available |
| --- | --- | --- |
| T2 | 2011- June 22nd | Completion of tool integration, test configuration announcement Start of cross-check |
| T3 | 2011-June 29th | Cross-check complete Reporting of all results to CE1 coordinators complete |
| T4 | 2011-July 1st | Start of the tests of joint approaches (category A + category B) |
| T5 | Documents registration deadline | Input documents uploaded |
| T6 | Documents registration deadline + 3 days | CE1 summary report uploaded |

Table 4: time-line