| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E702 |
| --- | --- |

| Title: | Core Experiment 2: Motion Partitioning and OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng1 (Primary Coordinator), Philippe Bordes2 (Coordinator) Peisong Chen3 (Coordinator) Liwei Guo3 Il-Koo Kim4 (Coordinator) Wen-Hsiao Peng5 | Tel: Email: | +86-755-28422009 zhengxiaozhen@huawei.com philippe.bordes@technicolor.com peisongc@qualcomm.com ilkoo.kim@samsung.com pawn@mail.si2lab.org |
| Source: | Huawei & HiSilicon Technologies1, Technicolor2, Qualcomm3, Samsung4,NCTU/ITRI5 |  |  |

_____________________________

- Introduction

This Core Experiment (CE) is an ongoing work of previous CE2 – Flexible Motion Partitioning in Error: Reference source not found. Its goal is to further investigate block partition issue.

Several related techniques have been considered in the 5th JCTVC meeting, with promising BD-rate performance, but with some remaining concerns regarding encoder and decoder complexity.

The techniques considered in this CE are:

- Asymmetric Motion Partitioning (AMP), proposed in [2];

- Non-rectangular Motion Partitioning (NRMP), proposed in [3];

- Overlap Block Motion Compensation (OBMC), proposed in [2], [3], [4], [7], [8]

- Non-Square TU, proposed in [5]

Further improvements are required to find the best solution of those four methods with the optimal trade-off coding efficiency/encoding complexity.

# Participants List

| Participant | Person, Email address |
| --- | --- |
| Technicolor | Philippe Bordes, philippe.bordes@technicolor.com |
| Qualcomm | Peisong Chen, peisongc@qualcomm.com Liwei Guo, liweig@qualcomm.com |
| Huawei/HiSilicon | Xiaozhen Zheng, zhengxiaozhen@huawei.com Haoping Yu, haopingyu@huawei.com |
| Samsung | Il-Koo Kim, ilkoo.kim@samsung.com Jianle Chen, jianle.chen@samsung.com |
| Tsinghua | Yuan Yuan, y-yuan08@mails.tsinghua.edu.cn |
| HHI | Benjamin Bross, benjamin.bross@hhi.fraunhofer.de Martin Winken, Martin.Winken@hhi.fraunhofer.de |
| Intel | Y.-J Chiu, Yi-jen.chiu@intel.com L. Xu, Lidong.xu@intel.com W. Zhang, Wenhao.zhang@intel.com |
| NCTU/ITRI | Wen-Hsiao Peng, pawn@mail.si2lab.org Chun-Chi Chen, cheerchen.cs98g@g2.nctu.edu.tw Chung-Lin Lee, yoyoli.cs97g@nctu.edu.tw |
| TI | Minhua Zhou, zhou@ti.com |
| Microsoft | Jizheng Xu, jzxu@microsoft.com |
| NTT Docomo | T.K. Tan, ttk@pacific.net.sg |
| Motorola | Krit Panusopone, krit@motorola.com |
| Nokia | Jani Lainema, jani.lainema@nokia.com |
| Panasonic | Hisao Sasai, sasai.hisao@jp.panasonic.com |

# Tools under Test

At the next meeting cycle, four related techniques will be tested at CE2:

- Asymmetric Motion Partitioning (AMP);

- Non-rectangular Motion Partitioning (NRMP);

- Overlapped Block Motion Compensation (OBMC)

- Non-Square TU

## Asymmetric Motion Partitioning

The related JCTVC contribution is:

- JCTVC-E316, by Samsung & Qualcomm

Asymmetric motion partition (AMP) was initially proposed in JCTVC-A124 [2], and fast AMP method was proposed in JCTVC-E316 [2]. The tool consists of partitioning a square block into two non-symmetric horizontal or vertical rectangles shapes, as illustrated in Figure 1 for the 64x64 block size. In the current design, the square block is split using AMP in one rectangle of 3/4 of the square width or height, and another rectangle of 1/4 of the square width or height.

Figure 1: Example of asymmetric motion partitioning for a 64x64 block.

For AMP encoding speed up, additional conditions are checked before doing motion estimation for AMP. If the certain conditions are met, additional motion estimation for AMP can be skipped.

## Non-Rectangular Motion Partitioning (NRMP)

The related JCTVC contribution is:

- JCTVC-E373 [3], by Huawei & HiSilicon, Technicolor, Qualcomm

Non-rectangular motion partitioning (NRMP) was initially proposed in JCTVC-D230 and JCTVC-D297. NRMP enables to partition a square block into 2 regions with non-rectangular shape. Limited sets of diagonal-only partitions (45 degree or 135 degree partitioning) and non-diagonal partitions will be tested. Figure 2 and figure 3 illustrate two NRMP approaches.

Figure 2: Example of 2 diagonal partitioning modes

Figure 3: Example of set of 6 not-only diagonal partitioning modes

## Overlapped Block Motion Compensation (OBMC)

The related JCTVC contribution is:

- JCTVC-E304, by Qualcomm

In OBMC, predictions in the overlapped regions are derived by weighted averaging prediction pixels from two different motion vectors. The overlapped region in the luma component is defined as 2 rows (columns) of pixels on both sides of the horizontal (vertical) partition boundary. For chroma components, the overlapped region is reduced to 1 row (column) of pixels. The weighting factors are (3/4, 1/4) for the pixels which are 1 row (column) apart from the boundary and (7/8, 1/8) for pixels which are 2 row (column) apart from the boundary. In this core experiments, OBMC will be tested with AMP and/or with non-rectangular motion partition and/or with regular motion partitions (e.g., 2NxN and Nx2N).

Additional contribution related to OBMC is “OBMC with Motion Merging”

- JCTVC-E154, by NCTU/ITRI

JCTVC-E154 [7] (and JCTVC-D175 [8]) introduced an OBMC scheme which forms a bi-prediction for a 2Nx2N PU based on two sets of motion parameters, one is signaled in uni-prediction syntax and the other is specified by the motion merging mechanism. The predictors derived from these motion parameters are linearly combined using OBMC with a window function dependent on MV constellation. Additionally, for optimizing the motion parameters to be signaled in syntax, the block matching criterion was modified to consider the OBMC effect. This technique was observed to achieve BD-rate reductions of 1.4% for RA-HE, 1.6% for RA-LC, 1.9% for LD-HE, and 2.6% for LD-LC. The improvements come with a 50-100% increase in encoding time and a 4-6% increase in decoding time. Discussion in this meeting showed some concerns about the memory bandwidth and the encoding complexity of this technique. This subtest aims to study its performance versus the number of hypotheses and the number of merge candidates to be considered, and to seek a way to harmonize it with the other techniques studied in this CE.

## Non-Square TU

The related JCTVC contribution is:

- JCTVC-E376, by Tsinghua, Huawei & HiSilicon, Microsoft, USTC

Residual quad transform (RQT) is used at block residual transform. A transform block can be split recursive. When TU cross PU's boundary, high frequency coefficients are generated which will impose negative influence on coding performance. For the case of AMP, this problem will be more serious because TU at first and second level will cross PU's boundary. To solve this problem, non-square transform unit (TU) is applied to asymmetric motion partitions.

At non-square TU design, 2Nx2N transform is still used at RQT’s first level, 0.5Nx2N transform or 2Nx0.5N transform is applied to the second level when asymmetric partition is used. At the case of RQT’s third level, the transform block inside non-square TU can be further split. Non-Square TU can also be extended to 2NxN, Nx2N and non-rectangular partitions. Figure 3 illustrates the non-square TU structure for AMP.

Figure 3: Illustration of non-square TU design

## Combination test

CE2 will also test the combination of different coding tools, which includes AMP, NRMP, OBMC and Non-Square TU.

To signal the different partitioning modes efficiently, INTER_2Nx2N is used to signal non-rectangular motion partitioning. An additional flag, inserted after the INTER_2Nx2N signaling, allows to select INTER_2Nx2N or INTER_NRMP. If INTER_NRMP is selected, additional bits are inserted to signal the INTER_NRMP mode. INTER_2NxN and INTER_Nx2N are used to signal horizontal asymmetric motion partitioning and vertical asymmetric motion partitioning respectively.

# Experiments

## Tests description

The goal of the tests of CE2 is to identify the best combination and settings.

The reference anchors will be generated using the HM3.0 with the tests conditions as specified in JCTVC-E700 [9].

The test will contain four subtests.

- Evaluation of motion partitioning. The following subtests will be made:

  - Turn off AMP, OBMC and Non-Square TU, and test NRMP with 2 diagonal partitioning;

  - Turn off AMP, OBMC and Non-Square TU, and test NRMP with 6 not-only diagonal partitioning;

  - Turn off NRMP, OBMC and Non-Square TU, and test AMP.

- Evaluation of OBMC. The following subtests will be made:

  - Turn on AMP and OBMC, turn off NRMP and Non-Square TU.

  - Turn off AMP and Non-Square TU, turn on Best (1a, 1b) and OBMC.

  - Turn off AMP, NRMP and Non-Square TU, test OBMC at regular partitions;

  - Turn off AMP, NRMP and Non-Square TU, test OBMC with Motion Merging

- Evaluation of Non-Square TU. The following subtest will be made:

  - Turn on AMP, turn off NRMP partitioning, turn off OBMC, turn on Non-Square TU.

- Evaluation of tools combination.

  - Test Best (1a, 1b) + 1c

  - Test 2a + 2b.

  - Test 2a + 2b + 3a.

The following table summarizes the planed tests.

| Test number | AMP | NRMP | OBMC | OBMC with Motion Merging | Non-Square TU | Cross-checker |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1a | Off | 2 diagonal | Off | Off | Off |  |  |
| 1b | Off | 6 not-only diagonal | Off | Off | Off |  |  |
| 1c | On | Off | Off | Off | Off |  |  |
| 2a | On | Off | On | Off | Off |  |  |
| 2b | Off | Best(1a,1b) | On | Off | Off |  |  |
| 2c | Off | Off | On | Off | Off |  |  |
| 2d | Off | Off | Off | On | Off | Intel |  |
| 3a | On | Off | Off | Off | On |  |  |
| 4a | On | Best(1a,1b) | Off | Off | Off |  |  |
| 4b | On | Best(1a,1b) | On | Off | Off |  |  |
| 4c | On | Best(1a,1b) | On | Off | On |  |  |

## Software

The CE tools are going to be implemented into the HM-SDIP branch software.

## Test Sequences, Bit Rates and Coding Conditions

The CE tools will be tested and evaluated under the common official configurations recommended by JCT-VC at the conclusion of the 5th JCT-VC meeting in document JCTVC-E700. In particular the reference anchors will be generated using the recommended configuration files provided by the JCT-VC software group.

Restrictions / modifications for this CE:

- No test with intra-only configuration

- High efficiency configuration

- Low complexity configuration (optional)

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and encoding/decoding run time comparisons as defined in document JCTVC-B312 Error: Reference source not found.

Following comparisons will be made:

- Proposal against HM3.0+SDIP with default configuration

# Time-line

- 8 April 2011		Final CE-description

- 4 May 2011         Start of tool integration (After release of HM3.0+SDIP branch)

- 20 June 2011		Completion of tool integration and optimization, start of testing

- 28 June 2011	 	Document registration

- 1st July 2011   	Input Documents upload

# References

- X. Zheng, P. Bordes, P. Chen, I-K Kim, “Core Experiment 2: Flexible Motion Partitioning,” Doc. JCTVC-D602, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, KR, January 2011

- I.-K. Kim (Samsung), P. Chen, L, Guo (Qualcomm), “CE2: Test results of asymmetric motion partition (AMP) with overlapped block motion compensation (OBMC)”, Doc. JCTVC-E316, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011.

- X. Zheng, H. Yu (Huawei & HiSilicon), P. Bordes (Technicolor), L. Guo, P. Chen (Qualcomm), “CE2: Test results of non-rectangular motion partitioning (NRMP) with overlapped block motion compensation (OBMC)”, Doc. JCTVC-E373, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011.

- L. Guo, P. Chen, R. Joshi, M. Karczewicz, “Overlapped Block Motion Compensation for Regular Block Partitions”, Doc. JCTVC-E304, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011.

- Y. Yuan (Tsinghua), X. Zheng (HiSilicon), X. Peng (USTC), L. Liu (HiSilicon), Y. Wang (Tsinghua), X. Cao (Tsinghua), J. Xu (Microsoft), C. Lai (HiSilicon), J. Zheng (HiSilicon), Y. He (Tsinghua), H. Yu (Huawei), “Asymmetric Motion Partition with OBMC and Non-Square TU”, Doc. JCTVC-E376, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011.

- C.-C. Chen, C.-L. Lee, W.-H. Peng and H.-M. Hang, (NCTU/ITRI), “CE1: Report of DMVD-based Bi-prediction”, Doc. JCTVC-E154 (m19671), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, March 2011.

- C.-L. Lee, C.-C. Chen, Y.-W. Chen, M.-H. Wu, C.-H. Wu, W.-H. Peng and H.-M. Hang, (NCTU/ITRI), “Bi-prediction combining template and block motion compensations”, Doc. JCTVC-D175 (m18927), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- Edouard Francois, Philippe Bordes (Technicolor), Liwei Guo, Marta Karczewicz (Qualcomm), “CE2: Simplified Geometry Block Partitioning”, Doc. JCTVC-D230 (m18990), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- X.Zheng (HiSilicon), H.Yu (Huawei), “CE2: Huawei & HiSilicon report on Flexible Motion Partitioning”, Doc. JCTVC-D297 (m19063), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.