| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 19-28 January, 2011 | Document: JCTVC-E703r3 WG11 Number: m20242 |
| --- | --- |

| Title: | Core Experiment 3: Motion Compensation Interpolation |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Elena Alshina | Email: | takeshi.chujoh@toshiba.co.jp elena_a.alshina@samsung.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

The goal of this Core Experiment (CE) is to further investigate following aspects of interpolation filtering for motion compensation (Luma) in HM:

- Simplification of interpolation MC and reduction of reference frame memory access bandwidth;

- Coding performance improvement by interpolation MC optimization;

- Study interdependency of interpolation and adaptive loop filter techniques;

- Study performance of interpolation filter techniques for “only-P” test case;

- Study visual quality effect of interpolation filters.

Interpolation filters for motion compensation (Luma) techniques will be evaluated according to BD-rate gain, complexity and subjective picture quality.

# Participants

Coordinators: Takeshi Chujoh (TOSHIBA Corporation) and Elena Alshina (Samsung)

| No. | Affiliation | Contact | Email | Remark: P - proponent C - cross-checker O -observer |
| --- | --- | --- | --- | --- |
| 1 | eBrisk Video | Faouzi Kossentini | faouzi@ebriskvideo.com | P/C |
| 2 | HHI | Haricharan Lakshman Heiko Schwarz | haricharan.lakshman@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de | O |
| 3 | KDDI | Tomonobu Yoshino | to-yoshino@kddilabs.jp | P/C |
| 4 | Motorola Mobility | Krit Panusopone Jian Lou Koohyar Minoo | krit@motorola.com lou@motorola.com kminoo@motorola.com | P/C |
| 5 | NTT | Shohei Matsuo | jctvc-te@lab.ntt.co.jp | P/C |
| 6 | Qualcomm | In Suk Chong Liwei Guo | ichong@qualcomm.com liweig@qualcomm.com | P/C |
| 7 | Sony | Teruhiko Suzuki Kenji Kondo | teruhikos@jp.sony.com kenji.kondo@jp.sony.com | P/C |
| 8 | TOSHIBA | Takeshi Chujoh Kazuyo Kanou | takeshi.chujoh@toshiba.co.jp kazuyo.kanou@toshiba.co.jp | P/C |
| 9 | Nokia | Kemal Ugur | Kemal.Ugur@nokia.com | P/C |
| 10 | Samsung | Elena Alshina Jianle Chen | elena_a.alshina@samsung.com / elena.alshina@gmail.com jianle.chen@samsung.com | P/C |
| 11 | TI | Zhou Minhua | zhou@ti.com | O |
| 12 | Intel | Y.-J. Chiu | yi-jen.chiu@intel.com | O |
| 13 | Canon | Edouard Francois | edouard.francois@crf.canon.fr | O |
| 14 | Docomo | TK Tan | ttk@pacific.net.sg | O |

# Description of tool

Details are provided in JCTVC-E134, JCTVC-D344, JCTVC-E241, JCTVC-E128, JCTVC-E129, JCTVC-E358, JCTVC-E359, JCTVC-D376, JCTVC-E078, JCTVC-E284, JCTVC-E188. According to discussion of motion compensation techniques during 5th JCTVC meeting following two categories of tests were recommended to be conducted in this CE:

- Simplification of interpolation filter for HEVC

- Study the adaptive interpolation filter application in HEVC

3.1. Simplified interpolation filter techniques for HEVC

The worst cases of complexity of interpolation process are bi-directional two dimensional quarter pixel positions of bidirectional prediction. Reduction of number of filter taps for certain positions or prediction modes (Single/Bi-directional) was proposed in several contributions. This would be advantageous in terms of memory bandwidth as well as computational complexity. Tests in this category are targeting to searching better trade-off between complexity and performance of interpolation MC.

Tool 1: JCTVC-E134 (TOSHIBA Corporation) [1]

JCTVC-E134 is an interpolation method that has non-uniform tap length filters for computational complexity reduction. In general, filter with asymmetric coefficients requires a larger number of multiplication operations compared to filter with symmetric coefficients supposing that they are the same tap length N. Then, the worst-case number of operations is able to be reduced by using shorter tap length (M<=N) of filter with coefficients in some cases.

Tool 2: JCTVC- D344 (Samsung)[2]

DCT based interpolation filter coefficients can be easily generated for arbitrary number of taps and any precision of filter coefficients. In this test different combination of 6 taps and 8 taps DCTIF will be tested:

- Solid 6 taps DCTIF for all fractional positions and all types of prediction: Single/Bi-directional.

- 6 taps DCTIF for bi-directional prediction and 8 taps DCTIF for uni-directional prediction (similar with JCTVC-E241).

- 6 taps DCTIF for 2D ¼ pel fractional position in bi-directional prediction and 8 taps DCTIF for other cases (similar with JCTVC-E134).

- 6 taps DCTIF for 2D ¼ pel fractional position regardless direction of prediction and 8 taps DCTIF for other cases.

- Solid 8 taps DCTIF for all fractional positions and all types of prediction: Single/Bi-directional.

This set of tests will be performed in order to get clear understanding about performance and complexity ratio of solid filter and multi-filter solution for interpolation MC.

Tool 3: JCTVC- E241 (Nokia) [3]

It is proposed to use a lower tap length filter for interpolation of bi-prediction samples than the interpolation filter used for uni-direction case.

This test will compare the performance and memory bandwidth of this tool with various filter coefficients as described in JCTVC-D344, JCTVC-E358.

Tool 4: JCTVC- E129 (Sony)[4]

In order to reduce complexity for memory bandwidth and computation, MC interpolation with compact support was proposed in this JCTVC-E129. In this CE, we further investigate impacts when DCTIF is always used in only ME to check complexity to check an encoder complexity.

Tool 5: JCTVC- E128 (Sony)[5]

In order to improve quality of prediction at MC, bi/single (BiS) implicitly switching was proposed. For BiS, two pre-defined filter coefficients sets are switched either inter mode is bi-prediction or not. Because, current filter for single-prediction didn’t care P-slice, this experiment tests the BiS interpolation filter including P slice.

Tool 6: JCTVC-E358 (Motorola Mobility)[6]

In order to improve trade-off between coding efficiency and complexity, interpolation filter based on windowed sinc kernels was proposed in JCTVC-E358 [6]. This experiment will study the tradeoffs between the efficiency of fixed interpolation filters based on windowed sinc kernels and the number of taps and precision of the coefficients.

Tool 7: JCTVC-E359 (Motorola Mobility)[7]

This study will study the interaction between the choice of optimal interpolation filters and the temporal prediction parameters (e.g. PU size). More specifically in this experiment we will design sets of interpolation filter based on windowed sinc kernels [7] and use them to study the correlation between the temporal prediction parameters (such as PU size, number of prediction reference blocks, etc.) and the condition of signal and correlated and independent prediction noises. The result of this experiment will be used to propose different filter sets for different MC categories. The MC categorization will also be proposed based on aforementioned temporal prediction parameters.

Tool 8: JCTVC- D376 (Qualcomm)[8]

Qualcomm proposes interpolation filters with different tap sizes, based on the fractional pixel locations as in [8]. The basic idea is to use longer filters for fractional pixel positions that require 1-D filtering and shorter filters for positions that require 2-D filtering.

3.2. Adaptive interpolation filter techniques for HEVC

From several contributions it was observed that adaptive interpolation filters (AIF) for MC are beneficial in terms of performance (especially for LC case). Adaptive filters with reasonable degree of freedom in parameters could minimize the danger of tuning the standard for a set of test sequences. Typically encoder complexity of AIF tools was indicated as problematic aspect of practical application.

Tool 9: JCTVC-E078 (NTT)[9]

This test proposes a region-based adaptive interpolation filter as an extension of AIF.  A frame is segmented into multiple regions, and filter coefficients for each region are derived. The basic idea is that when the frame consists of multiple regions with different characteristics, filter coefficients adapt for each region.

In this experiment following aspects will be studied:

1) further complexity reduction while maintaining coding gain,
2) interdependency with ALF by testing with ALF on in LC and ALF off in HE.

Tool 10: JCTVC- E284 (eBrisk Video Inc.)[10]

This contribution proposes an Adaptive Interpolation Filtering (AIF) technique for interpolation filtering of the luminance samples of video sequences. In the proposed technique, for each video picture, the encoder selects, for each sub-sample position, either one/two HM 2.0 predefined 8-tap filters or corresponding newly-generated 8-tap filters. The encoder transmits up to four 8-tap filters per video picture.

In this experiment, we will study (1) the impact of further encoding complexity reductions on the AIF BD-Rate gains and (2) the interdependency between AIF and ALF by testing AIF on LC+ALF and HE-ALF.

Tool 11: JCTVC- E188 (KDDI)[11]

JCTVC-E188 proposes switching interpolation filter (SIF) approach. In this approach, eight candidates of 8-tap interpolation filter are pre-defined and transmitted to decoder in advance. Encoder selects one filter for each slice, and the selected filter identifier is signaled in slice layer.

In this experiment, we will study (1) the father improving coding performance while maintaining encoding/decoding time, and (2) the interdependency between AIF and ALF by testing with ALF on in LC and ALF off in HE.

4. Recommendation

If more than one filter is used in proposed algorithm than it is recommended that proponents test different variations of de-coupling ME and MC stages.

5. Cross-verification

5.1. Cross-versification activity assignment

| Proponents | Cross-checkers |
| --- | --- |
| Tool 1 (Toshiba) | Sony |
| Tool 2 (Samsung) | 1) Sony 2) Nokia 3) Toshiba 4) Qualcomm 5) Motorola Mobility |
| Tool 3 (Nokia) | Sony |
| Tool 4 (Sony) | Nokia |
| Tool 5 (Sony) | Toshiba |
| Tool 6 (Motorola Mobility) | Samsung |
| Tool 7 (Motorola Mobility) | Samsung |
| Test 8 (Qualcomm) | eBrisk Video Inc |
| Tool 9 (NTT) | KDDI |
| Tool 10 (eBrisk Video Inc) | Motorola Mobility |
| Tool 11 (KDDI) | NTT |

5.2. Cross-checker responsibility.

Cross-checker should

- study s/w provided for cross-check,

- run test under condition described below,

- check the number of operation counting provided by component to match with implementation,

- check visual quality and include informal comment about this into cross-checker report.

# 6. Core experiments test conditions

# 6.1. Software

# HM version 3.0 will be used for testing in this CE.

# 6.2. Test cases

Following test cases mandatory should tested by proponents in this CE.

High coding efficiency anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Low complexity anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Following test cases recommended to be presented in order to show independency of proposed tool and Adaptive Loop Filter (ALF):

High coding efficiency anchor with ALF off:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Low complexity anchor with ALF on:

Random access configuration

Low delay configuration

“P only” (GPB off case)

These optional tests are more relevant for AIF proposals.

Common test condition is specified in JCTVC-E700[12].

# 6.3. Evaluation criteria

- Measure impact on bitrate/PSNR using provided data. Use 4-point BD-PSNR and BD-Rate [13].

- Complexity (encoding/decoding times, and use a common Excel sheet that counts number of operations instruction cycles and memory bandwidth compared to another)

- Subjective visual quality (informal comments).

# 7. Time line

| April, 14, 2011: | Deadline for sending email to coordinator expressing interest in participating in tool experiment. |
| --- | --- |
| April, 15, 2011: | Distribution of description for Core experiment. |
| June, 22, 2011: | Test results representation form announcement and start to cross-check of experimental results and check subjective quality |
| June, 29, 2011: | Complete cross-checking of experimental results and checking subjective quality |
| July, 1, 2011: | Upload contributions |

# 8. References

[1] T.Chujoh, K.Kanou, T.Yamakage (Toshiba), “CE3: Non-uniform tap length filtering for bidirectional prediction,” JCTVC-E134, 5th. JCT-VC Meeting, Geneva, Mar. 2011.

[2] E.Alshina, J.Chen, A.Alshin, N.Shlyakhov, Woo-Jin Han (Samsung), “CE3: Experimental results of DCTIF by Samsung,” JCTVC-D344, 4th. JCT-VC Meeting, Daegu, Jan., 2011.

[3] K. Ugur, J. Lainema (Nokia) “Low complexity bi-predictive interpolation with 6-tap DCT-IF filter”, JCTVC-E241, 5th. JCT-VC Meeting, Geneva, Mar., 2011.

[4] K.Kondo and T.Suzuki (Sony), “CE3: Results on Bi/Single MC interpolation filter” JCTVC-E128,

5th. JCT-VC Meeting, Geneva, Mar. 2011.

[5] K.Kondo and T.Suzuki (Sony), “Memory bandwidth reduction MC filter” JCTVC-E129, 5th. JCT-VC

Meeting, Geneva, Mar. 2011.

[6] J. Lou, K. Minoo, D. Baylon, K. Panusopone and L. Wang (Motorola Mobility) “CE3: Report on Motorola Mobility's interpolation filter for HEVC”, JCTVC-E358, 5th. JCT-VC Meeting, Geneva, Mar., 2011.

[7] J. Lou, K. Minoo, D. Baylon, K. Panusopone and L. Wang (Motorola Mobility) “Motorola Mobility’s Adaptive Interpolation Filter”, JCTVC-E359, 5th. JCT-VC Meeting, Geneva, Mar., 2011.

[8] I.S. Chong, L. Guo and M. Karczewicz, “CE3: Qualcomm’s proposal for luma interpolation filters,” JCTVC-D376, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[9] Shohei Matsuo, Yukihiro Bandoh, Takeshi Ito, Seishi Takamura, Hirohisa Jozawa (NTT), “CE3: Region-based adaptive interpolation filter” JCTVC-E078, 5th. JCTVC, Meeting, Geneva, Mar. 2011

[10] Faouzi Kossentini, Nader Mahdi,Hsan Guermazi, Mohammed Ali Ben Ayed, “An Adaptive Interpolation Filtering Technique” JCTVC-E284, 5th. JCTVC, Meeting, Geneva, Mar. 2011.

[11] T. Yoshino, S. Naito (KDDI), “CE3: Switching interpolation filter scheme” JCTVC-E188, 5th. JCTVC, Meeting, Geneva, Mar. 2011

[12] F. Bossen Common HM test conditions and software reference configurations JCTVC-E700, 5th. JCTVC, Meeting, Geneva, Mar. 2011.

[13] G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves,” VCEG-M33, 13th VCEG Meeting, Austin, USA, April 2001.