| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E704r1 |
| --- | --- |

| Title: | Description of Core Experiment 4: Quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | K.Sato (Sony) M. Budagavi (TI) M. Coban (Qualcomm) H.Aoki (NEC) X.Li (MediaTek) | Email: | Kazushi.Sato@jp.sony.com madhukar@ti.com mcoban@qualcomm.com hao@bu.jp.nec.com x.li@mediatek.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

At 5th JCTVC meeting DeltaQP signaling at Sub-LCU-level has been adopted, and there are some contributions for improvement of better QP prediction. There is also proposal on de-quantization offset, which is suggested to study further in CE.

The goal of this Core Experiment (CE) is to evaluate proposals on

- Spatial granularity at which CU-level to signal deltaQP

- Delta QP prediction from neighboring CU QP values

- De-quantization offset

- Adaptive Quantization Level

and further investigation for improvement by merging the ideas of different proposals and so on.

# Participants

| Participant | Person, Email address | P | C |
| --- | --- | --- | --- |
| Canon | M. Kobayashi (kobayashi.masaaki517@canon.co.jp) M. Shima (shima.masato@canon.co.jp) | X | X |
| ETRI | S.-C. Lim (sclim@etri.re.kr) J. Lee (jinosoul@etri.re.kr) H. Y. Kim (hykim5@etri.re.kr) |  | X |
| HKUST | C. Pang (pcece@ust.hk) O. C. Au (eeau@ust.hk) X. Wen (wxxab@ust.hk) | X |  |
| LGE | J. Jia (jie.jia@lge.com) |  |  |
| MediaTek | X. Li (x.li@mediatek.com) X. Guo (xun.guo@mediatek.com) S. Lei (shawmin.lei@mediatek.com) | X | X |
| Microsoft | J. Xu (jzxu@microsoft.com) |  | X |
| Mitsubishi | K.Sugimoto(Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp) S.Sekiguchi(Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp) |  | X |
| NEC | H. Aoki (hao@bu.jp.nec.com) K.Chono (chono@ct.jp.nec.com) | X | X |
| Qualcomm | M. Coban, (mcoban@qualcomm.com) | X |  |
| RIM | G. Martin-Cocher (gmartincocher@rim.com) X. Yu (xiayu@rim.com) D. He (dhe@rim.com) | X | X |
| Samsung | E. Alshina (elena.alshina@gmail.com) |  | X |
| Sony | K.Sato (Kazushi.Sato@jp.sony.com) A.Cheung (Cheung.Auyeung@am.sony.com) | X | X |
| Texas Instruments | M. Zhou (Zhou@ti.com) M. Budagavi (Madhukar@ti.com) |  | X |
| Zenverge | D. Hoang (Dzung.Hoang@Zenverge.com) |  | X |
| Zhejiang Univ. | X. Shen (babylion@zju.edu.cn) |  | X |

# Tools under Test

## Subtest 1: Spatial granularity at which CU-level to signal deltaQP

### JCTVC-E436: Sub-LCU QP representation (HKUST)

In this test, the signaling of minCUDQPsize at LCU level as described in JCTVC-E436 will be tested. The anchor is the method of signaling of minCUDQPsize at pps level which has four situations: minCUDQPsize = 0, 1, 2, 3. Note that the QP maps used in this test and the anchor will be the same for fair comparison.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 3.1.1 | HKUST : C. Pang (pcece@ust.hk) O. C. Au (eeau@ust.hk) X. Wen (wxxab@ust.hk) | Zenverge: D. Hoang (Dzung.Hoang@Zenverge.com) |

## Subtest 2: Delta QP prediction from neighboring CU QP values

### Subtest 2.1: Delta QP prediction from left CU

In this subtest, following two methods are tested in order to evaluate effectiveness of using left QP as defined in HM3.

- JCTVC-E217 (HKUST) method 1: HKUST Method 1 uses only the previous QP according to decoding order as predictor.

- HM3 anchor: QP of left CU is used as predictor. If left CU is unavailable, the previous QP according to decoding order as predictor.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 2.1.a Reference | HKUST: C. Pang (pcece@ust.hk) O. C. Au (eeau@ust.hk) X. Wen (wxxab@ust.hk) | NEC: H. Aoki (hao@bu.jp.nec.com) |
| 2.1.b | (HM3 anchor) |  |

### Subtest 2.2: Delta QP prediction using depth-dependent previous QPs

In this subtest, following two methods are tested in order to evaluate effectiveness of using depth-dependent QP as described in JCTVC-E198.

- Same as 2.1.a.

- JCTVC-E198 (Canon) method 2: Delta QP is calculated as the difference between the QP of current CU and the QP calculated from QPs of left and top CUs with the same "PredMode" as the current CU. "PredMode" indicates whether CU is inter, intra, or other mode.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 2.2.a = 2.1.a Reference | HKUST: C. Pang (pcece@ust.hk) O. C. Au (eeau@ust.hk) X. Wen (wxxab@ust.hk) | NEC: H. Aoki (hao@bu.jp.nec.com) |
| 2.2.b | Canon : M. Shima (shima.masato@canon.co.jp) | Sony: K.Sato (Kazushi.Sato@jp.sony.com) |

### Subtest 2.3: Delta QP prediction from spatially neighboring CUs

This subtest is planned to evaluate following two aspects.

  - Effectiveness of employing spatially neighboring QPs other than left QP

  - How delta QP should be derived from multiple spatially-neighboring QPs

In this subtest, following seven methods described in JCTVC-D038, JCTVC-E198, JCTVC-E215, JCTVC-E220 and JCTVC-E391 are tested.

- Same as 2.1.b.

- JCTVC-E391 (Qualcomm) method 1: Method 1 uses left and top neighbors QP as a predictor.

- JCTVC-E391 (Qualcomm) method 2: Method 2 uses left and top neighboring CU within a quadtree split as QP predictors.

- JCTVC-E220 (Sony) method 1: Method 1 used median of QPa, QPb and QPc. (QPa, QPb, QPc are QP values for upper, left and upper-left blocks) as

- if (Left and Above and AboveLeft) predQP=Median(Left, Above, AboveLeft)

- else if (Left and Above) predQP=(Left + Above) / 2

- else if (Left and Aboveleft) predQP=(Left + Aboveleft) / 2

- else if (Above and Aboveleft) predQP=(Above + Aboveleft) / 2

- else if (Left) predQP=Left

- else if (Above) predQP=Above

- else if (AboveLeft) predQP=Aboveleft

- else predQP=prevQP

- JCTVC-E220 (Sony) method 2: Method 2 is modification of 2.3.d as

- if (Left and Above) predQP=(Left + Above) / 2

- else if (Left and Aboveleft) predQP=(Left + Aboveleft) / 2

- else if (Above and Aboveleft) predQP=(Above + Aboveleft) / 2

- else if (Left) predQP=Left

- else if (Above) predQP=Above

- else if (AboveLeft) predQP=Aboveleft

- else predQP=prevQP

- JCTVC-E198 (Canon) method 2: Delta QP is calculated as the difference between the QP of current CU and the QP calculated from QPs of left and top CUs with the same "PredMode" as the current CU. "PredMode" indicates whether CU is inter, intra, or other mode.

- JCTVC-E198 (Canon) method 3 and JCTVC-E215 (NEC) method 1: For intra CU, QP of either top or left neighboring CU indicated by direction of intra prediction is employed as predictor. For inter CU, delta QP is calculated as defined in HM 3.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 2.3.a = 2.1.b Reference | (HM3 anchor) |  |
| 2.3.b | Qualcomm: M. Coban (mcoban@qualcomm.com) | LGE: J. Jia (jie.jia@lge.com) |
| 2.3.c | Qualcomm: M. Coban (mcoban@qualcomm.com) | LGE: J. Jia (jie.jia@lge.com) Zenverge: D. Hoang (Dzung.Hoang@Zenverge.com) |
| 2.3.d | Sony: K.Sato (Kazushi.Sato@jp.sony.com) | ETRI: S.-C. Lim (sclim@etri.re.kr) Texas Instruments Madhukar Budagavi (TI) |
| 2.3.e | Sony: K.Sato (Kazushi.Sato@jp.sony.com) | ETRI: S.-C. Lim (sclim@etri.re.kr) Texas Instruments Madhukar Budagavi (TI) |
| 2.3.f | Canon : M. Shima (shima.masato@canon.co.jp) | Sony: K.Sato (Kazushi.Sato@jp.sony.com) |
| 2.3.g | Canon : M. Shima (shima.masato@canon.co.jp) NEC: H. Aoki (hao@bu.jp.nec.com) | Huawei: H. Yang (haitao.yang@huawei.com) |

### Delta QP prediction from QP of temporally neighboring CUs

In this subtest, following two methods are tested in order to evaluate effectiveness of employing temporally neighboring QPs in addition to spatially neighboring QPs.

- Same as 2.3.g.

- JCTVC-E215 (NEC) method 2: For intra CU, delta QP is calculated in the same way as (2.4.a). For inter CU, QP of temporally neighboring CU indicated by motion vector is employed as predictor.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 2.4.a Reference | Canon : M. Shima (shima.masato@canon.co.jp) NEC: H. Aoki (hao@bu.jp.nec.com) | Huawei: H. Yang (haitao.yang@huawei.com) |
| 2.4.b | NEC: H. Aoki (hao@bu.jp.nec.com) | Huawei: H. Yang (haitao.yang@huawei.com) |

## Subtest 3: De-Quantization Offset and Adaptive Quantization Level

### JCTVC-E091: Adaptive De-Quantization Offset (MediaTek)

In this subtest, adaptive de-quantization offset proposed in JCTVC-E091 is to be evaluated. The basic idea of the method is briefly described below.

Let  be the percentage of transformed coefficients which are quantized to -1, 0, or 1, the normalized de-quantization offset  can be derived as follows.

Firstly, the base offsets Y, U, and V components are calculated at slice level according to statistics of previously coded same-type-same-temporal-level frame. Then the offsets are adapted at LCU level. Moreover, the Lagrange multiplier used in RDO process is refined according to the base offsets at slice level.

In addition to the method proposed in JCTVC-E091 (with improvement if possible), the interaction with other tools shall be also tested. In total, there are three test cases as follows.

- JCTVC-E091

In this test case, the performance of JCTVC-E091 is to be tested. Since the method proposed in JCTVC-E091 is actually a combination of adaptive de-quantization offset and de-quantization offset based lambda refinement, the performance of both tools as well as the combination shall be evaluated separately. To independently test the lambda refinement, the de-quantization offset shall be suppressed to zero after the lambda refinement.

- JCTVC-E091 with RDOQ off

In this test case, the performance of JCTVC-E091 is to be tested with RDOQ disabled.

- JCTVC-E091 with AO off.

In this test case, the performance of JCTVC-E091 is to be tested with AO disabled.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 3.3.1.a | MediaTek: X. Li (x.li@mediatek.com) | Qualcomm: M. Karczewicz (martak@qualcomm.com) Samsung: E. Alshina (elena.alshina@gmail.com) Zhejiang Univ.: X. Shen (babylion@zju.edu.cn) |
| 3.3.1.b | MediaTek: X. Li (x.li@mediatek.com) | RIM: D. He (dhe@rim.com) |
| 3.3.1.c | MediaTek: X. Li (x.li@mediatek.com) | Sony: A.Cheung(Cheung.Auyeung@am.sony.com) |

### JCTVC-D384 Adaptive Quantization Level (RIM)

The AQL algorithm is described in JCTVC-D384 as follow:

- Initialize the reconstruction levels.

The reconstruction levels are computed from the data on the encoder side and are selectively signaled to the decoder. Denote the data to be quantized as S. The forward quantization partitions S into S0 , S1, S2, …, Si, … with sizes N0 , N1, N2, …, Ni, … . Denote the i-th reconstruction level as centroid cu=i,  with:

cu=i=1Nin=1Nic(i)n.

- Collect reconstruction levels from the current frame.

Represent the centroid as:

δi=1Nin=1Nic(i)n−i⋅q.

A threshold Ti is defined so that δiis transmitted when   Ni >Ti.

For all subsets corresponding to Ni <=Ti , an average is calculated and used for reconstruction, as

δ=Ni≤Tiδi⋅NiNi≤TiNi.

- Transmitting selected reconstruction levels {δi} and an average level δ.

- Update the reconstruction levels for the next frame based on the transmission. Go the step 2.

It should be noted that the lagrangian multiplier is not modified by this scheme.

The following modifications were added to JCTVC-D384:

AQL corresponds in fact to an adaptive reconstruction level, more than to an adaptive quantization level. It is proposed to call it ARL.

- ARL is applied at the LCU level instead of the frame level.

- ARL is available with and without RDOQ

- ARL results are available for the following LC and HE settings: LD and RA.

In this subset the following tests will be performed:

- Anchor + ARL

- Anchor + ARL without RDOQ

- Anchor+ fixed lambda + QP search per LCU (CU-based multi-QP optimization). This latest test is to compare with a) to evaluate the potential impact of the proposal on the RD balance. The test is to be carried out by setting “MaxDeltaQP” to 2.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 3.3.2.a | RIM: X. Yu (xiayu@rim.com) | TI: M. Budagavi (Madhukar@ti.com) MediaTek: X. Li (x.li@mediatek.com) |
| 3.3.2.b | RIM: X. Yu (xiayu@rim.com) | TI: M. Budagavi (Madhukar@ti.com) MediaTek: X. Li (x.li@mediatek.com) |
| 3.3.2.c | RIM: X. Yu (xiayu@rim.com) | Samsung E. Alshina (elena_a.alshina@samsung.com) MediaTek: X. Li (x.li@mediatek.com) |

# Core Experiment Tasks

The following tasks shall be performed in this CE.

- For subtests 1 and 2, implement tools for evaluation into HM3.0 and will be distributed to CE participants

- For subtests 1 and 2, implement proposed tools into HM3.0 with QP adaptation functionality

- For subtest 3, proposed tools shall be implemented into HM3.0 and distributed among CE participants.

- Improve the tools if possible

- Conduct Test & Cross-Checking

- Report the results

# Time-line and Responsibilities

T1:	2011-Apr-15:	Final CE description and test condition definition

HM3.0 available

T2: 2011-May-13: 	HM3.0 with Sub-LCU-Level dQP coding and QP adaptation functionality available

T3:	2011-Jun-10:	Completion of tool integration, test configuration announcement

Start of cross-check

T4:	2011-Jun-24:	Report cross-checkers’ results to coordinators

T5:	2011-Jul-01:	Input document upload

# Software and Configuration

For subtests 1 and 2, software shall be based on HM-3.0 that with Sub-LCU-Level dQP representation adopted in this mtg and implemented by MediaTek. As of the QP adaptation method, TM5-step3-like algorithm as mentioned in JCTVC-D308 and JCTVC-E215 is used for test. NEC will volunteer for implementation on the software that include sub-LCU-level dQP, and it will be cross-checked by Mitsubishi (MaxCUDepthDQP=0&2) and Sony (MaxCUDepthDQP=1&3) with the timeline as shown in section 5.

For subtest 1 and 2, all the test sequences (Class A, B, C, D and E) shall be tested with all test conditions (Intra_HE, Intra_LC, RandomAccess_HE, RandomAccess_LC, Lowdelay_HE and Lowdelay_LC). BaseQP for each of the frame is same as the one defined in common test conditions (QP=22, 27, 32 and 37 for the first frame) but QP adaptation within a frame is employed.

For subtest 2, following settings shall be used.

- minCUSizeDQP = 3 (8x8) is mandatory.  Other minCUSizeDQP values; i.e. 0 (64x64), 1 (32x32) and/or 2 (16x16) are optional, but encouraged.

- At encoder side, counting cu_qp_delta bits is disabled for RDO-based mode decision in order to purely evaluate coding gains, eliminating side effects like perceptual quality variation.

- Note that an alternative dQP derivation, JCTVC-E217 (HKUST) method 1 is also implemented. This is disabled at default in the provided software but can be enabled by setting REFERENCE_QP_TYPE = 0 in TypeDef.h. (At default, REFERENCE_QP_TYPE is set to 1. It means that the WD3 anchor method is used)

For subtest 3, HM-3.0 with the bug fix of RDOQ off in rev 850, shall be used as the anchor. The related changes to HM-3.0 can be obtained at:

http://hevc.kw.bbc.co.uk/trac/changeset/850/branches/HM-3.0-dev

Four inter configurations (RAHE, RALC, LDHE, and LDLC) defined in JCTVC-E700 shall be tested. For lowdelay configurations, RVM values (VCEG-AM10) shall be provided, Moreover, for the four inter configurations, PSNR and bit allocation of different frame types and temporal levels shall be provided as well.

# Evaluation Criteria

Subtests 1 and 3 are evaluated based on

- BD-rate

- Enc/Dec time

with TM5-like QP adaptation applied both for the anchor and to be tested.

For subtest 2, in addition to above two measures, amount of cu_qp_delta bits is used as supplementary measure to evaluate coding efficiency. Functionality of counting cu_qp_delta bits will be included with the base code used for subtest 2, i.e. HM3.0 with Sub-LCU-Level dQP coding and QP adaptation functionality.

# References

[1] “WD3: Working Draft 3 of High-Efficiency Video Coding”, JCTVC-E603, Mar 2011

[2] “Common test conditions and software reference configurations”, JCTVC-E700, Mar 2011

[3] “HEVC Test Model 3 (HM 3) Encoder Description”, JCTVC-E602, Mar 2011