| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E705 |
| --- | --- |

| Title: | CAVLC Entropy Coding Improvements |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Xianglin Wang | Tel: Email: | +1-858-651-5135 xianglin@qualcomm.com |
| Source: | CE coordinator |  |  |

___________________________

# Participants

Coordinators: 	Xianglin Wang 	xianglin@qualcomm.com

Ping Wu	ping.wu@zte.com.cn

| Proponent/Partner | Contact | Email |
| --- | --- | --- |
| Sharp | Yukinobu Yasugi Tomoyuki Yamamoto | yasugi.yukinobu@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp |
| ZTE | Ping Wu Ming Li Wen Zhang | pwuzte@gmail.com li.ming42@zte.com.cn zhang.wen4@zte.com.cn |
| Panasonic | Virginie Drugeon | Virginie.Drugeon@eu.panasonic.com |
| Yonsei Univ. | Seongwan Kim Jaeho Lee Sangyoun Lee | knauer@yonsei.ac.kr jhlee82@yonsei.ac.kr syleee@yonsei.ac.kr |
| LG Electronics | Jaehyun Lim Jie Jia | jaehyun.lim@lge.com jie.jia@lge.com |
| Qualcomm | Liwei Guo Xianglin Wang | liweig@qualcomm.com xianglin@qualcomm.com |
| Cisco | Arild Fuldseth | arilfuld@cisco.com |
| Nokia | Kemal Ugur Jani Lainema | kemal.ugur@nokia.com jani.lainema@nokia.com |
| Microsoft | Jizheng Xu | jzxu@microsoft.com |
| MediaTek | Yu-Wen Huang Shan Liu Ximin Zhang | yuwen.huang@mediatek.com Shan.Liu@mediatek.com ximin.zhang@mediatek.com |
| HHI | Tung Nguyen | tung.nguyen@hhi.fraunhofer.de |
| TI | Vivienne Sze | sze@ti.com |
| Zhejiang Univ | Xingguo Zhu | 05xxgcsgx@zju.edu.cn |
| NTT Docomo | TK Tan | ttk@pacific.net.sg |
| Sony | Munsi Haque Jun Xu | munsi.haque@am.sony.com Jun.Xu@am.sony.com |
| Samsung | Jianle Chen | jianle.chen@samsung.com |
| NEC | Keiichi Chono | chono@ct.jp.nec.com |

# Related proposals

In JCTVC-E156, a run mode bypassing scheme was proposed in which run mode is skipped in case level of last coefficient is large in intra TU. It is asserted complexity can be reduced by the scheme.

In JCTVC-E446, it was proposed to use separate tables in run-level coding for large block (i.e. 16x16 or larger) that’s truncated to 8x8. This is applied to both intra and inter blocks.

In JCTVC-E258, modifications were proposed for inter mode and split flags coding in CAVLC. In inter mode coding, the table to code symbol inter_partition is both CU depth and context dependent. The contexts defined for CABAC are used. Furthermore, a joint coding of the split flags for I frames was proposed. A sequence of four split flags, corresponding to the split flags of four CUs of the same depth in Zscan order, is coded using a codeword that depends on a context.

In JCTVC-E072, adaptive and joint coding of the syntax indicating prediction information was proposed. Based on the scheme, some new syntax elements are introduced in slice header to specify the coding order of the prediction related syntax in current slice. The value of new syntax elements are determined based on the estimated statistical features of the previous coded slices.

In JCTVC-E160, joint coding of inter prediction mode is limited to prediction mod. Partitioning types are signaled separately. Additionally, initial mapping tables between symbol and codeword index are rearranged to reflect the statistical distribution of each prediction mode.

# Functionality addressed

The core experiment will test coding efficiency improvement in CAVLC on the various syntaxes listed section 2, which includes the following sub-topics.

- Coefficient coding based on JCTVC-E156 and JCTVC-E446.

- Inter mode coding based on In JCTVC-E072, JCTVC-E160 and JCTVC-E258.

- Joint coding of CU split flags for I frame based on JCTVC-E258.

# Core experiment test conditions

- Software

Use HM 3.0, which will be available approximately three weeks after Geneva meeting, as base software.

- Coding conditions

The same as that specified in JCTVC-E700 using low complexity settings.

- Test set

- The same as that specified in JCTVC-E700.

- Evaluation criteria

Report coding efficiency improvement relative to reference software, evaluate coding complexity.

- Cross check

| Proposals | Cross-checkers |
| --- | --- |
| JCTVC-E156 (Sharp) | Yonsei Univ. |
| JCTVC-E446(Yonsei Univ.) | Sharp |
| JCTVC-E072 (ZTE) | LGE |
| JCTVC-E258 (Panasonic) | ZTE, NEC |
| JCTVC-E160(LGE) | Panasonic |

# Timelines

Apr. 8, 2011: Deadline for sending email to coordinator expressing interest in participating in core experiment.

Apr. 8, 2011: Final CE description.

Jun. 23, 2011: Software available for cross-verification.

Jul. 1, 2011: Report the verification results.

# References

- Y. Yasugi, T Yamamoto (SHARP) “Simplification of transform coefficient coding in LCEC”, JCTVC-E156, Geneva, Switzerland, Mar. 2011.

- Virginie Drugeon, Thomas Wedi, Matthias Narroschke (Panasonic), “Improvement of inter mode coding and split flags coding for LCEC”, JCTVC-E258, Geneva, Switzerland, Mar. 2011.

- Seongwan Kim, Jaeho Lee, Sangyoun Lee (Yonsei Univ.), “Coefficient coding table improvement in LCEC”, JCTVC-E446, Geneva, Switzerland, Mar. 2011.

- J. Lim, B. Jeon (LGE), “CE5: Modified Joint Coding on Prediction Mode Signaling”, JCTVC-E160, Geneva, Switzerland, Mar. 2011.

- Wen Zhang, Ming Li, Ping Wu (ZTE), “CE5: Adaptive Coding of InterPredMode Syntax Elements”, JCTVC-E072, Geneva, Switzerland, Mar. 2011.