| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E707_r4_draft WG11 Number: m20244 |
| --- | --- |

| Title: | Description of Core Experiment 7: Additional Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories Chuohao Yeo Institute for Infocomm Research Rajan Joshi Qualcomm Felix Fernandes Samsung Electronics Co., Ltd. | Email: | cohen@merl.com chyeo@i2r.a-star.edu.sg rajanj@qualcomm.com felix.f@sta.samsung.com |
| Source: | Mitsubishi Electric Research Laboratories, Institute for Infocomm Research, Qualcomm, Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document defines Core Experiment 7 (CE7) on Additional Transforms, to be performed for the upcoming July 2011 JCT-VC meeting. During the 5th JCT-VC meeting in March, 2011, the 4x4 mode-dependent DCT/DST from JCTVC-E125 in Core Experiment 7 was adopted. The remaining transforms such as the 8x8 mode-dependent DCT/DST and the 4x4 mode-dependent secondary transform were recommended for further study. The purpose of this CE is to examine further improvements to these transforms and to determine if their architectures are compatible with the HM encoder and decoder.

# Introduction

For the March 2011 JCT-VC meeting in Geneva, Core Experiment 7 (CE7) [1] investigated the performance of several proposed transforms for coding Intra, and in one case, Inter blocks. In addition to being reviewed during the regular JCT-VC sessions, details were discussed in a breakout group (BoG) [2]. The resulting recommendations were:

- The BoG consented to refer to JCTVC-E125 [3] and JCTVC-E098 [4] as JCTVC-E125, due to their similarity. The differences between these contributions are in some coefficients of the fast 4x4 transform and four entries in the mode-to-transform mapping table.

- The 4x4 mode-dependent DCT/DST of JCTVC-E125 was proposed for adoption by the proponents.

- The proponents recommended that JCTVC-E074 [5] and the 8x8 transforms of JCTVC-E125 be further studied in a CE.

JCT-VC agreed to adopt JCTVC-E125 for 4x4 blocks only, subject to text review. The purpose of this CE is to examine the additional transforms described in Section 3. This CE will also determine if their architectures are compatible with the rest of HM encoder and decoder.

# Participant list

| Name | Affiliation | Email |
| --- | --- | --- |
| Robert Cohen (coordinator) | Mitsubishi Electric Research Labs | cohen@merl.com |
| Chuohao Yeo (coordinator) | Institute for Infocomm Research | chyeo@i2r.a-star.edu.sg |
| Rajan Joshi (coordinator) | Qualcomm | rajanj@qualcomm.com |
| Felix Fernandes (coordinator) | Samsung | felix.f@sta.samsung.com |
| Akiyuki Tanizawa | Toshiba | akiyuki.tanizawa@toshiba.co.jp |
| Jun Yamaguchi | Toshiba | jun2.yamaguchi@toshiba.co.jp |
| Elena Alshina | Samsung | elena_a.alshina@samsung.com |
| Ankur Saxena | Samsung | asaxena@sta.samsung.com |
| Il-Koo Kim | Samsung | ilkoo.kim@samsung.com |
| Youji Shibahara | Panasonic | shibahara.youji@jp.panasonic.com |
| Takahiro Nishi | Panasonic | nishi.takahiro@jp.panasonic.com |
| Atsuro Ichigaya | NHK | ichigaya.a-go@nhk.or.jp |
| Shinichi Sakaida | NHK | sakaida.s-gq@nhk.or.jp |
| Haitao Yang | Huawei | haitao.yang@huawei.com |
| Tung Nguyen | HHI | tung.nguyen@hhi.fraunhofer.de |
| Madhukar Budagavi | TI | madhukar@ti.com |
| TK Tan | NTT DOCOMO | ttk@pacific.net.sg |
| Kiran Misra | Sharp Labs of America | misrak@sharplabs.com |

# Description of tools

The following tools shall be evaluated for CE 7. Full details can be found in the related documents. The “I” column indicates that the transform operates on Intra blocks, and “B” corresponds to Inter blocks.

| Tool # | Proponent(s) | Related Docs. | I | B | Tool Description | Cross-checkers |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Qualcomm Toshiba I2R | JCTVC-E098 JCTVC-D392 JCTVC-D046 |  |  | For 8x8 blocks, this 2-D separable transform applies either the HM DCT or an odd type-III (type-VII) DST vertically and horizontally. The choice of transform to use in each direction is determined by the Intra prediction mode. | 1. Samsung |
| 2 | Samsung | JCTVC-E125 JCTVC-D033 |  |  | For 8x8 and larger blocks, this 2-D separable transform applies either the HM DCT or DST vertically and horizontally. The choice of transform is dependent on the intra prediction mode. | 1. I2R |
| 3 | Panasonic Samsung | JCTVC-E074 JCTVC-E380 |  |  | A mode-dependent secondary transform operates on the low-frequency coefficients output by the HM core transform. Various secondary transforms will be evaluated, including a 4x4 pseudo-DST and 4x4/8x8 ROT. | 1. Huawei 2. NHK 3. Qualcomm |
| 4. | Samsung | JCTVC-E380 |  |  | The HM transform is applied first, followed by a secondary rotational transform [6] applied separably to vertical and horizontal dimensions. The secondary transform will be tested for various block-size combinations. | 1. Qualcomm |
| 5. | NHK | JCTVC-E107 |  |  | A Rate-distortion optimized selection of HM DCT or type-II DST is used. The butterfly operations of the horizontal DCT and DST transforms are merged so that the DCT and DST coefficients are output simultaneously. | 1. Panasonic 2. MERL |
| 6. | Samsung | JCTVC-E125 |  |  | A modified version of the 4x4 DST adopted from JCTVC-E125 is used. The DST is scaled to match the scaling of the JCTVC-E243 DCT, and some coefficients are modified to reduce the number of arithmetic operations. | 1. Toshiba |

# Core Experiment tasks

The objectives for this Core Experiment are to:

- Specify the software platform, common software configurations and common test conditions for these experiments;

- Evaluate compression efficiency and complexity of the additional transforms studied in this CE;

- Provide informal comments regarding subjective differences, if any.

- Investigate interactions between the additional transforms and coefficient scanning schemes;

- Investigate the compatibility of the additional transforms with the HM encoder and decoder.

- Report the results and conclusions of these experiments.

# Test conditions

The following test conditions shall be used for this CE:

## Software

HM version 3.0 shall be used for these experiments, as specified in JCTVC-E700 [7].

## Test configurations

This CE shall use the same encoder and decoder configurations as described in JCTVC-E700. Experiments shall be run for the Intra High-Efficiency, Intra Low-Complexity, Random Access High-Efficiency, and Random Access Low-Complexity coding configurations. Additional transforms that operate on Inter slices shall also use the Low-Delay configurations.

## Test sequences

The test sequences as described in JCTVC-E700 shall be used for these tests.

## Training of transforms (if applicable)

The test sequences used from Section 5.3 shall not be used for training any proposed additional transforms. Proponents shall report which sequences they use for training of their transforms, if applicable.

# Evaluation Criteria

This section contains details about how the transforms shall be evaluated. If a proposed transform includes scan orders not already included in HM, simulation results at a minimum shall be reported for the additional transform without these additional scan orders. Results with the additional scan orders may be reported as well. This is done to help JCT-VC determine the benefits of these additional scans. Proponents are welcome to include additional results at their discretion.

## Compression efficiency

To obtain objective rate-distortion measurements, four point BD-Rate and BD-PSNR metrics [8] shall be used to evaluate coding efficiency. Rates and PSNR values obtained for all test points shall be provided as well. Compression efficiency and encoder/decoder run time results shall also be reported with the common spreadsheet template that is being used for most JCT-VC contributions. The references for these computations shall be generated from the unmodified HM version 3.0 software.

## Complexity evaluation

### Tool description

In the JCT-VC text document associated with each proposed tool, the abstract shall include a brief text description of the proposed additional transform, along with a brief summary of the compression performance and complexity evaluation.

### Additional transform description

The additional transforms for each transform size shall be described in such a way that makes it clear what operations are being performed. Pseudo-code or data-flow diagrams are preferred.

### Arithmetic operations

For each size of each inverse transform, the worst-case number of arithmetic operations per block (multiplies, adds, and shifts) shall be reported. These counts shall be based upon the pseudo-code or data-flow diagrams. The operations of the final scaling performed during the inverse transform process shall also be included in the operations count. Data for the JCTVC-E243 partial butterfly implementation are provided in Table 1 for reference. Note that the number of additions reported includes the offset before the shift. The correct number of shifts is also shown.

The following table format shall be used. The contents of the shaded areas are fixed, and the non-shaded areas shall contain data for each of the proposed transforms. Additional columns may be added as needed. If the forward and inverse transforms do not have similar operation counts, forward transform data shall also be reported in a separate column. For secondary transforms applied in addition to the core HM transform, the operation counts of the secondary transform shall be preceded by a “+” sign. Example entries for additional transforms are shown in the last two columns of Table 1.

Table 1. Operation count examples

| Transform size |  | HM 2 core 2-D transform | JCTVC-E243 partial butterfly implementation of 2-D inverse DCT | Example transform that replaces the core HM 2-D transform | Example secondary transform applied after 8x8 and larger HM 2-D transforms. For 4x4, a replacement transform is used |
| --- | --- | --- | --- | --- | --- |
| 4x4 | Mults | 0 | 48 | 72 | 128 |
|  | Adds | 80 | 96 | 160 | 144 |
|  | Shifts | 32 | 32 | 80 | 32 |
| 8x8 | Mults | 0 | 352 | 1024 | +128 |
|  | Adds | 640 | 576 | 1024 | +128 |
|  | Shifts | 352 | 128 | 352 | +16 |
| 16x16 | Mults | 1408 | 2752 |  | +128 |
|  | Adds | 2624 | 3712 |  | +128 |
|  | Shifts | 1600 | 512 |  | +16 |
| 32x32 | Mults | 7424 | 21888 |  | +128 |
|  | Adds | 13440 | 25856 |  | +128 |
|  | Shifts | 7296 | 2048 |  | +16 |

### Bit precision

Report the maximum bit precision needed for each transform size.

### Memory requirements

For each transform size, report any additional memory requirements in bytes.

### Quantization

Report the presence and number of additional quantization matrices, for each transform size.

Any additional bit-depth required by use of these additional quantization matrices shall also be reported.

### R-D optimization

If any additional rate-distortion loops or iterations are being performed in the encoder or decoder, they shall be described.

### Encoder/Decoder run times

Encoder and decoder run times shall be reported using the common spreadsheet as described in Section 6.1.

### Other statistics

If present, memory transfers, reordering, and other data operations not included above shall be reported.

## Subjective effects

If sequences coded using a proposed additional transform exhibit subjective improvements or degradations, proponents and/or cross-verifiers shall describe these subjective differences. These descriptions are to be considered as informal comments, as formal subjective testing is not required by CE7 at this time.

# Timeline

April 6, 2011: Final CE7 description uploaded

April 15, 2011: HM 3.0 software expected to be available

June 20, 2011: Cross-verification begins by this date

June 29, 2011: Proponents report results to CE7 coordinators

July 1, 2011: Input documents uploaded

# References

- R. Cohen, C. Yeo, R. Joshi, F. Fernandes, “CE7: Summary Report of Core Experiment on Alternative Transforms,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E027, 5th JCT-VC Meeting, Geneva, CH, Mar. 2011.

- R. Cohen, “BoG Report on Transforms,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E480, 5th JCT-VC Meeting, Geneva, CH, Mar. 2011.

- A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E125, 5th JCT-VC Meeting, Geneva, CH,  Mar. 2011.

- R. Joshi , P. Chen , M. Karczewicz, A. Tanizawa , J. Yamaguchi, C. Yeo , Y. H. Tan, H. Yang , H. Yu,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E098, 5th JCT-VC Meeting, Geneva, CH,  Mar. 2011.

- Y. Shibahara and  T. Nishi, “CE7: Mode Dependent 2-step Transform for Intra Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E074, 5th JCT-VC Meeting, Geneva, CH,  Mar. 2011.

- E. Alshina, A. Alshin, F. Fernandes, A. Saxena, V. Seregin ,  Z. Ma , W.-J. Han, “CE7: Experimental results of ROT by Samsung,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E380, 5th JCT-VC Meeting, Geneva, CH,  Mar. 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E700, 5th JCT-VC Meeting, Geneva, CH, March 2011.

- G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SG16/Q6, VCEG-M33, 13th VCEG Meeting, Austin, USA, April 2001.