| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E710r5 WG11 Document: m20234 |
| --- | --- |

| Title: | CE10: Core transform design |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala FastVDO LLC Madhukar Budagavi Texas Instruments Arild Fuldseth Cisco Rajan Joshi Qualcomm Il-Koo Kim Samsung Electronics | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-443-455-0394 pankajtva@yahoo.com madhukar@ti.com arilfuld@cisco.com rajanj@qualcomm.com ilkoo.kim@samsung.com |
| Source: | FastVDO LLC, Texas Instruments, Cisco, Qualcomm, Samsung Electronics |  |  |

_____________________________

# Objective

This CE shall investigate the design of core spatial transforms, and study the tradeoff between coding efficiency and complexity, return evidence and a recommendation for design selection. The anchor for comparison shall be the transforms in the HM. Proposals that use dynamic range control methods (such as clipping) should incorporate them into the complexity analyses below.

# Participant list

| Name |  | Affiliation |  | Email |
| --- | --- | --- | --- | --- |
| Pankaj Topiwala |  | FastVDO LLC |  | pankajtva@yahoo.com |
| Madhukar Budagavi Mangesh Sadafale |  | Texas Instruments Texas Instruments |  | madhukar@ti.com mangesh@ti.com |
| Arild Fuldseth |  | Cisco |  | arilfuld@cisco.com |
| Rajan Joshi |  | Qualcomm |  | rajanj@qualcomm.com |
| Il-Koo Kim Jianle Chan |  | Samsung Electronics |  | ilkoo.kim@samsung.com jianle.chen@samsung.com |
| Dzung Hoang |  | Zenverge |  | Dzung.Hoang@Zenverge.com |
| David Flynn |  | BBC |  | davidf@rd.bbc.co.uk |
| Louie Kerofsky |  | Sharp |  | lkerofsky@sharplabs.com |
| Jian Lou |  | Motorola |  | lou@motorola.com |
| Robert Cohen |  | Mitsubishi Electric Research Laboratories |  | cohen@merl.com |
| Tung Nguyen |  | HHI |  | tung.nguyen@hhi.fraunhofer.de |
| Ankur Saxena |  | Samsung Telecommunications America |  | asaxena@sta.samsung.com |
|  | TK Tan |  | NTT DOCOMO, Inc. | tktan.p@rd.nttdocomo.co.jp |

# Description of tools

Transform design for HEVC with 16 bit intermediate data representation (JCTVC-E243, A. Fuldseth, G. Bjøntegaard (Cisco), M. Sadafale, M. Budagavi (TI))

This contribution proposes a set of transform matrices for HEVC, covering all transform sizes from 4x4 to 32x32. The proposed transform design is reported to have the following properties: 16 bit data representation before and after each transform stage (applies to both IBDI off and IBDI on), 16 bit multipliers for all internal multiplications, no need for correction of different norms of basis vectors during quantization/dequantization, all transform sizes above 4x4 can reuse arithmetic operations for smaller transform sizes, and implementations using either pure matrix multiplication or a combination of matrix multiplication and butterfly structures are possible. This experiment will evaluate the coding performance and complexity of the transform.

Integer transform based on modified Loeffler’s factorization (JCTVC-E277, I.-K. Kim, T. Lee, J. Chen, V. Seregin, Y. Hong, W.-J. Han (Samsung), P. Topiwala (FastVDO))

This contribution proposes transform based on modified Loeffler’s factorization without lifting scheme. The main features of the proposed transform are reported to include, a) it can be implemented with both factorized butterfly form or matrix multiplication form; b) 4x4, 8x8 and 16x16 transforms are embedded in 32x32 transform. c) input signal and intermediate transpose buffer for inverse transform are represented within 16 bit; d) six scalar values are used to replace de-quantization scaling matrix for all transform size. The proposed transform is reported to support two different forms of implementation with the same performance so that the optimal implementation can be chosen depending on which platform will be used. This experiment will evaluate the coding performance and complexity of the transform.

Core Transforms, Proposal 1 (JCTVC-E353, Pankaj Topiwala (FastVDO), Wei Dai (FastVDO), Ilkoo Kim (Samsung))

This contribution proposes integer transforms and several variants, which are reported to offer substantial computational savings with effectively no performance loss.  The current 4-pt and 8-pt transforms are reported to be initially retained, while the 16-pt and 32-pt transforms are reported to be systematically simplified, even including multiplication-free designs. The multiplication-free designs are reported to afford the lowest complexity as well as exact invertibility in limited bitdepth arithmetic, yet have identical performance.  This experiment will evaluate the coding performance and complexity of the transform.

Scaled orthogonal integer transforms supporting recursive factorization structure (JCTVC-E370, R. Joshi, Y. Reznik, J. Sole, M. Karczewicz (Qualcomm))

In JCTVC-E370, scaled orthogonal integer transforms were proposed for all block sizes from 4×4 to 32×32 The 16 and 32-point transforms are reported to use 36 and 92 multiplications, respectively. A multiplication-free implementation for the proposed transforms is also reported. This experiment will evaluate the coding performance and complexity of the proposed transforms. Clipping and right shifts to reduce the dynamic range at intermediate stages of the proposed transforms will also be explored..

# Core Experiment Tasks

# Proposals and Cross-Checks

B.1. Coding efficiency

B.1.1. Measure the RD performance of candidate transform designs using common conditions E700 and HM 3.0 software. If a transform design uses clipping etc. on intermediate variables, this clipping should be included in coding efficiency tests.

B.1.2. Also test at low and high QP values.   Provide BD-Rate results for the following QP sets: 1, 5, 9, 13 and 36, 42, 47, 51 to see if there is unusual behavior at these uncommon test points.  Run this test on 2 sec of video to speed up this test. Number of frames to encode is provided in  table 1 below.

B.1.3 (Optional). Also, report BD-rate performance using “12 bit input” (InternalBitDepth=12) with high efficiency settings. Provide BD-rate results for the low-, normal-, and high QP ranges described in B1.1 and B1.2.

Table 1. Number of test frames for B.1.2.

| Sequence name | No. frames to encode |
| --- | --- |
| Traffic | 65 |
| PeopleOnStreet | 65 |
| Kimono | 49 |
| Nebuta | 129 |
| SteamLocomotive | 129 |
| ParkScene | 49 |
| Cactus | 97 |
| BasketballDrive | 97 |
| BQTerrace | 129 |
| BasketballDrill | 97 |
| BQMall | 129 |
| PartyScene | 97 |
| RaceHorses (C) | 65 |
| BasketballPass | 97 |
| BQSquare | 129 |
| BlowingBubbles | 97 |
| RaceHorses (D) | 65 |
| Vidyo1 | 129 |
| Vidyo3 | 129 |
| Vidyo4 | 129 |

| Proponent | Cross-Check Normal Qp | Cross-Check Low/High Qp |
| --- | --- | --- |
| Cisco/TI (E243) | Qualcomm | Samsung/FastVDO |
| Samsung/FastVDO butterfly (E277) | Motorola | Zenverge ¦ Sharp |
| FastVDO/Samsung Lifting (E353) | Cisco/TI | Qualcomm |
| Qualcomm (E370) | Samsung/FastVDO | Cisco/TI |

B.2. Complexity

Analyze the complexity of candidate transform designs using the following parameters:

- Arithmetic operations

- Software execution speed

- Dynamic range

- Parallelizability on SIMD

- Number of sequential operations

- Quantization/dequantization matrices (optional)

- HW

Proponents are encouraged to provide additional complexity analyses voluntarily.

B.2.1. Arithmetic Operations

Report the number of arithmetic operations needed for implementing the 2D inverse transform, in terms of multiplications, additions, shifts, compares. Optionally, multiplications using 16-bit and 32-bit multipliers can be counted separately. If the design allows flexibility in the way it is implemented, report the range of arithmetic operations available (for example, methods with and without multiplications).  Include in the count TPE and final rounding if used.

In addition, report the number of arithmetic operations for IDCT pruning for the following cases (See JCTVC-E386 for definition of L4, L8, L16):

16-pt: L8, L4.

32-pt: L16, L8, L4.

[Paper study]

B.2.2. Parallelizability on SIMD

Produce optimized versions of following transforms:

- HM3,

- Samsung/FastVDO-loeffler,

- FastVDO/Samsung-lifting,

- Qualcomm

- Cisco/TI

Using SIMD targeted at the Intel Nehalem instruction set architecture.

Produce run time comparisons of all proposals.

Produce code size comparisons of all proposals.

Volunteers: David Flynn (BBC) volunteers to conduct this test - which is greatly appreciated by the group.  Recommend that this be done in a common environment so anyone can test, crosscheck.

B.2.3. Dynamic Range Analysis

B.2.3.1. (Optional) Report if the candidate transform design is exactly invertible in fixed-point arithmetic, and if so, at what bitdepth.    [Paper study]

B.2.3.2. Test the dynamic range of the data at four points: prior to dequantization, prior to inverse transform, after first-stage inverse transform, and after second-stage inverse transform using common test framework.  Example input may be data consisting of +/- 255, either random, or Hadamard-like.

B.2.4. Number of sequential operations

Report the number of cascaded operators (multipliers/adders) in the longest execution path.  [Paper study]

B.2.5. Quantization/dequantization matrices [Optional]

Report the size of quantization matrices used to obtain the BD-rate results reported in section B.1.

Report the size of dequantization matrices proposed for normative decoding and used to obtain the BD-rate results reported in section B.1

B.2.6. HW [Optional]

B.2.6.1. Develop optimized hardware models (RTL) of candidate transform designs, and determine the silicon area required, where optimization is for a common hardware condition.

B.2.6.2. Determine the hardware speed.  From area and speed, power consumption can be estimated. This subtask is intended to be a paper study, given B.2.6.1.

Samsung and TI plan to do RTL implementation and verification of the proposals.

Conditions in consideration are as follows.

- Use case of 4Kx2K (4096x2048) 60 fps.

- N transform coefficients/second throughput for each transform size (N TBD).

- Frequency sweep (i.e. total area v/s freq) will be presented. Frequency range TBD.

- 45nm LP process.

# Timeline

2011-Apr-15: 	HM 3.0 made available

2011-Apr-15:	Final CE description and upload

2011-June-17:	Completion of tool integration, release of software and results for cross-check

2011-July–01:	Complete cross-check, input document upload