| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F004 WG11 Number: m21173 |
| --- | --- |

| Title: | JCT-VC AHG report: Slice support development and characterization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg KI/EAB/TVV/V 164 80 Stockholm Sweden Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA Kimihiko Kazui 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki Japan Michael Horowitz eBrisk Video, Inc. 1055 W Hastings St, Suite 1450 Vancouver, BC V6E 2E9, Canada Andrew Segall Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607 U.S.A. | Tel: Email: Tel: Email: Tel: Email: | +46761153563 rickard sjoberg@ericsson com +1.778.331.3432 michael@ebriskvideo.com +1 360 817 8487 asegall@sharplabs.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This report summarizes the slice support development and characterization ad-hoc activities between the 5th and the 6th JCT-VC meeting and the input documents to this meeting related to this ad-hoc group.

# Mandate

Table 1	- AHG mandates

| AHG Title and Email Reflector | Chairs |
| --- | --- |
| Slice support development and characterization (jct-vc@lists.rwth-aachen.de) - Identify and work to resolve issues relating to the draft text description of slice functionality and the associated reference software functionality - Study technical proposals relating to slice structured coding - Study the coding efficiency and loss resilience impact of slice-structured coding - Identify and discuss additional issues relating to slice-structured coding | R. Sjöberg (chair), Y. Chen, M. Horowitz, K. Kazui, A. Segall (vice chairs) |

# Working draft and source code issues

Working draft and source code issues are described in the sections that follow.  The labels “[SW]” and “[WD]” are used to indicate that an issue exists in the source code and working draft respectively. Working draft version E603_d8 has been compared with the HM-3.3 source code.

## HM encoder settings

Issues:

- Decide what POC type to use for our coding conditions [SW]
e.g. type 0 for RA and type 2 for AI and LD

- Decide how many bits to use for frame_num and POC in slice headers [SW]
Currently frame_num is not sent and 10 bits is used for POC

- Decide what value to use for pic_init_qp_minus26 in the PPS [SW]
e.g. same as the config file QP

Tasks:

- Take decisions on these three issues and implement them [SW]

## Order of slice header syntax elements

Issues:

- Slice header syntax elements in HM is not in the same order as in WD [SW]

Tasks:

- Put slice syntax element in the order as specified in WD [SW]

## Picture parameter set

Issues:

- pic_parameter_set_id is not present [SW]

Tasks:

- Add pic_parameter_set_id=0 to PPS [SW]

- Add pic_parameter_set_id=0 to slice header [SW]

## frame_num

Issues:

- frame_num is not present [SW]

Tasks:

- Add log2_max_frame_num_minus4=4 to SPS (gives 8 bit frame_num) [SW]

- Add frame_num to slice header using log2_max_frame_num_minus4+4 bits [SW]
Encoder: use wrap-around, increment by 1 relative previous reference picture in coding order
Decoder: Parse log2_max_frame_num_minus4+4 bits for frame_num in slice header

## idr_pic_id

Issues:

- idr_pic_id is not present [SW]

Tasks:

- Add idr_pic_id=0 to the slice header for IDR pictures [SW]

## POC

Issues:

- POC type 1 is not present [WD]

- POC types are not signaled [SW]

- Source code always uses 10-bits for POC [SW]

Tasks:

- Add description of POC type 1 [WD]

- Add pic_order_cnt_type=0 to SPS [SW]

- Add log2_max_pic_order_cnt_lsb_minus4=4 to SPS (gives 8 bit POC) [SW]

- Make source code use value of log2_max_pic_order_cnt_lsb_minus4 and write/parse the correct number of bits [SW]

## Reference picture list modification

Issues:

- Reference picture list modification syntax elements are not present [SW]

Tasks:

- Add ref_pic_list_modification_flag_l0=0 to slice header [SW]

- Add ref_pic_list_modification_flag_l1=0 to slice header [SW]

## Reference picture marking

Issues:

- Decoded reference picture marking syntax not present [SW]

- A reference picture flag is used instead of using nal_ref_idc=0 [SW]

Tasks:

- Add no_output_of_prior_pics_flag=0 to slice header [SW]

- Add long_term_reference_flag=0 to slice header [SW]

- Add adaptive_ref_pic_marking_mode_flag=0 to slice header [SW]

- Remove reference picture flag from slice header and use nal_ref_idc instead [SW]

## Entropy coding

Issues:

- cabac_init_idc missing [SW]

- symbol mode signaled in slice header [SW]

Tasks:

- Add cabac_init_idc=0 to slice header [SW]

- Move symbol mode signaling from slice header to PPS [SW]

## Quantizer

Issues:

- pic_init_qp_minus26 is missing [SW]

- slice_qp_delta is missing [SW]

Tasks:

- Add pic_init_qp_minus26=<QP from config file> to PPS [SW]

- Add slice_qp_delta to slice header [SW]

## Things yet to be checked

- correct coding of alf_param() and sao_param()

- deblocking filter control syntax

- DRBFlag and ERBIndex

- ColDirFlag

# Coding efficiency of slices

Table 2 shows the BDR performance of the HM-3.3rc1 source code with and without 1500 byte slices at a slice granularity of 64x64. As shown in Table 3, the slice overhead with HM-3.3rc1 is lower compared to the slice results based on HM 2.0 (see JCTVC-E004.doc).

Table 2	- Performance of 1500 byte slices (HM-3.3) compared to an anchor without slices (HM 3.3)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 4.0 | 6.5 | 4.2 | 3.4 | 8.1 | 6.7 |
| Class B | 4.9 | 4.3 | 3.0 | 4.6 | 6.2 | 5.9 |
| Class C | 4.9 | 4.5 | 4.4 | 5.1 | 6.0 | 6.1 |
| Class D | 3.2 | 3.1 | 3.3 | 3.7 | 3.7 | 3.8 |
| Class E | 7.3 | 4.9 | 5.4 | 7.0 | 11.6 | 10.2 |
| Overall | 4.7 | 4.6 | 3.9 | 4.6 | 6.9 | 6.3 |
| Enc Time[%] | 118% |  |  | 132% |  |  |
| Dec Time[%] | 103% |  |  | 109% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 5.6 | 12.1 | 9.3 | 3.7 | 9.7 | 8.5 |
| Class B | 4.5 | 6.1 | 4.2 | 4.0 | 5.9 | 5.2 |
| Class C | 3.8 | 5.1 | 4.7 | 3.7 | 4.5 | 4.4 |
| Class D | 2.5 | 3.3 | 3.2 | 2.3 | 2.7 | 2.8 |
| Class E |  |  |  |  |  |  |
| Overall | 4.1 | 6.6 | 5.3 | 3.4 | 5.7 | 5.2 |
| Enc Time[%] | 105% |  |  | 113% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 3.5 | 6.3 | 4.6 | 2.6 | 3.6 | 3.3 |
| Class C | 2.5 | 4.2 | 4.2 | 2.2 | 2.6 | 2.6 |
| Class D | 1.8 | 2.4 | 3.1 | 1.5 | 1.8 | 1.9 |
| Class E | 1.8 | 2.1 | 0.5 | 1.1 | 5.3 | 2.5 |
| Overall | 2.5 | 4.0 | 3.4 | 2.0 | 3.2 | 2.6 |
| Enc Time[%] | 109% |  |  | 110% |  |  |
| Dec Time[%] | 105% |  |  | 104% |  |  |

Table 3	- 1500 byte slice overhead for HM-2.0 and HM-3.3rc1

|  | HM-2.0 slice overhead |  | HM-3.3rc1 overhead |  |
| --- | --- | --- | --- | --- |
|  | HE | LC | HE | LC |
| AI | 4.9 | 5.5 | 4.7 | 4.6 |
| RA | 4.3 | 4.3 | 4.1 | 3.4 |
| LD | 2.5 | 2.6 | 2.5 | 2.0 |

The complexity numbers above come from cluster simulations using different machines and are therefore not reliable. The encoder complexity for the all-Intra configuration is high due to the fact that the encoder re-encodes LCUs rather frequently for Intra pictures at low QPs to reach the slice byte target.

There are some known issues with the current slice implementation. These are:

- The slice header is not accounted for when the code is configured for byte size limited slices. Therefore the slice sizes are larger than 1500 bytes more often than they should.

- Slice header syntax elements differ between source code and Working Draft, see Section 2.

# List of AHG related input documents

The following contributions related to slices were submitted to the current JCT-VC meeting:

JCTVC-F131 [V. Sze , M. Budagavi, A. Osamoto (TI)] "Modifications to slice header termination for low delay encoding"

JCTVC-F143 [T. Yamakage, S. Asaka, T. Chujoh (Toshiba), M. Karczewicz, I. S. Chong (Qualcomm)] "CE12: Deblocking filter parameter adjustment in slice level"

JCTVC-F174 [K. Sugimoto, S. Sekiguchi (Mitsubishi)] "Signaling of Max and Min QP in slice"

JCTVC-F187 [M. Li, P. Wu (ZTE)] "Comments on Slice Common Information Sharing"

JCTVC-F469 [C. Kim, Y. Park (Samsung)] "Syntax improvement for fine granularity slices"

# Recommendation

The Slice support development and characterization AHG recommends JCTVC to review the list of slice issues reported in section 2 and all slice related input document.