| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F006_r2 WG11 Number: m21292 |
| --- | --- |

| Title: | JCT-VC AHG report: In-loop and post-processing filtering (AHG 6) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Yamakage1 K. Chono2 Y. J. Chiu3 I. S. Chong4 M. Narroschke5 |  | tomoo.yamakage@toshiba.co.jp chono@ct.jp.nec.com yi-jen.chiu@intel.com ichong@qualcomm.com Matthias.Narroschke@panasonic.de |
| Source: | AHG chair and vice-chairs (Toshiba1, NEC2, Intel3, Qualcomm4, Panasonic5) |  |  |

_____________________________

# Abstract

The following summarizes the In-loop and post-processing filtering Ad hoc activities between the 5th JCT-VC meeting in Geneva, CH (16 to 23 March, 2011) and the current 6th JCT-VC meeting in Torino, IT (14 to 22 July, 2011).

# Mandate

| AHG Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| In-loop and post-processing filtering (jct-vc@lists.rwth-aachen.de) - Study enhancement schemes of in-loop filtering, including de-blocking/de-banding/de-noising filters, and adaptive Wiener-based filters including variants with various inputs, and combination of filters - Study trade-offs and characteristics of filter designs including complexity and subjective and objective performance - Discuss relationships and evaluation procedures for the filtering techniques - Identify possibilities for harmonization of enhanced in-loop filtering technologies - Study the relationship between in-loop and post-processing filtering | T. Yamakage (chair), K. Chono, Y. J. Chiu, I. S. Chong, M. Narroschke (vice chairs) | N |

# List of related input documents

There are 29 technical contributions and 22 cross-checks related to this AHG6 and 11 technical proposals to CE8 on non-deblocking loop filtering.  Most of the mandates are covered by AHG 6 contributions and CE8 contributions.

The document number is highlighted in yellow for complexity issue, in light blue for improvement of coding efficiency or subjective picture quality and in green for adding functionality according to the main subject of the contribution, although some contributions claim several aspects.

## AHG 6 related contributions

(Deblocking fitler, 10 proposals, 5 cross-checks)

- JCTVC-F044 [TI] Improving Deblocking filter Performance with SKIP-CU

      - Cross-check JCTVC-F219 (Sony)

        - It is proposed to decrease computation and memory access to increase performance without affecting the video quality.

- JCTVC-F053 [MediaTek] Deblocking Filter with Reduced Pixel Line Buffers for LCU-based Processing

        - Two vertical filtering methods are proposed only for the processing of horizontal LCU edges in order to reduce line buffers.

- JCTVC-F120 [MediaTek] Simplification of Chroma Deblocking Filter in F118

        - Simplification of the chroma filtering decision in JCTVC-F118 (CE8 contribution) by reusing the filtering decision results of luma component is proposed.

- JCTVC-F175 [Mitsubishi] Signaling of boundary filtering strength of deblocking filter for intra

        - Signaling of boundary filtering strength of deblocking filter for intra at the slice header is proposed.

- JCTVC-F214 [Sony] Parallel deblocking improvement

      - Cross-check: JCTVC-F267 [SKT/SKKU], JCTVC-F445 [TI]

        - Two methods, (1) improving and developing the parallel deblocking filter and (2) improving and simplifying the parallel deblocking filter, are proposed.

- JCTVC-F215 [Sony] Vertical tap length reduction to reduce line memory in deblocking filter

        - Reduction of one line memory with keeping BD-Rate and subjective quality is proposed by reducing one pixel regarding vertical tap length.

- JCTVC-F256 [TI] Improving Deblock filter efficiency

      - Cross-Check: JCTVC-F220 [Sony]

        - Chroma filtering improvements and removal of parallel decision are proposed.

- JCTVC-F359 [ETRI] Deblocking filtering modification for constrained intra prediction

      - Cross-cehck: JCTVC-F651 [Panasonic]

        - The proposal prevents the deblocking filter problem about constrained intra prediction by disabling deblocking filtering on pixels inside intra block when its neighboring block is inter block.

- JCTVC-F405 [Panasonic] Deblocking filter using adaptive weighting factors

        - Adaptive weighting factors are proposed into the deblocking filter.  Each weighting factor weights the modification value, by which the HM3.0 deblocking filter would modify a sample value.

- JCTVC-F484 [TI] Improving Worst-Case performance of Deblock filter

      - Cross-Check: JCTVC-F722 [Samsung]

        - With changing BS computation for BS=0 or 1, the large number of operation and memory access required in worst case is removed.

(SAO, 7 proposals, 7 cross-checks)

- JCTVC-F055 [MediaTek] Sample Adaptive Offset with Zero Pixel Line Buffers for LCU-based Decoding

      - Cross-check: JCTVC-F366 [Qualcomm]

        - SAO with skipping or padding at virtual boundaries is proposed to remove line buffer for decoding SAO.

- JCTVC-F056 [MediaTek/Qualcomm] Sample Adaptive Offset with LCU-based Syntax

      - Cross-check: JCTVC-F439 [Samsung]

        - A new syntax that allows SAO parameters to be adaptively changed at any largest coding unit (LCU) is proposed in order to support localization of SAO parameters with higher flexibility.

- JCTVC-F057 [MediaTek/LGE/Samsung] Sample Adaptive Offset for Chroma

      - Cross-check: JCTVC-F364 [Qualcomm]

        - The same techniques of band offset (BO) and edge offset (EO) used for luma are applied for chroma.  The three components are independent, and each component has its own partitions and offsets.

- JCTVC-F058 [MediaTek] Sample Adaptive Offset with PPS-level Syntax

      - Cross-check: JCTVC-F700 [Sharp]

        - Adding an option of moving SAO parameters to the picture parameter set (PPS) is proposed to enabled for multiple slices per picture or to disable for single slice per picture.

- JCTVC-F093 [MediaTek] Sample Adaptive Offset with Padding at LCU, Slice, and Image Boundaries

      - Cross-check: JCTVC-F365 [Qualcomm]

        - Replacement of the skipping technique used in SAO in HM3.2 by the padding technique as unification between ALF and SAO is proposed.

- JCTVC-F232 [TI] SAO LCU boundary processing

      - Cross-check: JCTVC-F442 [Samsung]

        - Techniques for SAO processing at LCU boundary to eliminate grid line type of visual artifact is proposed without using additional buffer.

- JCTVC-F396 [Sharp] Improvement of Sample Adaptive Offset with modified bit accuracy and restricted offsets

      - Cross-check: JCTVC-F693 [MediaTek]

        - A modified bit accuracy (10 bits) and offset range (4, 5, or 6 bits) used in Sample Adaptive Offset (SAO) are proposed.

(ALF, 8 proposals, 6 cross-checks)

- JCTVC-F054 [MediaTek] Adaptive Loop Filter with Zero Pixel Line Buffers for LCU-based Decoding

      - Cross-check: JCTVC-F367 [Qualcomm], JCTVC-F389[Sharp], JCTVC-665 [Toshiba]

        - Processing a to-be-filtered pixel on one side of a virtual boundary without using any pixel from the other side is proposed. Repetitive padding is used to replace pixels from the other side.

- JCTVC-F076 [Ericsson] ALF coefficient prediction

        - ALF luma coefficient prediction in spatial domain is proposed.

- JCTVC-F161 [ZTE] Complexity Scalable ALF

      - Cross-check: JCTVC-F209 [Microsoft]

        - Several patterns specifying ALF applications at picture/slice level are proposed to implement complexity scalable ALF, which aims at achieving tradeoff between coding efficiency and computational burden.

- JCTVC-F179 [Mitsubishi] An improvement on pixel classification for ALF based on edge direction

        - A modified pixel classification method for ALF is proposed.  Directional information is harmonized with the Laplacian activity metric based classification method in HM3.0.  4 levels of Laplacian activity are used for vertical and horizontal edge blocks, and 8 levels of Laplacian activity are used for non-directional blocks.

- JCTVC-F342 [Broadcom] ALF Complexity Analysis

        - The implementation complexity of the adaptive loop filter (ALF) is examined, and it is recommended to study of ways to reduce this complexity.

- JCTVC-F498 [PKU] Adaptive Loop Filter Merge in Temporal Domain

      - Cross-check: JCTVC-F207 [Microsoft]

        - A temporal adaptive loop filter (ALF) merging method is proposed.  The proposed method utilizes decoded ALF parameters from prior frames to implement the adaptive loop filtering.

- JCTVC-F522 [MediaTek/Qualcomm/Sharp/Toshiba] Enhancing block/region based Adaptive Loop Filter by MediaTek, Qualcomm, Sharp and Toshiba

      - Cross-check: JCTVC-F182 [Mitsubishi]

        - Merged version of two CE8 subtest1 proposals (JCTVC-F321 (temporal coefficient prediction and BA enhancement) and JCTVC-F384 (flexible class representation and additional block-based adaptive mode) is studied, which aims to improve block/region adaptive loop filter (ALF) adopted in Geneva meeting.

- JCTVC-F542 [Cisco] ALF with low latency and reduced complexity for HEVC

        - Some ideas and initial simulation tests on an ALF version that could be useful in connection with low latency applications are presented.

(SAO and ALF, 3 proposals)

- JCTVC-F271 [Panasonic] Grid displacements for in-loop filtering

        - A method to reduce the complexity of the LCU- (Largest Coding Unit) based processing of Sample Adaptive Offset (SAO) and Adaptive Loop Filter (ALF) operations via displacements in the filter grids is proposed.

- JCTVC-F320 [TI] Subjective tests on ALF and SAO

        - Informal subjective tests conducted on two loop filtering operations in HM-3.0: SAO and ALF are presented.

- JCTVC-F399 [Panasonic] Restart of CABAC after coding of ALF and SAO slice header data

        - It is proposed to restart CABAC after the coding of the ALF/SAO control parameters by the use of a cabac_restart_flag, which is coded in the terminating mode within the slice header in order to overcome large buffer and delay in encoder.

(Other in-loop filters, 1 proposal, 1 cross-check)

- JCTVC-F047 [NTT] Modifications of in-loop filter based on non-local means filter

      - Cross-check: JCTVC-F323 [KDDI]

        - A denoising filter called non-local means (NLM) filter is proposed in the in-loop filer process of the encoder/decoder in addition to original in-loop filter of HM3.0.

## CE8 contributions

Subtest 1: Block level filter adaptation with directional feature (BADIR)

  - JCTVC-F301 [Samsung] CE8 Subtest 1: Block-based filter adaptation with features on subset of pixels

    - Cross-check: JCTVC-F223 [eBrisk], JCTVC-F244 [TI], JCTVC-F387 [Sharp]

  - JCTVC-F321 [Toshiba/MediaTek/Qualcomm] CE8.1: Block based Adaptive Loop Filter by MediaTek, Qualcomm and Toshiba

    - Cross-check: JCTVC-F307 [Samsung], JCTVC-F516[Intel]

  - JCTVC-F384 [Sharp/Qualcomm] CE8.1:Block based Adaptive Loop Filter with flexible syntax and additional BA mode by Sharp and Qualcomm

    - Cross-check: JCTVC-F067 [MediaTek]

Subtest 2: Adaptive loop filter with fixed set of filters

  - JCTVC-F285 [Sony] CE8: Parametric Adaptive Loop Filter

    - Cross-check: JCTVC-F163 [Toshiba], JCTVC-E170 [Mitsubishi], JCTVC-F312 [Qualcomm]

Subtest 3: Memory reduction in ALF decoding

  - JCTVC-F272 [Panasonic] CE8 subtest 3: Line memory reduction for in-loop filtering

    - Cross-check: JCTVC-F521 [Qualcomm]

Subtest 4: Filter size and shape

  - JCTVC-F041 [eBrisk] CE8 Subtest 4: Adaptive Loop Filtering Using Two Filter Shapes

    - Cross-check: JCTVC-F305 [Samsung]

  - JCTVC-F234 [TI] CE8, Subset 4, Tool 3: ALF decode with reduced vertical filter size (JCTVC-E060)

    - Cross-check: JCTVC-F304 [Samsung]

  - JCTVC-F303 [Samsung/eBrisk] CE8 Subtest 4: ALF using vertical-size 5 filters with up to 9 coefficients

    - Cross-check: JCTVC-F324 [Qualcomm]

Subtest 5: Filter size and shape

  - JCTVC-F042 [eBrisk] CE8 Subtest 5: Adaptive Loop Filtering of Luminance and Chrominance Samples Using Same Filtering Shape, Structure and Map

    - Cross-check: JCTVC-F188 [Toshiba]

  - JCTVC-F157 [Toshiba/MediaTek/Qualcomm] CE8.5: Unified Chroma Filter Shapes with Luma's Shapes for ALF

    - Cross-check: JCTVC-F245 [TI], JCTVC-F454 [Panasonic]

  - JCTVC-F235 [TI] CE8, Subset 5, Tool 3: Chroma ALF with reduced vertical filter size (JCTVC-E287)

    - Cross-check: JCTVC-F666 [eBrisk Video]

## CE12 contributions

- JCTVC-F118 [Ericsson/MediaTek] CE12: Ericsson's and MediaTek's deblocking filter.

  - Cross-check: JCTVC-F189 [Toshiba] CE12: Verification results of Ericsson/MediaTek's proposal JCTVC-F118

  - Cross-check: JCTVC-F263 [SKT/SKKU] CE12: Cross-verification Results of Ericsson and MediaTek Deblocking Filter (JCTVC-F118) by SKT/SKKU

  - Cross-check: JCTVC-F401 [Panasonic] CE12: Cross-check results of the deblocking filter of MediaTek and Ericsson (JCTVC-F118)

  - Cross-check: JCTVC-F609 [Samsung] CE12: Cross-verification of the Deblocking Filter Proposed by MediaTek and Ericsson (JCTVC-F118)

- JCTVC-F143 [Toshiba/Qualcomm] CE12: Deblocking filter parameter adjustment in slice level

  - Cross-check: JCTVC-F530 [Ericsson] CE12: Cross-verification of proposal E143 by Ericsson

- JCTVC-F258 [SK Telecom/SKKU] CE12: SK Telecom/SKKU Deblocking Filter

  - Cross-check JCTVC-F218 [Sony] CE12: Cross-verification of SK Telecom/SKKU's proposal JCTVC-F258

  - Cross-check JCTVC-F529 [Ericsson] CE12: Cross-check of SKKU deblocking by Ericsson

  - Cross-check JCTVC-F625 [ETRI] CE12: Cross-verification of SKT/SKKU deblocking filter (JCTVC-F258) by ETRI

- JCTVC-F262 [SK Telecom/SKKU] CE12: SK Telecom/SKKU Harmonized Deblocking Filter with Additional Chroma Processing

  - Cross-check: JCTVC-F206 [Microsoft] CE12: Cross-check of harmonized deblocking filter with additional chroma processing (JCTVC-F262)

  - Cross-check: JCTVC-F611 [Samsung] CE12: Cross-verification of SK Telecom/SKKU Harmonized Deblocking Filter with Additional Chroma Processing (JCTVC-F262)

- JCTVC-F191 [Panasonic] CE12: Results for decisions for deblocking

  - Cross-check: JCTVC-F217 [Sony] CE12: Cross-verification of Panasonic's proposal JCTVC-F191

  - Cross-check: JCTVC-F580 [Qualcomm] CE12: Cross Check of Panasonic’s Deblocking Decisions (JCTVC-F191)

- JCTVC-F198 [Microsoft] CE12, Subset 1: Report of Deblocking for Large Size Blocks

  - Cross-check: JCTVC-F264 [SKT/SKKU] CE12: Cross-verification Results of Microsoft Deblocking Filter (JCTVC-F198) by SKT/SKKU

  - Cross-check: JCTVC-F403 [Panasonic] CE12: Cross-check results of the deblocking filter of Microsoft (JCTVC-F198)

  - Cross-check: JCTVC-F633 [Nokia] CE12: Cross-check of deblocking for large size blocks (JCTVC-F198 by Microsoft)

# Recommendations

JCT-VC to review all input contributions.

JCT-VC to include some proposals in the relevant CEs (CE8 and CE12) for the next term.

JCT-VC to re-create a CE for SAO.

JCT-VC to create a BoG to study reduction of line buffers may be necessary to have throughout consideration of deblocking filter, SAO and ALF.