| .Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F007-r2 |
| --- | --- |

| Title: | JCT-VC AHG report: Coding block structures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Krit Panusopone (Motorola Mobility) Woo-Jin Han (Samsung Electronics) T. K. Tan (NTT Docomo) Thomas Wiegand (Fraunhofer HHI) | Email: | krit@motorola.com wjhan.han@samsung.com tktan.p@rd.nttdocomo.co.jp thomas.wiegand@hhi.fraunhofer.de |
| Source: | Coding block structures AhG |  |  |

_____________________________

# Abstract

The following report summarizes the Coding block structures Ad hoc activities between the 5th JCT-VC meeting in Geneva, Switzerland (16 to 23 March, 2011) and the 6th JCT-VC meeting in Torino, Italy (14 to 22 July, 2011).

- Mandate

| AHG Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Coding block structures (jct-vc@lists.rwth-aachen.de) - Study techniques of HM relating to coding block structure - Characterize the trade-offs involved in coding block structure issues, including complexity, redundancy and compression performance aspects - Identify opportunities for harmonization and simplification of coding block structure | K. Panusopone, W.-J. Han, T. K. Tan, T. Wiegand [chairs] | N |

- Status

Several changes were made to coding block structure at the Geneva meeting including modified definition of TU partition when max RQT depth is equal to 1 and modified CAVLC coded block flag coding under residual quadtree. Common test conditions were also changed to unified maximum RQT depth across all test conditions. Table 1 reports coding performance of HM-3.2 with maximum RQT depths set to 1 and 2 against common test condition.

There were some coordinated activities relating to Coding block structure AhG occurred between the Geneva meeting and the Torino meeting. CE-2 [1] studied non-square quadtree transform for symmetric motion partitions. CE-6 [2] investigated RQT and SDIP harmonization. List of related input documents to the Toriono meeting is shown in Section 3.

Table 1. Coding performance of RQT at various maximum depth for Luma component

| Condition | D=1 | D=2 |
| --- | --- | --- |
| AIHE | 0.4% | 0.1% |
| AILC | 0.6% | 0.3% |
| RAHE | 0.9% | 0.2% |
| RALC | 1.2% | 0.3% |
| LDHE | 0.9% | 0.2% |
| LDLC | 1.4% | 0.3% |

- List of related input documents

JCTVC-F045 [R. H. Gweon, Y.-L. Lee (Sejong Univ.), J. Lim (SK Telecom)] Early Termination of CU Encoding to Reduce HEVC Complexity

JCTVC-F070 [M. Zhou, M. Budagavi (TI)] Sub-8x8 PU coding with fixed reference index

JCTVC-F077 [M. Mrak, A. Gabriellini, N. Sprljan, D. Flynn (BBC)] Transform skip mode

JCTVC-F092 [K. Choi, E. S. Jang (Hanyang Univ.)] Coding tree pruning based CU early termination

JCTVC-F107 [J. Park, B. Jeon (LGE)] Redundancy removal of residual information for CAVLC in merge 2Nx2N

JCTVC-F192 [Y. Sugito, A. Ichigaya, S. Sakaida (NHK), K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] A study on addition of 64x64 transform to HM3.0

JCTVC-F302 [Y. Zheng, X. Wang, W.-J. Chien, M. Karczewicz (Qualcomm)] Merge candidate selection in 2NxN, Nx2N, and NxN mode

JCTVC-F343 [J. Xu, A. Tabatabai, T.Suzuki (Sony)] Evaluation of SDIP

JCTVC-F418 [W. Zhang (ZTE)] Joint coding of CU splitting flag and inter modes based on HM3.1

JCTVC-F563 [L. Guo, J. Sole, R. Joshi, P. Chen, X. Wang, M. Karczewicz (Qualcomm)] Non-Square Transform for 2NxN and Nx2N Motion Partitions

JCTVC-F578 [K. Panusopone, X. Fang, V. Kung, L. Wang (Motorola Mobility)] RQT with rectangular transform unit support

JCTVC-F596 [M. Horowitz, S. Xu (eBrisk Video), E. S. Ryu, Y. Y (InterDigital)] The effect of LCU size on coding efficiency in the context of MTU size matching

- Recommendations

The recommendations of the Coding block structures AhG are to:

- Study interaction of Non-square transform and RQT.

- Study complexity reduction methods for coding block structures processing including early termination technique and transform skip mode.

- Encourage more experts to volunteer to contribute on improving coding efficiency and simplification of Coding block structures.

# References

[1] X Zheng , P. Bordes , P. Chen , I.-K Kim, “Description of Core Experiment 2 (CE2): Motion Partitioning and OBMC,” JCTVC-E702, Geneva, CH, Mar. 2011

[2] A. Tabatabai, M. Budagavi, K. Chono, R. Joshi, A. Segall, H. Yu, “Description of Core Experiment 6 (CE6): Intra Prediction Improvement,” JCTVC-F706, Geneva, CH, Mar. 2011