| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F008-r1 |
| --- | --- |

| Title: | JCT-VC AHG report: Reference pictures memory compression (AHG8) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono (Chair) NEC Corporation Takeshi Chujoh (Vice Chair) Toshiba Corporation ChongSoon Lim (Vice Chair) Panasonic Corporation Ali Tabatabai (Vice Chair) Sony Corporation Minhua Zhou (Vice Chair) Texas Instruments Inc., USA | Email: | chono@ct.jp.nec.com takeshi.chujoh@toshiba.co.jp ChongSoon.Lim@sg.panasonic.com Ali.Tabatabai@am.sony.com zhou@ti.com |
| Source: | [Reference pictures memory compression AHG] |  |  |

_____________________________

# Abstract

This document summarizes the AHG activities between the 5th Meeting: Geneva, CH, 16-23 March 2011, and the current 6th Meeting: Torino, 14-22 July, 2011.

# Mandate

| AHG Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Reference pictures memory compression (jct-vc@lists.rwth-aachen.de) - Study motion compensation memory access bandwidth of HM design and proposed reference picture memory compression schemes - Study reference picture memory compression schemes proposed for the HM design - Study data format alignment between reference picture memory compression and display processing - Study the visual quality impact of reference picture memory compression - Report on conclusions reached | K. Chono (chair), T. Chujoh, C. S. Lim, A. Tabatabai, M. Zhou (vice chairs) | N |

# Related activities and input contributions

Related to Mandate 1:

Decoder-side motion compensation memory access bandwidth of HM 3.0 Anchor streams was measured by using a module in HM2.0 branch software:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/HM-2.0-ahg-memory

It was observed that decoder-side motion compensation memory access bandwidths of HM2.0 and HM 3.0 are similar since HM3.0 does not adopt new tools that drastically increase or decrease motion compensation memory access bandwidth. The average memory bandwidth increase was -0.22% for HE Random access, -0.02% for LC Random access, -2.12% for HE Low delay, and -0.05% for LC Low delay. The detailed results are found in the attached XLS sheet.

There is no contribution document.

Related to Mandate 2:

There are 4 technical proposals (and 4 cross-check contributions) from Huawei, MediaTek, Toshiba, Xidian University, and Zenverge.

- JCTVC-F073 [MediaTek] Joint Luma-Chroma adaptive reference picture memory compression

      - Cross-check: JCTVC-F079 [Zenverge]

        - The proposal is based on adaptive scaling and uses one scaling factor for each of luma and chroma blocks. Three results using different bit allocation for chroma blocks with luma offset on/off are presented.

- JCTVC-F075 [Zenverge] Unified scaling with adaptive offset for reference frame compression

      - Cross-check: JCTVC-F078 [MediaTek] and JCTVC-F548[Santa Clara University and Hisilicon]

        - The proposal is based on Unified Scaling with Adaptive Offset (USAO). Two results based on USAO are presented.

- JCTVC-F319 [Toshiba] Adaptive scaling with offset for reference pictures memory compression

      - Cross-check: JCTVC-F620 [Zenverge]

        - The proposal is based on adaptive scaling with adaptive offset.

- JCTVC-F496 [Xidian University and Huawei] 2x2 block-based reference pictures memory compression

        - The joint proposal is based on adaptive scaling.

The performance of the technical proposals is summarized as follows:

Table 1	Summary of JCTVC-F073, JCTVC-F075, JCTVC-F319, and JCTVC-F496 results in comparison to HM3.0 Anchor for HE RA.

Table 2	Summary of JCTVC-F073, JCTVC-F075, JCTVC-F319, and JCTVC-F496 results in comparison to HM3.0 Anchor for HE LD.

Related to Mandate 3 and 4:

There was no discussion among reflector. There is no contribution document.

# Recommendations

The AHG recommends reviewing the related input documents.