| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F009 |
| --- | --- |

| Title: | JCT-VC AHG report: Entropy coding (AHG9) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., 12500 TI Blvd, Dallas, TX, USA Gaëlle Martin-Cocher 175 Columbia St. West Waterloo, Ontario N2L 5Z5 Canada Andrew Segall SHARP Labs of America 5750 NW Pacific Rim Blvd Camas, WA USA Wade Wan Broadcom Corporation | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | madhukar@ti.com gmartincocher@rim.com asegall@sharplabs.com +1-949-926-6387 wwan@broadcom.com |
| Source: | JCTVC Entropy coding AhG |  |  |

_____________________________

# Entropy coding AhG overview

The mandate of this AHG is as follows:

| Entropy coding (AHG9) (jct-vc@lists.rwth-aachen.de) - Study the entropy coding complexity and compression characteristics of CABAC, CAVLC, PIPE/V2V, and other entropy coding designs - Consider the potential for harmonization of HE and LC entropy coding designs - Characterize entropy coding throughput, memory, silicon area, power requirements, etc. - Study interdependencies between entropy coding and other processes and the consequences of these interdependencies - Study parallel context processing, syntax-element partitioning, and other parallelism approaches for entropy coding - Study and develop approaches for hardware and software evaluation of entropy coding methods - Identify and discuss additional issues on entropy coding | M. Budagavi (chair), G. Martin-Cocher, A. Segall, W. Wan (vice chairs) |
| --- | --- |

# Entropy coding related activity and emails

There was a kick-off message related to this AhG on the JCTVC reflector. There were no other email exchanges. Several AhG members interested in this area were actively participating in proposal/cross-verification of entropy coding within CE5 and CE11.

# Entropy coding related contributions

There are several contributions to the Torino meeting that are related to the mandates of the Ad Hoc Group. They can be broadly categorized as follows:

- CE5 (CAVLC entropy coding improvements) related contributions

- CE11 (coefficient scanning and coding) related contributions

- CAVLC related contributions

- CABAC context reduction contributions

- Other CABAC contributions

- PIPE/V2V related contribution

- Contributions related to parsing dependencies between entropy coding and other processes

A summary of the contributions is provided below.

## CE5 (CAVLC entropy coding improvements)

### Proposals

| JCTVC-F025 | CE5: Summary report of Core Experiment on CAVLC entropy coding improvements | X. Wang, P. Wu (CE coordinators) |
| --- | --- | --- |
| JCTVC-F390 | CE5: Simplification of transform coefficient coding in CAVLC | Y. Yasugi, T. Yamamoto (SHARP) |
| JCTVC-F408 | CE5: Run and level mode coding improvement in CAVLC | S. Kim, J. Lee, S. Lee (Yonsei Univ.), J. Chen, J. Park (Samsung) |
| JCTVC-F524 | CE5: Results on modified inter mode coding and joint coding of split flags for CAVLC | A. Kotra, V. Drugeon, T. Wedi (Panasonic) |

Bottom of Form

### Cross-checks

| JCTVC-F345 | CE5: Cross-check report on modifications for inter mode and split flags coding in CAVLC (JCTVC-F524) | K. Chono, H. Aoki (NEC) |
| --- | --- | --- |
| JCTVC-F404 | CE5: Cross-check of Yonsei Univ. and Samsung's proposal (JCTVC-F408) | Y. Yasugi (SHARP) |
| JCTVC-F409 | CE5: Crosscheck for Sharp's proposal (JCTVC-F390) | J. Lee, S. Kim, S. Lee (Yonsei Univ.), J. Chen, J. Park (Samsung) |

## CE11 (coefficient scanning and coding)

### Proposals

| JCTVC-F031 | CE11: Summary report of Core Experiment on coefficient scanning and coding | V. Sze, J. Chen, T. Nguyen, K. Panusopone, J. Sole (CE coordinators) |
| --- | --- | --- |
| JCTVC-F128 | CE11: Reduced neighboring dependency in context selection of significant_coeff_flag for parallel processing (JCTVC-E330) | V. Sze, M. Budagavi (TI) |
| JCTVC-F129 | CE11: Parallelization of HHI_TRANSFORM_CODING (Fixed Diagonal Scan) | V. Sze, M. Budagavi (TI) |
| JCTVC-F288 | CE11: Unified scans for the significance map and coefficient level coding in high efficiency | J. Sole, R. Joshi, M. Karczewicz (Qualcomm) |

Bottom of Form

### Cross-checks

| JCTVC-F136 | CE11: Cross-check of Qualcomm's Unified scans for the significance map and coefficient level coding in high coding efficiency (JCTVC-E335 Part 2) | V. Sze (TI) |
| --- | --- | --- |
| JCTVC-F149 | CE11: Cross-check of TI’s proposal on diagonal coefficient scans (JCTVC-F129) | C. Yeo, Y. H. Tan (I2R) |
| JCTVC-F185 | CE11: Cross-check result of TI proposal on context simplification of the significance map (JCTVC-F128). | C. Rosewarne, M. Maeda (Canon) |
| JCTVC-F293 | CE11: Cross-check of TI’s fixed diagonal scan (JCTVC-F129) | J. Sole (Qualcomm) |
| JCTVC-F311 | CE11.A: Cross checking of TI proposal JCTVC-E330 on context simplification of significance map | C. Auyeung (Sony) |
| JCTVC-F369 | CE11:Cross-check report for Qualcomm's proposal JCTVC-F288 part2 | H. Sasai, T. Nishi (Panasonic) |
| JCTVC-F451 | CE11: Crosscheck - Reduced neighboring dependency in context selection of significant_coeff_flag for parallel processing (E330/F128) | T. Nguyen |
| JCTVC-F607 | CE11:Cross-check report for Qualcomm's proposal JCTVC-F288 part1 | J. Chen, V. Seregin (Samsung) |

### Other CE11 related contributions

| JCTVC-F134 | CE11.A: Cross checking of JCTVC-C227 and proposal on semantic, syntax, and implementation | C. Auyeung (Sony) |
| --- | --- | --- |
| JCTVC-F597 | CE11: Hardware complexity of large zig-zag scan for level-coding of transform coefficients | C. Auyeung, T. Suzuki (Sony) |

## CAVLC related contributions

### Proposals

Table below also shows summary of BD-Rate savings of different proposals. Rows not shaded are luma BD-Rates and rows shaded yellow are chroma BD-Rates.

|  |  |  | AI | RA | LD |
| --- | --- | --- | --- | --- | --- |
| JCTVC-F106 | CAVLC coding for Intra Pred mode | J. Park, B. Jeon (LGE) | -0.2 | -0.1 | 0 |
| JCTVC-F107 | Redundancy removal of residual information for CAVLC in merge 2Nx2N | J. Park , B. Jeon (LGE) |  | -0.1 | -0.1 |
| JCTVC-F123 | Updated CAVLC Tables of Intra Mode Coding with Separated DC and Planar | M. Guo, X. Guo, S. Lei (MediaTek) | -0.2 |  |  |
| JCTVC-F160 | CAVLC table-size reduction for Inter/chroma run-level coding | T. Davies (Cisco) | 0 | 0 | 0 |
| JCTVC-F199 | A further improvement of inter prediction direction and reference frame index combined coding in CAVLC | B. Li (USTC), J. Xu (Microsoft), H. Li (USTC) |  | -0.2 | -0.2 |
| JCTVC-F286 | Redundancy removal for Run-mode in CAVLC | J. Xu, A. Tabatabai (Sony) | -0.1 | -0.1 | 0 |
| JCTVC-F287 | Improvements on last nonzero position coding of 4x4 TU in CAVLC | J. Xu, A. Tabatabai (Sony) | -1.7 | -1.4 | -0.8 |
| JCTVC-F296 | Modifications for CAVLC RDOQ | M. Karczewicz, L. Guo, X. Wang (Qualcomm) | -0.08 | -0.09 | -0.09 |
| JCTVC-F298 | Run-level Table Reduction for CAVLC | L. Guo, X. Wang , M. Karczewicz (Qualcomm) | 0 | -0.14 | 0.03 |
| JCTVC-F395 | CAVLC Adaptation using Difference Counter | T. Yamamoto (SHARP) | 0 | 0 | 0 |
| JCTVC-F406 | Coefficient coding in CAVLC | S. Kim, J. Lee, K. Lim, S. Lee (Yonsei Univ.), J. Chen, J. Park (Samsung) |  |  |  |
| JCTVC-F407 | Run-mode coding improvement with table reduction in CAVLC | J. Lee, S. Kim, K. Lim, D. Pak, S. Lee (Yonsei Univ.), J. Chen, J. Park (Samsung) | 0 | -0.2 | 0 |
| JCTVC-F458 | Improvement of CAVLC run- coding by prediction mode | C. Kim, Y. Park (Samsung) | -0.2 | -0.1 | 0 |
| JCTVC-F466 | Handling for exception cases regarding to code-word larger than 32bit in CAVLC | C. Kim, Y. Park(Samsung) | 0 | 0 | 0 |
| JCTVC-F467 | Improvement of CAVLC table adaptation for coefficient coding | C. Kim, Y. Park (Samsung) | -0.2 | -0.1 | -0.1 |
| JCTVC-F526 | CAVLC improvements | Anand Kotra, Virginie Drugeon, Thomas Wedi (Panasonic) |  |  |  |
| JCTVC-F543 | Tableless run-length coding for transform coefficients in CAVLC | A. Hallapuro, J. Kang, J. Lainema, K. Ugur (Nokia) |  |  |  |
| JCTVC-F608 | Removing Chroma Zonal Coding in CAVLC | M. Karczewicz, X. Wang, W.-J. Chien, L. Guo (Qualcomm) | 0 | -0.1 | -0.2 |

### Cross-checks

| JCTVC-F313 | Verification Results of run-level CAVLC table-size reduction (JCTVC-F160) | E. Maani (Sony) |
| --- | --- | --- |
| JCTVC-F621 | Crosscheck report for Samsung's JCTVC-F458 on CAVLC run coding | X. Wang (Qualcomm) |
| JCTVC-F622 | Crosscheck report for Samsung's JCTVC-F466 on CAVLC long codeword handling | X. Wang (Qualcomm) |
| JCTVC-F623 | Crosscheck report for Samsung's JCTVC-F467 on CAVLC table adaptation | X. Wang (Qualcomm) |
| JCTVC-F635 | Crosscheck report for JCTVC-F407 on CAVLC run-mode coding from Yonsei Univ. and Samsung | X. Wang (Qualcomm) |

## CABAC context reduction contributions

### Proposals

Contributions below deal with CABAC context reductions. Attached spreadsheet (JCTVC-F009.xls) provides a comparison of various simplifications in the proposals.

| JCTVC-F060 | Reducing Line Buffers for Motion Data and CABAC | T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| --- | --- | --- |
| JCTVC-F132 | Reduction in contexts used for significant_coeff_flag and coefficient level | V. Sze (TI) |
| JCTVC-F133 | Simplified MVD context selection (Extension of JCTVC-E324) | V. Sze (TI), A. P. Chandrakasan (MIT) |
| JCTVC-F148 | Context simplification for coefficients entropy coding | X. Che, W. Ding, Y. Shi (Beijing Univ. Tech.) |
| JCTVC-F429 | Modified Context Derivation for neighboring dependency reduction | H. Sasai, T. Nishi (Panasonic) |
| JCTVC-F497 | Simplified context model selection for block level syntax coding | J. Chen, T. Lee (Samsung) |
| JCTVC-F606 | Memory and Parsing Friendly CABAC Context | W.-J. Chien, M. Karczewicz, X. Wang, (Qualcomm) |

### Cross-checks

| JCTVC-F650 | Crosscheck for Samsung's Context Model Selection in JCTVC-F497 | T.-D. Chuang, Y.-W. Huang (MediaTek) |
| --- | --- | --- |
| JCTVC-F658 | Crosscheck for TI's MVD Context in JCTVC-F133 | T.-D. Chuang, Y.-W. Huang (MediaTek) |
| JCTVC-F672 | Cross-check of Panasonic’s proposal on modified context derivation for neighboring dependency reduction (JCTVC-F429) | J. Sole (Qualcomm) |
| JCTVC-F691 | Verification of JCTVC-F132: reduction of the number of CABAC contexts | F. Bossen (DOCOMO USA Labs) |

## Other CABAC contributions

### Proposals

| JCTVC-F046 | Efficient binary representation of cu_qp_delta syntax for CABAC | K. Chono, H. Aoki (NEC) |
| --- | --- | --- |
| JCTVC-F059 | CABAC with Constrained Outstanding Bits | T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-F061 | CABAC with a Reduced LPS Range Table | T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-F131 | Modifications to slice header termination for low delay encoding | V. Sze, M. Budagavi, A. Osamoto (TI) |
| JCTVC-F177 | Fast bypass mode for CABAC | R. Hattori, K. Sugimoto, S. Sekiguchi (Mitsubishi) |
| JCTVC-F186 | Predicted neighbour for context selection of significant_coeff_flag for parallel processing | C. Rosewarne, M. Maeda (Canon) |
| JCTVC-F254 | Multi-parameter probability up-date for CABAC | A. Alshin, E. Alshina (Samsung) |
| JCTVC-F275 | Wavefront and Cabac Flush: Different Degrees of Parallelism Without Transcoding | G. Clare, F. Henry, S. Pateux (Orange FT) |
| JCTVC-F376 | Utilisation of CABAC equal probability mode for intra mode coding | V. Seregin, I.-K Kim (Samsung) |
| JCTVC-F399 | Restart of CABAC after coding of ALF and SAO slice header data | M. Narroschke, T. Wedi, S. Esenlik (Panasonic) |
| JCTVC-F423 | Modified MVD coding for CABAC | H. Sasai, T. Nishi (Panasonic) |
| JCTVC-F455 | Modified binarization and coding of MVD for PIPE/CABAC | T. Nguyen, D. Marpe, H. Schwarz, T. Wiegand |
| JCTVC-F593 | Improved CABAC Context Initialization | K. Misra, A. Segall (Sharp) |

### Crosschecks

| JCTVC-F452 | Crosscheck - CABAC with Constrained Outstanding Bits (JCTVC-F059) | T. Nguyen |
| --- | --- | --- |
| JCTVC-F453 | Crosscheck - CABAC with a Reduced LPS Range Table (JCTVC-F061) | T. Nguyen |
| JCTVC-F137 | Cross-check results of Panasonic’s Modified MVD coding for CABAC (JCTVC-F423) | V. Sze (TI) |
| JCTVC-F691 | Verification of JCTVC-F132: reduction of the number of CABAC contexts | F. Bossen (DOCOMO USA Labs) |
| JCTVC-F636 | Cross-check of JCTVC-F254: Multi-Parameter Probability Update for CABAC | J. Zan, D. He (RIM) |
| JCTVC-F668 | Cross-check results of Canon’s Predicted neighbour for context selection of significant_coeff_flag for parallel processing (JCTVC-F186) | V. Sze (TI) |
| JCTVC-F669 | Cross-check of Mitsubishi's Fast bypass mode for CABAC (JCTVC-F177) | V. Sze (TI) |
| JCTVC-F670 | Cross-check of Mediatek's Reducing Line Buffers for Motion Data and CABAC (JCTVC-F060) | V. Sze (TI) |

## PIPE/V2V related contributions

### Proposals

| JCTVC-F176 | Improved PIPE/V2F for low complexity entropy coding | K. Sugimoto, R. Hattori, S. Sekiguchi, K. Asai (Mitsubishi) |
| --- | --- | --- |
| JCTVC-F268 | Unified PIPE-Based Entropy Coding for HEVC | D. Marpe, H. Kirchhoffer, B. Bross, V. George, T. Nguyen, M. Preiß, M. Siekmann, J. Stegemann, T. Wiegand (Fraunhofer HHI) |

Summary of BD-Rate results:

|  | AI-HE | RA-HE | LB-HE | AI-LC | RA-LC | LB-LC | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| JCTVC-F176 |  |  |  | -3.9 | -4.3 | -3.2 | Chroma loss between 6-16.7% |
| JCTVC-F268 | 0.3 | 0.3 | 0.2 | 0 | 0.5 | 1.2 | Chroma loss between 8.6-17.4% |

### Cross-checks

| JCTVC-F284 | Cross-check of Mitsubishi Electric’s improved PIPE/V2F for low complexity entropy coding proposal JCTVC-F176 | J. Stegemann (Fraunhofer HHI) |
| --- | --- | --- |

## Parsing dependencies between entropy coding and other processes

In HEVC, it is reported that there are parsing dependencies between entropy coding and other processes. For example, merge_idx and mvp_idx parsing (which is part of entropy decoding) requires full reconstruction of motion data (vectors, ref_idx, part_size, etc). This is asserted to reduce decoder speed (so-called parsing throughput issue, because now motion data reconstruction cannot run parallel to entropy decoding) and also makes bitstream very sensitive to corruption in temporal and spatial neighboring motion data (so-called parsing robustness issue, any error in motion data can cause parsing stop).

There are 7 contributions that address parsing dependencies for inter coding and 3 contributions that address parsing dependencies for Intra coding. The inter coding contributions can be divided into two categories. All of them use truncated unary coding for merge_idx and mvp_idx coding, but the table size signaling is different,   F470, F068, F402, and F347 signal table size in slice header, while F474, F052 and F341 still use implicit signaling at PU level, but changed to using neighboring PU availability only, temporal MVP candidate is always assumed available in this so-called availability based method.  Methods of both categories make the motion data reconstruction independent of entropy decoding.

The signaled table size (either fixed signaling at slice level  or availability based implicit signaling at PU-level) is oftentimes larger than actual merge/skip and AMVP list size of individual PUs. For example, if table size of 5 is signaled for merge_idx coding, then the following table will be used. If the actual MVP list size of a PU is 3 (marked in yellow) then the last two entries of the table go unused. The proposed methods add additional MVP candidates to fill the holes in the MVP list and make full use of the signaled table. The way of adding additional MVP candidates are different.

| MVP candidate | Merge_idx | Truncated unary coding |
| --- | --- | --- |
| (MVf0, MVb0) | 0 | 0 |
| (MVf1, MVb1) | 1 | 10 |
| (MVf2, MVb2) | 2 | 110 |
|  | 3 | 1110 |
|  | 4 | 1111 |

### Proposals

| JCTVC-F052 | CE9: Results of Experiment ROB04 | J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| --- | --- | --- |
| JCTVC-F068 | A study on HEVC parsing throughput issue | M. Zhou, V. Sze, Y. Matsuba (TI) |
| JCTVC-F341 | Decoder Performance Restrictions due to Merge/MVP Index Parsing | T. Hellman, Y. Yu (Broadcom) |
| JCTVC-F347 | A method of decoupling motion data reconstruction from entropy decoding | M. Zhou, V. Sze, Y. Matsuba (TI), T. Sugio, T. Nishi (Panasonic), J. Chen, T. Lee (Samsung), W. Wan, Y. Yu (Broadcom) |
| JCTVC-F402 | MVP index parsing with fixed number of candidates | J. Chen, T. Lee, I.-K. Kim (Samsung) |
| JCTVC-F470 | Parsing Robustness for Merge/AMVP | T. Sugio, T. Nishi (Panasonic) |
| JCTVC-F474 | CE9: Description of experiments ROB01 and ROB02 | G. Laroche, P. Onno, T. Poirier, C. Gisquet (Canon) |
| JCTVC-F378 | Intra mode parsing without access neighbouring information | V. Seregin, T. Lee, J. Chen (Samsung) |
| JCTVC-F459 | Parsing friendly intra mode coding | W.-J. Chien, X. Wang, M.Karczewicz (Qualcomm) |
| JCTVC-F550 | Removal of the parsing dependency of residual coding on intra mode | J. Sole, Y. Zheng, W.-J. Chien, R. Joshi, X. Wang, M. Karczewicz (Qualcomm) |

### Crosscheck

| JCTVC-F074 | Cross-check of TI's study on HEVC parsing throughput issue JCTVC-F068 | B. Bross (Fraunhofer HHI) |
| --- | --- | --- |
| JCTVC-F090 | Cross-verification of Panasonic's proposal JCTVC-F470 on parsing robustness for merge/AMVP | M. Zhou (TI) |
| JCTVC-F212 | Cross-check of intra mode parsing (JCTVC-F378) | J. Xu (Microsoft) |
| JCTVC-F504 | Cross check of Samsung proposal on MVP index parsing with fixed number of candidates (F402) | G. Laroche, Tangi Poirier (Canon) |

# Recommendations

The AhG recommends to review all relevant input contributions.

# Acknowledgements

AhG chairs would like to acknowledge help from Vivienne Sze for creating table in Section 3.3.1 and spreadsheet in JCTVC-F009 and from Minhua Zhou for Section 3.7.