| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F016_v2 WG11 Number: m21320 |
| --- | --- |

| Title: | JCT-VC AHG report: Decoder-side motion vector derivation (DMVD) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu Intel Corp. Elena Alshina Samsung Electronics Co., Ltd. Haoping Yu Huawei Technologies Co., Ltd. | Tel: Email: | yi-jen.chiu@intel.com elena_a.alshina@samsung.com haopingyu@huawei.com |
| Source: | [AHG Chairs] Intel Corp., Samsung Electronics Co., Ltd., Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Introduction

The decoder-side motion vector derivation (DMVD) AhG was formed at the 5th JCT-VC meeting at Geneva, Switzerland with the following mandates:

- Study the techniques of decoder side motion vector derivation to improve video compression

- Investigate inference processing techniques such as optical-flow at the decoder side

- Study the performance/complexity tradeoffs of uni/bi-predictional-based methods to derive motion vector information at decoder side

- Study complexity/performance of parallel processing for decoder-side inference techniques to improve video compression

# Related contributions

The following contributions related to DMVD were submitted to the current JCT-VC meeting:

- JCTVC-F500, “Report of self derivation of motion estimation techniques at video decoder side on HM 3.0”

- Two cross-check reports for JCTVC-F500: JCTVC-F724 (Huawei) and JCTVC-F735 (MERL)

# Result of coding efficiency improvement

Table 1 summarizes the performance of Skip-mode 9-candidate set based SDME of JCTVC-F500 compared to the performance of HM 3.0 anchors. In HE case, the average BD bitrate reduction for Luma component is about 0.5% (up to 1.3% for clip PeopleOnStreet) with about 2% increase in encoding time and almost no increase in decoding time. In low complexity (LC) case, the average BD bitrate reduction for Luma component is about 0.1% with about 3% increase in encoding time and about 4% increase in decoding time.

Table 1 - The performance of Skip-mode only 9-candidate set based SDME without MV refinement in JCTVC-F500

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.6 | -0.6 | -0.1 | -0.2 | 0.1 |
| Class B | -0.5 | -0.6 | -0.5 | 0.1 | 0.2 | 0.1 |
| Class C | -0.5 | -0.4 | -0.5 | -0.2 | -0.2 | -0.3 |
| Class D | -0.7 | -0.8 | -0.8 | -0.4 | -0.3 | -0.4 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.6 | -0.6 | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 104% |  |  |

Table 2 summarizes the performance of Skip-mode only 9-candidate set based SDME with 8-point-MV-refinement in JCTVC-F500 compared with the HM3.0 anchors. In HE case, the average BD bitrate reduction for Luma component is about 1.2% (up to 2.5% for clip Cactus) with about 5% increase in encoding time and about 13% increase in decoding time. In LC case, the average BD bitrate reduction for Luma component is about 0.8% with about 5% increase in encoding time and about 17% increase in decoding time.

Table 2 - The performance of Skip-mode only 9-candidate set based SDME with 8-point MV refinement in JCTVC-F500

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -1.2 | -1.2 | -0.6 | -0.6 | -0.5 |
| Class B | -1.3 | -1.1 | -1.1 | -0.8 | -0.5 | -0.6 |
| Class C | -1.1 | -1.1 | -1.1 | -0.8 | -0.8 | -0.9 |
| Class D | -1.4 | -1.5 | -1.6 | -1.2 | -1.0 | -0.7 |
| Class E |  |  |  |  |  |  |
| All | -1.2 | -1.2 | -1.2 | -0.8 | -0.8 | -0.8 |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 113% |  |  | 117% |  |  |

# Recommendations

The DMVD AhG recommends:

- To review all DMVD related input contributions

- To continue to study DMVD techniques to improve coding efficiency of HM software