| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F022 |
| --- | --- |

| Title: | Motion Partitioning and OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng Huawei & HiSilicon Technologies Philippe Bordes Technicolor Peisong Chen Qualcomm Il-Koo Kim Samsung Wen-Hsiao Peng NCTU/ITRI Liwei Guo Qualcomm | Tel: Email: | +86-755-28422009 zhengxiaozhen@huawei.com philippe.bordes@technicolor.com peisongc@qualcomm.com ilkoo.kim@samsung.com pawn@mail.si2lab.org liweig@qualcomm.com |
| Source: | Huawei & HiSilicon Technologies, Technicolor, Qualcomm, Samsung, NCTU/ITRI |  |  |

_____________________________

# Abstract

This document summarizes the activities of CE2 related to Motion Partitioning and OBMC [1]. The description of the experiment can be found in JCTVC-E702. In this CE, five tools have been studied and evaluated: Asymmetric Motion Partitioning (AMP) [2], Non-rectangular Motion Partitioning (NRMP) [3], Overlapped Block Motion Compensation (OBMC) [4], Non-Square Quadtree Transform (NSQT) [5] and OBMC with Motion Merging [6]. 14 companies have been involved as proponent or cross-checker.

# CE Documents

The purpose of Core Experiment 2 (CE2) is to investigate block partition issue at current HEVC coding structure. The contributions submitted for CE2 along with the corresponding cross-verification documents and related contributions are summarized as follow:.

- Summary document (this document)

  - JCTVC-F022[CE coordinators] CE2: Summary report on Motion Partitioning and OBMC

- Proposals and verification documents

Tool 1: Asymmetric Motion Partitioning (AMP)

  - Proposal :	JCTVC-F379 [Samsung and HiSilicon] CE2: Test result of asymmetric motion partition (AMP)

  - Tool Description: AMP consists of partitioning a square block into two non-symmetric horizontal or vertical rectangles shapes. In the current design, the square block is split using AMP in one rectangle of 3/4 of the square width or height, and another rectangle of 1/4 of the square width or height.

  - Cross-check: 	JCTVC-F435 [HHI] CE2: Cross-check of Non-Square Quadtree Transform JCTVC-F410 & JCTVC-F412

Tool 2: Non-rectangular Motion Partitioning (NRMP)

  - Proposal :	JCTVC-F415 [Huawei & HiSilicon, Tsinghua, Technicolor] CE2: Non-rectangular motion partitioning

  - Tool Description: NRMP enables to partition a square block into 2 regions with non-rectangular shape. Limited sets of diagonal-only partitions (45 degree or 135 degree partitioning) are used for the partitioning.

  - Cross-check: 	JCTVC-F520 [Microsoft] CE2: Cross-check of non-rectangular motion partitioning (JCTVC-F415)

Tool 3: Non-Square Quadtree Transform (NSQT)

  - Proposal1 :	JCTVC-F410 [Huawei & HiSilicon, Microsoft, Tsinghua, USTC] CE2: Non-Square Quadtree Transform for symmetric partitions

  - Proposal2 :	JCTVC-F412 [Huawei & HiSilicon, Microsoft, Tsinghua, USTC, Samsung] CE2: Non-Square Quadtree Transform for symmetric and asymmetric partitions

  - Tool Description: NSQT applies non-square TU to symmetric partitions (2NxN and Nx2N) and asymmetric partitions. The width and height of a non-square TU is based on RQT's depth and PU type.

  - Cross-check1: 	JCTVC-F435 [HHI] CE2: Cross-check of Huawei’s study on Non-Square Quadtree Transform JCTVC-F410 & JCTVC-F412

  - Cross-check2: JCTVC-F579 [Motorola Mobility] Cross-check report for Huawei’s proposal on inter non-square TU technique by Motorola Mobility

Tool 4: OBMC with Motion Merging

  - Proposal1 :	JCTVC-F049 [NCTU/ITRI)] CE2: Report of OBMC with Motion Merging

  - Tool Description: This technique performs OBMC for a 2Nx2N PU based on two sets of motion parameters, one is derived from motion merging with a reduced candidate set containing only Left, Above and Co-located merge candidates and the other is signaled in prediction_unit syntax for the current PU. Two predictors are created from these motion parameters and combined linearly using an adaptive OBMC window function to form a prediction block.

  - Cross-check: 	JCTVC-F515 [Intel] CE2: Cross verification results of NCTU’s OMBC with motion merging (JCTVC-F049) by Intel

  - Proposal2:    JCTVC-F687 [NCTU/ITRI] CE2: Report on the Combination of OBMC with Motion Merging and Non-Square Quadtree Transform

  - Tool Description: Combination of OBMC with Motion Merging and NSQT. Additionally, NSQT is enabled for symmetric partitions.

  - Cross-check:  JCTVC-F704 [Intel] CE2: Cross verification of JCTVC-F687

Tool 5: OBMC

  - Proposal1 :	JCTVC-F299 [Qualcomm] CE2: Overlapped Block Motion Compensation for 2NxN and Nx2N Motion Partitions

  - Tool Description: OBMC is applied to 2NxN and Nx2N motion partitions. If a CU is partitioned into 2 2NxN or Nx2N PUs, OBMC is applied to the horizontal boundary of the two 2NxN prediction blocks, and the vertical boundary of the two Nx2N prediction blocks.

  - Cross-check:  JCTVC-F385 [Samsung] CE2: Cross-check report for Qualcomm's OBMC for 2NxN and Nx2N motion partitions (JCTVC-F299)

  - Proposal2:    JCTVC-F582 [Qualcomm, Tsinghua, Huawei] CE2: Asymmetric Motion Partition, Non-Square Quadtree Transform and Overlapped Block Motion Compensation

  - Tool Description: Tools combination of AMP, NSQT for 2NxN, Nx2N and asymmetric partitions and OBMC for 2NxN, Nx2N and asymmetric partitions.

  - Cross-check:  JCTVC-F642 [Samsung] CE2: Cross-check report for a combination of AMP, NSQT and OBMC (JCTVC-F582)

# Summary of Results

Test conditions as specified in JCTVC-E700 were used [7]. The results for each tools is summarized as follow:

2.1 Asymmetric Motion Partitioning (AMP) with out encoding speed-up

|  | Random access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9 | -1.2 | -0.9 | -0.7 | -0.7 | -0.5 |
| Class B | -0.9 | -1.0 | -1.0 | -0.7 | -0.7 | -0.6 |
| Class C | -0.9 | -1.0 | -1.1 | -0.7 | -0.9 | -0.9 |
| Class D | -0.8 | -1.0 | -0.9 | -0.5 | -0.7 | -0.6 |
| Class E |  |  |  |  |  |  |
| Overall | -0.9 | -1.0 | -1.0 | -0.7 | -0.7 | -0.7 |
| Enc Time[%] | 144% |  |  | 151% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.1 | -1.5 | -1.5 | -0.9 | -0.8 | -0.6 |
| Class C | -1.0 | -1.2 | -1.3 | -0.7 | -0.6 | -0.7 |
| Class D | -1.1 | -1.3 | -1.5 | -0.6 | -0.5 | -0.9 |
| Class E | -2.3 | -2.2 | -2.4 | -1.7 | -1.1 | -1.3 |
| Overall | -1.3 | -1.5 | -1.6 | -0.9 | -0.7 | -0.8 |
| Enc Time[%] | 144% |  |  | 150% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

2.2 Asymmetric Motion Partitioning (AMP) with encoding speed-up

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5 | -0.8 | -0.5 | -0.4 | -0.6 | -0.2 |
| Class B | -0.5 | -0.8 | -0.7 | -0.4 | -0.5 | -0.5 |
| Class C | -0.6 | -0.8 | -0.8 | -0.5 | -0.6 | -0.7 |
| Class D | -0.5 | -0.9 | -0.8 | -0.4 | -0.5 | -0.6 |
| Class E |  |  |  |  |  |  |
| Overall | -0.5 | -0.8 | -0.7 | -0.4 | -0.6 | -0.5 |
| Enc Time[%] | 112% |  |  | 112% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | -1.1 | -1.2 | -0.5 | -0.4 | -0.3 |
| Class C | -0.7 | -1.0 | -0.9 | -0.4 | -0.4 | -0.7 |
| Class D | -0.7 | -1.2 | -0.8 | -0.5 | -0.7 | -0.2 |
| Class E | -1.5 | -1.9 | -1.7 | -1.0 | -1.0 | -0.9 |
| Overall | -0.8 | -1.2 | -1.1 | -0.6 | -0.6 | -0.5 |
| Enc Time[%] | 111% |  |  | 111% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

2.3 Non-rectangular Motion Partitioning (NRMP)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3 | -0.9 | -0.8 | -0.4 | -0.8 | -0.6 |
| Class B | -0.3 | -0.8 | -0.8 | -0.4 | -0.5 | -0.5 |
| Class C | -0.9 | -1.5 | -1.6 | -1.0 | -1.5 | -1.6 |
| Class D | -1.2 | -1.8 | -1.9 | -1.3 | -1.9 | -1.9 |
| Class E |  |  |  |  |  |  |
| Overall | -0.7 | -1.2 | -1.2 | -0.8 | -1.1 | -1.1 |
| Enc Time[%] | 119% |  |  | 122% |  |  |
| Dec Time[%] | 109% |  |  | 111% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.6 | -0.8 | -0.4 | -0.6 | -0.6 |
| Class C | -1.1 | -1.6 | -1.5 | -1.1 | -1.4 | -1.5 |
| Class D | -1.4 | -2.1 | -1.4 | -1.4 | -1.7 | -2.0 |
| Class E | -0.4 | -0.3 | -0.5 | -0.4 | -0.4 | -0.5 |
| Overall | -0.8 | -1.1 | -1.1 | -0.8 | -1.0 | -1.2 |
| Enc Time[%] | 117% |  |  | 120% |  |  |
| Dec Time[%] | 111% |  |  | 111% |  |  |

2.4 Non-Square Quadtree Transform (NSQT) for symmetric partitions

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2 | -0.6 | -0.5 | -0.2 | -0.5 | -0.3 |
| Class B | -0.2 | -1.4 | -1.1 | -0.5 | -1.2 | -0.9 |
| Class C | -0.2 | -0.5 | -0.5 | -0.4 | -0.6 | -0.5 |
| Class D | -0.2 | -0.5 | -0.4 | -0.4 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | -0.2 | -0.8 | -0.7 | -0.4 | -0.7 | -0.5 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | -3.5 | -3.1 | -1.3 | -3.0 | -2.9 |
| Class C | -0.7 | -2.0 | -2.0 | -1.2 | -1.6 | -1.5 |
| Class D | -0.8 | -1.8 | -1.5 | -1.2 | -1.1 | -0.7 |
| Class E | -1.4 | -2.5 | -3.0 | -2.8 | -2.3 | -2.9 |
| Overall | -0.9 | -2.5 | -2.4 | -1.6 | -2.0 | -2.0 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6 | -3.8 | -3.7 | -1.1 | -2.3 | -2.0 |
| Class C | -0.8 | -2.3 | -2.3 | -1.2 | -1.5 | -1.2 |
| Class D | -0.9 | -2.1 | -1.6 | -1.3 | -0.9 | -0.3 |
| Class E | -1.6 | -2.8 | -3.9 | -2.7 | -2.1 | -2.7 |
| Overall | -0.9 | -2.8 | -2.9 | -1.5 | -1.8 | -1.5 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

2.5 AMP + NSQT for asymmetric partitions

|  | Random access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -1.3 | -1.3 | -0.6 | -1.0 | -0.8 |
| Class B | -0.7 | -2.1 | -1.9 | -0.9 | -1.8 | -1.6 |
| Class C | -0.8 | -1.5 | -1.5 | -0.9 | -1.3 | -1.2 |
| Class D | -0.7 | -1.4 | -1.4 | -0.7 | -1.0 | -1.0 |
| Class E |  |  |  |  |  |  |
| All | -0.7 | -1.6 | -1.6 | -0.8 | -1.3 | -1.2 |
| Enc Time[%] | 112% |  |  | 113% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.2 | -4.4 | -4.4 | -1.6 | -3.2 | -3.0 |
| Class C | -1.3 | -3.4 | -3.4 | -1.4 | -2.2 | -2.2 |
| Class D | -1.5 | -3.3 | -3.0 | -1.4 | -1.4 | -1.5 |
| Class E | -2.6 | -3.9 | -4.1 | -3.1 | -2.1 | -2.8 |
| All | -1.6 | -3.8 | -3.7 | -1.8 | -2.3 | -2.4 |
| Enc Time[%] | 111% |  |  | 112% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

2.6 AMP + NSQT for symmetric and asymmetric partitions

|  | Random access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | -1.5 | -1.3 | -0.6 | -1.2 | -1.0 |
| Class B | -0.8 | -2.4 | -2.1 | -1.0 | -1.9 | -1.7 |
| Class C | -0.9 | -1.6 | -1.6 | -1.0 | -1.4 | -1.3 |
| Class D | -0.8 | -1.5 | -1.4 | -0.8 | -0.9 | -0.8 |
| Class E |  |  |  |  |  |  |
| All | -0.8 | -1.8 | -1.6 | -0.9 | -1.4 | -1.2 |
| Enc Time[%] | 114% |  |  | 114% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.5 | -4.7 | -5.0 | -1.9 | -3.4 | -3.4 |
| Class C | -1.5 | -3.4 | -3.6 | -1.8 | -2.3 | -2.3 |
| Class D | -1.7 | -3.4 | -2.8 | -1.9 | -1.7 | -1.3 |
| Class E | -3.1 | -4.3 | -4.8 | -4.0 | -2.3 | -3.2 |
| All | -1.9 | -4.0 | -4.1 | -2.3 | -2.5 | -2.6 |
| Enc Time[%] | 113% |  |  | 113% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

2.7 OBMC with Motion Merging

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.3 | -2.5 | -2.5 | -1.2 | -1.2 | -1.4 |
| Class B | -1.2 | -2.1 | -2.0 | -1.1 | -1.7 | -1.8 |
| Class C | -1.7 | -2.5 | -2.7 | -1.5 | -2.3 | -2.4 |
| Class D | -1.7 | -2.3 | -2.5 | -1.4 | -2.1 | -2.1 |
| Class E |  |  |  |  |  |  |
| Overall | -1.5 | -2.4 | -2.4 | -1.3 | -1.8 | -1.9 |
| Enc Time[%] | 122% |  |  | 122% |  |  |
| Dec Time[%] | 104% |  |  | 107% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.3 | -2.4 | -2.3 | -1.8 | -2.4 | -2.6 |
| Class C | -1.9 | -2.6 | -2.7 | -2.1 | -2.5 | -2.8 |
| Class D | -2.0 | -2.7 | -2.7 | -2.2 | -3.0 | -3.0 |
| Class E | -2.0 | -2.3 | -2.7 | -3.0 | -3.5 | -4.2 |
| Overall | -1.8 | -2.5 | -2.6 | -2.2 | -2.8 | -3.0 |
| Enc Time[%] | 128% |  |  | 127% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |

2.8 OBMC for 2NxN and Nx2N motion partitions

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | -1.0 | -1.0 | -0.5 | -0.6 | -0.4 |
| Class B | -0.5 | -0.8 | -0.6 | -0.5 | -0.6 | -0.6 |
| Class C | -0.7 | -1.0 | -1.1 | -0.6 | -1.0 | -1.1 |
| Class D | -0.7 | -1.0 | -1.0 | -0.5 | -0.9 | -0.8 |
| Class E |  |  |  |  |  |  |
| Overall | -0.5 | -0.9 | -0.9 | -0.5 | -0.8 | -0.7 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6 | -0.6 | -0.8 | -0.5 | -0.6 | -0.8 |
| Class C | -0.9 | -1.3 | -1.4 | -0.8 | -0.9 | -1.1 |
| Class D | -0.9 | -0.9 | -0.9 | -0.8 | -1.0 | -1.0 |
| Class E | -1.1 | -0.9 | -1.0 | -1.1 | -0.9 | -1.2 |
| Overall | -0.8 | -0.9 | -1.0 | -0.8 | -0.8 | -1.0 |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.0 | -0.9 | -1.4 | -1.3 | -1.1 | -0.7 |
| Class C | -1.3 | -1.6 | -1.6 | -1.3 | -1.4 | -1.4 |
| Class D | -1.8 | -1.5 | -2.0 | -1.7 | -1.5 | -1.3 |
| Class E | -1.6 | -1.5 | -1.8 | -1.9 | -1.4 | -1.5 |
| Overall | -1.4 | -1.4 | -1.7 | -1.5 | -1.3 | -1.2 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 105% |  |  |

2.9 AMP + NSQT + OBMC

|  | Random access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | -2.3 | -2.2 | -1.3 | -1.8 | -1.6 |
| Class B | -1.3 | -3.2 | -2.9 | -1.5 | -2.5 | -2.4 |
| Class C | -1.8 | -2.6 | -2.7 | -1.7 | -2.3 | -2.4 |
| Class D | -1.6 | -2.2 | -2.3 | -1.5 | -1.7 | -1.6 |
| Class E |  |  |  |  |  |  |
| All | -1.4 | -2.6 | -2.5 | -1.5 | -2.1 | -2.0 |
| Enc Time[%] | 116% |  |  | 116% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -2.1 | -5.6 | -5.4 | -2.6 | -4.0 | -4.1 |
| Class C | -2.6 | -4.6 | -4.5 | -2.7 | -3.2 | -3.1 |
| Class D | -2.9 | -3.7 | -4.0 | -2.7 | -2.4 | -2.0 |
| Class E | -4.4 | -5.2 | -5.7 | -5.2 | -3.8 | -4.3 |
| All | -2.9 | -4.8 | -4.9 | -3.1 | -3.3 | -3.4 |
| Enc Time[%] | 115% |  |  | 114% |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |

2.10 OBMC with Motion Merging + NSQT

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.4 | -2.8 | -3.0 | -1.5 | -1.7 | -1.8 |
| Class B | -1.4 | -3.4 | -3.2 | -1.6 | -2.9 | -2.8 |
| Class C | -2.0 | -3.2 | -3.3 | -1.9 | -2.8 | -2.7 |
| Class D | -2.0 | -2.8 | -2.7 | -1.9 | -2.2 | -2.2 |
| Class E |  |  |  |  |  |  |
| Overall | -1.7 | -3.1 | -3.0 | -1.7 | -2.4 | -2.4 |
| Enc Time[%] | 128% |  |  | 128% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.9 | -5.4 | -5.4 | -3.2 | -5.0 | -5.1 |
| Class C | -2.8 | -4.8 | -4.7 | -3.4 | -4.2 | -4.2 |
| Class D | -3.0 | -4.2 | -4.2 | -3.5 | -3.3 | -3.7 |
| Class E | -3.7 | -4.9 | -5.8 | -5.8 | -5.9 | -6.6 |
| Overall | -2.7 | -4.9 | -5.0 | -3.8 | -4.6 | -4.8 |
| Enc Time[%] | 132% |  |  | 132% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |

# Results analysis

The following tables summarize the average BD-rate gain for Y, U, V component, encoding time and decoding time for the different tools.

Random Access HE

| subtest | Y-BD-rate [%] | U-BD-rate [%] | V-BD-rate [%] | Enc Time [%] | Dec Time [%] |
| --- | --- | --- | --- | --- | --- |
| 2.1 AMP without encoding speed-up | -0.9 | -1.0 | -1.0 | 144% | 99% |
| 2.2 AMP with encoding speed-up | -0.5 | -0.8 | -0.7 | 112% | 99% |
| 2.3 NRMP | -0.7 | -1.2 | -1.2 | 119% | 109% |
| 2.4 NSQT for symmetric partitions | -0.2 | -0.8 | -0.7 | 100% | 100% |
| 2.5 AMP + NSQT for asymmetric partitions | -0.7 | -1.6 | -1.6 | 112% | 100% |
| 2.6 AMP + NSQT for symmetric and asymmetric partitions | -0.8 | -1.8 | -1.6 | 114% | 100% |
| 2.7 OBMC with Motion Merging | -1.5 | -2.4 | -2.4 | 122% | 104% |
| 2.8 OBMC for 2NxN and Nx2N motion partitions | -0.5 | -0.9 | -0.9 | 102% | 101% |
| 2.9 AMP + NSQT + OBMC | -1.4 | -2.6 | -2.5 | 116% | 102% |
| 2.10 OBMC with Motion Merging + NSQT | -1.7 | -3.1 | -3.0 | 128% | 102% |

Low delay B HE

| subtest | Y-BD-rate [%] | U-BD-rate [%] | V-BD-rate [%] | Enc Time [%] | Dec Time [%] |
| --- | --- | --- | --- | --- | --- |
| 2.1 AMP without encoding speed-up | -1.3 | -1.5 | -1.6 | 144% | 99% |
| 2.2 AMP with encoding speed-up | -0.8 | -1.2 | -1.1 | 111% | 100% |
| 2.3 NRMP | -0.8 | -1.1 | -1.1 | 117% | 111% |
| 2.4 NSQT for symmetric partitions | -0.9 | -2.5 | -2.4 | 100% | 101% |
| 2.5 AMP + NSQT for asymmetric partitions | -1.6 | -3.8 | -3.7 | 111% | 101% |
| 2.6 AMP + NSQT for symmetric and asymmetric partitions | -1.9 | -4.0 | -4.1 | 113% | 101% |
| 2.7 OBMC with Motion Merging | -1.8 | -2.5 | -2.6 | 128% | 100% |
| 2.8 OBMC for 2NxN and Nx2N motion partitions | -0.8 | -0.9 | -1.0 | 101% | 103% |
| 2.9 AMP + NSQT + OBMC | -2.9 | -4.8 | -4.9 | 115% | 104% |
| 2.10 OBMC with Motion Merging + NSQT | -2.7 | -4.9 | -5.0 | 132% | 102% |

Random access LC

| subtest | Y-BD-rate [%] | U-BD-rate [%] | V-BD-rate [%] | Enc Time [%] | Dec Time [%] |
| --- | --- | --- | --- | --- | --- |
| 2.1 AMP without encoding speed-up | -0.7 | -0.7 | -0.7 | 151% | 99% |
| 2.2 AMP with encoding speed-up | -0.4 | -0.6 | -0.5 | 112% | 98% |
| 2.3 NRMP | -0.8 | -1.1 | -1.1 | 122% | 111% |
| 2.4 NSQT for symmetric partitions | -0.4 | -0.7 | -0.5 | 101% | 101% |
| 2.5 AMP + NSQT for asymmetric partitions | -0.8 | -1.3 | -1.2 | 113% | 101% |
| 2.6 AMP + NSQT for symmetric and asymmetric partitions | -0.9 | -1.4 | -1.2 | 114% | 101% |
| 2.7 OBMC with Motion Merging | -1.3 | -1.8 | -1.9 | 122% | 107% |
| 2.8 OBMC for 2NxN and Nx2N motion partitions | -0.5 | -0.8 | -0.7 | 101% | 103% |
| 2.9 AMP + NSQT + OBMC | -1.5 | -2.1 | -2.0 | 116% | 104% |
| 2.10 OBMC with Motion Merging + NSQT | -1.7 | -2.4 | -2.4 | 128% | 105% |

Low delay B LC

| subtest | Y-BD-rate [%] | U-BD-rate [%] | V-BD-rate [%] | Enc Time [%] | Dec Time [%] |
| --- | --- | --- | --- | --- | --- |
| 2.1 AMP without encoding speed-up | -0.9 | -0.7 | -0.8 | 150% | 99% |
| 2.2 AMP with encoding speed-up | -0.6 | -0.6 | -0.5 | 111% | 99% |
| 2.3 NRMP | -0.8 | -1.0 | -1.2 | 120% | 111% |
| 2.4 NSQT for symmetric partitions | -1.6 | -2.0 | -2.0 | 101% | 102% |
| 2.5 AMP + NSQT for asymmetric partitions | -1.8 | -2.3 | -2.4 | 112% | 101% |
| 2.6 AMP + NSQT for symmetric and asymmetric partitions | -2.3 | -2.5 | -2.6 | 113% | 102% |
| 2.7 OBMC with Motion Merging | -2.2 | -2.8 | -3.0 | 127% | 103% |
| 2.8 OBMC for 2NxN and Nx2N motion partitions | -0.8 | -0.8 | -1.0 | 99% | 103% |
| 2.9 AMP + NSQT + OBMC | -3.1 | -3.3 | -3.4 | 114% | 105% |
| 2.10 OBMC with Motion Merging + NSQT | -3.8 | -4.6 | -4.8 | 132% | 104% |

# Conclusion

CE2 proponents suggest considering the different proposals for potential introduction into HM.

# References

[1] X. Zheng, P. Bordes, P. Chen, I-K Kim, “Core Experiment 2: Motion Partitioning and OBMC,” Doc. JCTVC-D702, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March 2011.

[2] JCT-VC, “Samsung’s Response to the Call for Proposals on Video Compression Technology”, JCTVC-A124, JCT-VC Meeting, Dresden, DE, April 2010.

[3] X. Zheng, H. Yu (Huawei & HiSilicon), P. Bordes (Technicolor), L. Guo, P. Chen (Qualcomm), “CE2: Test results of non-rectangular motion partitioning (NRMP) with overlapped block motion compensation (OBMC)”, Doc. JCTVC-E373, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011.

[4] L. Guo, P. Chen, R. Joshi, M. Karczewicz, “Overlapped Block Motion Compensation for Regular Block Partitions”, Doc. JCTVC-E304, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011.

[5] Y. Yuan (Tsinghua), X. Zheng (HiSilicon), X. Peng (USTC), L. Liu (HiSilicon), Y. Wang (Tsinghua), X. Cao (Tsinghua), J. Xu (Microsoft), C. Lai (HiSilicon), J. Zheng (HiSilicon), Y. He (Tsinghua), H. Yu (Huawei), “Asymmetric Motion Partition with OBMC and Non-Square TU”, Doc. JCTVC-E376, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011.

[6] C.-C. Chen, C.-L. Lee, W.-H. Peng and H.-M. Hang, (NCTU/ITRI), “CE1: Report of DMVD-based Bi-prediction”, Doc. JCTVC-E154 (m19671), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, March 2011.

[7] Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011.

Annex: list of CE participants

| Participant | Person, Email address |
| --- | --- |
| Technicolor | Philippe Bordes, philippe.bordes@technicolor.com |
| Qualcomm | Peisong Chen, peisongc@qualcomm.com Liwei Guo, liweig@qualcomm.com |
| Huawei/HiSilicon | Xiaozhen Zheng, zhengxiaozhen@huawei.com Haoping Yu, haopingyu@huawei.com |
| Samsung | Il-Koo Kim, ilkoo.kim@samsung.com Jianle Chen, jianle.chen@samsung.com |
| Tsinghua | Yuan Yuan, y-yuan08@mails.tsinghua.edu.cn |
| HHI | Benjamin Bross, benjamin.bross@hhi.fraunhofer.de Martin Winken, Martin.Winken@hhi.fraunhofer.de |
| Intel | Y.-J Chiu, Yi-jen.chiu@intel.com L. Xu, Lidong.xu@intel.com W. Zhang, Wenhao.zhang@intel.com |
| NCTU/ITRI | Wen-Hsiao Peng, pawn@mail.si2lab.org Chun-Chi Chen, cheerchen.cs98g@g2.nctu.edu.tw Chung-Lin Lee, yoyoli.cs97g@nctu.edu.tw |
| TI | Minhua Zhou, zhou@ti.com |
| Microsoft | Jizheng Xu, jzxu@microsoft.com |
| NTT Docomo | T.K. Tan, ttk@pacific.net.sg |
| Motorola | Krit Panusopone, krit@motorola.com |
| Nokia | Jani Lainema, jani.lainema@nokia.com |
| Panasonic | Hisao Sasai, sasai.hisao@jp.panasonic.com |