| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F027_r1 WG11 Number: m20736 |
| --- | --- |

| Title: | CE7: Summary Report of Core Experiment on Additional Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories Chuohao Yeo Institute for Infocomm Research Rajan Joshi Qualcomm Felix Fernandes Samsung Electronics Co., Ltd. | Email: | cohen@merl.com chyeo@i2r.a-star.edu.sg rajanj@qualcomm.com felix.f@sta.samsung.com |
| Source: | CE7 coordinators |  |  |

_____________________________

# Abstract

The purpose of Core Experiment 7 (CE7) is to characterize the performance, in terms of both compression efficiency and complexity, of several transforms other than those currently defined in HM 3.0. Some of these proposed transforms are secondary transforms applied after the HM transforms, and some are used in place of the HM transforms, under certain conditions. Some combinations of the proposed transforms are tested as well. This document, JCTVC-F027, provides a summary of activities and results for this core experiment.

# Introduction

The purpose of Core Experiment 7 (CE7) [1] is to investigate the performance of transforms other than the transforms currently defined in HM (HM 3.0) [2]. The proposals submitted for CE7 along with their corresponding cross-verification documents and any related contributions are summarized in Table 1.

Table 1. CE7-related JCT-VC documents

| Tool # | Proponent (Tool Type) | CE7 Doc. | Cross-verification |  |  | Other related docs. | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Qualcomm Toshiba I2R (8x8 mode-dep. DCT/KLT; Intra) | F138 | F510 Huawei | F281 Samsung |  |  | 2 variants |
| 2 | Samsung (8x8 mode-dep. DCT/DST; Intra) | F282 | F141 I2R |  |  |  |  |
| 3 | Panasonic (Secondary mode dep. transform; Intra) | F224 | F230 NHK | F511 Huawei | F558 Qualcomm | F554 |  |
|  |  |  |  |  | F734 Samsung |  |  |
| 4 | Samsung (R-D selection of secondary rotational transform; Intra) | F294 | F559 Qualcomm |  |  |  |  |
| 5 | NHK (R-D selection of DCT or DST; Intra and Inter) | F229 | F226 Panasonic | F600 MERL |  |  | 4 variants |
| 6 | Samsung (Modified 4x4 mode-dep. DCT/KLT; Intra) | F283 | F141 I2R | F329 Toshiba |  | F153 F553 |  |

# Summary of proposed alternative transforms

The types of proposed transforms and methods generally fall into one or more of the following categories:

- Mode-dependent DCT/DST or DCT/KLT, where the horizontal and vertical transform depends upon the Intra prediction mode. 8x8 versions and improvements to the existing 4x4 version are tested.

- Mode-dependent secondary 4x4 KLT, used horizontally and/or vertically depending upon mode.

- Secondary rotational transform, rate-distortion optimized selection.

- Adaptive selection of 2-D DCT or DST, rate-distortion optimized or coefficient-based selection.

Brief descriptions for each of the proposals are shown in Table 2. For more details, please refer to the document number associated with each tool.

Table 2. Brief descriptions of tools and their variants

| Tool # | Proponent | JCTVCDoc. | Tool Description |
| --- | --- | --- | --- |
| 1 | Qualcomm Toshiba I2R | F138 | (Intra) An 8-point derived KLT is proposed as the additional transform for mode-dependent transform coding of 8x8 intra blocks, using the same Intra prediction mode mapping table already defined for 4x4 blocks in HM3. Two variants: 1. Proposed transform added to HM 3.0, with HM 3.0 as reference. 2. Proposed transform added to SDIP+DST4 software, using SDIP+DST4 as the reference. |
| 2 | Samsung | F282 | (Intra) For 8x8 blocks, this 2-D separable transform applies either the HM 3.0 DCT or DST Type-7 vertically and horizontally. The choice of the transform is dependent on the intra prediction mode. |
| 3 | Panasonic | F224 | (Intra) A mode-dependent secondary transform operates on the low-frequency coefficients output by the HM core transform. The secondary transforms is a 4x4 pseudo-DST and operates on 8x8 and larger blocks. |
| 4 | Samsung | F294 F295 | (Intra) A rate-distortion optimized selection of an 8x8 secondary rotational transform is applied after the HM transform. The secondary transform is applied to the upper-left, 8x8 DCT-coefficient block of 8x8, 16x16 and 32x32 TUs. The implementation uses a lifting factorization of the rotational transform. Two variants: 1. Proposed transform added to HM 3.0, with HM 3.0 as reference. 2. Proposed transform added to SDIP software, using the HM 3.0 SDIP development branch as the reference. |
| 5 | NHK | F229 | (Intra and Inter) A rate-distortion optimized or fast coefficient-based selection of HM DCT or type-II DST is used to select the transform type. The butterfly operations of the horizontal DCT and DST transforms are merged so that the DCT and DST coefficients are output simultaneously. Four variants: 1. R-D selection for 8x8 and larger Intra and Inter TUs; 2. Same as variant 1, except fast selection is used for 16x16 and larger Intra TUs; 3. R-D selection for 4x4 and larger Inter TUs; 4.: R-D selection for 16x16 and larger Inter TUs. Parallel implementations are described as well. |
| 6 | Samsung | F283 | (Intra) A lower-complexity version of the 4x4 (29,55) DST in HM 3.0 is presented in this proposal. The number of multiplications are reduced from 8 for the (29,55) DST to 5 for the proposed (28,56) DST. |

# Summary of Experimental Results

Table 3 summarizes the BD-Rate, encoding time, and decoding time summaries for each tool listed in the CE description. Low-delay results are optional for tools that do not operate on Inter blocks. “N/A” indicates that a given configuration was not part of that particular experiment. For tools with multiple variants, results for each variant are separated by a vertical bar character (|).

Table 3. Performance summaries for all proposed tools

|  | Overall BD-Rate change for the luma (Y) components (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tool | 1 (F138) 8x8 MD DCT/KLT | 2 (F282) 8x8 MD DCT/DST | 3 (F224) MD secondary 4x4 | 4 (F294) RD secondary ROT | 5 (F229) RD or fast 2-D Intra/Inter DST-II | 6 (F283) Reduced-complex. 4x4 DST |
| Reference Config | HM3.0 | HM 3.0 | HM 3.0 | HM3.0 | HM 3.0 | HM 3.0 |
| AI-HE | -0.4 | -0.2 | -0.4 | -0.9 | -0.3 ¦ -0.2 ¦ N/A ¦ N/A | 0.0 |
| AI-LC | -0.2 | 0.0 | -0.3 | -1.0 | 0.4 ¦ 0.5 ¦ N/A ¦ N/A | 0.0 |
| RA-HE | -0.2 | -0.2 | -0.3 | -0.5 | -0.5 ¦ -0.5 ¦ -0.4 ¦ -0.2 | 0.0 |
| RA-LC | -0.1 | 0.0 | -0.2 | -0.6 | 0.1 ¦ 0.1 ¦ -0.2 ¦ -0.1 | 0.0 |
| LD-HE |  |  |  |  | -0.7 ¦ -0.7 ¦ -0.7 ¦ -0.4 |  |
| LD-LC |  |  |  |  | -0.2 ¦ -0.2 ¦ -0.4 ¦ -0.2 |  |
| LD-HE(P) |  |  |  |  | -0.6 ¦ -0.6 ¦ -0.7 ¦ -0.3 |  |
| LD-LC(P) |  |  |  |  | -0.2 ¦ -0.2 ¦ -0.4 ¦ -0.2 |  |

|  | Encoding time as compared to reference (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tool | 1 (F138) 8x8 MD DCT/KLT | 2 (F282) 8x8 MD DCT/DST | 3 (F224) MD secondary 4x4 | 4 (F294) RD secondary ROT | 5 (F229) RD or fast 2-D Intra/Inter DST-II | 6 (F283) Reduced-complex. 4x4 DST |
| Reference Config | HM3.0 ¦ SDIP | HM 3.0 | HM 3.0 | HM3.0 | HM 3.0 | HM 3.0 |
| AI-HE | 101 | 102 | 101 | 125 | 122 ¦ 111 ¦ N/A ¦ N/A | 101 |
| AI-LC | 101 | 100 | 101 | 157 | 139 ¦ 123 ¦ N/A ¦ N/A | 101 |
| RA-HE | 100 | 102 | 100 | 105 | 115 ¦ 114 ¦ 117 ¦ 107 | 102 |
| RA-LC | 100 | 102 | 100 | 105 | 115 ¦ 113 ¦ 115 ¦ 108 | 99 |
| LD-HE |  |  |  |  | 113 ¦ 112 ¦ 115 ¦ 107 |  |
| LD-LC |  |  |  |  | 112 ¦ 111 ¦ 112 ¦ 106 |  |
| LD-HE(P) |  |  |  |  | 119 ¦ 116 ¦ 120 ¦ 109 |  |
| LD-LC(P) |  |  |  |  | 120 ¦ 117 ¦ 120 ¦ 111 |  |
|  | Decoding time as compared to reference (%) |  |  |  |  |  |
| Tool | 1 (F138) 8x8 MD DCT/KLT | 2 (F282) 8x8 MD DCT/DST | 3 (F224) MD secondary 4x4 | 4 (F294) RD secondary ROT | 5 (F229) RD or fast 2-D Intra/Inter DST-II | 6 (F283) Reduced-complex. 4x4 DST |
| Reference Config | HM3.0 | HM 3.0 | HM 3.0 | HM3.0 | HM 3.0 | HM 3.0 |
| AI-HE | 102 | 103 | 101 | 101 | 101 ¦ 101 ¦ N/A ¦ N/A | 100 |
| AI-LC | 102 | 100 | 101 | 100 | 101 ¦ 102 ¦ N/A ¦ N/A | 101 |
| RA-HE | 102 | 102 | 100 | 101 ¦ 101 | 99 ¦ 99 ¦ 99 ¦ 100 | 102 |
| RA-LC | 101 | 104 | 100 | 100 ¦ 100 | 99 ¦ 100 ¦ 99 ¦ 99 | 100 |
| LD-HE |  |  |  |  | 100 ¦ 99 ¦ 100 ¦ 99 |  |
| LD-LC |  |  |  |  | 99 ¦ 98 ¦ 99 ¦ 99 |  |
| LD-HE(P) |  |  |  |  | 100 ¦ 100 ¦ 100 ¦ 100 |  |
| LD-LC(P) |  |  |  |  | 98 ¦ 99 ¦ 98 ¦ 98 |  |

- Performance with SDIP

Although not formally required by CE7, some proponents provided results for their tools when software including short-distance Intra prediction (SDIP) was used as a reference. For Tool 1, the AI-HE and AI-LC BD-Rate changes are -0.5% and -0.4% respectively, and the reference is the HM 3.0 + SDIP + harmonization with DST4 software (so, the DST4 was used for both square and rectangular blocks in the reference). Using that reference as well, Tool 3 results are -0.5% and -0.5% for AI-HE and AI-LC respectively. For Tool 4, the AI-HE, AI-LC, RA-HE, and RA-LC BD-Rate changes are -0.7%, -0.8%, -0.4%, and -0.4% respectively, and the reference is the HM 3.0 development branch  (so, the DST4 was used for square blocks). Encode/decode complexities are similar to those obtained with HM 3.0 as a reference. Tool 3 also provided results compared to SDIP. Please refer to the contributions associated with each tool for more details regarding SDIP.

# Summary of complexity

Tools 1, 2, 5 and 6 propose the use of replacement transforms, while Tools 3 and 4 propose the use of secondary transforms. Furthermore, RDO is used in Tool 4 and portions of Tool 5 to select a set of transforms to be used.

Table 4 below summarizes the breakdown by transform size and operations type of worst-case number of operations for each Tool. For Tools 1, 2, 5 and 6, the worst-case number of operations of the replacement transform is shown. For Tools 3 and 4, the worst-case number of additional operations needed to perform the secondary transform is shown; the “+” in front indicates this. Counts for HM2, HM3 (DCT) and HM3 (DST) are also shown for reference. Note that the operations of the final scaling performed during the inverse transform process are included in the operations count; in particular, the number of additions reported includes the offset before the shift.

Table 4. Inverse transform operations count summary for all tools.

|  | Operations count |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tool Trans-form Size |  | HM2 | HM3 (DCT) | HM3 (DST) | Tool 1 (F138) 8x8 MD DCT/ KLT | Tool 2 (F282) 8x8 MD DCT/ DST | Tool 3 (F224) MD secondary 4x4 | Tool 4 (F294) RD secondary ROT | Tool 5 (F229) RD or fast 2D Intra/Inter DST-II | Tool 6 (F283) Reduced-complex. 4x4 DST |
| 4x4 | Mults | 0 | 48 | 64 |  |  |  |  | +0 ¦ 48 | 40 |
|  | Adds | 80 | 96 | 120 |  |  |  |  | +8 ¦ 96 | 120 |
|  | Shifts | 32 | 32 | 32 |  |  |  |  | +0 ¦ 32 | 56 |
| 8x8 | Mults | 0 | 352 |  | 1024 | 1024 | +128 | +240 | +0 ¦ 352 |  |
|  | Adds | 640 | 576 |  | 1024 | 1024 | +128 | +288 | +32 ¦ 576 |  |
|  | Shifts | 352 | 128 |  | 128 | 128 | +32 | +336 | +0 ¦ 128 |  |
| 16x16 | Mults | 1408 | 3752 |  |  |  | +128 | +240 | +0 ¦ 3752 |  |
|  | Adds | 2624 | 3712 |  |  |  | +128 | +288 | +128 ¦ 3712 |  |
|  | Shifts | 1600 | 512 |  |  |  | +32 | +336 | +0 ¦ 512 |  |
| 32x32 | Mults | 7424 | 21888 |  |  |  | +128 | +240 | +0 ¦ 21888 |  |
|  | Adds | 13440 | 25856 |  |  |  | +128 | +288 | +512 ¦ 25856 |  |
|  | Shifts | 7296 | 2048 |  |  |  | +32 | +336 | +0 ¦ 2048 |  |

# Summary

Tools 1 and 2 are mode-dependent replacements for the 8x8 core transform, for Intra TUs. Tools 3 and 4 are secondary transforms applied for Intra TUs after the core transform, where Tool 3 is mode-dependent, and Tool 4 is selected through R-D optimization. Tool 5 adaptively selects a DCT or DST-II to replace the core transform, for both Intra and Inter TUs, where the selection is either R-D optimized or based upon coefficient metrics. Tool 6 is a replacement for the 4x4 DST of HM 3, reducing the number of multiplies of the 1-D DST by 3.  The changes in compression efficiency and complexity for each of these tools are summarized in Tables 3 and 4, which can be used as a reference for the CE7 summary discussion during the upcoming JCT-VC meeting.

# References

- R. Cohen, C. Yeo, R. Joshi, F. Fernandes, “Description of Core Experiment 7: Additional Transforms”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E707, Geneva, CH, March 2011.

- HM 3.0 software, available at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware

Two implementations were described for Tool 5. In the first, the IDST is implemented as a secondary transform by performing the HM3 IDCT together with coefficients switching and sign alternations. In the second, the IDST is implemented as a replacement transform but with the same number of operations as HM3 IDCT.