| cJoint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F030-r3 MPEG Doc. m20789 |
| --- | --- |

| Title: | CE10: Summary report on core transform design |  |  |
| --- | --- | --- | --- |
| Status: | Input document JCT-VC |  |  |
| Purpose: | CE Report |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala FastVDO LLC Madhukar Budagavi Texas Instruments Arild Fuldseth Cisco Rajan Joshi Qualcomm Il-Koo Kim Samsung Electronics | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-443-455-0394 pankajtva@yahoo.com madhukar@ti.com arilfuld@cisco.com rajanj@qualcomm.com ilkoo.kim@samsung.com |
| Source: | FastVDO LLC, Texas Instruments, Cisco, Qualcomm, Samsung Electronics |  |  |

_____________________________

# 1 Objective

This CE investigates the design of core spatial transforms, and studies the tradeoff between coding efficiency and complexity, return evidence and a recommendation for design selection. The anchors for comparison are the transforms in the HM (from E243).

# Participant list

| Name |  | Affiliation |  | Email |
| --- | --- | --- | --- | --- |
| Pankaj Topiwala |  | FastVDO LLC |  | pankajtva@yahoo.com |
| Madhukar Budagavi Mangesh Sadafale |  | Texas Instruments Texas Instruments |  | madhukar@ti.com mangesh@ti.com |
| Arild Fuldseth |  | Cisco |  | arilfuld@cisco.com |
| Rajan Joshi |  | Qualcomm |  | rajanj@qualcomm.com |
| Il-Koo Kim Jianle Chan |  | Samsung Electronics |  | ilkoo.kim@samsung.com jianle.chen@samsung.com |
| Dzung Hoang |  | Zenverge |  | Dzung.Hoang@Zenverge.com |
| David Flynn |  | BBC |  | davidf@rd.bbc.co.uk |
| Louie Kerofsky |  | Sharp |  | lkerofsky@sharplabs.com |
| Jian Lou |  | Motorola |  | lou@motorola.com |
| Robert Cohen |  | Mitsubishi Electric Research Laboratories |  | cohen@merl.com |
| Tung Nguyen |  | HHI |  | tung.nguyen@hhi.fraunhofer.de |
| Ankur Saxena |  | Samsung Telecommunications America |  | asaxena@sta.samsung.com |
|  | TK Tan |  | NTT DOCOMO, Inc. | tktan.p@rd.nttdocomo.co.jp |

# 2 Description of tools

HM 3.0 Transforms

JCTVC-E243, A. Fuldseth, G. Bjøntegaard (Cisco), M. Sadafale, M. Budagavi (TI), Transform design for HEVC with 16 bit intermediate data representation.

This contribution proposes a set of transform matrices for HEVC, covering all transform sizes from 4x4 to 32x32. The proposed transform design is reported to have the following properties: 16 bit data representation before and after each transform stage (applies to both IBDI off and IBDI on), 16 bit multipliers for all internal multiplications, no need for correction of different norms of basis vectors during quantization/dequantization, all transform sizes above 4x4 can reuse arithmetic operations for smaller transform sizes, and implementations using either pure matrix multiplication or a combination of matrix multiplication and butterfly structures are possible. This experiment will evaluate the coding performance and complexity of the transform.

JCTVC-E277, I.-K. Kim, T. Lee, J. Chen, V. Seregin, Y. Hong, W.-J. Han (Samsung), P. Topiwala (FastVDO), Integer transform based on modified Loeffler’s factorization

This contribution proposes transform based on modified Loeffler’s factorization without lifting scheme. The main features of the proposed transform are reported to include, a) it can be implemented with both factorized butterfly form or matrix multiplication form; b) 4x4, 8x8 and 16x16 transforms are embedded in 32x32 transform. c) input signal and intermediate transpose buffer for inverse transform are represented within 16 bit; d) six scalar values are used to replace de-quantization scaling matrix for all transform size. The proposed transform is reported to support two different forms of implementation with the same performance so that the optimal implementation can be chosen depending on which platform will be used. This experiment will evaluate the coding performance and complexity of the transform.

JCTVC-E353, Pankaj Topiwala (FastVDO), Wei Dai (FastVDO), Ilkoo Kim (Samsung), Core Transforms, Proposal

This contribution proposes integer transforms and several variants, which are reported to offer substantial computational savings with effectively no performance loss.  The current 4-pt and 8-pt transforms are reported to be initially retained, while the 16-pt and 32-pt transforms are reported to be systematically simplified, even including multiplication-free designs. The multiplication-free designs are reported to afford the lowest complexity as well as exact invertibility in limited bitdepth arithmetic, yet have identical performance.  This experiment will evaluate the coding performance and complexity of the transform.

JCTVC-E370, R. Joshi, Y. Reznik, J. Sole, M. Karczewicz (Qualcomm), Scaled orthogonal integer transforms supporting recursive factorization structure

In JCTVC-E370, scaled orthogonal integer transforms were proposed for all block sizes from 4×4 to 32×32 The 16 and 32-point transforms are reported to use 36 and 92 multiplications, respectively. A multiplication-free implementation for the proposed transforms is also reported. This experiment will evaluate the coding performance and complexity of the proposed transforms. Clipping and right shifts to reduce the dynamic range at intermediate stages of the proposed transforms will also be explored..

# 3	CE Proposals

Of these, four core transform proposals were generated:

1. Cisco/TI (HM3.0, F446),

2. Samsung/FastVDO (F251),

3. FastVDO/Samsung (F363),

4. Qualcomm (F352).

These proposed designs offer a variety of transform computational paradigms, from full matrix multiplication to full factorization and totally multiplication-free designs.  All achieve approximately the same RD performance as the HM transforms, but offer complexity reductions in various areas.  These proposals were tested in a variety of ways to measure both coding efficiency and complexity.

# 4  Additional Relevant Documents

The area of core transforms received intense interest, and a large number of documents have been submitted that are directly related to the work area of CE10. These are listed below in numerical order. Part of the activity during the meeting will be to understand these contributions, as no early copies were made available prior to the meeting.

F192. A study on addition of 64x64 transform to HM 3.0

F193. Accuracy improvement of Cisco and TI's transform (JCTVC-E243)

F236.  IDCT pruning and scan dependent transform order

F447.  SIMD optimization of proposed HEVC core transforms.

F545.  Core spatial transform: improved coding efficiency

F592.  Recursive factorization for 16 and 32-point transforms using 4 and 8-point HM 3.0 core transforms

F595.  New results for guaranteeing 16-bit transform dynamic range.

F710.  The result of SIMD implementation for Core Transforms

| # 5 Core Experiment Tasks |  |  |
| --- | --- | --- |

B.1. Coding efficiency - Description

B.1.1. Measure the RD performance of candidate transform designs using common conditions E700 and HM 3.0 software. If a transform design uses clipping etc. on intermediate variables, this clipping should be included in coding efficiency tests.

B.1.2. Also test at low and high QP values.   Provide BD-Rate results for the following QP sets: 1, 5, 9, 13 and 36, 42, 47, 51 to see if there is unusual behavior at these uncommon test points.  Run this test on 2 sec of video to speed up this test. Number of frames to encode is provided in the table below.

|  |  | Number of frames to encode |
| --- | --- | --- |
| S01 | Traffic | 65 |
| S02 | PeopleOnStreet | 65 |
| S03 | Kimono | 49 |
| S04 | ParkScene | 49 |
| S05 | Cactus | 97 |
| S06 | BasketballDrive | 97 |
| S07 | BQTerrace | 129 |
| S08 | BasketballDrill | 97 |
| S09 | BQMall | 129 |
| S10 | PartyScene | 97 |
| S11 | RaceHorses (C) | 65 |
| S12 | BasketballPass | 97 |
| S13 | BQSquare | 129 |
| S14 | BlowingBubbles | 97 |
| S15 | RaceHorses (D) | 65 |
| S16 | Vidyo1 | 129 |
| S17 | Vidyo3 | 129 |
| S18 | Vidyo4 | 129 |

# Proposals and Cross-Checks

| Proponent | Cross-Check Normal Qp | Cross-Check Low/High Qp | Match Success (Normal/Lo-Hi) |
| --- | --- | --- | --- |
| FastVDO/Samsung (F363) | Motorola (F571) | Zenverge ¦ Sharp | Yes ¦ Yes |
| Samsung/FastVDO (F251) | Cisco/TI (F241) | Qualcomm (F560) | Yes ¦ Yes |
| Qualcomm (E352) | Samsung/FastVDO (F643) | Cisco/TI (F243) | Yes ¦ Yes |

# Results Overview

<<Brief Table Overview>>

We have attempted to capture the gist of our experiments in a single table for convenient comparison. This table captures some performance metrics, arithmetic complexity, as well as dynamic range limitation of 16-bit intermediates. We find that all proposals perform as well or better than HM transforms, while there is a significant variation in arithmetic complexity. There are some differences in whether proposals meet the 16-bit intermediate limitation, which has been suggested to be useful; however, this particular feature is not a differentiator as dynamic range limitation can be achieved without noticeable coding performance loss.    We await speed analysis in optimized software. Also, we have postponed hardware optimization for future work.  The number of cascades in a computational paradigm, which indicates how many stages one may expect in a pipelined architecture, remains to be completed, but should be available at this meeting.

## B.1. Results

BD-Rate results for normal and low/high QP are summarized below.

Remarks:

1.  These results confirm that all proposals perform nearly identically in the Normal Qp range of the common conditions, with only very minor variations in R-D performance for the special cases of Low Qp; these were intended mainly as a check for unusual behavior in any case. It is reasonable to deduce that no truly unusual behavior is observed. (In fact, the only notable variation seems to occur for the LC case.)

2. Please note that no visual tests were planned or performed, since with this tight RD performance no visual differences were anticipated.  (If the RD tests had indicated some significant variation, visual testing may have been indicated.)

B.2. Complexity - Description

Analyze the complexity of candidate transform designs using the following parameters:

- Arithmetic operations

- Software execution speed

- Dynamic range

- Parallelizability on SIMD

- Number of sequential operations

- HW

Proponents are encouraged to provide additional complexity analyses voluntarily.

Discussion:

Complexity is multidimensional, and it is important to capture the essence of this space, but at the same time, keep the analysis space limited in size. A number of complexity metrics are listed below, and it was the sense of the group to do more, rather than less, testing.  These tests are constructed to explore suitability of designs in both software and hardware.  Some of these tests are "paper" tests, while others require actual coding and optimization. At this time, some tests are considered required, while others are optional. This is based on the fact that some tests require optimization, in software or hardware, which it may be unreasonable to require proponents to produce.  Nevertheless, it is the intent of the group to conduct even the optional tests, by recruiting volunteers. It is recognized that we are dealing with an exceptionally short meeting cycle (6 weeks), which makes completing all tasks in one meeting cycle challenging.  It is hoped to collect all relevant evidence in at most two meeting cycles.

B.2.1. Arithmetic Operations - Description

Report the number of arithmetic operations needed for implementing the 2D inverse transform, in terms of multiplications, additions, shifts, compares. If the design allows flexibility in the way it is implemented, report the range of arithmetic operations available (for example, methods with and without multiplications).  Include in the count TPE and final rounding if used.  [Paper study]

### B.2.1. Results

Note first that among the various transform of size 4 – 32, the 32-pt transform dominates the complexity, and it is reasonable (for convenience) to focus attention on the complexity of the 32-pt transform.

a. Cisco/TI (F446)

These transforms can be implemented using matrix multiplication, or using a partial butterfly.

Table 7. Number of arithmetic operations for the 2D inverse transform

|  | Matrix Multiplication |  |  |  | Partial Butterfly |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transform size | Mult, 16 bit | Mult, 32 bit | Add | Shift | Mult, 16 bit | Mult, 32 bit | Add | Shift |
| 4x4 | 128 | 0 | 128 | 32 | 48 | 0 | 64 | 32 |
| 8x8 | 1024 | 0 | 1024 | 128 | 352 | 0 | 448 | 128 |
| 16x16 | 8192 | 0 | 8192 | 512 | 2752 | 0 | 3200 | 512 |
| 32x32 | 65536 | 0 | 65536 | 2048 | 21888 | 0 | 23808 | 2048 |

Table 8. Number of arithmetic operations for 2D inverse transforms of size NxN where only the KxK low frequency sub-block is non-zero (partial butterfly structure)

| NxN | KxK | Multiplications (16-bit) | Add | Shift |
| --- | --- | --- | --- | --- |
| 8x8 | 8x8 | 352 | 448 | 128 |
| 8x8 | 4x4 | 132 | 228 | 96 |
| 16x16 | 16x16 | 2752 | 3200 | 512 |
| 16x16 | 8x8 | 1032 | 1512 | 384 |
| 16x16 | 4x4 | 420 | 820 | 320 |
| 32x32 | 32x32 | 21888 | 23808 | 2048 |
| 32x32 | 16x16 | 8208 | 10320 | 1536 |
| 32x32 | 8x8 | 3400 | 5320 | 1280 |
| 32x32 | 4x4 | 1476 | 3060 | 1152 |

b. FastVDO/Samsung Prop 1 (F363)

This proposal reuses the HM 3.0 transforms of size 4 and 8, and results of (a) above apply.  Results for the 16-pt transform are provided.  The 16-pt transform can be implemented with multiplications, with lifting (and multiplications), and with lifting without multiplications.

|  | 16x16 Integer transform | 16x16 Integer Transform+lifting | 16x16 Integer Transform +lifting +Mult-free |
| --- | --- | --- | --- |
| Mults | 1344 | 768 | 0 |
| Adds | 2304 | 2496 | 3776 |
| Shift | 768 | 864 | 2784 |

Transform of size 32 is identical to that of proposal F251, and results of (c) apply.

c. Samsung/FastVDO Prop 2 (F251)

This proposal reuses the 4-pt transform of HM3.0, and results of (a) apply.  Complexity information is provided in a multiplication form of the transform, although a multiplication-free approach (for transforms of size 8-32) is also available.

|  |  | HM 3.0 |  | Proposed |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Matrix multiplication | Partial-butterfly | Matrix multiplication | Partial-butterfly | Full-factorization |
| 8-point | Additions | 896 | 448 | 896 | 448 | 464 |
|  | Multiplications | 1024 | 384 | 1024 | 384 | 176 |
| 16-point | Additions | 7680 | 3200 | 7680 | 3200 | 2592 |
|  | Multiplications | 8192 | 2816 | 8192 | 2816 | 992 |
| 32-point | Additions | 63488 | 23808 | 63488 | 23808 | 14656 |
|  | Multiplications | 65536 | 22016 | 65536 | 22016 | 5568 |

- Qualcomm (F352)

This proposal reuses the HM 3.0 4-pt transform, and results of (a) apply for that case.  A version of transforms with multiplications is provided, although a multiplication-free implementation is also available.

|  |  | HM 3.0 | Proposed transform |
| --- | --- | --- | --- |
| 8-point | Additions | 448 | 416 |
|  | Multiplications | 352 | 192 |
| 16-point | Additions | 3200 | 2304 |
|  | Multiplications | 2752 | 1152 |
| 32-point | Additions | 23808 | 11904 |
|  | Multiplications | 21888 | 5888 |

Remarks:

1. All three proposals (F251, F352, F363) provide transforms that have a multiplication-free implementation.

2. Estimates on the arithmetic complexity of HM 3.0 transforms appear to vary slight between the counts provided in (a) vs. (c) and (d), but these are immaterial.

B.2.2. Software Execution Speed

Develop a software model of candidate (transform + quantization modules) in single-thread C++ compatible with current HM software.  Develop a common test environment for testing all candidates.  In software tests below, run the tests N times, for N sufficiently large to smooth variations (e.g., N = 100K). Report total execution times.

B.2.2.1. IBDI Off

Develop a test input frame of a random noise (9-bit, [-255, 255]), and run N trials of the modules:

[forward transform, quantization, dequantization, inverse transform].

B.2.2.2. IBDI On

Develop a test input frame of a random noise (11-bit, [-1020, 1020]), and run N trials of the modules:

[forward transform, quantization, dequantization, inverse transform].

### B.2.2 Results.

Cisco/TI volunteered to develop a common test environment, which has been distributed during the previous meeting cycle (Geneva).  Individual transform proposals were integrated into the environment at that time.  Test results were not generated, anticipating a more valuable optimized software analysis. This test was not repeated during this cycle.

B.2.3. Dynamic Range Analysis - Description

B.2.3.1. (Optional) Report if the candidate transform design is exactly invertible in fixed-point arithmetic, and if so, at what bitdepth.    [Paper study]

B.2.3.2. In the software module developed for B.2.2., test the dynamic range of the data at four points: prior to dequantization, prior to inverse transform, after first-stage inverse transform, and after second-stage inverse transform.  Example input may be data consisting of +/- 255, either random, or Hadamard-like. (TBD)

### B.2.3. Results

This test was not repeated during this meeting cycle. However, we note that:

1. F595 (Sharp).  This document and its predecessors provide a generic approach which is asserted to guarantee 16-bit intermediate dynamic range in transforms, applicable to all proposed designs. This current document focuses on making the intermediates in the HM transforms conform to 16-bits, and asserts that it can do so with no loss of performance. It is also asserted by example that without their method, the current HM transforms do not actually satisfy 16-bit limitations.

2. F446 (Cisco/TI). This document asserts that for the case of its proposed transforms, the transform buffer is equal to 16 bits and is independent of the internal bit depth of the encoding/decoding process. Other proposal cases are not studied, so a comparison is not available.

B.2.4. Parallelizability on SIMD

Develop an optimized implementation for a target SIMD architecture (e.g., Intel Core i7 platform) for candidate designs [forward transform, quantization, dequantization, inverse transform]. Combine source codes into one environment (machine, OS, compiler, SW package) for comparative testing.

### B.2.4. Results

David Flynn (BBC) volunteered to conduct this test.  Test results are not available at reporting time, but are forthcoming. Meanwhile,

- F710 from Samsung addresses this issue for the core transforms in F251 and F446 (HM). The conclusion, after testing matrix multiply and partial butterfly methods, are that these transforms have nearly identical SIMD complexity.  Other transform proposals, or the study of full factorization, are not provided. These results are not cross-verified.

- F447 from Cisco addresses the SIMD implementability just of the F446 transforms on the latest gen Intel Sandy Bridge (3.5 Ghz) multi-core processor, and finds that for class B sequences, the transform-related load on the decoder appears to be acceptable (0.3 – 5.6%).  A further conclusion of this analysis is that a partial butterfly implementation is more appropriate for SIMD than a matrix multiply.  These results are not cross-verified.

B.2.5. Number of sequential operations

Report the number of cascaded operators (multipliers/adders) in the longest execution path.  [Paper study]

### B.2.5. Results

This is to be reported by individual proponents.

B.2.6. HW [Optional]

B.2.6.1. Develop optimized hardware models (RTL) of candidate transform designs, and determine the silicon area required, where optimization is for a common hardware condition.  Details to be determined.

B.2.6.2. Determine the hardware speed.  From area and speed, power consumption can be estimated. This subtask is intended to be a paper study, given B.2.6.1.

### B.2.6. Results

This task was volunteered by Samsung and TI.  Some tests were conducted by Samsung, and will be reported elsewhere (some details appear in F251).

# Recommendations

- Review input contributions,

- evaluate results, draw conclusions, and

- plan future experiments (including HW-centric analysis) to help narrow-in on a solution.

- Vision: HM2.0 transforms were suboptimal, HM 3.0 used transforms from E243 to push the envelope, and we envision recommending a solution this cycle.

- Analysis: Candidate solutions cover a performance/complexity space which:

  - offer strong and nearly equal RD performance

  - vary in complexity (arithmetic as well as in implementation)

  - some solutions offer a matrix multiplication model, while some offer a multiplication-free model (beyond the lower 4 or 8pt transforms); one solution offers both.

  - some solutions can potentially offer convenience for lossless coding