| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F032_r4 WG11 Number: m20965 |
| --- | --- |

| Title: | CE12: Summary report of core experiment on deblocking filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Xun Guo Byeungwoo Jeon Matthias Narroschke | Email: | andrey.norkin@ericsson.com xun.guo@mediatek.com bjeon@skku.edu matthias.narroschke@eu.panasonic.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

This contribution is a summary report on Core Experiment 12 on deblocking filtering. The goal of proposals is to enhance deblocking filtering techniques in the HEVC Test Model. This is not limited to improving coding efficiency and subjective quality but also reducing complexity and harmonization of the various schemes that are technically feasible. Six proposals have participated in the CE. All the proposals are based on HM3.0 and experiments are performed according to the common test conditions provided in JCTVC-E700. All results are verified by cross-checkers.

# Document list

## Summary document (this document)

- JCTVC-F032 [CE coordinators] CE12: Summary report of core experiment on deblocking filtering.

## Proposals and cross-verification documents

- JCTVC-F118 [Ericsson/MediaTek] CE12: Ericsson's and MediaTek's deblocking filter.

  - Cross-check: JCTVC-F189 [Toshiba] CE12: Verification results of Ericsson/MediaTek's proposal JCTVC-F118

  - Cross-check: JCTVC-F263 [SKT/SKKU] CE12: Cross-verification Results of Ericsson and MediaTek Deblocking Filter (JCTVC-F118) by SKT/SKKU

  - Cross-check: JCTVC-F401 [Panasonic] CE12: Cross-check results of the deblocking filter of MediaTek and Ericsson (JCTVC-F118)

  - Cross-check: JCTVC-F609 [Samsung] CE12: Cross-verification of the Deblocking Filter Proposed by MediaTek and Ericsson (JCTVC-F118)

- JCTVC-F143 [Toshiba/Qualcomm] CE12: Deblocking filter parameter adjustment in slice level

  - Cross-check: JCTVC-F530 [Ericsson] CE12: Cross-verification of proposal E143 by Ericsson

- JCTVC-F258 [SK Telecom/SKKU] CE12: SK Telecom/SKKU Deblocking Filter

  - Cross-check JCTVC-F218 [Sony] CE12: Cross-verification of SK Telecom/SKKU's proposal JCTVC-F258

  - Cross-check JCTVC-F529 [Ericsson] CE12: Cross-check of SKKU deblocking by Ericsson

  - Cross-check JCTVC-F625 [ETRI] CE12: Cross-verification of SKT/SKKU deblocking filter (JCTVC-F258) by ETRI

- JCTVC-F262 [SK Telecom/SKKU] CE12: SK Telecom/SKKU Harmonized Deblocking Filter with Additional Chroma Processing

  - Cross-check: JCTVC-F206 [Microsoft] CE12: Cross-check of harmonized deblocking filter with additional chroma processing (JCTVC-F262)

  - Cross-check: JCTVC-F611 [Samsung] CE12: Cross-verification of SK Telecom/SKKU Harmonized Deblocking Filter with Additional Chroma Processing (JCTVC-F262)

- JCTVC-F191 [Panasonic] CE12: Results for decisions for deblocking

  - Cross-check: JCTVC-F217 [Sony] CE12: Cross-verification of Panasonic's proposal JCTVC-F191

  - Cross-check: JCTVC-F580 [Qualcomm] CE12: Cross Check of Panasonic’s Deblocking Decisions (JCTVC-F191)

- JCTVC-F198 [Microsoft] CE12, Subset 1: Report of Deblocking for Large Size Blocks

  - Cross-check: JCTVC-F264 [SKT/SKKU] CE12: Cross-verification Results of Microsoft Deblocking Filter (JCTVC-F198) by SKT/SKKU

  - Cross-check: JCTVC-F403 [Panasonic] CE12: Cross-check results of the deblocking filter of Microsoft (JCTVC-F198)

  - Cross-check: JCTVC-F633 [Nokia] CE12: Cross-check of deblocking for large size blocks (JCTVC-F198 by Microsoft)

# Cross-verification assignment

Table 1 Cross-verification assignment

| Proponent(s) | Cross-verifiers(s) |
| --- | --- |
| Ericsson/MediaTek (F118) | SKKU, Samsung, Toshiba, Panasonic |
| SKT/SKKU (F258) | Sony, Ericsson, ETRI |
| SKT/SKKU (F262) | Samsung, Microsoft |
| Microsoft (F198) | SKKU, Nokia, Panasonic |
| Panasonic (F191) | Qualcomm, Sony |
| Toshiba, Qualcomm (F143) | Ericsson |

# Deblocking filter modifications

Table 2 Parts that have been modified compared to HM3.0 are marked with an “x”

| Proponent | Ericsson/ MediaTek (F118) | SKKU/SKT (F258) | SKKU/SKT (F262) | Microsoft (F198) | Toshiba/ Qualcomm (F143) | Panasonic (F191) |
| --- | --- | --- | --- | --- | --- | --- |
| Number of lines for block boundary decisions |  |  |  |  |  | x |
| Line-wise filtering decisions (on/off) | x |  |  |  |  | x (only in v2) |
| Decision weak/strong filter |  | x | x | x |  |  |
| Luma filter operation (weak) | x | x | x |  |  |  |
| Luma filter operation (strong) |  |  |  | x |  |  |
| Chroma filter operation | x |  | x |  |  |  |
| Bs derivation | x | x | x |  |  |  |
| Tc values | x | x | x |  | x |  |
| Beta values |  |  |  |  |  |  |
| Optimization of original source code parts |  |  |  |  |  |  |
| Remarks |  |  |  |  |  |  |

The modifications relative to HM3.0 are described below. The contribution numbers in brackets indicate the origin of the tools in the proposals since some contributions are combinations of tools from different proposals.

- JCTVC-F118 [Ericsson/MediaTek] CE12: Ericsson's and MediaTek's deblocking filter.

- Luma filter:

  - Weak filter is modified an has different  values for p1 and q1 [E276]

  - Check for natural edges (line-wise) based on the weak filter  value for p0/q0 [E276]

  - Separate decisions whether to filter one or two pixels on each side of the block boundary (on 8-line edge level) [E276]

- Chroma filter:

  - Add filtering when the block has coefficients (chroma Bs = 2) [E276]

  - Filtering grid size. For 4:2:0, 4x4 chroma grid is used (corresponds to 8x8 luma grid) [E079]

  - Decision whether to filter the boundary on the 4-pel edge level (based on |p0-p1|+|q0-q1| for two central lines) [E079]

  - Modification of chroma filter (delta is multiplied with a constant factor compared to HM3.0) [E079]

- Bs and tc_values:

  - Do not set Bs = 2 for PU boundaries that are not the TU boundaries [E079]

  - tc_offset = 2 [E193].

- JCTVC-F143 [Toshiba/Qualcomm] CE12: Deblocking filter parameter adjustment in slice level

- tc_values:

  - tc_offset (for intra) is adjusted [E193]

  - new_tc_offset is introduced (for all frames) that is transmitted in slice header [E193]

- JCTVC-F258 [SK Telecom/SKKU] CE12: SK Telecom/SKKU Deblocking Filter

- Luma filter:

  - Weak filter is modified. [E417]

  - Separate decisions whether to filter one or two pixels on each side of the block boundary (on 8-line edge level). [E417]

  - Delta or half of delta values are used for p0 and q0 depending on the fact if one or two pixels are filtered. [new]

  - strong filter decision: additional condition for the 8-pel edge is added: the same test as in a test for filtering the block boundary as in HM but shifted 1 pixel away from the block boundary |p3-2*p2+p1|. [new]

- Chroma filter:

  - Add filtering when the block has chroma coefficients (chroma Bs = 2) [E276]

- Bs and tc_values:

  - Bs derivation is modified for intra blocks (Bs for intra depends on the relative intra-prediction direction of the blocks) [E417]

  - tc_offset for intra is either 1 or 3 depending on the Bs strength [E417]

- JCTVC-F262 [SK Telecom/SKKU] CE12: SK Telecom/SKKU Harmonized Deblocking Filter with Additional Chroma Processing

- Luma filter (as in F258):

  - Weak filter is modified. [E417]

  - Separate decisions whether to filter one or two pixels on each side of the block boundary (on 8-line edge level). [E417]

  - Delta or half of delta values are used for p0 and q0 depending on the fact if one or two pixels are filtered. [new]

  - strong filter decision: additional condition for the 8-pel edge is added: the same test as in a test for filtering the block boundary as in HM but shifted 1 pixel away from the block boundary |p3-2*p2+p1|. [new]

- Chroma filter (as in F118):

  - Add filtering when the block has coefficients (chroma Bs = 2) [E276]

  - Filtering grid size. For 4:2:0, 4x4 chroma grid is used (correspond to 8x8 luma grid) [E079]

  - Decision whether to filter the chroma boundary (based on spatial activity) [E079]

  - Modified chroma filter (delta value is down-scaled compared to HM3.0) [E079]

- Bs and tc_values (as in F258):

  - Bs derivation is modified for intra blocks (Bs for intra depends on the relative intra-prediction direction of the blocks) [E417]

  - tc_offset for intra is either 1 or 3 depending on the Bs strength [E417]

- JCTVC-F191 [Panasonic] CE12: Results for decisions for deblocking, Proposal 1

- Luma filter:

  - Individual on/off-decisions for each line/column based on a linear combination of 4 calculated decision values per 8 line/column segment [E251]

- JCTVC-F191 [Panasonic] CE12: Results for decisions for deblocking, Proposal 2

- Luma filter:

  - Individual on/off-decisions for each line/column based on a linear combination of 4 calculated decision values per 8 line/column segment [E251]

  - For the filter on/off decision, a check for natural edges is added using the same functions as in HM3.0 but crossing the block boundary: |p1-2*p0+q0| [new]

- JCTVC-F198 [Microsoft] CE12, Subset 1: Report of Deblocking for Large Size Blocks

- Luma filter:

  - Strong filtering mode (mode B) is introduced for large blocks [E144]

  - A criterion for applying mode B is introduced including block size and calculating the variance of the block based on the block pixel [E144]

  - The information of the block variance is added as a CU parameter [E144]

  - A long filter (filtering of 4 to 12 pixels away from block boundary is used) in proposed mode [E144]

- Experimental results

Based on common test conditions [1], results for all six mandatory test cases shall be provided for CE12 proposals. The following evaluation criteria are used:

- BD-rate relative to the anchor(s)

- Complexity (decoding time measurement)

- Subjective video quality

In the tables below, the following abbreviations are used:

AI – all intra

RA – random access

LD B – low delay (B)

LD P – low delay (P)

## Coding efficiency

Table 3 BD-Rate (Y)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Ericsson/MediaTek (F118) | -1.2 | -1.1 | -1.5 | -1.4 | -1.0 | -1.1 | -1.3 | -1.2 |
| SKT/SKKU (F258) | -1.2 | -1.1 | -1.6 |  | -1.0 | -0.8 | -1.0 |  |
| SKT/SKKU (F262) | -1.2 | -1.2 | -1.7 |  | -1.0 | -0.9 | -1.0 |  |
| Microsoft (F198) | 0.0 | 0.0 | 0.0 |  | 0.0 | 0.0 | 0.0 |  |
| Panasonic (F191) v1 | -0.1 | -0.2 | -0.3 | -0.3 | -0.1 | -0.2 | -0.3 | -0.2 |
| Panasonic (F191) v2 | -0.4 | -0.5 | -1.0 | -0.9 | -0.2 | -0.3 | -0.6 | -0.2 |
| Toshiba/Qualcomm (F143) (a) | -0.4 | -0.1 | 0.0 | 0.0 | -0.2 | -0.1 | 0.0 | 0.1 |
| Toshiba/Qualcomm (F143) (b) | -0.7 | -0.4 | -0.3 | -0.3 | -0.5 | -0.3 | -0.3 | -0.2 |
| Toshiba/Qualcomm (F143) (c) | -0.7 | -0.3 | -0.1 | -0.1 | -0.5 | -0.2 | 0.0 | 0.0 |

Table 4 BD-Rate (U)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Ericsson/MediaTek (F118) | -1.7 | -2.8 | -2.6 | -3.0 | -2.8 | -2.9 | -3.0 | -3.7 |
| SKT/SKKU (F258) | -0.5 | -0.7 | -1.1 |  | -0.6 | -0.3 | -1.1 |  |
| SKT/SKKU (F262) | -1.7 | -2.8 | -2.6 |  | -2.8 | -2.8 | -3.0 |  |
| Microsoft (F198) | 0.0 | -0.1 | 0.0 |  | 0.0 | 0.0 | 0.0 |  |
| Panasonic (F191) v1 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.2 |
| Panasonic (F191) v2 | 0.0 | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Toshiba/Qualcomm (F143) (a) | -0.5 | -0.5 | 0.0 | 0.0 | -0.6 | -0.5 | -0.1 | 0.0 |
| Toshiba/Qualcomm (F143) (b) | -0.5 | -0.6 | -0.1 | -0.1 | -0.6 | -0.8 | -0.4 | -0.2 |
| Toshiba/Qualcomm (F143) (c) | -0.5 | -0.6 | -0.2 | -0.1 | -0.6 | -0.8 | -0.2 | -0.1 |

Table 5 BD-Rate (V)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Ericsson/MediaTek (F118) | -1.7 | -2.6 | -2.7 | -3.2 | -3.1 | -2.8 | -3.2 | -3.4 |
| SKT/SKKU (F258) | -0.5 | -0.6 | -1.2 |  | -0.6 | -0.3 | -1.1 |  |
| SKT/SKKU (F262) | -1.7 | -2.7 | -2.7 |  | -3.1 | -2.7 | -.3.2 |  |
| Microsoft (F198) | 0.0 | 0.1 | 0.1 |  | 0.0 | 0.0 | -0.1 |  |
| Panasonic (F191) v1 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | -0.2 | 0.0 |
| Panasonic (F191) v2 | 0.0 | -0.2 | -0.2 | -0.2 | 0.0 | 0.0 | -0.4 | -0.1 |
| Toshiba/Qualcomm (F143) (a) | -0.5 | -0.4 | -0.1 | -0.2 | -0.6 | -0.5 | -0.2 | 0.0 |
| Toshiba/Qualcomm (F143) (b) | -0.5 | -0.7 | 0.0 | -0.5 | -0.6 | -0.7 | -0.5 | -0.3 |
| Toshiba/Qualcomm (F143) (c) | -0.5 | -0.6 | -0.1 | -0.4 | -0.6 | -0.7 | -0.3 | -0.2 |

## Complexity – relative decoding time against the anchor -

Table 6 Summary of relative decoding time (%) (against HM3.0 anchor)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Ericsson/MediaTek (F118) | 100% | 100% | 100% | 101% | 102% | 103% | 103% | 102% |
| Ericsson/MediaTek (F118) updated code | 100% | 100% | 99% |  | 101% | 102% | 102% |  |
| SKT/SKKU (F258) | 100% | 99% | 99% |  | 101% | 100% | 100% |  |
| SKT/SKKU (F262) | 101% | 101% | 101% |  | 103% | 103% | 103% |  |
| Microsoft (F198) | 99% | 100% | 100% |  | 100% | 100% | 100% |  |
| Panasonic (F191) v1 | 100% | 100% | 101% | 99% | 100% | 100% | 100% | 99% |
| Panasonic (F191) v2 | 100% | 100% | 101% | 100% | 100% | 100% | 101% | 100% |
| Toshiba/Qualcomm (F143) (a) | 100% | 100% | 99% | 101% | 100% | 100% | 99% | 100% |
| Toshiba/Qualcomm (F143) (b) | 100% | 100% | 99% | 101% | 100% | 100% | 99% | 100% |
| Toshiba/Qualcomm (F143) (c) | 100% | 100% | 99% | 101% | 100% | 100% | 99% | 100% |

### Comments from cross-checkers

Ericsson/MediaTek (F118)

- JCTVC-F263 [SKT/SKKU] Cross-checker has verified that there was no mismatch between encoder and decoder YUV files; PSNR and bitrate results were identical to the results provided by the proponents. The slight fluctuation in encoding time measurements in the verification test was due to encoding operated on several PCs, but the decoding time which was measured on a single PC was similar to what the proponents have provided. Subjective quality was also inspected and reported to be similar to the anchor on average.

- JCTVC-F119 [Toshiba] Cross-checker has verified that coding efficiency results are exactly matched with the proponent’s results.  Relative encoding and decoding time are similar to those of proponent’s for both the original code and the improved code.  Verification was performed on several PCs that were identically configured and Turbo Boost and Hyper Threading were disabled so that the clock frequency is kept when the number of threads is smaller than the number of CPU cores. Source code was reviewed and confirmed the consistency with JCTVC-F118. Since the tc offset value adjustment for intra blocks is not included in the original contributions (JCTVC-E079 and JCTVC-E276), the spirit of the cross-checker’s contribution (JCTVC-E193 and JCTVC-F143) can be considered to be incorporated to the proponents’ contribution (JCTVC-F118).

- JCTVC-F401 [Panasonic] Cross-checker has completely cross checked the first software version provided by the proponent. Exactly the same BD-bit-rate results as the proponent are measured. Measured run time ratios of the cross-checker match approximately to those reported by the proponent. For run-time measurements, cross-checker used PCs of identical configurations. Cross-checker has also measured decoder run-time ratios using a revised software version provided by the proponent.

- JCTVC-F609 [Samsung] BDR results are confirmed and no mismatch observed. Cross-verifier reports that it is possible to see more artifacts on edges in one of the studied sequences comparing to the anchor, in particular on static images.

SKT/SKKU (F258)

- JCTVC-F529 [Ericsson] Cross-checker confirms a perfect match for BDR results. The decoding time on a single PC was similar to that of the proponent. Subjective video quality was inspected and found to be similar to the anchor on average.

- JCTVC-F218 [Sony] Cross-checker has verified that the coding efficiency results are perfectly matched with SKT/SKKU’s results. The encoding and decoding time are similar to those of proponent’s results and the differences are due to the simulation environment. The measurement is performed by using cluster PCs with considering the load average of CPU. Though the difference with anchor in subjective quality is visible, it is difficult to say which one is better.

SKT/SKKU (F262)

- JCTVC-F206 [Microsoft] Cross-checker confirms the proponent’s RD results and observes similar encoding/decoding time behaviors.

- JCTVC-F611 [Samsung] BDR results are confirmed and no mismatch observed. Cross-verifier reports that it is possible to see more artifacts on edges in one of the studied sequences comparing to the anchor, in particular on static images.

Microsoft (F198)

- JCTVC-F633 [Nokia] Cross-checker confirmed both objective gains in Intra configurations as well as the reported subjective gains in Kimono and ParkScene sequences.

- JCTVC-F264 [SKT/SKKU] Cross-checkers have reviewed that for both versions of the software, there was no mismatch between encoder and decoder YUV files; PSNR and bitrate results were identical to the results provided by the proponents.

- JCTVC-F403 [Panasonic] Cross-checker has cross checked both versions of software provided by the proponent. Exactly the same BD-bit-rate results as the proponent are measured. Measured run time ratios of the cross-checker match approximately to those reported by the proponent. For run-time measurements, cross-checker used PCs of identical configurations.

Panasonic (F191)

- JCTVC-F217 [Sony] Cross-checker has verified that the coding efficiency results are perfectly matched with Panasonic’s results. The encoding and decoding time are similar to those of proponent’s results and the differences are due to the simulation environment. The measurement is performed by using cluster PCs with considering the load average of CPU. Though the subjective quality is similar to anchor on average, the trailing edges and so on, which seem not to be caused directly, are found in two sequences regarding “Individual Decision Second Derivation at Border”. These aren’t found in the same sequences regarding “PANASONIC_INDIVIDUAL_DECISIONS”.

- JCTVC-F580 [Qualcomm] The BD-rate results match those reported by Panasonic. The encoding times are similar to Panasonic’s, when the variable execution speed of the computing cluster is taken into consideration. The decoding times are measured on a single CPU and they are similar to JCTVC-F191 decoding times.

Toshiba, Qualcomm (F143)

- JCTVC-F530 [Ericsson] Cross-checker confirms that the BDR results exactly match with the proponent’s results for all three proposed variants of tc parameter adjustment. Relative decoding time on a single PC is similar. Source code was reviewed and found consistent with the description in JCTVC-F143. The subjective quality was on average similar to that of the anchor.

# Subjective quality

A subjective quality test (informal viewing by experts) will be held during the Torino meeting.

# Conclusions

In CE12, six proposals are evaluated based on the common test conditions. The results are summarized based on BD-rates, decoding time and modifications to the current HM3.0 deblocking. The informal subjective viewing by experts will be held in Torino in order to assess how the proposals affect the subjective quality.

# References

- F. Bossen, Common test conditions and software reference configurations, JCTVC-E700, Geneva, CH, March 2011.

- A. Norkin, X. Guo, B. Jeon, M. Narroschke, Description of Core Experiment 12 (CE12): Deblocking filtering, JCTVC-E712, Geneva, CH, March 2011.

* Proponents have provided an updated source code that is a bit-exact match of the first version but has shorter decoding time due to better implementation.