| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, Italy, 14-22 July, 2011 | Document: JCTVC-F041 |
| --- | --- |

| Title: | CE8 Subtest 4: Adaptive Loop Filtering Using Two Filter Shapes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Hsan Guermazi Nader Mahdi Mohammed Ali Ben Ayed Michael Horowitz | Email: | faouzi@ebriskvideo.com hsan@ebriskvideo.com nader@ebriskvideo.com medali@ebriskvideo.com michael@ebriskvideo.com |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

This contribution presents the advantages of utilizing the cross 11x5 shape (instead of the cross 19x5 shape) in the Adaptive Loop Filtering part (that is applied to the luminance samples) of the HM3.1-dev-adcs [1] encoder/decoder. We show that the shorter-width cross 11x5 shape reduces the average decoding time by 4.5% while also achieving similar BD-Rate performance levels to those of HM3.1-dev-adcs. Moreover, when AIF is used, the resulting HM-3.0 encoder achieves essentially the same BD-Rate performance of HM3.0, while achieving substantially lower decoding complexity (~4.5% reduction in average decoding times, ~30% lower in memory complexity).

# Introduction

In HM3.1-dev-adcs [1], TI’s 7×5 modified-diamond-shaped and eBrisk’s 19x5 cross-shaped symmetrical filters are used in Adaptive Loop Filtering (ALF) of the reconstructed luminance samples [2]. Such filters are limited to 5 lines and 12 independent coefficients, reducing both decoder memory and decoder computational complexity. The two underlying shapes are shown in Figure 1.

Figure 1 – 7×5 modified-diamond-shaped and 19x5 cross-shaped symmetrical filters

# Proposed Modification: Cross 11x5 Shape

During the Geneva meeting, it was decided that the ALF algorithm [3] that employs TI’s 7×5 modified-diamond- and eBrisk’s 19x5 cross-shaped symmetrical filters be integrated in the latest HM3.x software. Such algorithm was integrated in HM3.1-dev-adcs [1]. To reduce the width of the 19x5 cross-shaped filter, we propose that the cross 11x5 shape be used instead of the cross 19x5 shape. The new 11-width shape is limited to 5 lines, but it also consists of only 8 independent coefficients, reducing appreciably the average decoding time. The new two-shape configuration is shown in Figure 2.

Figure 2 – 7×5 modified-diamond-shaped and 11x5 cross-shaped symmetrical filters

In HM-3.1, an adaptation between Block-based and Region-based Classification is used for ALF [5]. Therefore, the HM-3.1-dev-adcs ALF algorithm consists of three steps as shown in Figure 3. The two parts of the first step, which employs only block-based classification, can be executed in parallel using a set of 7x5 modified-diamond-shaped filters and a set of 11×5 cross-shaped filters (respectively). A decision between the two shapes is then made, and the shape candidate that yields the best rate-distortion trade-offs is selected. The region-based classification is then applied in Step 2 to the filters of the selected shape, and a second rate-distortion-based decision is performed in order to determine the best classification method. Finally, a CU-adaptive decision is performed using the filters of the selected shape (from Step 1) and classification method (from Step 2).

In this proposed technique, the new ALF filtering is applied to only the luminance component, and the same HM-3.1-dev-adcs (HM 3.0) ALF is applied to the chrominance components.

Figure 3 – Proposed HM3.1-dev-adcs ALF Algorithm – Multi-pass

# Simulation Results

The simulation experiments were conducted following the testing conditions in [6]. The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core 2 Quad Q8300 @ 2.50 GHz.

- RAM: 4 Go

- OS:  Windows XP Professional.

Our proposal has been compared against HM3.1-dev-adcs multi-pass ALF algorithm where the modified-diamond 7x5 and cross 19x5 shapes are used as candidates in ALF filtering. The coding efficiency (BD-Rate) numbers for the proposed HM3.1-dev-adcs multi-pass ALF algorithm (using the modified-diamond 7x5 and cross 11x5 shapes) were obtained for the Low Delay P HE (Table 1), Low Delay HE (Table 2), Random Access HE (Table 3) and Intra Only HE (Table 4) profiles. The detailed luminance Y results for all video sequences are shown in Table 5. Those BD-Rate numbers relative to HM 3.1 are shown in Table 6 (Low Delay P HE), Table 7 (Low Delay HE), Table 8 (Random Access HE) and Table 9 (Intra Only HE). The detailed luminance Y results for all video sequences are shown in Table 10.

Our results have been cross-verified by Samsung and the BD rates matched exactly. Run-times are slightly different ().

It is clear from the tables that the cross 11x5 shape yields a good compromise between filter width, computational complexity and BD-Rate performance. Moreover, an informal subjective analysis shows that the original and modified HM3.1-dev-adcs ALF algorithms yield essentially the same video quality. The same analysis also shows that HM3.0’s video quality is similar to that of the modified HM3.1-dev-adcs in the IO/RA HE configurations and is slightly better than that of the modified HM3.1-dev-adcs in the LD/LD P HE configurations.

Table 1 – Simulation Results: Low Delay P HE (D + Cross 11x5 relative to HM3.1-dev-adcs)

|  | Low Delay P HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.06 | 0.16 | 0.05 |
| Class C | 0.06 | 0.20 | 0.08 |
| Class D | -0.04 | 0.49 | -0.34 |
| Class E | -0.14 | -0.67 | 0.18 |
| All | 0.00 | 0.10 | -0.02 |
| Enc Time[%] | 97.09% |  |  |
| Dec Time[%] | 99.12% |  |  |

Table 2 – Simulation Results: Low Delay HE (D + Cross 11x5 relative to HM3.1-dev-adcs)

|  | Low Delay HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.10 | -0.18 | 0.00 |
| Class C | 0.12 | 0.21 | -0.06 |
| Class D | 0.01 | -0.24 | -0.57 |
| Class E | -0.25 | -0.28 | -0.34 |
| All | 0.02 | -0.12 | -0.22 |
| Enc Time[%] | 96.88% |  |  |
| Dec Time[%] | 99.08% |  |  |

Table 3 – Simulation Results: Random Access HE (D + Cross 11x5 relative to HM3.1-dev-adcs)

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.07 | 0.10 | -0.10 |
| Class B | 0.09 | 0.04 | 0.04 |
| Class C | 0.01 | -0.04 | -0.05 |
| Class D | -0.01 | -0.08 | 0.00 |
| Class E |  |  |  |
| All | 0.04 | 0.00 | -0.02 |
| Enc Time[%] | 97.41% |  |  |
| Dec Time[%] | 99.55% |  |  |

Table 4 – Simulation Results: Intra Only HE (D + Cross 11x5 relative to HM3.1-dev-adcs)

|  | Intra Only HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.02 | -0.01 | -0.01 |
| Class B | 0.01 | -0.01 | -0.01 |
| Class C | 0.00 | -0.01 | -0.01 |
| Class D | -0.01 | -0.02 | -0.02 |
| Class E | -0.01 | 0.00 | 0.00 |
| All | 0.00 | -0.01 | -0.01 |
| Enc Time[%] | 95.78% |  |  |
| Dec Time[%] | 100.05% |  |  |

Table 5 – Detailed Results (Luminance Y Only) (D + Cross 11x5; Multi-pass)

|  | Sequences | Intra Only HE | Random Access HE | LowDelay HE | LowDelay P HE |
| --- | --- | --- | --- | --- | --- |
| S01 | Traffic | 0.0 | 0.0 |  |  |
| S02 | PeopleOnStreet | 0.0 | 0.0 |  |  |
| S0X | Nebuta | 0.0 | 0.0 |  |  |
| S0Y | SteamLocomotive | 0.1 | 0.2 |  |  |
| S03 | Kimono | 0.0 | 0.0 | 0.0 | 0.0 |
| S04 | ParkScene | 0.0 | 0.0 | 0.1 | 0.0 |
| S05 | Cactus | 0.0 | 0.1 | 0.2 | 0.1 |
| S06 | BasketballDrive | 0.0 | 0.1 | 0.1 | 0.1 |
| S07 | BQTerrace | 0.0 | 0.2 | 0.2 | 0.1 |
| S08 | BasketballDrill | 0.0 | 0.0 | 0.1 | 0.0 |
| S09 | BQMall | 0.0 | -0.1 | 0.0 | -0.1 |
| S10 | PartyScene | 0.0 | 0.0 | 0.1 | 0.1 |
| S11 | RaceHorses | 0.0 | 0.1 | 0.2 | 0.2 |
| S12 | BasketballPass | 0.0 | 0.0 | -0.1 | 0.0 |
| S13 | BQSquare | 0.0 | -0.1 | 0.0 | 0.0 |
| S14 | BlowingBubbles | 0.0 | 0.1 | 0.1 | -0.2 |
| S15 | RaceHorses | 0.0 | 0.0 | 0.1 | -0.1 |
| S16 | Vidyo1 | 0.0 |  | -0.3 | -0.2 |
| S17 | Vidyo3 | 0.0 |  | -0.3 | 0.0 |
| S18 | Vidyo4 | 0.0 |  | -0.1 | -0.1 |
| Average | Class A | 0.02 | 0.07 |  |  |
|  | Class B | 0.01 | 0.09 | 0.10 | 0.06 |
|  | Class C | 0.00 | 0.01 | 0.12 | 0.06 |
|  | Class D | -0.01 | -0.01 | 0.01 | -0.04 |
|  | Class E | -0.01 |  | -0.25 | -0.14 |
|  | All | 0.00 | 0.04 | 0.02 | 0.00 |

Table 6 – Simulation Results: Low Delay P HE (D + Cross 11x5 relative to HM-3.1)

|  | Low Delay P HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.51 | 0.03 | -0.01 |
| Class C | 0.11 | -0.12 | 0.07 |
| Class D | 0.02 | 0.39 | -0.11 |
| Class E | 0.54 | 0.17 | 0.08 |
| All | 0.29 | 0.11 | 0.00 |
| Enc Time[%] | 100.94% |  |  |
| Dec Time[%] | 95.26% |  |  |

Table 7 – Simulation Results: Low Delay HE (D + Cross 11x5 relative to HM-3.1)

|  | Low Delay HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.53 | 0.05 | 0.36 |
| Class C | 0.18 | -0.10 | -0.18 |
| Class D | 0.16 | -0.21 | -0.21 |
| Class E | 0.75 | 0.58 | -0.23 |
| All | 0.39 | 0.05 | -0.03 |
| Enc Time[%] | 100.69% (100.74%) |  |  |
| Dec Time[%] | 96.02% (94.73 %) |  |  |

Table 8 – Simulation Results: Random Access HE (D + Cross 11x5 relative to HM-3.1)

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.41 | -0.30 | 0.16 |
| Class B | 0.25 | 0.03 | 0.03 |
| Class C | 0.04 | 0.04 | -0.01 |
| Class D | 0.08 | 0.06 | 0.05 |
| Class E |  |  |  |
| All | 0.20 | -0.04 | 0.06 |
| Enc Time[%] | 101.07% (103.38%) |  |  |
| Dec Time[%] | 98.43% (96.75%) |  |  |

Table 9 – Simulation Results: Intra Only HE (D + Cross 11x5 relative to HM-3.1)

|  | Intra Only HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.42 | -0.02 | -0.02 |
| Class B | 0.13 | -0.05 | -0.05 |
| Class C | 0.07 | -0.06 | -0.06 |
| Class D | 0.09 | -0.06 | -0.06 |
| Class E | 0.43 | -1.00 | -0.10 |
| All | 0.22 | -0.05 | -0.05 |
| Enc Time[%] | 102.38% (105.89%) |  |  |
| Dec Time[%] | 93.36% (93.51%) |  |  |

Table 10 – Detailed Y Results (D + Cross 11x5 relative to HM-3.1)

|  | Sequences | Intra Only HE | Random Access HE | LowDelay HE | LowDelay P HE |
| --- | --- | --- | --- | --- | --- |
| S01 | Traffic | 0.4 | 0.6 |  |  |
| S02 | PeopleOnStreet | 0.2 | 0.1 |  |  |
| S0X | Nebuta | 0.7 | 0.6 |  |  |
| S0Y | SteamLocomotive | 0.4 | 0.3 |  |  |
| S03 | Kimono | 0.1 | 0.1 | 0.3 | 0.4 |
| S04 | ParkScene | 0.2 | 0.0 | 0.2 | 0.2 |
| S05 | Cactus | 0.1 | 0.2 | 0.3 | 0.4 |
| S06 | BasketballDrive | 0.1 | 0.2 | 0.4 | 0.5 |
| S07 | BQTerrace | 0.2 | 0.7 | 1.3 | 1.1 |
| S08 | BasketballDrill | 0.4 | 0.2 | 0.7 | 0.5 |
| S09 | BQMall | 0.0 | -0.1 | 0.0 | 0.0 |
| S10 | PartyScene | -0.1 | 0.2 | 0.2 | 0.1 |
| S11 | RaceHorses | -0.1 | -0.2 | -0.1 | -0.1 |
| S12 | BasketballPass | 0.0 | 0.0 | 0.1 | 0.2 |
| S13 | BQSquare | 0.0 | 0.4 | 0.6 | 0.0 |
| S14 | BlowingBubbles | -0.1 | 0.0 | -0.1 | -0.2 |
| S15 | RaceHorses | 0.5 | 0.0 | 0.0 | 0.1 |
| S16 | Vidyo1 | 0.4 |  | 0.2 | 0.3 |
| S17 | Vidyo3 | 0.5 |  | 1.6 | 1.0 |
| S18 | Vidyo4 | 0.4 |  | 0.5 | 0.3 |
| Average | Class A | 0.42 | 0.41 |  |  |
|  | Class B | 0.13 | 0.25 | 0.53 | 0.51 |
|  | Class C | 0.07 | 0.04 | 0.18 | 0.11 |
|  | Class D | 0.09 | 0.08 | 0.16 | 0.02 |
|  | Class E | 0.43 |  | 0.75 | 0.54 |
|  | All | 0.22 | 0.20 | 0.39 | 0.29 |

In order to demonstrate the ability of the proposed technique in decreasing worst-case decoder complexity, an evaluation against the highest-complexity HM-3.1 (i.e., where the ALF employs only 9×7 modified-diamond-shaped filters) has been conducted on Class D & Class C sequences, and in this case, we found that our proposed technique reduces the average worst-case decoding time by approximately 9 % (Table 11).

Table 11 – Decoding times in the worst case

|  | Dec Time[%] – Class D | Dec Time[%] – Class C |
| --- | --- | --- |
| Low Delay P HE | 89% | 90% |
| Low Delay HE | 91% | 92% |
| Random Access HE | 92% | 92% |
| Intra Only HE | 92% | 91% |

When the Adaptive Interpolation Filter (AIF) algorithm of [4] is used in conjunction with the proposed multi-pass ALF algorithm (using the modified-diamond and cross 11x5 shapes), we obtain the BD-rate numbers (relative to HM3.0) shown in Table 12 (Low Delay P HE), Table 13 (Low Delay HE) and Table 14 (Random Access HE). The detailed luminance Y results for all video sequences are shown in Table 15.

Table 12 – Simulation Results: Low Delay P HE (D + Cross 11x5 +AIF relative to HM-3.0)

|  | Low delay P HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.18 | -0.33 | -0.61 |
| Class C | -0.35 | -0.27 | -0.59 |
| Class D | -0.38 | -0.19 | -0.14 |
| Class E | -0.30 | -0.18 | -0.71 |
| All | -0.18 | -0.26 | -0.52 |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 95% |  |  |

Table 13 – Simulation Results: Low Delay HE (D + Cross 11x5 +AIF relative to HM-3.0)

|  | Low Delay HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.27 | 0.00 | -0.09 |
| Class C | -0.05 | -0.10 | -0.22 |
| Class D | 0.03 | 0.17 | 0.12 |
| Class E | -0.01 | 0.10 | -0.17 |
| All | 0.08 | 0.03 | -0.09 |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 95% |  |  |

Table 14 – Simulation Results: Random Access HE (D + Cross 11x5 +AIF relative to HM-3.0)

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.15 | -0.75 | -0.35 |
| Class B | 0.24 | -0.08 | -0.12 |
| Class C | 0.04 | 0.10 | 0.03 |
| Class D | 0.10 | 0.02 | -0.07 |
| Class E |  |  |  |
| All | 0.06 | -0.18 | -0.17 |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 97% |  |  |

Table 15 – Detailed Y Results (D + Cross 11x5 +AIF relative to HM-3.0)

|  | Sequences | Random Access HE | Low Delay HE | Low Delay-P HE |
| --- | --- | --- | --- | --- |
| S01 | Traffic | 0.6 |  |  |
| S02 | PeopleOnStreet | 0.1 |  |  |
| S0X | Nebuta | 0.6 |  |  |
| S0Y | SteamLocomotive | 0.3 |  |  |
| S03 | Kimono | 0.1 | 0.3 | 0.4 |
| S04 | ParkScene | 0.0 | 0.2 | 0.2 |
| S05 | Cactus | 0.2 | 0.3 | 0.4 |
| S06 | BasketballDrive | 0.2 | 0.4 | 0.5 |
| S07 | BQTerrace | 0.7 | 1.3 | 1.1 |
| S08 | BasketballDrill | 0.2 | 0.3 | 0.2 |
| S09 | BQMall | 0.0 | -0.2 | -0.5 |
| S10 | PartyScene | 0.2 | 0.1 | -0.2 |
| S11 | RaceHorses | -0.3 | -0.2 | -0.4 |
| S12 | BasketballPass | 0.0 | 0.0 | -0.1 |
| S13 | BQSquare | 0.4 | 0.6 | -0.4 |
| S14 | BlowingBubbles | -0.1 | -0.3 | -0.7 |
| S15 | RaceHorses | 0.1 | 0.1 | 0.2 |
| S16 | Vidyo1 |  | 0.1 | -0.2 |
| S17 | Vidyo3 |  | 0.8 | 0.1 |
| S18 | Vidyo4 |  | 0.1 | -0.2 |
| Average | Class A | -0.08 |  |  |
|  | Class B | 0.19 | 0.32 | 0.28 |
|  | Class C | 0.04 | -0.01 | -0.19 |
|  | Class D | 0.10 | 0.08 | -0.25 |
|  | Class E |  | 0.34 | -0.12 |
|  | All | 0.07 | 0.18 | -0.07 |

In order to demonstrate the ability of the combination (ALF + AIF) in decreasing worst-case decoder complexity, an evaluation against the highest-complexity HM-3.0 (i.e., where the ALF employs only 9×7 modified-diamond-shaped filters) has been conducted on Class D & Class C sequences, and in this case, we found that our proposed technique reduces the average worst-case decoding time by approximately 9 % (Table 11).

Table 16 – Decoding times in the worst case

|  | Dec Time[%] – Class D | Dec Time[%] – Class C |
| --- | --- | --- |
| Low Delay | 89% | 90% |
| Random access | 91% | 92% |
| Intra Only | 92% | 92% |

# Syntax & Semantics

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_filter_shape_flag | 2 | u(1) |
| alf_nofilters_minus1 | 2 | ue(v) |
| if (alf_noFilters_minus1 == 1) |  |  |
| alf_start_second_filter | 2 | ue(v) |
| else if (alf_noFilters_minus1 > 1) { |  |  |
| for (i=0; i< AlfMaxNumFilters; i++) |  |  |
| alf_filter_pattern[i] | 2 | u(1) |
| } |  |  |
| ..... |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

alf_filter_shape_flag: 1 means that the 11x5 cross-shaped symmetrical filters are used; 0 means that the 7×5 modified-diamond-shaped filters are used.

# Proposed Draft Text

#### 8.6.2.3 Filtering process for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a filter index array of (nS)x(nS), fIdx.

Output of this process is the filtered reconstruction of luma picture.

Let s’ be a variable specifying luma sample array recPictureL.

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfTap is set equal to ( alf_length_luma_minus_5_div2 << 1 ) + 5 and a variable  alfFilterShape equal to (alf_filter_shape_flag == 1).

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, is derived as follows.

recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=0N−1s'[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]	(8-1)
where N is set equal to AlfNumCoeffLuma-1 and horPos[i] and verPos[i] are specified in Error: Reference source not found and Table 8-2, respectively.

Table 8-1 – Specification of horPos[ i ] according to alfFilterShape for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfFilterShape = 0 | -1 | 0 | 1 | -2 | -1 | 0 | 1 | 2 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | -2 | -1 | 0 | 1 | 2 |
| alfFilterShape = 1 | 0 | 0 | -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 | 0 | 0 | - | - | - | - | - |

| i | 20 | 21 | 22 |
| --- | --- | --- | --- |
| alfFilterShape = 0 | -1 | 0 | 1 |
| alfFilterShape = 1 | - | - | - |

Table 8-2 – Specification of verPos[ i ] according to alfFilterShape for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfFilterShape = 0 | -2 | -2 | -2 | -1 | -1 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| alfFilterShape = 1 | -2 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | - | - | - | - | - |

| i | 20 | 21 | 22 |
| --- | --- | --- | --- |
| alfFilterShape = 0 | 2 | 2 | 2 |
| alfFilterShape = 1 | - | - | - |

Figure 8-2 Mapping between geometric position and luma adaptive loop filter index according to alfFilterShape (informative)

# Conclusions

Our simulation results show that the shorter-width cross 11x5 shape reduces the average decoding time (0.5% - 1%) while also achieving similar BD-Rate performance levels to those of HM-3.1-dev-adcs. Moreover, when AIF is used, the modified HM-3.1-dev-adcs ALF algorithm also achieves similar BD-Rate performance to that of HM-3.0, while achieving substantially lower decoding complexity (2% - 7% reduction  in average decoding times, ~30% lower in memory complexity).

# References

[1]	 https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/HM-3.1-dev-adcs/

[2] 	M. Karczewicz, P. Chen, R. Joshi, X. Wang, W. Chien, R. Panchal, “Video coding technology  proposal by Qualcomm Inc.”, JCTVC-A121.

[3] 	F. Kossentini, H. Guermazi, N. Mahdi, MedA BenAyed, M. Budagavi, V. Sze, M. Zhou, “CE8   Subset 5: Results on combination of JCTVC-E342 + JCTVC-E060”, JCTVC-E492.

[4] 	F. Kossentini, N. Mahdi, H. Guermazi, MedA BenAyed, “An Adaptive Interpolation Filtering Technique”, JCTVC-E284.

[5] 	C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I. S. Chong, T. Yamakage, T. Chujoh, T. Watanabe, “CE8 Subtest 2: Block based adaptive loop filter (ALF)”, JCTVC-E323.[6]  F. Bossen, “Common Test Conditions and Software Reference Configurations”, JCTVC-E700.

# Patent rights declaration(s)

eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).