| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, Italy, 14-22 July, 2011 | Document: JCTVC-F042 |
| --- | --- |

| Title: | CE8 Subtest 5: Adaptive Loop Filtering of Luminance and Chrominance Samples Using Same Filtering Shape, Structure and Map |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Hsan Guermazi Nader Mahdi Mohammed Ali BenAyed Michael Horowitz | Email: | faouzi@ebriskvideo.com hsan@ebriskvideo.com nader@ebriskvideo.com medali@ebriskvideo.com michael@ebriskvideo.com |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

This contribution presents a technique for the Adaptive Loop Filtering (ALF) of chrominance samples that employs the same filtering shape, structure (frame-based or CU-based) and map used in ALF of the luminance samples. The proposed technique reduces substantially the average decoding time (~6.5%) while also achieving slightly worse Y BD-Rate (~0.26%) and significantly better U/V BD-Rate (~0.63%-0.60%) performance than those of HM3.0 [1]. Moreover, when AIF is applied to the luminance and chrominance samples, the resulting encoder achieves similar Y (~0.10%) BD-Rate to, and substantially better U/V BD-Rate performance (~1.07%-1.27%) than, that of HM3.0, while achieving substantially lower decoding complexity (~6% reduction in average decoding times, ~30% lower in memory complexity).

# Proposed ALF Technique

In HM3.0, Adaptive Loop Filtering (ALF) employs a 5x5 square filter shape for filtering of the chrominance samples (Figure 1). The same square filter shape is applied to both U & V chrominance samples. A two-bit chroma_idc field is used to signal which of the chroma components is/are filtered. ALF of the chrominance samples is applied only when ALF filtering is applied to the luminance samples. In HM3.0, CU-adaptive filtering is not applied; ALF is applied to a whole frame of the chrominance samples, or it is not applied at all.

Figure 1 – HM3.0 Filter Shape for ALF of Chrominance Samples

In the proposed technique, we employ the same filter shape (one of the two shapes shown in Figure 2) used during ALF of the luminance samples for ALF of the chrominance samples. Moreover, unlike in HM3.0, where the chrominance samples can be only frame-filtered, we apply either frame-based or CU-based ALF as follows (see Figure 3):

- If ALF is not applied to the luminance samples, it is also not applied to the chrominance samples.

- If frame-based ALF is applied to the luminance samples, either ALF is not applied at all, or frame-based ALF is also applied to the chrominance samples.

- If CU-based AIF is applied to the luminance samples, either ALF is not applied at all, or CU-based ALF is also applied to the chrominance samples.

- When CU-based ALF is applied to the chrominance samples, we apply the same CU filter map generated for the luminance samples.

Figure 2 – 7×5 modified-diamond-shaped and 11x5 cross-shaped symmetrical filters

OFF

ON

on

OFF

OFFF

ON

OFF

LUMA

CHROMA

Figure 3 - ALF of the Luminance and Chrominance Samples

The Luma part of the proposed ALF technique is described in [2].

Figure 4 illustrates the proposed ALF technique at the encoder.

Figure 4 – Proposed ALF Technique (Encoder)

# Simulation Results

The simulation experiments were conducted following the testing conditions in [3]. The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core 2 Quad Q8300 @ 2.50 GHz.

- RAM: 4 Go

- OS:  Windows XP Professional.

The coding efficiency (BD-Rate) numbers for the proposed multi-pass ALF Algorithm (using the modified-diamond and cross 11x5 shapes and using the proposed ALF-chroma technique) were obtained for the Low Delay P HE (Table 1), Low Delay HE (Table 2), Random Access HE (Table 3) and Intra Only HE (Table 4) profiles. The detailed Y/U/V results for all video sequences are shown in Table 5. The multi-pass results were cross-verified by Toshiba and the BD rates matched exactly. Run-times are slightly different ().

It is clear from the tables that the proposed ALF technique yields slightly worse Y performance and substantially better U/V performance than HM3.0, while maintaining lower decoder complexity (5% - 7%). Moreover, an informal subjective analysis shows that the proposed technique yields better overall visual quality than that of HM3.0.

Table 1 – Low Delay P HE (D + Cross 11x5 relative to HM3.0)

|  | Low Delay P HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.53 | -1.15 | -1.06 |
| Class C | 0.05 | -1.27 | -1.42 |
| Class D | -0.03 | -0.50 | -0.68 |
| Class E | 0.71 | -1.33 | -2.13 |
| All | 0.30 | -1.05 | -1.25 |
| Enc Time[%] | 100.99% (100.12%) |  |  |
| Dec Time[%] | 93.18% (93.79%) |  |  |

Table 2 – Low Delay HE (D + Cross 11x5 relative to HM3.0)

|  | Low Delay HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.45 | -0.76 | -0.44 |
| Class C | 0.05 | -1.44 | -1.02 |
| Class D | 0.11 | -0.62 | -0.19 |
| Class E | 0.82 | -0.80 | -1.80 |
| All | 0.34 | -0.90 | -0.78 |
| Enc Time[%] | 100.67% (99.70%) |  |  |
| Dec Time[%] | 95.03% (93.54%) |  |  |

Table 3 – Simulation Results: Random Access HE (D + Cross 11x5 relative to HM3.0)

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.40 | -0.30 | -0.22 |
| Class B | 0.21 | -0.20 | -0.06 |
| Class C | 0.03 | -0.81 | -0.42 |
| Class D | 0.08 | -0.68 | -0.58 |
| Class E |  |  |  |
| All | 0.18 | -0.48 | -0.30 |
| Enc Time[%] | 101% (100.95%) |  |  |
| Dec Time[%] | 94.63% (95.32%) |  |  |

Table 4 – Simulation Results: Intra Only HE (D + Cross 11x5 relative to HM3.0)

|  | Intra Only HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.42 | -0.11 | 0.01 |
| Class B | 0.13 | -0.02 | -0.01 |
| Class C | 0.07 | 0.01 | 0.07 |
| Class D | 0.08 | 0.08 | 0.10 |
| Class E | 0.43 | -0.43 | -0.60 |
| All | 0.21 | -0.07 | -0.06 |
| Enc Time[%] | 102.45% (101.78%) |  |  |
| Dec Time[%] | 90.84% (91.58%) |  |  |

Table 5 – Detailed Y/U/V Results (D + Cross 11x5 relative to HM3.0)

|  | Sequences | Intra Only HE | Random Access HE | Low Delay HE | Low Delay P HE |
| --- | --- | --- | --- | --- | --- |
| S01 | Traffic | 0.4 | 0.6 |  |  |
| S02 | PeopleOnStreet | 0.2 | 0.2 |  |  |
| S0X | Nebuta | 0.7 | 0.6 |  |  |
| S0Y | SteamLocomotive | 0.4 | 0.2 |  |  |
| S03 | Kimono | 0.1 | 0.1 | 0.3 | 0.4 |
| S04 | ParkScene | 0.2 | 0.1 | 0.2 | 0.2 |
| S05 | Cactus | 0.1 | 0.2 | 0.1 | 0.3 |
| S06 | BasketballDrive | 0.1 | 0.1 | 0.4 | 0.5 |
| S07 | BQTerrace | 0.2 | 0.6 | 1.3 | 1.3 |
| S08 | BasketballDrill | 0.4 | 0.3 | 0.5 | 0.5 |
| S09 | BQMall | 0.0 | 0.0 | -0.2 | 0.0 |
| S10 | PartyScene | -0.1 | 0.1 | 0.1 | 0.1 |
| S11 | RaceHorses | -0.1 | -0.3 | -0.2 | -0.4 |
| S12 | BasketballPass | 0.0 | 0.0 | 0.1 | 0.1 |
| S13 | BQSquare | 0.0 | 0.4 | 0.5 | -0.3 |
| S14 | BlowingBubbles | -0.1 | -0.1 | -0.2 | -0.1 |
| S15 | RaceHorses | 0.5 | 0.0 | 0.0 | 0.1 |
| S16 | Vidyo1 | 0.4 |  | 0.5 | 0.6 |
| S17 | Vidyo3 | 0.5 |  | 1.3 | 1.1 |
| S18 | Vidyo4 | 0.4 |  | 0.6 | 0.4 |
| Average | Class A | 0.42 | 0.40 |  |  |
|  | Class B | 0.13 | 0.21 | 0.45 | 0.53 |
|  | Class C | 0.07 | 0.03 | 0.05 | 0.05 |
|  | Class D | 0.08 | 0.08 | 0.11 | -0.03 |
|  | Class E | 0.43 |  | 0.82 | 0.71 |
|  | All | 0.21 | 0.18 | 0.34 | 0.30 |

In order to demonstrate the ability of the proposed technique in decreasing worst-case decoder complexity, an evaluation against the highest-complexity HM-3.0 (i.e., where the ALF employs only 9×7 modified-diamond-shaped filters) has been conducted on Class D & Class C sequences, and in this case, we found that our proposed technique reduces the average worst-case decoding time by approximately 10% (Table 6).

Table 6 – Decoding times in the worst case

|  | Dec Time[%] – Class D | Dec Time[%] – Class C |
| --- | --- | --- |
| Low Delay P HE | 88% | 89% |
| Low Delay HE | 90% | 89% |
| Random Access HE | 90% | 91% |
| Intra Only HE | 92% | 91% |

When the Adaptive Interpolation Algorithm (AIF) algorithm is used in conjunction with the proposed multiple-pass ALF algorithm, we obtain the BD-rate numbers (relative to HM3.0) shown in Table 7 (Low Delay P HE), Table 8 (Low Delay HE) and Table 9 (Random Access HE). The detailed luminance Y results for all video sequences are shown in Table 10.

Table 7 – Simulation Results: Low Delay P HE (D + Cross 11x5 + AIF relative to HM-3.0)

|  | Low delay P HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.38 | -1.44 | -1.52 |
| Class C | -0.19 | -1.63 | -1.68 |
| Class D | -0.18 | -1.06 | -0.84 |
| Class E | -0.02 | -1.93 | -3.17 |
| All | 0.00 | -1.51 | -1.80 |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 94% |  |  |

Table 8 – Simulation Results: Low Delay HE (D + Cross 11x5 + AIF relative to HM-3.0)

|  | Low delay HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.30 | -0.75 | -0.61 |
| Class C | -0.05 | -1.52 | -1.45 |
| Class D | 0.05 | -0.44 | -0.76 |
| Class E | 0.40 | -1.23 | -2.50 |
| All | 0.18 | -0.99 | -1.33 |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 94% |  |  |

Table 9 – Simulation Results: Random Access (D + Cross 11x5 + AIF relative to HM-3.0)

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.17 | -0.26 | -0.10 |
| Class B | 0.25 | -0.15 | 0.04 |
| Class C | 0.03 | -0.81 | -0.42 |
| Class D | 0.08 | -0.68 | -0.58 |
| Class E |  |  |  |
| All | 0.12 | -0.71 | -0.67 |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 95% |  |  |

Table 10 – Detailed Results (Luminance Y Only) (D + Cross 11x5 + AIF relative to HM-3.0)

|  | Sequences | Random Access HE | Low Delay HE | Low Delay P HE |
| --- | --- | --- | --- | --- |
| S01 | Traffic | 0.6 |  |  |
| S02 | PeopleOnStreet | 0.2 |  |  |
| S0X | Nebuta | 0.6 |  |  |
| S0Y | SteamLocomotive | 0.2 |  |  |
| S03 | Kimono | 0.1 | 0.3 | 0.4 |
| S04 | ParkScene | 0.1 | 0.2 | 0.2 |
| S05 | Cactus | 0.2 | 0.1 | 0.3 |
| S06 | BasketballDrive | 0.1 | 0.4 | 0.5 |
| S07 | BQTerrace | 0.6 | 1.3 | 1.3 |
| S08 | BasketballDrill | 0.3 | 0.3 | 0.3 |
| S09 | BQMall | 0.1 | -0.3 | -0.5 |
| S10 | PartyScene | 0.3 | 0.0 | -0.2 |
| S11 | RaceHorses | -0.5 | -0.3 | -0.4 |
| S12 | BasketballPass | 0.2 | -0.1 | 0.0 |
| S13 | BQSquare | 0.6 | 0.5 | -0.3 |
| S14 | BlowingBubbles | 0.2 | -0.2 | -0.6 |
| S15 | RaceHorses | 0.3 | 0.0 | 0.2 |
| S16 | Vidyo1 |  | 0.2 | 0.0 |
| S17 | Vidyo3 |  | 0.8 | 0.1 |
| S18 | Vidyo4 |  | 0.2 | -0.2 |
| Average | Class A | 0.17 |  |  |
|  | Class B | 0.25 | 0.30 | 0.38 |
|  | Class C | 0.03 | -0.05 | -0.19 |
|  | Class D | 0.08 | 0.05 | -0.18 |
|  | Class E |  | 0.40 | -0.02 |
|  | All | 0.12 | 0.18 | 0.00 |

In order to demonstrate the ability of the combination (ALF + AIF) in decreasing worst-case decoder complexity, an evaluation against the highest-complexity HM-3.0 (i.e., where the ALF employs only 9×7 modified-diamond-shaped filters) has been conducted on Class D & Class C sequences, and in this case, we found that our proposed technique reduces the average worst-case decoding time by approximately 10% (Table 11).

Table 11 – Decoding times in the worst case

|  | Dec Time[%] – Class D | Dec Time[%] – Class C |
| --- | --- | --- |
| Low Delay P HE | 88% | 89% |
| Low Delay HE | 90% | 89% |
| Random Access HE | 90% | 91% |

# Syntax & semantics

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_length_luma_minus_5_div2 | 2 | ue(v) |
| alf_filter_shape_flag | 2 | u(1) |
| alf_no_filters_minus1 | 2 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |  |
| alf_start_second_filter | 2 | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |  |
| for (i=0; i< 16; i++) |  |  |
| alf_filter_pattern[i] | 2 | u(1) |
| } |  |  |
| if (AlfNumFilters > 1) |  |  |
| alf_pred_method | 2 | u(1) |
| alf_min_kstart_minus1 | 2 | ue(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |  |
| alf_golomb_index_bit[i] | 2 | u(1) |
| for (i=0; i< AlfNumFilters; i++) |  |  |
| for (j=0; j< AlfCodedLengthLuma; j++) |  |  |
| alf_coeff_luma[i][j] |  | ge(v) |
| alf_chroma_idc | 2 | ue(v) |
| if ( alf_chroma_idc ) { |  |  |
| alf_length_chroma_minus_5_div2 | 2 | ue(v) |
| for( i = 0; i< AlfCodedLengthChroma; i++ ) |  |  |
| alf_coeff_chroma[i] |  | se(v) |
| } |  |  |
| alf_cu_control_flag | 2 | u(1) |
| if ( alf_cu_control_flag ) { |  |  |
| alf_cu_control_max_depth | 2 | ue(v) |
| if ( alf_cu_control_max_depth ) |  |  |
| alf_length_cu_control_info | 2 | u(v) |
| for( i = 0; i< NumAlfCuFlag; i++ ) |  |  |
| alf_cu_flag [i] | 2 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

alf_filter_shape_flag: 1 means that the 11x5 cross-shaped symmetrical filters are used; 0 means that the 7×5 modified-diamond-shaped filters are used.

# Proposed draft text

#### 8.6.2.3 Filtering process for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a filter index array of (nS)x(nS), fIdx.

Output of this process is the filtered reconstruction of luma picture.

Let s’ be a variable specifying luma sample array recPictureL.

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfTap is set equal to ( alf_length_luma_minus_5_div2 << 1 ) + 5 and a variable  alfFilterShape equal to (alf_filter_shape_flag == 1).

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, is derived as follows.

recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=0N−1s'[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]	(8-1)
where N is set equal to AlfNumCoeffLuma-1 and horPos[i] and verPos[i] are specified in Error: Reference source not found and Table 8-2, respectively.

Table 8-1 – Specification of horPos[ i ] according to alfFilterShape for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfFilterShape = 0 | -1 | 0 | 1 | -2 | -1 | 0 | 1 | 2 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | -2 | -1 | 0 | 1 | 2 |
| alfFilterShape = 1 | 0 | 0 | -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 | 0 | 0 | - | - | - | - | - |

| i | 20 | 21 | 22 |
| --- | --- | --- | --- |
| alfFilterShape = 0 | -1 | 0 | 1 |
| alfFilterShape = 1 | - | - | - |

Table 8-2 – Specification of verPos[ i ] according to alfFilterShape for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfFilterShape = 0 | -2 | -2 | -2 | -1 | -1 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| alfFilterShape = 1 | -2 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | - | - | - | - | - |

| i | 20 | 21 | 22 |
| --- | --- | --- | --- |
| alfFilterShape = 0 | 2 | 2 | 2 |
| alfFilterShape = 1 | - | - | - |

Figure 8-1 Mapping between geometric position and luma adaptive loop filter index according to alfFilterShape (informative)

#### Filtering process for chroma samples

Inputs of this process are:

–	a chroma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

–	a variable cIdx specifying the chroma component index.

Output of this process is the filtered reconstruction of chroma picture.

Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture and recFiltPicture represents the filtered reconstruction of chroma picture recFiltPictureCb.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture and recFiltPicture represents the filtered reconstruction of chroma picture recFiltPictureCr.

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfTapChroma is set equal to ( alf_length_chroma_minus_5_div2 << 1 ) + 5 and a variable  alfFilterShape equal to (alf_filter_shape_flag == 1)..

Filtered samples of chroma picture recFiltPicture[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, are derived as follows.

recFiltPicture[xC+x][yC+y]=cC[N]+i=0N−1s'[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]	(8-2)
where

N = AlfNumCoeffChroma – 1,		(8-3)

and horPos[i] and verPos[i] are specified in Error: Reference source not found and Table 8-2, respectively.

horPos[ i ] = ( i %  alfTapChroma ) – ( alfTapChroma >> 1 ), and	(8-4)

verPos[ i ] = ( i /  alfTapChroma ) – ( alfTapChroma >> 1 )	(8-5)

# Conclusions

We presented a technique for the Adaptive Loop Filtering (ALF) of chrominance samples that employs the same filtering shape, structure (frame-based or CU-based) and map used in ALF of the luminance samples. The proposed technique reduces substantially the average decoding time (~6.5%) while also achieving slightly worse Y BD-Rate (~0.26%) and significantly better U/V BD-Rate (~0.63%-0.60%) performance than those of HM3.0. Moreover, when AIF is applied to the luminance and chrominance samples, the resulting encoder achieves similar Y (~0.10%) BD-Rate to, and substantially better U/V BD-Rate performance (~1.07%-1.27%) than, that of HM3.0, while achieving substantially lower decoding complexity (~6% reduction in average decoding times, ~30% lower in memory complexity).

# Reference

[1]	 https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0/

[2] 	F. Kossentini, H. Guermazi, N. Mahdi, MedA BenAyed, M. Horowitz, “CE8 Subtest 4: Adaptive Loop Filtering Using Two Filter Shapes”, JCTVC-F041.

[3]  F. Bossen, “Common Test Conditions and Software Reference Configurations”, JCTVC-E700.

# Patent rights declaration(s)

eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).