| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F044 |
| --- | --- |

| Title: | Improving Deblocking-filter Performance with SKIP-CU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mangesh Sadafale Texas Instruments Inc., India | Tel: Email: | mangesh@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

Encode/Decode time is very important factor in video processing. The time required for video processing is directly proportional to average number of computation and memory access done per LCU. We know by increasing computation and memory access we will increase video quality. If we consider “clocks/LCU” as performance matrix, then video quality is inversely proportional to performance. Challenge is to decrease computation and memory access to increase performance without affecting the video quality.

# Problem Statement

Deblocking-filter is used to remove block artifacts that get introduced by the block-based transforms used in video compression. But, there are certain conditions where transforms are not required. So we should be able to avoid Deblocking-filter when transforms are not applied. I_PCM is one case where there is no transform and so we do not need to apply Deblocking-filter by making qP=0. SKIP is a condition where transform are not applied and TU-Size and “qP” is don’t care. But, for SKIP-CU we use “qP” same as left-CU and apply Deblocking-filter with it. Also, for SKIP-CU all “CBF” are zero. So, we will have to read huge data of Reference-Picture Index and Motion-Vector from memory to decide “BS” value between “1” and “0”. Refer  Figure 1 for more detail.

In SKIP-CU, there is no transform and hence no block artifacts. But, we, still, apply Deblocking-filter and access memory to decide BS value. As a result we are doing lot of computation and memory accesses which can be avoided.

Figure 1 :BS calculation in HM-3.x

# Proposal

In the proposed solution we make Boundary-Strength (BS) calculation dependent on SKIP-flag. As given in Figure 2 if edge does not belong to INTRA-block, we first check if block (P and Q) of an edge belongs to SKIP-block. If both P and Q blocks belong to SKIP-block then we will not apply Deblocking filter by making BS=0. As a result, we don’t need to apply Deblocking filter to any edge inside the SKIP-CU. If the adjacent CU is SKIP then we don’t need to apply filter at that CU-boundary also.

//-- Set BS for not Intra MB : BS = 2 or 1 or 0

if ( !pcCUP->isIntra(uiPartP) && !pcCUQ->isIntra(uiPartQ) )

#if DEBLK_FILTER_SKIP

if ( pcCUP->isSkipped(uiPartP) && pcCUQ->isSkipped(uiPartQ) ) {

uiBs = 0;

} else

#endif

{

BS computed value in proposed solution will vary with HM-3.0 when following conditions are true.

- Q-Block and P-Block are SKIP

- Q-Block and P-Block belongs to different Predication Unit (PU-edge)

- Q-Block and P-Block Reference-Picture Index.

  - Are Different (BS = 1)

  - If Same, Q-Block and P-Block MV difference is greater than 4.

Deblocking in HM is done at frame Level as edge-by-edge filtering. The software and hardware implementation in product will be using LCU or CU-based processing as frame based processing needs huge on-chip memory and corresponding memory access in frame-based processing. So, in final product the gain will be huge as we can skip all filters in CU just by looking at Skip-flag. We know in conference and surveillance application we will have huge number of SKIP in P and B frame.

With this change we will save/reduce good amount of computation and memory access in P and B frame. This will result in performance improvement without affecting BD-rates. This will also improve power consumption, which is very important in battery operated application.

As a result we improve performance and power by avoiding:

- Filter computation for all internal edges of SKIP-CU.

- Filter computation for SKIP-CU edges.

- Memory accesses to read Reference-Picture index and motion vector of SKIP-CU.

Figure 2 : Proposed Changes in HM-3.x

# Required Changes in Working Draft

#### Derivation process of boundary filtering strength

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process is an array of size (2)x(nS)x(nS), bS specifying the boundary filtering strength.

Let ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CUSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 4 and yEj+1 = yEj + 4.

For ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1, the following applies.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk ][ yC + yEj – 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 1.

- Otherwise, if verEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk – 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 0.

- Depending on the value of filterDir, the variable bS[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the block edge is also a coding unit edge and the following condition is true, the variable bS[ 0 ][ xEk ][ yEj ] is set equal to 4.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 3.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.

- The sample p0 or q0 is in a coding unit coded with SKIP.

- Otherwise, if the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

NOTE – The determination of whether the reference pictures used for the two prediction are the same or different is based on which pictures are referenced, without regard to whether a prediction is formed using an index into list 0 or an index into list 1, and also without regard to whether or not the index position within a reference picture list is different or not.

- One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples.

- [Ed.: (WJ) needs to be checked again whether this condition covers all 2-motion cases] Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition:

- The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to 4 in units of quarter luma samples.

- Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.

# Simulation Results

## In HM-3.0

## In HM-3.2

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).