| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F045 |
| --- | --- |

| Title: | Early Termination of CU Encoding to Reduce HEVC Complexity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ryeong Hee Gweon Yung-Lyul Lee Jeongyeon Lim | Tel: Email: Tel: Email: | +82-2-3408-3753 rhkwon@dms.sejong.ac.kr yllee@sejong.ac.kr +82-31-710-5196 jeongyeon@sktelecom.com |
| Source: | Sejong university and SK Telecom, Rep. of Korea |  |  |

_____________________________

# Abstract

Comparing to AVC, the computational complexity in HEVC has been increased due to newly added efficient coding tools. One reason of the high complexity of HEVC is that every Prediction Unit such as 2Nx2N, 2NxN, Nx2N and NxN is encoded regardless of the performance of the previous encoded Prediction Unit. By using the signaling information coded_block_flag in the HEVC syntax, the Prediction Unit which could have the best performance can be predicted. Therefore, this contribution introduces a method to reduce encoding complexity of HEVC by simply investigating the coded_block_flag. The encoder complexity of the proposed method is reduced to about 58.5% with Y BD-rate loss of 0.85 compared with that of the HM3.2 encoder.

# Introduction

When a CU is encoded in an Inter picture, the RD costs for total 6 PUs, Inter 2Nx2N, Inter 2NxN, Inter Nx2N, Inter NxN, Intra 2Nx2N and Intra NxN, are examined. And the RD costs for Inter NxN and Intra NxN are examined only for 8x8 CU.

In this proposal, if cbf of an Inter PU except Inter NxN PU in a CU is zero (cbf=0) for luma and two chromas (cbf_luma, cbf_u, cbf_v), the next PU encoding process of the CU is terminated.

According to the HM 3.2, PUs for Intra mode are encoded after all four Inter PUs are encoded. Encoding of these Intra mode PUs are also skipped when cbf of the previously encoded inter PU with the lowest RD cost is zero(0) in order to avoid the computation of Intra coding.

# Proposed changes

To speed up the HM3.2 encoder, the process of encoding a CU can be early terminated. That is, the process of encoding a CU is terminated after encoding a PU if cbf of the current PU except Inter NxN PU is zero for luma and two chromas(cbf_luma, cbf_u, cbf_v).

If cbf is zero(0) after encoding a PU for luma and two chromas, the next PU encoding process of the CU is terminated. The coding order of PUs within a CU is Inter 2Nx2N, Inter 2NxN, Inter Nx2N, Inter NxN, Intra 2Nx2N, Intra NxN. However, the early termination on Inter NxN PU is not taken into account.

As a reference, if cbf=0 of Inter NxN PU is considered in the proposed method, the encoder complexity of the proposed method is reduced to about 58.8% with Y BD-rate loss of 1.0 compared with that of the HM3.2 encoder.

# Simulation Results

The experimental results comparing speed and rate-distortion performance between the proposed method and the HM3.2 anchor are summarized in Table 1. The experimental results comparing speed and rate-distortion performance between the proposed method considering cbf=0 of Inter NxN PU setting and the HM3.2 anchor are summarized in Table 2. Detailed results are included in the attachment (JCTVC-F045-Table1_r1.xls) and (JCTVC-F045-Table2_r1.xls).

Table 1 Rate-distortion and speed performance comparisons of the proposed method where cbf=0 of NxN PU is not considered

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.56 | 0.01 | 0.02 | 0.74 | 0.12 | -0.04 |
| Class B | 0.50 | -0.06 | -0.07 | 0.73 | 0.13 | 0.02 |
| Class C | 0.88 | 0.18 | 0.30 | 0.89 | 0.15 | 0.27 |
| Class D | 1.13 | -0.14 | 0.08 | 1.01 | 0.04 | 0.14 |
| Class E |  |  |  |  |  |  |
| All | 0.75 | -0.01 | 0.07 | 0.84 | 0.11 | 0.10 |
| Enc Time[%] | 55% |  |  | 57% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.54 | 0.17 | 0.11 | 0.74 | 0.23 | 0.25 |
| Class C | 0.89 | 0.42 | 0.40 | 0.73 | 0.36 | 0.58 |
| Class D | 1.19 | 0.28 | 0.27 | 0.94 | 0.70 | 0.95 |
| Class E | 0.71 | 0.50 | 0.23 | 1.15 | 0.47 | 0.11 |
| All | 0.82 | 0.33 | 0.24 | 0.86 | 0.43 | 0.48 |
| Enc Time[%] | 60% |  |  | 52% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay P |  |  | Low delay LoCo P |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.73 | -0.19 | 0.31 | 0.84 | 0.27 | 0.32 |
| Class C | 0.88 | 0.41 | 0.31 | 0.83 | 0.46 | 0.50 |
| Class D | 1.11 | 0.25 | 1.05 | 0.96 | 0.32 | 0.39 |
| Class E | 0.95 | 0.35 | -0.39 | 1.10 | 0.75 | 0.21 |
| All | 0.90 | 0.17 | 0.36 | 0.91 | 0.42 | 0.36 |
| Enc Time[%] | 67% |  |  | 60% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2 Rate-distortion and speed performance comparisons of the proposed method where cbf=0 of NxN PU is considered

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.63 | 0.11 | 0.13 | 0.84 | 0.20 | 0.13 |
| Class B | 0.57 | -0.02 | 0.08 | 0.78 | 0.16 | 0.06 |
| Class C | 1.04 | 0.43 | 0.50 | 1.07 | 0.32 | 0.49 |
| Class D | 1.32 | 0.23 | 0.34 | 1.13 | 0.19 | 0.18 |
| Class E |  |  |  |  |  |  |
| All | 0.87 | 0.18 | 0.25 | 0.94 | 0.21 | 0.21 |
| Enc Time[%] | 61% |  |  | 57% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.63 | 0.42 | 0.35 | 0.81 | 0.24 | 0.18 |
| Class C | 1.06 | 0.53 | 0.57 | 0.88 | 0.43 | 0.73 |
| Class D | 1.44 | 0.48 | 0.73 | 1.07 | 0.59 | 0.95 |
| Class E | 0.92 | 0.17 | 0.60 | 1.25 | -0.04 | 0.30 |
| All | 0.99 | 0.42 | 0.55 | 0.98 | 0.32 | 0.54 |
| Enc Time[%] | 61% |  |  | 56% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

|  | Low delay P |  |  | Low delay LoCo P |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.80 | 0.10 | 0.26 | 0.95 | 0.42 | 0.21 |
| Class C | 1.11 | 0.47 | 0.50 | 0.98 | 0.52 | 0.64 |
| Class D | 1.44 | 0.38 | 1.14 | 1.18 | 0.08 | 0.53 |
| Class E | 1.22 | 0.36 | -0.10 | 1.45 | 0.70 | 0.42 |
| All | 1.12 | 0.31 | 0.47 | 1.11 | 0.41 | 0.44 |
| Enc Time[%] | 67% |  |  | 61% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

The encoder computational complexity of the proposed method is reduced to about 58.5% with Y BD-rates loss of 0.85 compared with that of the HM3.2 encoder. The proposed method will help the speed up of HEVC experiments.

# References

- Frank Bossen, “Common test conditions and software reference configurations,” JCT-VC document, JCTVC-E700, Jan. 2011

- “HEVC Specification Working Draft 3,” JCT-VC document, JCTVC-E603, Mar. 2011.

- http://hevc.kw.bbc.co.uk/trac/browser/jctvc-hm/tags/HM-3.2

Sejong University and SK telecom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).