| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F047 |
| --- | --- |

| Title: | Modifications of in-loop filter based on non-local means filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Mtsumura Yukihiro Bandoh Seishi Takamura Hirohisa Jozawa 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This contribution reports the performance of a technique that utilizes an adaptive denosing filter as the in-loop filter of HM codec. In the proposed method, a denoising filter called non-local means (NLM) filter is added in the in-loop filer process of the encoder/decoder in addition to original in-loop filter of HM3.0. The proposal was implemented in HM3.0 software (for High Efficiency) to evaluate its performance. Compared to the anchor of HM3.0, the average BD-rate gains were “intra (Y: –1.0%, U: –0.9 % and V: –1.2 %)”, “random access (Y:–1.0 %, U: –2.2% and V: –2.1 %)” and “lowdelay B(Y: – 1.2 %, U: –2.8 % and V: –3.3 %)”, and “lowdelay P (Y: –1.3 %, U: –3.2 % and V: –3.9 %)”, respectively. And the average decoding time increased 7..11%. The maximum gain was about “lowdelay P (Y: –1.5 %, U: –9.2 % and V: –8.8 %)” for the sequence “BasketballDrill”.

# Introduction

The in-loop filter that supports to reduce the distortion of a decoded picture is one of important coding tools for improving coding efficiency of HM. HM3.0 adopts two kinds of in-loop filter; deblocking filter and adaptive loop filter (ALF).

Proposed in-loop filter reduces the distortion by combination (block-based adaptive filtering) of non-local means (NLM) filter [1] and deblocking filter. NLM filter applies to not only few boundary pixels of each block but also inner pixels of each block. And for denoising of the decoded picture, NLM filter derives the filter coefficients of each neighboring samples on-the-fly though ALF derives them explicitly. By derivation of the filter coefficients suitable for since NLM filter cofficients are locally adapted, favorable denoising result was obtained [1]. However, one of the problems of NLM filter is its high-complexity. When NLM filter is implemented into HM decoder, the maximum gain of NLM filter was (Y: –4.1%, U: –8.8% and V: –8.9%). However, the decoding time increased 52% (for intra slice) and 24..31% (for inter slice) were reported in JCTVC-E206 [2].

This contribution addresses the decoding complexity by following techniques:

–	Reduction of the number of tap elements according to edge direction (described in 2.2.1)

–	Restricting the template size according to local activity (described in 2.2.2)

–	Restriction of the number of NLM-filtered block (described in 2.2.3)

In this contribution, Section 2 contains conventional/enhanced non-local means filter algorithm and overview of the encoding/decoding. The syntax and semantics are provided in section 3. Section 4 and 5 describe decoding/encoding process. Experimental results and subjective/objective evaluation are provided in Section 6. The conclusion is contained in Section 7, references in Section 8.

NOTE – Deblocking filter is described in 8.6.1 of Working Draft [3] and adaptive loop filter is described in 8.6.2 of Working Draft [3].

# Algorithm

In this section, the basic algorithm of conventional NLM filter [1] in subclause 2.1, and the approaches of reduction for computational complexity are described in subclause 2.2, 2.2 and 2.3 describe the overview of encoding/decoding algorithm, respectively.

## Conventional non-local means filter algorithm

In the basic algorithm of conventional NLM filter, for derivation of denoised sample NlmOutput, the following ordered steps apply:

–	Initially, SumFltCoeffs is assigned to 1 and SumSamples is assigned to ArrayRef[iT][jT], where ArrayRef with elements ArrayRef[iT][jT] specifies a sample array and a location (iT, jT) specifies the coordinate of the sample to be denoised.

–	For each tap element (generically, odd numbered NxN block around the sample is constructed as the search points) with location (iS, jS), a SSD (Sum of Square Difference) between all the sample values of template at a location (iT, jT) and all the sample values of template at the location (iS, jS) is derived, then MxM block (generically odd numbered) around the sample forms a template (Figure 1). SumFltCoeffs and SumSamples are derived by

FltCoeff = exp(–SSD  DenoiseCoef)		(2-1)

SumFltCoeffs += FltCoeff		(2-2)

SumSamples += FltCoeff * ArrayRef[iS][jS]	 	(2-3)

–	NlmOutput is derived by SumSamples SumWeights.

NOTE – The function exp() indicates mathematical exponential function. DenoiseCoef specifies the strength of NLM filter.

Figure 1: Relationship between the template, tap elements and target.

## Enhanced non-local means filter algorithm

Because this NLM filter requires O(N2M2) complexity, the reduction of the number of tap elements and the reduction of template size would lead to the reduction of the computational complexity.

For the reduction of the computational complexity, a half-resized sample array ArrayRefHalf with elements ArrayRefHalf[i2][j2] is prepared. Each element of ArrayRefHalf is derived by

ArrayRefHalf [i2][j2] = (ArrayRef [i][j    ] + ArrayRef [i + 1][j   ] 
		+  ArrayRef [i][j + 1] + ArrayRef [i + 1][j + 1] + 2) >> 2	(2-4)

### Reduction of the number of tap elements by sobel filtering

For each 2x2 sub-block with the location (i, j) specifying the top-left coordinate of the sub-block, EdgeIdx specifying the index of an edge direction is derived by sobel filtering applying to ArrayRefHalf. Figure 2 illustrates the mapping between EdgeIdx according to the edge direction, and Figure 3 illustrates reduced tap for locations (i, j), (i + 1, j), (i, j + 1) and (i + 1, j + 1) at the sub-block. Here, when the sub-block where a remarkable edge was not detected, EdgeIdx is assigned to 0.

Figure 2: The mapping of the EdgeIdx according to edge direction.

Figure 3: The mapping of the tap elements according to EdgeIdx

### Restricting the template size according to local activity

Using a large template to less-noisy areas causes increase of decoder complexity. Two thresholds (nlm_threshold_1, nlm_threshold_2) are prepared to restrict the template size.

For half-resized sample array ArrayRefHalf, the local activity (Activity) between a sample and neighboring samples is derived at a location (i2, j2) specifying the coordinate of the ArrayRefHalf.

Activity = 2 * (Abs(ArrayRefHalf[i2 – 1][ j2   ] – ArrayRefHalf[i2][ j2])
+ Abs(ArrayRefHalf[i2 + 1][ j2   ] – ArrayRefHalf[i2][ j2])
+ Abs(ArrayRefHalf[i2   ][ j2 – 1] – ArrayRefHalf[i2][ j2])
+ Abs(ArrayRefHalf[i2   ][ j2 + 1] – ArrayRefHalf[i2][ j2]))
+ Abs(ArrayRefHalf[i2 – 1][ j2 – 1] – ArrayRefHalf[i2][ j2])
+ Abs(ArrayRefHalf[i2 + 1][ j2 – 1] – ArrayRefHalf[i2][ j2])
+ Abs(ArrayRefHalf[i2 – 1][ j2 + 1] – ArrayRefHalf[i2][ j2])
+ Abs(ArrayRefHalf[i2 + 1][ j2 + 1] – ArrayRefHalf[i2][ j2])	(2-5)

If the Activity is less than or equal to nlm_threshold_1, NLM filter does not apply to the sample. Otherwise, if the Activity is less than or equal to nlm_threshold_2, NLM filter uses small template. Otherwise, NLM filter uses large template. Figuer 4 illustates the relationship between thresholds and filter patterns. The filters apply to locations (i2 << 1, j2 << 1), ((i2 << 1) + 1, j2 << 1), (i2 << 1, (j2 << 1) + 1) and ((i2 << 1) + 1, (j2 << 1) + 1) for the sample array ArrayRef.

Figure 4: Relationship between thresholds and filter patterns

### Restriction of the number of NLM-filtered block

For each block, if the flag that specifies whether NLM filter applies or not to the block is decided only by RD-optimization, the decoder complexity fluctuates widely, because NLM filter might occupy whole of a frame, depending on the distortion of the frame. To control the fluctuation, a limit threshold of occupancy ratio for NLM filter is defined to encoder. The detail of this part is described in 5.2.2.

## Overview of the encoding algorithm

This subclause describes overview of the encoding algorithm of the proposed adaptive filter. Firstly, an encoder generates two kinds of filtered pictures, one is generated by the deblocking filter of HM3.0 and the other is generated by NLM filter. Secondly, the encoder splits a frame into blocks, and selects which filtered picture is more suitable for each block. The selected filter is indicated using filter mode information (filter mode 0: deblocking filter, filter mode 1: NLM filter). The quadtree structure (constructed by split flags) is used to represent the segmentation information. The quadtree structure and filter mode information allocated to the inner node and the leaf node of the quadtree are determined based on RD cost minimization. Picture A in Figure 5 illustrates block segmentation in a quadtree structure, where green-colored areas and red-colored ones indicate blocks associated with “filter mode 0” and those associated with “filter mode 1”, respectively.

For derivation of a final decoded picture, the following ordered steps apply:

–	Deblocking filter of HM3.0 applies to the picture A, and picture B is derived.

–	The derivation process for combined picture C specified in subclause 4.3 is derived by the combination of picture A and picture B.

–	ALF of HM3.0 applies to the picture C, and picture D indicating the final decoded picture is derived.

The extra overhead of this algorithm is the information of the quadtree structure, filter modes of each block and denoising parameters controls the strength and applying range of NLM filter.

Figure 5: Picture flow for encoder.

## Overview of the decoding algorithm

This subclause describes the overview of the decoding algorithm corresponding to the encoding algorithm specified in subclause 2.2. When NLM filter applies to a decoded picture, after decoding a denoising parameter specifying the strength of NLM filter, two thresholds specified in subclause 2.1.2, and two scaling factors of the strength of NLM filter at the boundary of each transform unit specified in subclause 4.1.2, while decoding the information of the quadtree split flag for tracing the structure of the quadtree, each filter mode flag of block is decoded as the leaf node of the quadtree. For each block, if the filter mode flag indicates “Filter mode 1” in Figure 5, NLM filter applies to the block and a picture A' described in Figure 6 is derived.

For derivation of a final decoded picture, the following ordered steps apply:

–	Deblocking filter of HM3.0 applies to the picture A', and picture B' is derived.

–	The derivation process for combined picture C' specified in subclause 4.3 is derived by the combination of picture A' and picture B'.

–	ALF of HM3.0 applies to the picture C', and picture D' indicating the final decoded picture is derived.

NOTE – The syntax, semantics and the detail of the decoding algorithm are described in Section 3 and 4.

Figure 6: Picture flow for decoder.

# Syntax and Semanticsm

In this section, the syntax and semantics related to NLM filter are specified. The syntax is described in slice header, and NLM filter applies to each sample format (Y/Cb/Cr). The syntax of slice header is specified in Table 1.

Table 1: Syntax related to NLM filter for slice header

| slice_header(){ | Descriptor |
| --- | --- |
| … |  |
| nlm_proc_luma() |  |
| nlm_proc_chroma_cb() |  |
| nlm_proc_chroma_cr() |  |
| … |  |
| } |  |

## Non-local means filter luma/chroma (Cb/Cr) semantics and syntax

In this subclause, the semantics used in nlm_proc_luma(), nlm_proc_chroma_cb() and nlm_proc_chroma_cr() are specified. The syntax is specified in Table 2, 3 and 4.

The constant value NlmMaxBlkSize specifies the maximum block size for the quadtree-based division. In this contribution, NlmMaxBlkSize is assigned to 32.

nlm_filter_flag specifies whether NLM filter applies or not to the each sample array of the format (Y/Cb/Cr) of a decoded picture.

nlm_coeff specifies the strength of NLM filter. The value of nlm_coeff shall be in the range of 1 to 255, inclusive.

nlm_threshold_1 specifies the threshold that identifies whether NLM filter applies or not. The local activity (Activity) between a sample and neighboring samples is derived. If the Activity is greater than nlm_threshold_1, NLM filter (NlmFlt1) applies to the sample. Otherewise, NLM filter does not apply to the sample. The value of nlm_threshold_1 shall be in the range of 0 to 216 – 1, inclusive. This syntax element is used in 4.1.5.

nlm_threshold_2 specifies the threshold to switch the pattern of NLM filter. nlm_threshold_2 takes greater than nlm_threshold_1, constantly. If the Activity is greater than nlm_threshold_2, NLM filter employs a filter (NlmFlt2 different from NlmFlt1) to the sample. The value of nlm_threshold_2 shall be in the range of 0 to 216 – 1, inclusive. Figure 4 illustrates the relationship between nlm_threshold_1 and nlm_threshold_2. This syntax element is used in 4.1.5.

nlm_boundary_scaler[i] specifies the scaling factor of the strength of NLM filter at the boundary of each transform unit. The value of nlm_boundary_scaler[i] shall be in the range of 0 to 31, inclusive. This syntax element is used in 4.1.3 and 4.1.4.

Table 2: Syntax for luma.

| nlm_proc_luma(){ | Descriptor |
| --- | --- |
| nlm_filter_flag | ae(v) |
| if(nlm_filter_flag == 1){ |  |
| nlm_coeff | ae(v) |
| nlm_threshold_1 | ae(v) |
| nlm_threshold_2 | ae(v) |
| for(i = 0; i < 2; i++) |  |
| nlm_ boundary_scaler[i] | ae(v) |
| for(y = 0; y < pic_height_in luma_samples; y += NlmMaxBlkSize) |  |
| for(x = 0; x < pic_width_in luma_samples; x += NlmMaxBlkSize) |  |
| nlm_tree(x, y, pic_width_in_luma_samples, pic_height_in_luma_samples, NlmMaxBlkSize) |  |
| } |  |
| } |  |

Table 3: Syntax for chroma (Cb).

| nlm_proc_chroma_cb(){ | Descriptor |
| --- | --- |
| nlm_filter_flag | ae(v) |
| if(nlm_filter_flag == 1){ |  |
| nlm_coeff | ae(v) |
| nlm_threshold_1 | ae(v) |
| nlm_threshold_2 = 216 - 1 |  |
| for(i = 0; i < 2; i++) |  |
| nlm_boundary_ scaler[i] | ae(v) |
| for(y = 0; y < pic_height_in_chroma_samples; y += NlmMaxBlkSize) |  |
| for(x = 0; x < pic_width_in_chroma_samples; x += NlmMaxBlkSize) |  |
| nlm_tree(x, y, pic_width_in_chroma_samples, pic_height_in_chroma_samples, NlmMaxBlkSize) |  |
| } |  |
| } |  |

Table 4: Syntax for chroma (Cr).

| nlm_proc_chroma_cr(){ | Descriptor |
| --- | --- |
| nlm_filter_flag | ae(v) |
| if(nlm_filter_flag == 1){ |  |
| nlm_coeff | ae(v) |
| nlm_threshold_1 | ae(v) |
| nlm_threshold_2 = 216 - 1 |  |
| for(i = 0; i < 2; i++) |  |
| nlm_boundary_ scaler[i] | ae(v) |
| for(y = 0; y < pic_height_in_chroma_samples; yB += NlmMaxBlkSize) |  |
| for(x = 0; x < pic_width_in_chroma_samples; xB += NlmMaxBlkSize) |  |
| nlm_tree(x, y, pic_width_in_chroma_samples, pic_height_in_chroma_samples, NlmMaxBlkSize) |  |
| } |  |
| } |  |

## Non-local means filter tree semantics and syntax

Non-local means filter tree semantics and syntax

In this subclause, the semantics used in nlm_tree() are specified. The syntax is identified in Table 5.

The constant value NlmMinBlkSize specifies the minimum block size for the quadtree-based division. In this contribution, NlmMinBlkSize is assigned to 16.

nlm_split_flag specifies whether a block is split into blocks with half horizontal and vertical size for the purpose of NLM filtering.

nlm_mode_flag specifies whether NLM filter applies to a block or not. If this syntax is equal to 1, NLM filter applies to the samples of the block, and deblocking filter does not applies to the samples of the block. Otherwise, deblocking filter applies to the samples of the block.

## Table 5: Syntax for tree structure.

| nlm_tree(x, y, Width, Height, BlkSize){ | Descriptor |
| --- | --- |
| if(x < Width && y < Height && NlmMinBlkSize <= BlkSize){ |  |
| nlm_split_flag = 0 |  |
| if(BlkSize != NlmMinBlkSize) |  |
| nlm_split_flag | ae(v) |
| if(nlm_split_flag == 1){ |  |
| BlkSizeHalf = BlkSize >> 1 |  |
| nlm_tree(x, y, Width, Height, BlkSizeHalf) |  |
| nlm_tree(x + BlkSizeHalf, y, Width, Height, BlkSizeHalf) |  |
| nlm_tree(x, y + BlkSizeHalf, Width, Height, BlkSizeHalf) |  |
| nlm_tree(x + BlkSizeHalf, y + BlkSizeHalf, Width, Height, BlkSizeHalf) |  |
| } |  |
| else |  |
| nlm_mode_flag | ae(v) |
| } |  |
| } |  |

# Decoding process

In this section, the decoding process of proposed in-loop filter is described in 4 steps below:

- Decoding process for non-local means filter (described in subclause 4.1)

- Deblocking filtering process (described in subclause 4.2)

- Combination process (described in subclause 4.3)

- ALF process (described in subclause 4.4)

PicRef indicates a decoded picture before deblocking filtering, and PicNlm indicates a denoized picture after NLM filtering.

SampleCh indicates a sample format, and is allocated a sample format from 3 formats (Y/Cb/Cr). SampleWidth and SampleHeight indicate the width and height of a sample array, respectively.

–	If SampleCh is equal to Y, SampleWidth and SampleHeight are derived as follows:

SampleWidth = pic_width_in_luma_samples.
SampleHeight = pic_height_in_luma_samples

–	Otherwise, SampleWidth and SampleHeight are derived as follows:

SampleWidth = pic_width_in_ chroma _samples.
SampleHeight = pic_height_in_ chroma _samples

ArrayRef with elements ArrayRef[i][j] (i = 0..SampleWidth – 1, j = 0..SampleHeight – 1) indicates the sample array of SampleCh of PicRef. ArrayNlm with elements ArrayNlm[i][j] indicates the sample array of SampleCh of PicNlm.

## Decoding process for non-local means filter

Inputs to this process are:

–	A decoded picture PicRef.

–	Decoded transform units.

Outputs of this process are:

–	A denoised picture PicNlm.

–	An NlmBlkList that includes the location (xB, yB) specifying the top-left coordinate of a block and a block size.

For each SampleCh (Y/Cb/Cr), the following ordered steps apply:

–	Copy all the elements of ArrayRef to ArrayNlm.

–	If nlm_filter_flag is equal to 1, the following ordered steps apply:

- The derivation process for a half-resized sample array as specified in subclause 4.1.1 is invoked with ArrayRef as the input and the output is sample array ArrayRefHalf.

- The derivation process for the filter indexes of each sample as specified in subclause 4.1.2 is invoked with ArrayRefHalf and decoded transform units as the inputs and the output is filter index FltIdx.

- The derivation process for the pre-computed filter strength of NlmFlt1 as specified in subclause 4.1.3 is invoked with nlm_coef and nlm_boundary_scaler as the inputs and the output is FltCoef1 specifying the pre-computed filter strength of NlmFlt1.

- If SampleCh is equal to Y, The derivation process for the pre-computed filter strength of NlmFlt2 as specified in subclause 4.1.4 is invoked with nlm_coef and nlm_boundary_scaler as the inputs and the output is FltCoef2 specifying the pre-computed filter strength of NlmFlt2.

- A quadtree structure is traced by decoding nlm_split_flag, and each nlm_mode_flag is derived as the leaf node of the quadtree structure.

- For each nlm_mode_flag, if nlm_mode_flag is equal to 1, the derivation process for a denoised block specified in subclause 4.1.5 is invoked with the location (xB, yB) specifying the top-left coordinate of a block, a block size, ArrayRef, ArrayRefHalf, FltIdx, FltCoef1 and FltCoef2 as the inputs and the output is a sample arrayArrrayNlm, then the location (xB, yB) and the block size are added to NlmBlkList.

### Derivation process for a half-resized sample array

Input to this process is sample array ArrayRef.

Output of this process is half-resized sample array ArrayRefHalf.

The sample values of the half-resized sample array ArrayRefHalf with elements ArrayRefHalf[i2][j2], i2 = 0..(SampleWidth >> 1) – 1, j2 = 0..(SampleHeight >> 1) – 1 are derived by

i = i2 << 1
j = j2 << 1

ArrayRefHalf [i2][j2] = (ArrayRef [i][j    ] + ArrayRef [i + 1][j   ] 
				+  ArrayRef [i][j + 1] + ArrayRef [i + 1][j + 1] + 2) >> 2	(4-6)

### Derivation process for the filter indexes of each sample

Input to this process is decoded transform units (each of the transform unit includes the location (xB, yB) specifying the top-left coordinate of a block and a block size).

Output of this process is the filter index of each sample FltIdx with elements FltIdx[i][j].

For the derivation of FltIdx, the following ordered steps apply:

–	If a slice type is equal to intra slice and a SampleCh is equal to Y, all the elements of FltIdx are assigned to 0.

–	Otherwise, all the elements of FltIdx are assigned to 1.

–	For all the decoded transform units, the following ordered steps apply:

- If the block size of transform unit is equal to 4, the FltIdx[i][j] of inner circumference 1 sample from the edge of transform unit is assigned to 2.

- Otherwise, the FltIdx[i][j] of inner circumference 2 samples from the edge of transform unit is assigned to 2. And the FltIdx[i][j] of inner circumference 1 sample from the edge of transform unit is assigned to 3.

Figure 7 illustrates the FltIdx assignment for each sample.

Figure 7: Derivation for FltIdx

NOTE – A location (i, j) is derived from the location (xB, yB) of a transform unit and the block size of the transform unit.

### Derivation process for the pre-computed filter strength of NlmFlt1

Inputs to this process are:

–	The strength of NLM filter nlm_coef.

–	nlm_boundary_scaler specifying the scaling factor of the strength of NLM filter at the boundary of each transform unit.

Output of this process is FltCoef1 with elements FltCoef1[i], i = 0..3 specifying the pre-computed filter strength of NlmFlt1.

Each FltCoef1[i] (i = 0..3) is derived by

FltCoef1[0] = 0					(4-7)

FltCoef1[1] = 0.00065 nlm_coef 		(4-8)

FltCoef1[2] = 0.00065 nlm_coef * (1 + nlm_boundary_scaler[0] ) 	(4-9)

FltCoef1[3] = 0.00065 nlm_coef * (1 + nlm_boundary_scaler[1] ) 	(4-10)

### Derivation process for the pre-computed filter strength of NlmFlt2

Inputs to this process are nlm_coef and nlm_boundary_scaler.

Output of this process is FltCoef2 with elements FltCoef2[i], i = 0..3 specifying the pre-computed filter strength of NlmFlt2.

Each FltCoef2[i] (i = 0..3) is derived by

FltCoef2[0] = 0					(4-11)

FltCoef2[1] = 0.00015 nlm_coef 		(4-12)

FltCoef2[2] = 0.00015 nlm_coef * (1 + nlm_boundary_scaler[0] ) 	(4-13)

FltCoef2[3] = 0.00015 nlm_coef * (1 + nlm_boundary_scaler[1] ) 	(4-14)

### Derivation process for denoised block

Inputs to this process are:

–	The location (xB, yB) specifying the top-left coordinate of a block.

–	A block size.

–	A sample array ArrayRef.

–	A half-resized sample array ArrayRefHalf.

–	The filter indexes of each sample FltIdx.

–	The pre-computed filter strength FltCoef1.

–	The pre-computed filter strength FltCoef2.

Output of this process is a sample array ArrayNlm.

The input block is split into 2x2 sub-blocks. For each 2x2 sub-block the following step applies:

–	If all FltIdx (FltIdx[xS][yS], FltIdx[xS + 1][yS], FltIdx[xS][yS + 1] and FltIdx[xS + 1][yS + 1]) are not equal to 0 (the location (xS, yS) specifies the top-left coordinate of the sub-block),

the derivation process for the difference between a sample and neighboring samples as specified in subclause 4.1.5.1 is invoked with a location (xS, yS) and ArrayRefHalf as the inputs and the output is the Activity. Here,

if the value of Activity is greater than nlm_threshold_1, the following ordered steps apply:

  - The derivation process for the index of an edge direction EdgeIdx as specified in subclause 4.1.5.2 is invoked with ArrayRefHalf and a location (xS, yS) as the inputs and the output is EdgeIdx derived from edge direction.

  - If the Activity is less than or equal to nlm_threshold_2, for each sample (xP, yP) of 2x2 sub-block, the derivation process for a denoised value by NlmFlt1 as specified in subclause 4.1.5.3 is invoked with the location (xP, yP) of a target point specifying the sample coordinate, ArrayRef, FltCoef1, FltIdx[xP][yP] and EdgeIdx as the inputs and the output is ArrayNlm[xP][yP].

  - Otherwise, for each sample (xP, yP) of 2x2 sub-block, the derivation process for a denoised value by NlmFlt2 as specified in subclause 4.1.5.4 is invoked with the location (xP, yP) of a target point specifying the sample coordinate, ArrayRef, FltCoef2, FltIdx[xP][yP] and EdgeIdx as the inputs and the output is ArrayNlm[xP][yP].

#### Derivation process for the difference between neighboring samples

Inputs to this process are:

–	A location (xS, yS).

–	A half-resized sample array ArrayRefHalf.

Output of this process is Activity

The local activity (Activity) between a sample and neighboring samples as is derived by

xS2 = xS >> 1
yS2 = yS >> 1

Activity = 2 * (Abs(ArrayRefHalf[xS2 – 1][yS2   ] – ArrayRefHalf[xS2][yS2])
+ Abs(ArrayRefHalf[xS2 + 1][yS2   ] – ArrayRefHalf[xS2][yS2])
+ Abs(ArrayRefHalf[xS2   ][yS2 – 1] – ArrayRefHalf[xS2][yS2])
+ Abs(ArrayRefHalf[xS2   ][yS2 + 1] – ArrayRefHalf[xS2][yS2]))
+ Abs(ArrayRefHalf[xS2 – 1][yS2 – 1] – ArrayRefHalf[xS2][yS2])
+ Abs(ArrayRefHalf[xS2 + 1][yS2 – 1] – ArrayRefHalf[xS2][yS2])
+ Abs(ArrayRefHalf[xS2 – 1][yS2 + 1] – ArrayRefHalf[xS2][yS2])
+ Abs(ArrayRefHalf[xS2 + 1][yS2 + 1] – ArrayRefHalf[xS2][yS2])	(4-15)

#### Derivation process for an index of edge direction

Inputs to this process are:

–	A location (xS, yS).

–	A half-resized sample array ArrayRefHalf.

Output of this process is the index of edge direction EdgeIdx. Figure 2 illustrates between the EdgeIdx and edge direction.

In this process, BitDepth specifies the bit depth of SampleCh.

For the derivation of EdgeIdx, the following ordered steps apply:

–	dx and dy are derived by

xS2 = xS >> 1
yS2 = yS >> 1

dx =  ArrayRefHalf[xS2 + 1][yS2 – 1] – ArrayRefHalf[xS2 – 1][yS2 – 1]
+   ArrayRefHalf[xS2 + 1][yS2 + 1] – ArrayRefHalf[xS2 – 1][yS2 + 1]
+ ((ArrayRefHalf[xS2 + 1][yS2   ] – ArrayRefHalf[xS2 – 1][yS2   ]) << 1)	(4-16)

dy =  ArrayRefHalf[xS2 – 1][yS2 + 1] – ArrayRefHalf[xS2 – 1][yS2 – 1]
+   ArrayRefHalf[xS2 + 1][yS2 + 1] – ArrayRefHalf[xS2 + 1][yS2 – 1]
+ ((ArrayRefHalf[xS2   ][yS2 + 1] – ArrayRefHalf[xS2   ][yS2 – 1]) << 1)	(4-17)

–	If Abs(dx) + Abs(dy) is less than 3 << (BitDepth – 5), EdgeIdx is assigned to 0.

–	Otherwise, if dy is equal to 0, EdgeIdx is assigned to 6.

–	Otherwise, the following step applies:

–	A gradient (Grad) is assigned to dx  dy, and EdgeIdx is derived by

EdgeIdx =  6;Grad<−5.0 7;Grad<−2.0 8;Grad<−1.0 9;Grad<−0.510;Grad<−0.2 1;Grad< 0.2 2;Grad< 0.5 3;Grad< 1.0 4;Grad< 2.0 5;Grad< 5.0 6;otherwise	 	(4-18)

#### Derivation process for a denoised value by NlmFlt1

Inputs to this process are:

–	A location (xP, yP)

–	A sample array ArrayRef.

–	A pre-computed filter strength FltCoef1.

–	The filter Index for a sample FltIdx[xP][yP].

–	The index of edge direction EdgeIdx.

Output of this process is ArrayNlm[xP][yP]

For the derivation of ArrayNlm[xP][yP], the following step applies:

–	If FltIdx[xP][yP] is not equal to 0, the following ordered steps apply:

- Initially, SumFltCoeffs is assigned to 215 – 1, and SumSamples is assigned to (215 – 1) * ArrayRef[xP][yP].

- For each tap element (the relative location of tap element (xP2, yP2) and the location (xP, yP) are shown in Figure 3) related to EdgeIdx, a SD (Square Difference) between the sample value of the location (xP, yP) and the sample value of the location (xP2, yP2) is derived. SumFltCoeffs and SumSamples are derived as follows.

FltCoeff = ExpLUT[Floor(Clip3(0, FltCoef1[FltIdx[i][j]] * SD, 999))]	(4-19)

SumFltCoeffs += FltCoeff		(4-20)

SumSamples  += FltCoeff * ArrayRef[xP2][yP2] 	(4-21)

- Denoised value ArrayNlm [xP][yP] is assigned to Floor(SumSamples  SumFltCoeffs + 0.5).

#### Derivation process for a denoised value by NlmFlt2

Inputs to this process are:

–	A location (xP, yP)

–	A sample array ArrayRef.

–	A pre-computed filter strength FltCoef2.

–	The filter Index for a sample FltIdx[xP][yP].

–	The index of edge direction EdgeIdx.

Output of this process is ArrayNlm[xP][yP]

For the derivation of ArrayNlm[xP][yP], the following step applies:

–	If FltIdx[xP][yP] is not equal to 0, the following ordered steps apply:

- Initially, SumFltCoeffs is assigned to 215 – 1, and SumSamples is assigned to (215 – 1) * ArrayRef[xP][yP].

- For each tap element (the relative location of tap element (xP2, yP2) and the location (xP, yP) are shown in Figure 3) related to EdgeIdx, a SSD between all the sample values of cross-shaped template at the location (xP, yP) and all the sample values of cross-shaped template at the location (xP2, yP2) is derived (Figure 8). SumFltCoeffs and SumSamples are derived as follows.

FltCoeff = ExpLUT[Floor(Clip3(0, FltCoef2[FltIdx[i][j]] * SSD, 999))]	(4-22)

SumFltCoeffs += FltCoeff		(4-23)

SumSamples  += FltCoeff * ArrayRef[xP2][yP2] 	(4-24)

- Denoised value ArrayNlm[xP][yP] is assigned to Floor(SumSamples  SumWeights + 0.5).

Figure 8: Derivation example for SSD between all the sample values of cross-shaped template at the location (xP, yP) and all the sample values of cross-shaped template at the location (xP2, yP2).

NOTE –ExpLUT[i] (i = 0..999) is assigned by

ExpLUT[i]= 0;i=999(215− 1)*exp(−i÷142.85073);otherwise	(4-25)

## Deblocking filtering process

Input to this process is PicNlm.

Output of this process is PicDF specifying the deblocked picture after deblocking filtering to PicNlm.

## Combination process

Input to this process is:

–	A denoised picture PicNlm.

–	A deblocked picture PicDF.

–	Each NlmBlkList (for the SmpleCh Y/Cb/Cr) that includes the location specifying the top-left coordinate of a block and a block size.

Output of this process is PicCmb specifying a combined picture of PicNlm and PicDF.

For the derivation of PicCmb, and for each SampleCh (Y/Cb/Cr), the following ordered steps apply:

–	All the elements of ArrayDF is copied to ArrayCmb, where ArrayDF with element ArrayDF[i][j] specifies the sample array of SampleCh of the PicDF and ArrayCmb with element ArrayCmb[i][j] specifies the sample array of SampleCh of the PicCmb.

–	For each element of NlmBlkList, all the samples of ArrayNlm at the block described in the element of NlmBlkList are overwritten to ArrayCmb.

## ALF process

Input to this process is PicCmb.

Output of this process is PicAlf specifying the denoised picture after adaptive loog filtering [1] to PicCmb and the picture is outputted as the final picture of proposed in-loop filter.

# Encoding process

In this section, the encoding process of proposed in-loop filter is described in 6 steps below:

- Pre-computed deblocking filtering process (described in subclause 5.1)

- Optimization process for parameters of non-local means filter (described in subclause 5.2)

- Coding process for non-local means filter parameters (described in subclause 5.3)

- Deblocking filtering process (described in subclause 5.4)

- Combination process (described in subclause 4.3)

- ALF process (described in subclause 4.4)

## Pre-computed deblocking filtering process

Deblocking filter specified in subclause 4.2 applies to PicRef. The output of this process is PicRefDF

## Optimization process for parameters of non-local means filter

Inputs to this process are:

–	A picture PicRef before filtering

–	A deblocked picture PicRefDF

–	Decoded transform units.

Outputs of this process are:

–	A denoised picture PicOpt.

–	Each optimum cost (for the SampleCh Y/Cb/Cr) of the optimum picture

–	Each optimum nlm_coef (for the SampleCh Y/Cb/Cr)

–	Each optimum nlm_threshold_1 (for the SampleCh Y/Cb/Cr)

–	Each optimum nlm_threshold_2 (for the SampleCh Y/Cb/Cr)

–	Each optimum nlm_boundary_scaler (for the SampleCh Y/Cb/Cr)

–	Each optimum NlmBlkList (for the SampleCh Y/Cb/Cr)

–	Each optimum quadtree information (for the SampleCh Y/Cb/Cr) including nlm_split_flag and nlm_mode_flag

For each SampleCh (Y/Cb/Cr), the following ordered steps apply:

–	Pre computed processes below:

- A half-resized sample array ArrayRefHalf (described in subclause 3.1.1)

- The filter index of each sample FltIdx (described in subclause 3.1.2)

–	The derivation process for nlm_threshold_1 and nlm_threshold_2 as specified in subclause 5.2.1 is invoked with ArrayRefHalf as the input and the outputs are nlm_threshold_1 and nlm_threshold_2.

–	The derivation process of each optimum coefficient and optimum NLM-filtered array ArrayNLM described in Figure 5 (written in Picture denoised by NLM filter) is invoked with ArrayRef, ArrayRefDF, ArrayRefHalf, FltIdx, nlm_threshold_1 and nlm_threshold_2 as the inputs and the outputs are optimum nlm_coef nlm_boundary_scaler and optimum ArrayNLM. Where each optimum coefficient and optimum ArrayNLM are derived by iterative operation.

–	The derivation process for restricting the NLM-filterd blocks as specified in subclause 5.2.2 is invoked with optimum ArrayNLM and ArrayRefDF as the inputs and the outputs are ArrayOpt, NlmBlkList and optimum quadtree information. Where ArrayOpt indicates the sample array of SampleCh of PicOpt.

–	If the RD-cost of deblocking filter is less than the optimum RD-cost, the optimum cost is assigned to –1 and ArrayRef is copied to ArrayOpt.

### The derivation process for nlm_threshold_1 and nlm_threshold_2

Input to this process is a half-resized sample array ArrayRefHalf.

Outputs of this process are:

–	nlm_threshold_1.

–	nlm_threshold_2.

nlm_threshold_1 and nlm_threshold_2 are derived according to the integral ratio using histogram of the Activity of ArrayRefHalf.

### The derivation process for restricting NLM-filterd blocks

Inputs to this process are:

–	A picture PicRef before filtering

–	A deblocked picture array ArrayRefDF

–	A denoised picture array ArrayNLM

Outputs of this process are:

–	An ArrayOpt

–	An NlmBlkList

–	A quadtree information

For the restricting NLM-filterd blocks by following ordered steps:

–	Copy all the elements of ArrayRef to ArrayOpt.

–	The ArrayRefDF and ArrayNLM are divided into given NxN blocks, for each block a SSD between the block and the block of an original frame is derived.

–	For each block, the block SSD of ArrayRefDF and the block SSD of ArrayNLM are compared; a flag is added to the block which indicates lower SSD.

–	For all the blocks, the blocks are sorted to descending order according to the amount of the SSD reduction of the NLM filter compared to deblocking filter.

–	Each block is referred in descending order, and then the number of NLM-filtered sample is counted. When the ratio of the number and the size of sample array exceeds the threshold of occupancy ratio, each the flag of trailing blocks is reset to the block of ArrayRefDF.

–	Only when the flag has been added to the block of ArrayNlm, the block is encoded as a block for the NLM filter application. ArrayOpt is overwritten by the blocks with the flag of ArrayNlm.

–	NlmBlkList and quadtree information are derived according to the flags.

  - Coding process for non-local means filter parameters

Inputs to this process are:

–	Each optimum cost (for the SampleCh Y/Cb/Cr) of the optimum picture

–	Each optimum nlm_coef (for the SampleCh Y/Cb/Cr)

–	Each optimum nlm_threshold_1 (for the SampleCh Y/Cb/Cr)

–	Each optimum nlm_threshold_2 (for the SampleCh Y/Cb/Cr)

–	Each optimum nlm_boundary_scaler (for the SampleCh Y/Cb/Cr)

–	Each optimum quadtree information including nlm_split_flag and nlm_mode_flag (for each SampleCh)

Output of this process is bitstream.

For each SampleCh (Y/Cb/Cr), the following ordered steps apply:

–	If the optimum cost is not equal to –1, nlm_filter_flag is assigned to 0, and is encoded.

–	Otherwise, nlm_filter_flag is assigned to 1, and is encoded, and then the following ordered steps apply:

- The optimum nlm_coef is encoded.

- The optimum nlm_threshold_1 is encoded.

- If SampleCh is equal to Y, The nlm_threshold_2 is encoded.

- The optimum two nlm_boundary_scaler[i], i = 0..1 are encoded.

- The optimum quadtree information (nlm_split_flag and nlm_mode_flag) is encoded.

## Deblocking filtering process

Deblocking filter specified in subclause 4.2 applies to PicOpt. The output of this process is PicDF.

# Experimental results

The proposal has been implemented in HM3.0-r832. High Efficiency configuration specified by the common test condition [4] was experimented.

## Objective evaluation

Table 6 presents BD-Rate [5] and encoder/decoder complexities compared to HM3.0-r832 anchors.

Table 6: Experimental results for proposed in-loop filter compared to HM3.0-r832

|  | All Intra HE |  |  |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  | Y | U | V |
| Class A | -1.0 | -1.2 | -1.1 |  | Class A |  |  |  |
| Class B | -1.0 | -0.9 | -1.3 |  | Class B | -1.0 | -2.7 | -2.6 |
| Class C | -1.0 | -1.5 | -1.6 |  | Class C | -1.3 | -4.7 | -5.4 |
| Class D | -0.7 | -0.7 | -1.4 |  | Class D | -1.2 | -2.2 | -2.9 |
| Class E | -1.0 | 0.2 | 0.0 |  | Class E | -1.4 | -1.0 | -2.3 |
| Overall | -1.0 | -0.9 | -1.2 |  | Overall | -1.2 | -2.8 | -3.3 |
| Enc Time[%] | 114% |  |  |  | Enc Time[%] | 113% |  |  |
| Dec Time[%] | 110% |  |  |  | Dec Time[%] | 110% |  |  |
|  | Random Access HE |  |  |  |  | Low delay P HE |  |  |
|  | Y | U | V |  |  | Y | U | V |
| Class A | -0.8 | -2.3 | -2.1 |  | Class A |  |  |  |
| Class B | -1.0 | -2.2 | -1.7 |  | Class B | -1.0 | -3.2 | -3.1 |
| Class C | -1.1 | -2.6 | -2.7 |  | Class C | -1.4 | -5.1 | -5.7 |
| Class D | -0.9 | -1.5 | -2.1 |  | Class D | -1.3 | -2.4 | -3.7 |
| Class E |  |  |  |  | Class E | -1.5 | -1.8 | -3.1 |
| Overall | -1.0 | -2.2 | -2.1 |  | Overall | -1.3 | -3.2 | -3.9 |
| Enc Time[%] | 118% |  |  |  | Enc Time[%] | 118% |  |  |
| Dec Time[%] | 107% |  |  |  | Dec Time[%] | 111% |  |  |

## Subjective evaluation

Figure 9 presents the comparison between HM anchor and the proposed method from the viewpoint of subjective quality.

Figure 9: BasketballDrill (WVGA), LD P, BD-rate:  (Y: −1.5%, U: −9.2%, V: −8.8%),

QP=32, Frame #499 (−296bits, Y:+0.024dB, U:+0.341dB, V:+0.445dB)

- Conclusion

In this contribution, an in-loop filter based on non-local means filter was proposed. Compared to the anchor of HM3.0, the average BD-rate gains were “intra (Y: –1.0%, U: –0.9 % and V: –1.2 %)”, “random access (Y:–1.0 %, U: –2.2% and V: –2.1 %)” and “lowdelay (Y: –1.2 %, U: –2.8 % and V: –3.3 %)”, and “lowdelay P (Y: –1.3 %, U: –3.2% and V: –3.9 %)”, respectively. Recommend the proposed scheme to be studied within CE to find the optimal combination of current in-loop filter. Thanks to KDDI’s cross-check [6] for confirming the coding results.

# References

- A. Buades, B. Coll, and J. M. Morel, “A Non Local Algorithm for Image Denoising,” Proc. IEEE Int. Conf. on Computer Vision and Pattern Recognition, vol. 2, pp. 60-65, June 2005.

- Masaaki Matsumura, Yukihiro Bandoh, Seishi Takamura, Hirohisa Jozawa, “In-loop filter based on non-local means filter,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011.

- Thomas Wiegand, Woo-Jin Han, Benjamin Bross, Jens-Rainer Ohm, Gary J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCTVC-E603_d7.doc ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-D600, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

- G. Bjontegaard, “Calculation of average PSNR differences between RD-curves,” ITU-T SG16 Q.6 Document, VCEG-M33, 13th VCEG Meeting, Austin, Texas, USA, April 2001.

- Tomonobu Yoshino, Kei Kawamura, Sei Naito, “Cross-check for NTT's proposal on in-loop filter based on non-local mean filter (JCTVC-F047),” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).