| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F049_r2 |
| --- | --- |

| Title: | CE2: Report of OBMC with Motion Merging |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report/Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen1, Yan-Yu Chen, Chung-Lin Lee, Wen-Hsiao Peng2 and Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2pawn@mail.si2lab.org |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |

_____________________________

# Abstract

This contribution provides test results for the technique of OBMC with Motion Merging described in Core Experiment 2 (JCTVC-E702). Conceptually, it performs OBMC for a 2Nx2N prediction unit (PU) based on two sets of motion parameters, one (Set #1) is derived from motion merging with a reduced candidate set containing only Left, Above and Col merging candidates and the other (Set #2) is signaled in prediction_unit syntax for the considered PU. Two predictors are created from these motion parameters and combined linearly using an adaptive OBMC window function to form a prediction block. In addition, the motion parameters in Set #2 are optimized by considering OBMC. When implemented in HM-3.0, the technique was observed to achieve Y-BD-rate savings of 1.5% for RA-HE, 1.3% for RA-LC, 1.8% for LB-HE, and 2.2% for LB-LC. It performed consistently better in all test cases, with an average of 1.7%, a minimum of 0.9% and a maximum of 4.2%. The highest gain was achieved in Vidyo3, Class E under the LB-LC condition. On average, the encoding time was increased by 25% and the decoding time by 4%.

# Introduction

In JCTVC-E154 [1], we introduced an OBMC scheme that forms a bi-prediction for a 2Nx2N PU based on two sets of motion parameters, one (Set #1) is specified by the motion merging mechanism while the other (Set #2) is signaled in prediction_unit syntax (See Figure 1). The predictors derived from these motion parameters are linearly combined using OBMC with a window function dependent on MV constellation. The technique was shown to achieve BD-rate savings ranging from 1.4-2.6% in the common test conditions. The improvements, however, came at a cost of a 1.5-2x increase in encoding time. Some concerns were also raised during the last meeting about the increased memory bandwidth.

In the context of CE2 [3], this contribution provides test results of a modified version of the algorithm in [1] on HM-3.0 reference software [2]. Aimed at striking a better balance between complexity and performance, the modifications include four different aspects:

- Restricting the merging candidate in Set #1 to three options only: Left, Above, and Co-located.

- Providing the options of bi-prediction and motion merging in motion parameters of Set #2.

- Performing partial OBMC with simplified window functions.

- Speeding up coding mode decisions.

- OBMC with motion merging.

| w1,1 | w1,2 | w1,3 | w1,4 |
| --- | --- | --- | --- |
| w2,1 | w2,2 | w2,3 | w2,4 |

- Window functions for 16x16 PUs, wn,m.

# OBMC with Motion Merging

## Selection of Window Functions

Each 2Nx2N PU, when coded in the proposed mode, can select between two sets of window functions, denoted by wn,m with n=1, 2 and m=1,…,4. Their waveforms are depicted in Figure 2, assuming that the considered PU is of size 16x16. The parameter n is indicated explicitly in syntax while the value of m is inferred implicitly according to the choice of the merging candidate specified in the parameters of Set #1. In the present case, the merging candidates are the same as those defined in mergeCandList (subclause 8.4.2.1.1) for PART_2Nx2N, except that candidates C and D are excluded; in other words, all that remain are candidates A, B and Col (See Figure 3).

When this reduced candidate set is non-empty, the value of m is inferred as follows:

- If A is selected

  - If A and B have identical motion parameters, m is set equal to 4.

  - Otherwise, m is set equal to 1.

- If B is selected, m is set equal to 2.

- If Col is selected, m is set equal to 3.

If none of A, B and Col is available, m is set equal to 3.

- Merging candidates selectable in Set #1.

## Derivation of Prediction Samples

With the proposed technique, each pixel in a 2Nx2N PU is a weighted sum of two prediction values r(i, j) and q(i, j), which are derived from motion parameters in Sets #1 and #2, respectively.

### Derivation of Prediction Samples from Motion Parameters in Set #1

Specifically, r(i, j) is computed as follows:

- If r(i, j) is a uni-predicted sample,

;

- Otherwise (r(i, j) is a bi-predicted sample),

,

where

- (x0, y0) is the location of the top-left pixel of the considered PU relative to the top-left pixel of the current picture.

- (i, j), with i, j=0, …, 2N-1, is an integer position of a pixel in the considered PU.

- (mvxt1, mvyt1) and, in the case of bi-prediction, (mvxt2, mvyt2) are MVs copied from some specific merging candidate specified in the parameters of Set #1 (See Figure 3).

- p1 and p2 are two previously coded pictures.

Particularly, if Col is selected for use and is a bi-predicted block, then r(i, j) will be classified as a uni-predicted sample. In this special case, the computation of its value involves only one of the two derived MVs, which is chosen according to the following rule:

- If collocated_from_l0_flag equals to 1, (mvxt1, mvyt1) is set to the derived MV for reference picture list L0.

- Otherwise, (mvxt1, mvyt1) is set to the derived MV for reference picture list L1.

### Derivation of Prediction Samples from Motion Parameters in Set #2

Using the same notation, q(i, j) is computed similarly:

- If q(i, j) is a uni-predicted sample,

;

- Otherwise (q(i, j) is a bi-predicted sample),

,

where

- (mvxb1, mvyb1) and, in the case of bi-prediction, (mvxb2, mvyb2) are MVs signaled by the parameters of Set #2.

- p3 and p4 are two previously coded pictures.

In the present case, (mvxb1, mvyb1) and (mvxb2, mvyb2) can also be derived by motion merging. The merging candidate(s) from which they can be derived is restricted to be: (See Figure 4)

- Col, if (mvxt1, mvyt1) and (mvxt2, mvyt2) are derived from A and A, B have identical motion parameters.

- C’ and Col, if (mvxt1, mvyt1) and (mvxt2, mvyt2) are derived from A.

- D’ and Col, if (mvxt1, mvyt1) and (mvxt2, mvyt2) are derived from B.

- A and B, if (mvxt1, mvyt1) and (mvxt2, mvyt2) are derived from Col.

- C’, if (mvxt1, mvyt1) and (mvxt2, mvyt2) are set to zero motion vector due to an empty merging candidate set (i.e., none of A, B and Col is available)

If these candidates have identical motion parameters, pruning is performed in the reverse candidate order as shown above. Moreover, additional check is added to ensure the remaining candidates have different motion parameters than (mvxt1, mvyt1) and (mvxt2, mvyt2), i.e., those specified in Set #1. If the resulting candidate set is empty, a full set of motion parameters must be signaled.

Similar to the derivation of r(i, j), if Col is selected for use and is a bi-predicted block, then q(i, j) will be classified as a uni-predicted sample. The computation of its value involves only one of the two derived MVs, which is chosen according to the following rule:

- If collocated_from_l0_flag equals to 1, (mvxb1, mvyb1) is set to the derived MV for reference picture list L0.

- Otherwise, (mvxb1, mvyb1) is set to the derived MV for reference picture list L1.

- Merging candidates selectable in Set #2.

### Combining Prediction Samples with OBMC

Given the q(i, j) and r(i, j) above, the creation of each pixel, ppred(x0+i, y0+j), in the considered 2Nx2N PU is governed by the following equation:

.

The computation of ppred(x0+i, y0+j) can be done in fixed-point arithmetic (involving only binary shifts and integer additions), as the values of the window functions are currently limited to be 0.875, 0.75, 0.5, 0.125, and 0. For example,

- If wn,m(i, j)=0.875,

;

- If wn,m(i, j)=0.75,

;

- If wn,m(i, j)=0.5,

;

- If wn,m(i, j)=0.125,

;

- Otherwise (wn,m(i, j)=0),

.

## Resizing Window Functions

In the proposed technique, the window functions wn,m need to match the size of the considered 2Nx2N PU, which currently ranges from 8x8 to 128x128 (or 4x4 to 64x64, respectively) for Luma (or Chroma, respectively). This is accomplished by using a table to record, for every possible 2Nx2N PU size, the integer pixel positions where a transition in the value of the window function takes place.

In the current design, a window function wn,m can take values {0, 0.125, 0.5, 0.875} or {0, 0.125, 0.5, 0.75}. The distribution of its weighting coefficients forms a partitioning of a PU into four non-overlapping regions, each corresponds to the application of some specific coefficient. As an illustration, Figure 5 presents the various partitioning patterns that could arise from the application of window functions shown in Figure 2. As can be seen, the coefficients 0.5 occupy a region (Region B) that may distribute vertically, horizontally, or diagonally. To resize a window function according to the size of the considered PU, the start position a and the width b of Region B are recorded (See Figure 5 for their definitions). Table 1 lists the value of a for every possible 2Nx2N PU size, with the value of b deduced as follows:

  - If the window function is either w1,3 or w2,3, b=2N;

  - Otherwise (neither w1,3 nor w2,3),

    - If the considered PU is larger than 4x4, b=2;

    - Otherwise (the considered PU is of 4x4),

      - For w1,m, b=1;

      - For w2,m, b=2.

Once these two parameters are specified, the coefficients for the other regions are given as follows:

- wn,m(i, j)=0.75 in Region A1;

- wn,m(i, j)=0.875 in Region A2;

- wn,m(i, j)=0.5 in Region B;

- wn,m(i, j)=0.125 in Region C;

- wn,m(i, j)=0 in Region D,

where n=1 or 2.

Note that Region D has a weighting coefficient equal to 0; in other words, pixels in that region are not compensated by OBMC.

| wn,1 | wn,2 | wn,3 | wn,4 |
| --- | --- | --- | --- |

- Partitioning of a 2Nx2N PU by an application of the proposed window functions.

- The start position a for various 2Nx2N PU sizes.

| PU Size | w1,1 | w1,2 | w1,3 | w1,4 | PU Size | w2,1 | w2,2 | w2,3 | w2,4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 128x128 | 31 | 31 | 0 | 95 | 128x128 | 63 | 63 | 0 | 126 |
| 64x64 | 15 | 15 | 0 | 47 | 64x64 | 31 | 31 | 0 | 62 |
| 32x32 | 7 | 7 | 0 | 23 | 32x32 | 15 | 15 | 0 | 30 |
| 16x16 | 3 | 3 | 0 | 11 | 16x16 | 7 | 7 | 0 | 14 |
| 8x8 | 1 | 1 | 0 | 5 | 8x8 | 3 | 3 | 0 | 6 |
| 4x4 | 1 | 1 | 0 | 2 | 4x4 | 1 | 1 | 0 | 2 |

# Block Matching Criterion

When the proposed prediction mode is in use, the motion search criteria for (mvxb1, mvyb1) and, in the case of bi-prediction, (mvxb2, mvyb2) are modified in order to consider the contributions from the merged MVs, i.e., (mvxt1, mvyt1) and (mvxt2, mvyt2). The main idea is as follows. For each merging candidate in Set #1, the corresponding prediction values r(i, j) are weighted by wn,m(i, j) and subtracted from the pixel values of the considered PU. Then, (mvxb1, mvyb1) and (mvxb2, mvyb2) are estimated by minimizing the prediction error between the resulting difference signal and the weighted q(i, j), i.e., (1- wn,m(i, j))q(i, j). That is, the search for (mvxb1, mvyb1) and (mvxb2, mvyb2) is accomplished by considering OBMC.

# Changes of Syntax and Semantics

The following table indicates the changes of syntax and semantics. For brevity, the proposed prediction mode is referred hereafter to as TB-mode.

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| …. |  |
| …. |  |
| } else { /* MODE_INTER */ |  |
| if( !InferredMergeFlag ) { |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && PartMode==PART_2Nx2N) { |  |
| tb_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( tb_flag[ x0 ][ y0 ] ) { |  |
| tb_table_idc[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| If( NumTBCand > 1 ) { |  |
| tb_cand_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| If( NumTBMergeCand > 0 ) |  |
| tb_merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| if( !tb_flag[ x0 ][ y0 ] && merge_flag[ x0 ][ y0 ] && NumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( tb_flag[ x0 ][ y0 ] && tb_merge_flag[ x0 ][ y0 ] ) { |  |
| if(NumTBMergeCand > 1 ) |  |
| tb_merge_cand_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if(!entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| … |  |
| … |  |
| } |  |
| } |  |
| } |  |

tb_flag[ x0 ][ y0 ] specifies whether the current prediction unit with PART_2Nx2N is coded in TB-mode or not.

- If tb_flag[ x0 ][ y0 ] equals to 1, a PART_2Nx2N PU is coded in TB-mode.

- Otherwise (tb_flag[ x0 ][ y0 ] equals to 0), the usual merge mode is applied.

The array indices x0 and y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When tb_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

tb_table_idc[ x0 ][ y0 ] indicates which of the two sets of window functions, w1,m and w2,m, is in use.

- If tb_table_idc[ x0 ][ y0 ] is 0, w1,m is selected;

- Otherwise (tb_table_idc[ x0 ][ y0 ] is 1), w2,m is selected.

The array indices x0 and y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When tb_table_idc[ x0 ][ y0 ] is not present, it is inferred to be equal to 1.

tb_cand_idx[ x0 ][ y0 ] specifies the merging candidate index in Set #1 (See Figure 3) where x0 and y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When tb_cand_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

The variable NumTBCand specifies the number of valid motion merging candidates in Set #1.

If NumTBCand is equal to 1, tb_cand_idx[ xP][ yP ] is inferred to be the same index as the first available merging candidate in Set #1.

tb_merge_flag[ x0 ][ y0 ] equal to 1 specifies that the inter prediction parameters for the current prediction unit, when coded by TB-mode, are inferred from a neighboring inter-predicted partition (See Figure 4). The array indices x0 and y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When tb_merge_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

tb_merge_cand_idx[ x0 ][ y0 ] specifies the merging candidate index in Set #2 (See Figure 4) where x0 and y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When tb_merge_cand_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

The variable NumTBMergeCand specifies the number of valid motion merging candidates in Set #2.

If NumTBMergeCand is equal to 1, tb_merge_cand_idx[ xP][ yP ] is inferred to be the same index as the first available merging candidate in Set #2.

# Experiments

Extensive experiments were carried out using the HM-3.0 software [1] to collect BD-rate savings and runtimes of the proposed technique in the common test conditions [4]. The platform used is summarized below:

OS: 64-bit Microsoft Windows 7 Enterprise + Microsoft Visual Studio 2008 (32-bit Compiler)

Platform: Intel(R) Core(TM) i7-860 (2.80GHz) processor and 16GB RAMs

## Performance Summary

- BD-rate savings and coding time ratios.

|  | Random Access |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | -2.5 | -2.5 | -1.2 | -1.2 | -1.4 |
| Class B | -1.2 | -2.1 | -2.0 | -1.1 | -1.7 | -1.8 |
| Class C | -1.7 | -2.5 | -2.7 | -1.5 | -2.3 | -2.4 |
| Class D | -1.7 | -2.3 | -2.5 | -1.4 | -2.1 | -2.1 |
| Class E |  |  |  |  |  |  |
| All | -1.5 | -2.4 | -2.4 | -1.3 | -1.8 | -1.9 |
| Enc Time[%] | 122% |  |  | 122% |  |  |
| Dec Time[%] | 104% |  |  | 107% |  |  |
|  | Low Delay B |  |  | Low Delay B LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.3 | -2.4 | -2.3 | -1.8 | -2.4 | -2.6 |
| Class C | -1.9 | -2.6 | -2.7 | -2.1 | -2.5 | -2.8 |
| Class D | -2.0 | -2.7 | -2.7 | -2.2 | -3.0 | -3.0 |
| Class E | -2.0 | -2.3 | -2.7 | -3.0 | -3.5 | -4.2 |
| All | -1.8 | -2.5 | -2.6 | -2.2 | -2.8 | -3.0 |
| Enc Time[%] | 128% |  |  | 127% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |

## Memory Bandwidth Assessment

As a rough indication of memory bandwidth increase on the decoder side, Table 3 shows the percentage of extra pixels fetched during the motion compensation process. The anchor is HM-3.0 with TB-mode disabled. From the table, enabling TB-mode consumes, on average, 2-4% more bandwidth. The maxima range roughly from 5% to 15% in all test classes, except Class A where a considerable increase (26-30%) of memory bandwidth is observed in Nebuta sequence. This is the only case that shows a more than 20% bandwidth increase.

- Memory bandwidth consumption on the decoder side

|  |  | RA-HE | RA-LC | LB-HE | LB-LC |
| --- | --- | --- | --- | --- | --- |
| Class A | Max. | 130% | 126% |  |  |
|  | Min. | 100% | 100% |  |  |
|  | Avg. | 107% | 107% |  |  |
| Class B | Max. | 115% | 114% | 109% | 107% |
|  | Min. | 100% | 100% | 96% | 97% |
|  | Avg. | 103% | 104% | 102% | 102% |
| Class C | Max. | 108% | 108% | 114% | 113% |
|  | Min. | 101% | 101% | 98% | 99% |
|  | Avg. | 104% | 104% | 104% | 104% |
| Class D | Max. | 106% | 107% | 111% | 110% |
|  | Min. | 101% | 100% | 99% | 98% |
|  | Avg. | 103% | 103% | 104% | 104% |
| Class E | Max. |  |  | 105% | 103% |
|  | Min. |  |  | 99% | 99% |
|  | Avg. |  |  | 101% | 100% |
| All | Max. | 130% | 126% | 114% | 113% |
|  | Min. | 100% | 100% | 96% | 97% |
|  | Avg. | 104% | 104% | 103% | 102% |

# Conclusion

In view of the results obtained in these experiments, we would recommend the group consider adopting the proposed technique into the next release of HM and Working Draft.

# Reference

- C.-C. Chen, C.-L. Lee, W.-H. Peng and H.-M. Hang, “CE1: Report of DMVD-based Bi-prediction,” JCTVC-E154, Geneva, Mar. 2011.

- JCT-VC, “HM-3.0,”  https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0/

- X. Zheng, P. Bordes, P. Chen, L. Guo, I.-K. Kim and W.-H. Peng, “Core Experiment 2: Motion Partitioning and OBMC,” JCTVC-E702, Geneva, Mar. 2011.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-E700, Geneva, Mar. 2011.

# Appendix

## Random Access, High Efficiency

|  | Sequences | BD-rate Y (%) | BD-rate Y (%) | BD-rate Y (%) | EncTime | DecTime (w/ yuv out) | DecTime (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -1.1 | -1.5 | -1.6 |  |  |  |
|  | PeopleOnStreet | -1.7 | -3.8 | -3.4 |  |  |  |
|  | Nebuta | -1.3 | -2.1 | -3.2 |  |  |  |
|  | SteamLocomotive | -1.2 | -2.9 | -1.7 |  |  |  |
| Class B | Kimono | -0.9 | -1.9 | -1.7 |  |  |  |
|  | ParkScene | -1.0 | -1.6 | -1.7 |  |  |  |
|  | Cactus | -1.5 | -2.6 | -2.6 |  |  |  |
|  | BasketballDrive | -1.3 | -2.9 | -2.7 |  |  |  |
|  | BQTerrace | -1.2 | -1.6 | -1.2 |  |  |  |
| Class C | BasketballDrill | -1.8 | -2.7 | -2.9 |  |  |  |
|  | BQMall | -1.9 | -2.4 | -2.3 |  |  |  |
|  | PartyScene | -1.6 | -2.1 | -2.1 |  |  |  |
|  | RaceHorses | -1.6 | -2.8 | -3.4 |  |  |  |
| Class D | BasketballPass | -1.4 | -2.6 | -2.7 |  |  |  |
|  | BQSquare | -1.8 | -1.5 | -2.0 |  |  |  |
|  | BlowingBubbles | -1.8 | -2.3 | -2.2 |  |  |  |
|  | RaceHorses | -1.8 | -2.8 | -3.2 |  |  |  |
| Average of Class A |  | -1.3 | -2.5 | -2.5 |  |  |  |
| Average of Class B |  | -1.2 | -2.1 | -2.0 |  |  |  |
| Average of Class C |  | -1.7 | -2.5 | -2.7 |  |  |  |
| Average of Class D |  | -1.7 | -2.3 | -2.5 |  |  |  |
| Overall Average |  | -1.5 | -2.4 | -2.4 | 122% | 104% | 104% |

## Low Delay B, High Efficiency

|  | Sequences | BD-rate Y (%) | BD-rate Y (%) | BD-rate Y (%) | EncTime | DecTime (w/ yuv out) | DecTime (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | Kimono | -0.9 | -2.0 | -2.0 |  |  |  |
|  | ParkScene | -1.1 | -1.8 | -1.8 |  |  |  |
|  | Cactus | -1.8 | -3.3 | -3.7 |  |  |  |
|  | BasketballDrive | -1.8 | -3.3 | -3.1 |  |  |  |
|  | BQTerrace | -0.9 | -1.7 | -1.1 |  |  |  |
| Class C | BasketballDrill | -2.2 | -3.4 | -3.7 |  |  |  |
|  | BQMall | -2.0 | -2.3 | -2.1 |  |  |  |
|  | PartyScene | -1.9 | -2.4 | -2.6 |  |  |  |
|  | RaceHorses | -1.5 | -2.4 | -2.6 |  |  |  |
| Class D | BasketballPass | -2.0 | -2.9 | -2.7 |  |  |  |
|  | BQSquare | -2.3 | -3.0 | -3.0 |  |  |  |
|  | BlowingBubbles | -1.9 | -2.4 | -2.6 |  |  |  |
|  | RaceHorses | -1.9 | -2.4 | -2.6 |  |  |  |
| Class E | Vidyo1 | -1.7 | -1.9 | -2.4 |  |  |  |
|  | Vidyo3 | -2.6 | -2.3 | -3.1 |  |  |  |
|  | Vidyo4 | -1.7 | -2.5 | -2.6 |  |  |  |
| Average of Class B |  | -1.3 | -2.4 | -2.3 |  |  |  |
| Average of Class C |  | -1.9 | -2.6 | -2.7 |  |  |  |
| Average of Class D |  | -2.0 | -2.7 | -2.7 |  |  |  |
| Average of Class E |  | -2.0 | -2.3 | -2.7 |  |  |  |
| Overall Average |  | -1.8 | -2.5 | -2.6 | 128% | 100% | 100% |

## Random Access, Low Complexity

|  | Sequences | BD-rate Y (%) | BD-rate Y (%) | BD-rate Y (%) | EncTime | DecTime (w/ yuv out) | DecTime (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -1.0 | -1.2 | -1.4 |  |  |  |
|  | PeopleOnStreet | -1.2 | -2.9 | -2.6 |  |  |  |
|  | Nebuta | -1.4 | 1.4 | -0.3 |  |  |  |
|  | SteamLocomotive | -1.3 | -2.0 | -1.3 |  |  |  |
| Class B | Kimono | -0.9 | -1.7 | -1.7 |  |  |  |
|  | ParkScene | -1.0 | -1.2 | -1.4 |  |  |  |
|  | Cactus | -1.4 | -2.4 | -2.3 |  |  |  |
|  | BasketballDrive | -1.2 | -2.3 | -2.3 |  |  |  |
|  | BQTerrace | -1.1 | -1.1 | -1.3 |  |  |  |
| Class C | BasketballDrill | -1.6 | -2.8 | -2.8 |  |  |  |
|  | BQMall | -1.7 | -2.2 | -2.5 |  |  |  |
|  | PartyScene | -1.4 | -1.9 | -1.8 |  |  |  |
|  | RaceHorses | -1.1 | -2.3 | -2.7 |  |  |  |
| Class D | BasketballPass | -1.0 | -1.9 | -2.0 |  |  |  |
|  | BQSquare | -1.6 | -1.8 | -2.1 |  |  |  |
|  | BlowingBubbles | -1.8 | -2.4 | -2.1 |  |  |  |
|  | RaceHorses | -1.1 | -2.2 | -2.1 |  |  |  |
| Average of Class A |  | -1.2 | -1.2 | -1.4 |  |  |  |
| Average of Class B |  | -1.1 | -1.7 | -1.8 |  |  |  |
| Average of Class C |  | -1.5 | -2.3 | -2.4 |  |  |  |
| Average of Class D |  | -1.4 | -2.1 | -2.1 |  |  |  |
| Overall Average |  | -1.3 | -1.8 | -1.9 | 122% | 107% | 107% |

## Low Delay B, Low Complexity

|  | Sequences | BD-rate Y (%) | BD-rate Y (%) | BD-rate Y (%) | EncTime | DecTime (w/ yuv out) | DecTime (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | Kimono | -2.0 | -2.9 | -2.4 |  |  |  |
|  | ParkScene | -1.6 | -1.5 | -2.0 |  |  |  |
|  | Cactus | -2.1 | -2.9 | -2.9 |  |  |  |
|  | BasketballDrive | -2.4 | -3.5 | -3.2 |  |  |  |
|  | BQTerrace | -1.0 | -1.3 | -2.3 |  |  |  |
| Class C | BasketballDrill | -2.2 | -2.6 | -3.1 |  |  |  |
|  | BQMall | -2.2 | -2.9 | -3.0 |  |  |  |
|  | PartyScene | -2.1 | -2.2 | -2.7 |  |  |  |
|  | RaceHorses | -1.6 | -2.4 | -2.3 |  |  |  |
| Class D | BasketballPass | -2.1 | -2.3 | -2.5 |  |  |  |
|  | BQSquare | -2.5 | -5.0 | -4.1 |  |  |  |
|  | BlowingBubbles | -2.3 | -2.9 | -3.0 |  |  |  |
|  | RaceHorses | -1.8 | -2.0 | -2.4 |  |  |  |
| Class E | Vidyo1 | -2.4 | -2.8 | -3.2 |  |  |  |
|  | Vidyo3 | -4.2 | -4.3 | -5.1 |  |  |  |
|  | Vidyo4 | -2.4 | -3.5 | -4.3 |  |  |  |
| Average of Class B |  | -1.8 | -2.4 | -2.6 |  |  |  |
| Average of Class C |  | -2.1 | -2.5 | -2.8 |  |  |  |
| Average of Class D |  | -2.2 | -3.0 | -3.0 |  |  |  |
| Average of Class E |  | -3.0 | -3.5 | -4.2 |  |  |  |
| Overall Average |  | -2.2 | -2.8 | -3.0 | 127% | 103% | 103% |

# Patent rights declaration(s)

NCTU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Exact values of these window coefficients are given in the accompanying archive, JCTVC-F049-Coeff.zip.