| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F051 |
| --- | --- |

| Title: | CE9: Results of Experiment ROB03 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reported results of CE9 ROB03 experiment. It was reported that disabling temporal motion vector predictors (MVPs) in all pictures suffered 2.3% bit rate increase, and disabling temporal MVPs in reference pictures caused 1.0% bit rate increase for random access while parsing errors could be constrained within non-reference pictures and did not affect other pictures. The results also reportedly showed that the proposed control flag of temporal MVPs in slice header provided multiple trade-off points between parsing error resilience and coding efficiency. The bit rate increases could be reduced to only 0.2% and 0.3% while the parsing error propagation was constrained within 16 and 8 pictures respectively.

# Introduction

The CE9 ROB03 experiment was approved at the 5th JCT-VC meeting. The purpose is to evaluate the tradeoff between coding efficiency and error resilience by disabling temporal motion vector predictors (MVPs) according to control flags [1]. The evaluation is based on the HM-3.0 with a fix of bug #146. Results of HM-3.0 with a fix of bug #146 under the JCTVC-E700 [2] common test conditions are used as the anchor data. The rest of this contribution is organized as follows. Details of experiments will be described in Section 2, syntax will be described in Section 3, simulation results will be shown in Section 4, and conclusions will be given in Section 5.

# Experiment description

In this proposal, three syntax elements are proposed to constrain the usage of MVPs. A first flag is proposed in the sequence parameter set (SPS) to disallow temporal MVPs for the entire sequence. When the first flag is enabled, no parsing error propagation due to temporal MVPs will occur, but the coding efficiency can be significantly decreased. When the first flag is disabled, a second flag is proposed in the SPS to disallow temporal MVPs for reference pictures. The second flag can provide another trade-off point between parsing error resilience and coding efficiency. When the second flag is enabled, only non-reference pictures which still allow temporal MVPs may cause parsing errors, which will not affect other pictures. Since temporal MVPs are conditionally used to constrain parsing errors within non-reference pictures, the coding efficiency is expected to be better than that of entirely disallowing temporal MVPs that completely stops parsing errors. A third flag is proposed in the slice header to disallow the temporal MVPs slice by slice. The flag can provide full flexibility of trade-off points between parsing error resilience and coding efficiency. When the third flag is enabled in one slice, the picture order counts (POCs) of all reference pictures in both reference lists are transmitted in the slice header. The POCs can be differentially coded by using the syntax of the ref_pic_list_modification. The purpose of sending POCs is to prevent the scaling of spatial MVPs according to POCs from causing parsing errors when a POC of a reference picture is not correctly received on the decoder side. Therefore, no parsing error propagation will be started from the current slice, and the parsing error caused from any prior slice will be terminated by the current slice.

# Syntax description

Table 1 shows the syntax design of parsing error control. Modifications are highlighted with yellow backgrounds. The first two flags, disable_temporal_mvps_in_seq and disable_temporal_mvps_in_ref_pic, are in the SPS, while the third flag, disable_temporal_mvps_in_slice and differences of POCs, are in the slice header.

Table 1.  Syntax design of the proposed parsing error control.

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| disable_temporal_mvps_in_seq | 1 | u(1) |
| if( !disable_temporal_mvps_in_seq ) |  |  |
| disable_temporal_mvps_in_ref_pic | 1 | u(1) |
| … |  |  |
| } |  |  |

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| ... |  |  |
| if( !disable_temporal_mvps_in_seq && ( !disable_temporal_mvps_in_ref_pic ¦¦ !nal_ref_idc ) ) { |  |  |
| disable_temporal_mvps_in_slice | 2 | u(1) |
| if( disable_temporal_mvps_in_slice && nal_ref_idc) { |  |  |
| for( i = 0; i < num_ref_idx_l0_default_active_minus1; i++ ) |  |  |
| diff_pic_num_minus1 |  | se(v) |
| if( slice_type = = B_SLICE ) { |  |  |
| for( i = 0; i < num_ref_idx_l1_default_active_minus1; i++ ) |  |  |
| diff_pic_num_minus1 |  | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| ... |  |  |
| } |  |  |

disable_temporal_mvps_in_seq equal to 1 denotes temporal MVPs are not used for the entire sequence; equal to 0 denotes temporal MVPs may be used in the sequence.

disable_temporal_mvps_in_ref_pic equal to 1 denotes temporal MVPs are not used in reference pictures but used in non-reference pictures; equal to 0 denotes temporal MVPs may be used in reference and non-reference pictures. If disable_temporal_mvps_in_seq is 1, disable_temporal_mvps_in_ref_pic will be set as 1.

disable_temporal_mvps_in_slice equal to 1 denotes temporal MVPs are not used in the current slice; equal to 0 denotes temporal MVPs are used in the current slice. If disable_temporal_mvps_in_slice of a slice is 1, the propagation of any parsing error prior to the current slice will be terminated.

diff_pic_num_minus1 plus 1 specifies the difference between the picture number of the picture pointed by the current index in the list and the picture number of the picture pointed by previous index in the list. When the picture pointed by the previous index in the list does not exist, the picture number of the picture pointed by previous index in the list is replaced by that of the current picture.

# Simulation results

Several experiments were conducted to evaluate the coding efficiency and error resilience by disabling the temporal MVPs. In Experiment 1, we disable temporal MVPs for all pictures. The results are shown in Table 2. Disabling temporal MVPs causes 2.3%, 2.2%, 2.4%, and 2.4% bit rate increases for high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD), respectively.

Table 2.  Results of Experiment 1: disabling temporal MVPs in all pictures.

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3.1 | 2.6 | 2.8 | 3.3 | 2.8 | 2.8 |
| Class B | 2.0 | 1.4 | 1.4 | 1.7 | 1.1 | 1.1 |
| Class C | 2.1 | 2.1 | 2.2 | 2.0 | 2.0 | 2.1 |
| Class D | 2.1 | 2.0 | 2.0 | 2.0 | 1.8 | 1.8 |
| Class E |  |  |  |  |  |  |
| All | 2.3 | 2.0 | 2.0 | 2.2 | 1.9 | 1.9 |
| Enc Time[%] | 92% |  |  | 93% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 2.1 | 1.8 | 1.2 | 2.3 | 1.6 | 1.5 |
| Class C | 2.5 | 2.6 | 2.7 | 2.6 | 2.4 | 2.4 |
| Class D | 2.2 | 1.8 | 2.1 | 2.2 | 1.2 | 1.3 |
| Class E | 2.9 | 2.7 | 2.1 | 2.6 | 1.4 | 2.0 |
| All | 2.4 | 2.2 | 2.0 | 2.4 | 1.7 | 1.8 |
| Enc Time[%] | 93% |  |  | 93% |  |  |
| Dec Time[%] | 98% |  |  | 94% |  |  |

In Experiment 2, we disable the temporal MVPs in reference pictures for random access conditions. The results are shown in Table 3. Disabling temporal MVPs for reference pictures causes 1.0% and 1.0% bit rate increases for HE-RA and LC-RA while parsing errors can thus be constrained within non-reference pictures and will not affect other pictures.

Table 3.  Results of Experiment 2: disabling temporal MVPs in reference pictures.

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.5 | 1.4 | 1.6 | 1.8 | 1.7 | 1.6 |
| Class B | 0.9 | 0.7 | 0.7 | 0.7 | 0.6 | 0.6 |
| Class C | 0.8 | 0.7 | 0.8 | 0.7 | 0.7 | 0.8 |
| Class D | 1.0 | 1.0 | 0.9 | 1.0 | 0.7 | 0.7 |
| Class E |  |  |  |  |  |  |
| All | 1.0 | 1.0 | 1.0 | 1.0 | 0.9 | 0.9 |
| Enc Time[%] | 96% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

In Experiments 3 and 4, we periodically disable the temporal MVPs in every 8 and 16 pictures, respectively. The results are shown in Table 4 and 5. The results of Experiment 3 shows 0.3% bit rate increase on average while the parsing error propagation can be constrained within eight pictures. The results of Experiment 4 shows 0.2% bit rate increase on average while the parsing error propagation can be constrained within 16 pictures. As shown in these results, multiple trade-off points between parsing error control and coding efficiency can be provided by different frequencies of disabling temporal MVPs in the slice header.

Table 4.  Results of Experiment 3: disabling temporal MVPs in every eight pictures.

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.8 | 0.6 | 0.8 | 0.8 | 0.5 | 0.6 |
| Class B | 0.1 | 0.1 | 0.2 | 0.1 | 0.1 | 0.2 |
| Class C | 0.1 | 0.1 | 0.2 | 0.1 | 0.1 | 0.2 |
| Class D | 0.3 | 0.3 | 0.2 | 0.3 | 0.1 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.3 | 0.3 | 0.3 | 0.2 | 0.3 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | 0.0 | -0.5 | 0.4 | 0.1 | 0.2 |
| Class C | 0.4 | 0.3 | 0.3 | 0.4 | 0.3 | 0.2 |
| Class D | 0.3 | 0.0 | 0.8 | 0.4 | 0.0 | 0.0 |
| Class E | 0.5 | 0.3 | 0.4 | 0.4 | -0.2 | 0.6 |
| All | 0.4 | 0.1 | 0.2 | 0.4 | 0.0 | 0.3 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 95% |  |  | 99% |  |  |

Table 5.  Results of Experiment 4: disabling temporal MVPs in every 16 pictures.

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | 0.3 | 0.6 | 0.6 | 0.4 | 0.4 |
| Class B | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class C | 0.0 | 0.1 | 0.1 | 0.1 | 0.0 | 0.2 |
| Class D | 0.2 | 0.2 | 0.1 | 0.1 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.1 | 0.2 | 0.2 | 0.1 | 0.2 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.0 | -0.1 | 0.2 | 0.2 | 0.0 |
| Class C | 0.2 | 0.2 | 0.1 | 0.2 | 0.1 | 0.0 |
| Class D | 0.2 | 0.0 | 0.2 | 0.2 | -0.4 | 0.0 |
| Class E | 0.3 | 0.7 | 0.4 | 0.2 | 0.1 | 0.7 |
| All | 0.2 | 0.2 | 0.1 | 0.2 | 0.0 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 92% |  |  | 99% |  |  |

# Conclusions

MediaTek proposed a first flag to disable temporal MVPs in all pictures and a second flag to disable temporal MVPs in all reference pictures. The first flag and the second flag were coded in the SPS. Besides, a third flag was also proposed in the slice header to disable temporal MVPs at the slice level, which could allow flexible trade-off points between parsing error resilience and coding efficiency. Experimental results reportedly showed that 0.3% and 0.2% bit rate increases were observed while the parsing error propagation was constrained within eight or 16 pictures, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J.-L. Lin, C.-L. Wu, Y.-W. Chen, Y.-P. Tsai, Y.-W. Huang, S. Lei, “Parsing Robustness: Constrained Usage of Temporal MV and MVP Candidates,” Document of Joint Collaborative Team on Video Coding, JCTVC-E050, Mar. 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-E700, Mar. 2011.