| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F052 |
| --- | --- |

| Title: | CE9: Results of Experiment ROB04 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reported the performance results of CE9 ROB04 experiment on parsing robustness. In this experiment, a basic scheme with parsing robustness by skipping the process of removing redundant motion vector predictors (MVPs) and forcing the availability of the temporal MVP was firstly provided. The experiment results reported that this basic scheme resulted in average 1.3%, 1.3%, 1.9%, and 2.0% bit rate increases for high HE-RA, LC-RA, HE-LD, and LC-LD, respectively. To reduce the bit rate increases, two approaches were tested in ROB04. The first approach was replacing the redundant MVPs by other substitutes, and the second approach was reducing redundant MVPs without comparing motion vector values. It was reported that combing both approaches caused 0.3%, 0.4%, 0.3%, and 0.5% bit rate increases for HE-RA, LC-RA, HE-LD, and LC-LD, respectively, while providing the ability to prevent all parsing errors caused by any data loss of reference pictures.

# Introduction

The ROB04 experiment was approved at the 5th JCT-VC meeting in CE9 [1]. The ROB04 experiment is to evaluate the coding efficiency of the parsing robustness solution for MVPs. In HM-3.0 [2] and WD-3.0 [3], to reduce the bits used to encode the index of motion vector predictor (MVP), redundant MVPs are removed from the MVP candidate set. However, this can cause parsing errors leading to severe error propagation. When a motion vector (MV) of a previous picture cannot be decoded correctly, a mismatch between the candidate set on the encoder side and that on the decoder side may occur, resulting in parsing error of the index of the best MVP. Then, the rest of the current picture cannot be parsed or decoded properly. What is even worse, this parsing error can affect subsequent inter pictures that also use temporal MVPs. ROB04 was proposed to achieve the parsing robustness. The evaluation was based on the HM-3.0 with a fix of bug #146. The rest of this contribution is organized as follows. The experiment of ROB04 will be described in Section 2, simulation results will be shown in Section 3, and finally, conclusions will be given in Section 4.

# Experiment description

To achieve parsing robustness, two constraints are applied for the basic approach of parsing robustness:

- Avoid the process of removing redundant MVPs

- Force the availability of the temporal MVP as long as it is inside the picture boundaries

In order to avoid the parsing problem caused by incorrect MV data, the process of removing redundant MVPs is avoided. Moreover, the temporal MVP is forced to be available as long as the temporal block is inside the boundaries of that co-located picture. Even if the temporal block in the co-located picture is an Intra block, the parsing robustness system still force the temporal MVP to be available to prevent the case that the mode type (Inter or Intra) of the block in the co-located picture is lost or wrong. When the temporal MVP is not available, a zero MV is simply used to represent the temporal MVP. Two tools are applied to improve the coding efficiency of the parsing robustness solution. The first tool replaces redundant MVPs by other substitutes, and the second tool reduces redundant MVPs without comparing MV values.

## Tool 1 – Replacing redundant MVPs

Since the process of removing redundant MVPs is avoided for parsing robustness, the redundant MVPs are replaced by other substitutes to reduce the redundancy.

In HM-3.0, the top MVP in Inter mode is defined as the first available and non-redundant one searching within B0, B1, B2 as shown in Figure 1 based on the following order:

  - B0’ MV with the same list and the same reference picture

  - B1’ MV with the same list and the same reference picture

  - B2’ MV with the same list and the same reference picture

  - B0’ MV with different list or/and different reference picture

  - B1’ MV with different list or/and different reference picture

  - B2’ MV with different list or/and different reference picture

Please note that “the same list,” “the same reference picture,” “different list,” and “different reference picture” mean the “the same list as the current list,” “the same reference picture as the current picture,” “different list from the current list,” and “different reference picture from the current reference picture.” Moreover, the description “MV with different list or/and different reference picture” actually means three sequential items with the order as: “MV with different list and the same reference picture,” “MV with the same list and different reference picture,” and “MV with different list and different reference picture.”

In our approach, the top MVP is only searched within

  - B0’ MV with the same list and the same reference picture

  - B1’ MV with the same list and the same reference picture

Therefore, the top MVP exists only when B0 or B1 has a MV with the same list and the same reference picture. If the top MVP exists and all the possible MVPs are identical to the left MVP, the substitute is searched based on the following order:

  - B2’ MV with the same list and the same reference picture

  - B0’ MV with different list or/and different reference picture

  - B1’ MV with different list or/and different reference picture

  - B2’ MV with different list or/and different reference picture

After all these substitutes are searched and the top MVP is still redundant, the left MVP plus 1/4 pixel in both horizontal and vertical directions is used.

Figure 1.  Spatial blocks.

For the spatial MVPs in Merge/Merge-Skip modes, if one spatial MVP exists and is identical to one of the previously derived MVPs, we simply use a zero MV to replace the redundant MVP.

For the temporal MVP in Inter and Merge/Merge-Skip modes, as an example shown in Figure 2 where the gray area denotes the current prediction unit (PU), if the temporal MVP derived from H is redundant, a substitute is searched within Tc, To, F, and G until the first non-redundant one is found. If after these blocks are searched and the temporal MVP is still redundant, the zero MV is used.

Figure 2.  Temporal blocks.

## Tool 2 – Reducing redundant MVPs without comparing MV values

Tool 2 tries to reduce redundant MVPs without using MV values before finding other substitutes for the redundancy reduction. In tool 2, motion identification (ID) is proposed to describe the inheritance process of motion information passed from a coded PU to the current PU to follow the trail of the MV. A unique motion ID is assigned for each new MV created in the Inter mode and can be inherited through the Merge and Merge-Skip modes. By comparing the motion ID, the MVP having the same motion ID as a previous MVP in the candidate set can thus be removed.

# Simulation results

Several experiments are conducted to evaluate the coding efficiency and parsing robustness. HM-3.0 software [2] is used for the evaluation and the common test conditions specified in JCTVC-E700 [4] are followed. The anchor is HM-3.0 with the fix of bug #146 as described in CE9 [1].

Since the parsing problem is caused by including the temporal MVP in the candidate set, in experiment 1, we simply disable the temporal MVP for all pictures to avoid the parsing problem. The results are shown in Table 1. It shows that disabling temporal MVPs causes 2.3%, 2.2%, 2.4%, and 2.4% bit rate increases for high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD), respectively.

In experiment 2, the basic approach for parsing robustness is implemented to avoid the parsing error by skipping the process of removing redundant MVPs and forcing the availability of the temporal MVP. The results are shown in Table 2. The basic approach for parsing robustness causes 1.3%, 1.3%, 1.9%, and 2.0% bit rate increases for HE-RA, LC-RA, HE-LD, and LC-LD while providing the ability to prevent parsing errors caused by data loss of reference pictures.

Table 1.  Results of experiment 1: disabling temporal MVPs for all pictures.

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3.1 | 2.6 | 2.8 | 3.3 | 2.8 | 2.8 |
| Class B | 2.0 | 1.4 | 1.4 | 1.7 | 1.1 | 1.1 |
| Class C | 2.1 | 2.1 | 2.2 | 2.0 | 2.0 | 2.1 |
| Class D | 2.1 | 2.0 | 2.0 | 2.0 | 1.8 | 1.8 |
| Class E |  |  |  |  |  |  |
| All | 2.3 | 2.0 | 2.0 | 2.2 | 1.9 | 1.9 |
| Enc Time[%] | 92% |  |  | 93% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 2.1 | 1.8 | 1.2 | 2.3 | 1.6 | 1.5 |
| Class C | 2.5 | 2.6 | 2.7 | 2.6 | 2.4 | 2.4 |
| Class D | 2.2 | 1.8 | 2.1 | 2.2 | 1.2 | 1.3 |
| Class E | 2.9 | 2.7 | 2.1 | 2.6 | 1.4 | 2.0 |
| All | 2.4 | 2.2 | 2.0 | 2.4 | 1.7 | 1.8 |
| Enc Time[%] | 93% |  |  | 93% |  |  |
| Dec Time[%] | 98% |  |  | 94% |  |  |

Table 2.  Results of experiment 2: basic approach – skipping the process of removing redundant MVPs and forcing the availability of the temporal MVP.

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.0 | 0.9 | 1.0 | 1.2 | 1.0 | 1.0 |
| Class B | 1.3 | 0.8 | 0.8 | 1.4 | 1.0 | 0.9 |
| Class C | 1.4 | 1.3 | 1.3 | 1.4 | 1.3 | 1.4 |
| Class D | 1.3 | 1.1 | 1.1 | 1.3 | 0.9 | 1.1 |
| Class E |  |  |  |  |  |  |
| All | 1.3 | 1.0 | 1.0 | 1.3 | 1.0 | 1.1 |
| Enc Time[%] | 108% |  |  | 107% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.6 | 1.2 | 0.7 | 1.7 | 1.2 | 1.2 |
| Class C | 1.5 | 1.3 | 1.2 | 1.4 | 1.0 | 1.2 |
| Class D | 1.5 | 1.0 | 1.6 | 1.4 | 1.0 | 0.9 |
| Class E | 3.6 | 3.4 | 2.7 | 4.1 | 2.9 | 3.5 |
| All | 1.9 | 1.6 | 1.4 | 2.0 | 1.4 | 1.6 |
| Enc Time[%] | 107% |  |  | 106% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

In experiment 3, the tool 1 described in section 2.1 is implemented to find other substitutes for redundant MVPs. According to the results shown in Table 3, tool 1 causes 0.9%, 0.9%, 1.4%, and 1.3% bit rate increases for HE-RA, LC-RA, HE-LD, and LC-LD, respectively.

In experiment 4, the tool 2 described in Section 2.2 is implemented to reduce redundant MVPs without using MV values. According to the results shown in Table 4, Tool 2 causes 0.5%, 0.7%, 0.6%, and 0.9% bit rate increases for HE-RA, LC-RA, HE-LD, and LC-LD, respectively.

In experiment 5, we combine tool 1 and tool 2. The motion ID is firstly applied to reduce redundant MVPs. For the remaining redundant MVPs that cannot be removed by using motion IDs, tool 1 is then applied to find the replacement. According to the results shown in Table 5, experiment 5 achieves 0.3%, 0.4%, 0.3%, and 0.5% bit rate increases for HE-RA, LC-RA, HE-LD, and LC-LD, respectively, while providing the ability of parsing robustness.

Table 3.  Results of experiment 3: proposed tool 1 – replacing redundant MVPs by the substitutes described in Section 2.1.

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 0.9 | 1.1 | 0.7 | 0.8 | 0.8 |
| Class B | 0.9 | 0.6 | 0.5 | 0.9 | 0.7 | 0.7 |
| Class C | 1.0 | 1.0 | 1.0 | 1.0 | 0.9 | 1.0 |
| Class D | 0.9 | 0.8 | 0.9 | 0.8 | 0.6 | 0.8 |
| Class E |  |  |  |  |  |  |
| All | 0.9 | 0.8 | 0.8 | 0.9 | 0.8 | 0.8 |
| Enc Time[%] | 109% |  |  | 108% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.1 | 0.6 | 0.4 | 1.0 | 0.6 | 0.7 |
| Class C | 1.0 | 0.6 | 0.7 | 0.8 | 0.6 | 0.5 |
| Class D | 1.0 | 0.8 | 0.9 | 0.9 | 0.5 | 0.5 |
| Class E | 3.0 | 2.8 | 3.1 | 3.0 | 2.4 | 3.2 |
| All | 1.4 | 1.1 | 1.1 | 1.3 | 0.9 | 1.1 |
| Enc Time[%] | 108% |  |  | 107% |  |  |
| Dec Time[%] | 99% |  |  | 105% |  |  |

Table 4.  Results of experiment 4: proposed tool 2 – reducing redundant MVPs by comparing motion IDs instead of MV values as described in Section 2.2.

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | 0.9 | 0.8 | 0.7 | 0.8 | 0.8 |
| Class B | 0.5 | 0.3 | 0.3 | 0.6 | 0.5 | 0.5 |
| Class C | 0.6 | 0.6 | 0.7 | 0.9 | 0.9 | 1.1 |
| Class D | 0.5 | 0.5 | 0.5 | 0.6 | 0.5 | 0.6 |
| Class E |  |  |  |  |  |  |
| All | 0.5 | 0.6 | 0.6 | 0.7 | 0.7 | 0.7 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.5 | 0.3 | 0.1 | 0.7 | 0.4 | 0.1 |
| Class C | 0.6 | 0.5 | 0.5 | 0.7 | 0.7 | 0.5 |
| Class D | 0.4 | 0.0 | 0.4 | 0.6 | 0.2 | 0.0 |
| Class E | 1.3 | 1.2 | 0.4 | 1.8 | 1.1 | 1.6 |
| All | 0.6 | 0.5 | 0.3 | 0.9 | 0.5 | 0.5 |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 5.  Results of experiment 5: proposed tool 1 and tool 2 – first reducing redundant MVPs by comparing motion IDs instead of MV values as described in Section 2.2 and then replacing redundant MVPs by the substitutes described in Section 2.1.

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.6 | 0.6 | 0.4 | 0.6 | 0.6 |
| Class B | 0.2 | 0.2 | 0.2 | 0.3 | 0.3 | 0.3 |
| Class C | 0.4 | 0.4 | 0.6 | 0.6 | 0.7 | 0.7 |
| Class D | 0.4 | 0.4 | 0.3 | 0.4 | 0.4 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.4 | 0.4 | 0.4 | 0.5 | 0.5 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | -0.1 | -0.2 | 0.3 | 0.1 | 0.1 |
| Class C | 0.3 | 0.1 | 0.1 | 0.4 | 0.4 | 0.3 |
| Class D | 0.2 | -0.1 | 0.8 | 0.3 | 0.3 | -0.2 |
| Class E | 0.8 | 1.0 | -0.2 | 1.3 | 1.2 | 1.2 |
| All | 0.3 | 0.2 | 0.1 | 0.5 | 0.4 | 0.3 |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

# Conclusions

ROB04 was tested in CE9. Experiment 2 could solve the parsing problem caused by data loss of reference pictures and two tools are proposed to improve the coding efficiency of the parsing robustness solution. The first tool is replacing redundant MVPs by substitutes, and the second tool is reducing redundant MVPs by comparing motion IDs instead of MV values. The experimental results reportedly showed that the parsing robustness solution combining the proposed two tools caused 0.3%, 0.4%, 0.3%, and 0.5% bit rate increases for HE-RA, LC-RA, HE-LD, and LC-LD, respectively, while providing the ability to prevent all parsing errors caused by any data loss of reference pictures.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-W. Huang, B. Bross, M. Zhou, W.-J. Chien, I.-K. Kim, “Description of Core Experiment 9 (CE9): MV Coding and Skip/Merge Operations,” Document of Joint Collaborative Team on Video Coding, JCTVC-E709, Mar. 2011.

- K. McCann, S. Sekiguci, B. Bross, W.-J. Han, “HEVC Test Model 3 (HM 3) Encoder Description,” Document of Joint Collaborative Team on Video Coding, JCTVC-E602, Mar. 2011.

- T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-E603, Mar. 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-E700, Mar. 2011.