### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I ) |  |
| { |  |
| disable_temporal_mvps_in_slice | u(1) |
| } |  |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

disable_temporal_mvps_in_slice equal to 1 denotes that the temporal MVP candidates are not used in the current slice; equal to 0 denotes that temporal MVP candidates are used in the current slice. If temporal_mvp_candidates_not_used_in_slice of a slice is equal to 1, the propagation of any parsing error prior to the current slice will be terminated.

##### Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1,

- the motion IDs mvIDL0 and mvIDL1.

- For N from 0 to 4, where N is the candidate index, the motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1, and the motion IDs mvIDL0 and mvIDL1 are derived as specified by the following ordered steps:

- If candidate index N is equal to 0, 1, 3 or 4, the derivation for merging candidates from neighboring prediction unit partitions is processed by the following ordered steps:

- Subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx and the candidate index N as inputs and the output is assigned to the availability flags availableFlagN, the motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and mvIDL0N and mvIDL1N.

- When availableFlagN is equal to 1 and for Idx from 0 to N-1, where Idx is used as the index for the previously derived candidates, the following applies repeatedly:

- When avaliableFlagIdx is equal to 1 and mvIDLXN is equal to mvIDLXIdx and refIdxLXN is equal to  refIdxLXIdx and predFlagLXN is equal to predFlagLXIdx (X being replaced by 0 or 1), availableFlagN is set equal to 0.

- When availableFlagN is equal to 1 and for Idx from 0 to N-1, where Idx is used as the index for the previously derived candidates, the following applies repeatedly:

- When avaliableFlagIdx is equal to 1 and mvLXN is equal to mvLXIdx and refIdxLXN is equal to  refIdxLXIdx and predFlagLXN is equal to predFlagLXIdx (X being replaced by 0 or 1), both components of mvLXN are set equal to 0.

- Otherwise, the derivation process for temporal luma motion vector prediction is processed by the following ordered steps:

- The derivation process of reference indices for temporal merging candidate in subclause  is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

- Variable colPu0 is derived in the following ordered steps:

  - Right-bottom luma position of the current prediction unit is defined by

xPTMVP = xP + nPSW	(8-1)

yPTMVP = yP + nPSH	              (8-2)

  - The variable colPu0 is set as the prediction unit covering the modified position given by ( ( xPTMVP >> 4 ) << 4, ( yTMVP >> 4 ) << 4 ) inside the colPic.

  - ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu0 relative to the top-left luma sample of the colPic

- When disable_temporal_mvps_in_slice is equal to 1 or colPu0 is at outside of the slice boundary, availableFlagN is set to 0 and predFlagLXN is set to 0.

- When availableFlagN is equal to 1, the variables  mvIDLXN is derived as specified below.

mvIDLXN  =  ( ( (yPCol>>4)<<2)+X) * (PicWidthInSamplesL <<2) + ( ( (xPCol>>4)<<2)+ refIdxLX) 	(8-3)

- When availableFlagN is equal to 1, for Idx from 0 to N-1, where Idx is used as the index for the previously derived candidates, the following applies repeatedly:

- When avaliableFlagIdx is equal to 1 and mvIDLXN is equal to mvIDLXIdx and refIdxLXN is equal to  refIdxLXIdx and predFlagLXN is equal to predFlagLXIdx (X being replaced by 0 or 1), availableFlagN is set equal to 0 and predFlagLXN is set ot 0.

- When availableFlagN is equal to 1, the derivation process for temporal luma motion vector prediction in subclause 4 is invoked with luma location ( xP, yP ), refIdxLX, colPu0 as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables mvLXN , availableFlagN and predFlagLXN (with X being 0 or 1, respectively) are derived as specified below.

mvLXN = mvLXCol	(8-4)

predFlagLXN = availableFlagLXIdx	(8-5)

- If one of the following conditions is true, the availableFlagN is set equal to 0.

  - PartMode of the current prediction unit is PART_2NxN and PartIdx is equal to 1 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 0 ) and Candidate N have identical motion parameters:

  - mvIDLX[ xP,  yP – 1 ]  = =  mvIDLXN

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX

    - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLXN

  - PartMode of the current prediction unit is PART_Nx2N and PartIdx is equal to 1 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 0 ) and Candidate N have identical motion parameters:

  - mvIDLX[ xP – 1,  yP ]  = =  mvIDLXN

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLXN

  - PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 )  have identical motion parameters:

  - mvIDLX[ xP – 1,  yP ]  = =  mvIDLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

  - and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and Candidate N have identical motion parameters:

  - mvIDLX[ xP,  yP – 1 ]  = =  mvIDLXN

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLXN

  - PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 ) have identical motion parameters:

  - mvIDLX[ xP,  yP – 1 ]  = =  mvIDLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

  - and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and Candidate N have identical motion parameters:

  - mvIDLX[ xP – 1,  yP ]  = =  mvIDLXN

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLXN

- When availableFlagN is equal to 1, for colPuk from colPu1 to colPu4, the following applies repeatedly to find a non-redundant candidate.

For Idx from 0 to N-1, where Idx is used as the index for the previously derived candidates, the following applies repeatedly:

- When avaliableFlagIdx is equal to 1 and mvLXN is equal to mvLXIdx and refIdxLXM is equal to  refIdxLXIdx and predFlagLXM is equal to predFlagLXIdx (X being replaced by 0 or 1), the derivation process for temporal luma motion vector prediction in subclause 4 is invoked with luma location ( xP, yP ), refIdxLX, colPuk as the inputs and with  the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables mvLXN (with X being 0 or 1, respectively) are derived as specified below.

mvLXN = mvLXCol	(8-6)

- When availableFlagN is equal to 1 and for Idx from 0 to N-1, where Idx is used as the index for the previously derived candidates, the following applies repeatedly:

- When avaliableFlagIdx is equal to 1 and mvLXN is equal to mvLXIdx and refIdxLXN is equal to  refIdxLXIdx and predFlagLXN is equal to predFlagLXIdx (X being replaced by 0 or 1), both components of mvLXN are set equal to 0.

- The merging candidate list, mergeCandList, is constructed of which elements are given as specified order:

  - 0, if availableFlag0 is equal to 1

  - 1, if availableFlag1 is equal to 1

  - 2, if availableFlag2 is equal to 1

  - 3, if availableFlag3 is equal to 1

  - 4, if availableFlag4 is equal to 1

- If several merging candidates have the motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- If the number of elements NumMergeCand within the mergeCandList is equal to 1, mergeIdx is set equal to 0, otherwise, mergeIdx is set equal to merge_idx[ xP][ yP ].

- The following assignments are made with N being the candidate at position mergeIdx in the merging candidate list mergeCandList ( N = mergeCandList[ mergeIdx ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-7)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-8)

refIdxLX = refIdxLXN	(8-9)

predFlagLX = predFlagLXN	(8-10)

mvIDLX[ 0 ] = mvIDLXN[ 0 ]	(8-11)

mvIDLX[ 1 ] = mvIDLXN[ 1 ]	(8-12)

- If all availability flags availableFlagN (with N being replaced by 0, 1, 2, 3, or 4) are equal to 0, mergeIdx is set equal to 0 and the variables mvLX, refIdxLX and predFlagLX (with X being replaced by 0 or 1) are inferred as follows.

If slice_type is equal to P, the following applies.

mvLX[ 0 ] = 0	(8-13)

mvLX[ 1 ] = 0	(8-14)

refIdxL0 = 0		(8-15)

refIdxL1 = -1	(8-16)

predFlagL0 = 1	(8-17)

prefFlagL1 = 0	(8-18)

mvIDLX[ 0 ] = -2	(8-19)

mvIDLX[ 1 ] = -2	(8-20)

Otherwise ( slice_type is equal to B ), the following applies.

mvLX[ 0 ] = 0	(8-21)

mvLX[ 1 ] = 0	(8-22)

refIdxL0 = 0		(8-23)

refIdxL1 = 0		(8-24)

predFlagL0 = 1	(8-25)

prefFlagL1 = 1	(8-26)

mvIDLX[ 0 ] = -2	(8-27)

mvIDLX[ 1 ] = -2	(8-28)

##### 8.4.2.1.2 Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit,

- a variable N indicating which candidate is going to be derived (0:A,1: B,3: C or 4:D).

Outputs of this process are (with X being replaced by 0 or 1)

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units,

- the motion IDs mvIDLXN of the neighbouring prediction units.

Figure 8-1 – Spatial neighbours that can be used as merging candidates (informative) illustrates the position of the spatial neighbours A, B, C and D relative to the current prediction unit.

For the derivation of availableFlagN, with N being A, B, C or D and ( xN, yN ) being ( xP – 1,  yP ), ( xP,  yP – 1 ), ( xP + nPSW,  yP – 1 ) or ( xP – 1,  yP+nPSH ) the following applies.

–	If one of the following conditions is true, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

  - The prediction unit covering luma location ( xN, yN ) is not available or PredMode is MODE_INTRA.

- PartMode of the current prediction unit is PART_2NxN and PartIdx is equal to 1 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvIDLX[ xP,  yP – 1 ]  = =  mvIDLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

    - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_Nx2N and PartIdx is equal to 1 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvIDLX[ xP – 1,  yP ]  = =  mvIDLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 )  have identical motion parameters:

  - mvIDLX[ xP – 1,  yP ]  = =  mvIDLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvIDLX[ xP,  yP – 1 ]  = =  mvIDLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 ) have identical motion parameters:

  - mvIDLX[ xP,  yP – 1 ]  = =  mvIDLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvIDLX[ xP – 1,  yP ]  = =  mvIDLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ],  predFlagLX[ xN, yN ] and mvIDLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN, predFlagLXN and mvIDLXN.

##### Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1),

- the motion IDs mvIDLX.

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause  is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN and the motion IDs mvIDLXN with N being replaced by A, B as the output.

- The derivation process of reference indices for temporal merging candidate in subclause  is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P), the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

- Variable colPu0 is derived in the following ordered steps:

  - Right-bottom luma position of the current prediction unit is defined by

xPTMVP = xP + nPSW	(8-29)

yPTMVP = yP + nPSH	              (8-30)

  - The variable colPu0 is set as the prediction unit covering the modified position given by ( ( xPTMVP >> 4 ) << 4, ( yTMVP >> 4 ) << 4 ) inside the colPic.

  - ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu0 relative to the top-left luma sample of the colPic

- When disable_temporal_mvps_in_slice is equal to 1 or colPu0 is at outside of the slice boundary, availableFlagCol is set to 0 and predFlagLXCol is set to 0.

- When availableFlagCol is equal to 1, the variables  mvIDLXCol is derived as specified below.

mvIDLXCol  =  ( ( (yPCol>>4)<<2)+X) * (PicWidthInSamplesL <<2) + ( ( (xPCol>>4)<<2)+ refIdxLX) 	(8-31)

- When availableFlagCol is equal to 1, for candidate N ( N being A or B), the following applies repeatedly:

- When avaliableFlagN is equal to 1 and mvIDLXCol is equal to mvIDLXN and refIdxLXCol is equal to  refIdxLXN and predFlagLXCol is equal to predFlagLXN (X being replaced by 0 or 1), availableFlagCol is set equal to 0 and predFlagLXCol is set ot 0.

- When availableFlagCol is equal to 1, the derivation process for temporal luma motion vector prediction in subclause 4 is invoked with luma location ( xP, yP ), refIdxLX, colPu0 as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

predFlagLXCol = availableFlagLXCol	(8-32)

- When availableFlagCol is equal to 1, for colPuk from colPu1 to colPu4, the following applies repeatedly to find a non-redundant candidate.

For thoses candidates N (N being A or B), when availableFlagN=1 and refIdxLX is equal to refIdxLXN and  mvLXCol is equal to mLXN, the derivation process for temporal luma motion vector prediction in subclause 4 is invoked with luma location ( xP, yP ), refIdxLX, colPuk as the inputs and with  the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol.

- When availableFlagCol is equal to 1 and for those N (N being A or B) being the candidate with avaliableFlagN equal to 1, if refIdxLX is equal to refIdxLXA and  mvLXCol is equal to refIdxLXN, both components of mvLXCol are set equal to 0.

- The motion vector predictor list, mvpListLX, is constructed of which elements are given as specified order:

- mvLXCol, if availableFlagLXCol is equal to 1

- mvLXA, if availableFlagLXA is equal to 1

- mvLXB, if availableFlagLXB is equal to 1

- If several motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- If the number of elements NumMVPCand( LX ) within the mvpListLX is equal to 1, mvpIdx is set equal to 0, otherwise, mvpIdx is set equal to mvp_idx_lX[ xP, yP ].

- The motion vector of mvListLX[ mvpIdx ] is assigned to mvpLX.

- The motion IDs mvIDLX is assigned to a new unique motion ID number.

##### Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A,or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units,

- the motion IDs mvIDLXN of the neighbouring prediction units.

Figure 8-2 – Spatial motion vector neighbours

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0 .. 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- When the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX and mvIDLXA is set equal to the motion ID mvIDLX[ xAk ][ yAk ].

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] and ListA is set equal to LY and mvIDLXA is set equal to the motion ID mvIDLX[ xAk ][ yAk ].

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX and mvIDLXA is set equal to -1.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY and mvIDLXA is set equal to -1.

- When availableFlagLXA is equal to 1, the following applies.

- If RefPicOrderCnt( currPic , refIdxA, ListA ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), mvLXA is set equal to mvLXA

- Otherwise, mvLXA is derived as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-33)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-34)

mvLXA = ClipMv( ( DistScaleFactor * mvLXA + 128 ) >> 8 )	(8-35)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxA, ListA ) )	(8-36)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-37)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) and ( xB1, yB1 ) where xB0 = xP +nPSW and xB1 = xB0 - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- When the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX and the motion ID mvIDLX[ xBk ][ yBk ] is not equal to mvIDLXA, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX and mvIDLXB is set equal to the motion ID mvIDLX[ xBk ][ yBk ].

- When availableFlagLXB is equal to 1 and mvLXB is identical to mvLXA, for ( xB2, yB2 ) where xB2 = xP – MinPuSize, the following applies:

- When the prediction unit covering luma location ( xB2, yB2  ) is available, PredMode is not MODE_INTRA, predFlagLX[ xB2 ][ yB2 ] is equal to 1, and the reference index refIdxLX[ xB2 ][ yB2 ] is equal to the reference index of the current prediction unit refIdxLX, the motion vector mvLXB is set equal to the motion vector mvLX[ xB2 ][ yB2 ], refIdxB is set equal to refIdxLX[ xB2 ][ yB2 ] and ListB is set equal to LX.

- When availableFlagLXB is equal to 1 and mvLXB is identical to mvLXA, for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until mvLXB is not identical to mvLXA:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ](with Y = !X) is equal to 1 and RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ] and ListB is set equal to LY.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA and RefPicOrderCnt( currPic , refIdxB, ListB ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), mvLXB is set equal to mvLXB.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA and RefPicOrderCnt( currPic , refIdxB, ListB ) is not equal to RefPicOrderCnt( currPic, refIdxLX, LX ),, mvLXB is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-38)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-39)

mvLXB = ClipMv( ( DistScaleFactor * mvLXB + 128 ) >> 8 )	(8-40)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxB, ListB ) )	(8-41)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-42)

- When availableFlagLXB is equal to 1 and mvLXB is identical to mvLXA, availableFlagLXB is set equal to 0.

##### Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1),

- the collocated PU colPuk.

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

For k from 0 to 4, variable colPuk is derived in the following ordered steps:

- If k is equal to 0, right-bottom luma position of the current prediction unit is defined by

xPTMVP = xP + nPSW	(8-43)

yPTMVP = yP + nPSH	              (8-44)

- Otherwise if k is equal to 1, central luma position of the current prediction unit is defined by

xPTMVP = ( xP + ( nPSW >> 1 ) – 1	(8-45)

yPTMVP = ( yP + ( nPSH >> 1 ) – 1	(8-46)

- Otherwise if k is equal to 2, left-top luma position of the current prediction unit is defined by

xPTMVP = xP 	(8-47)

yPTMVP = yP	(8-48)

- Otherwise if k is equal to 3, the luma position to the right of right-top of the current prediction unit is defined by

xPTMVP = xP + nPSW(8-49)

yPTMVP = yP	(8-50)

- Otherwise if k is equal to 4, the luma position to the below of left-bottom of the current prediction unit is defined by

xPTMVP = xP	(8-51)

yPTMVP = yP + nPSH	(8-52)

The variable colPuk is set as the prediction unit covering the modified position given by ( ( xPTMVP >> 4 ) << 4, ( yTMVP >> 4 ) << 4 ) inside the colPic. ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPuk relative to the top-left luma sample of the colPic. The variables mvLXCol and availableFlagLXCol are derived as follows.

  - If colPuk is coded in an intra prediction mode or colPuk is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

  - Otherwise (colPuk is not coded in an intra prediction mode and colPuk is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following applies.

    - The following asignments are made with X being 0 or 1.

      - RefIdxColLX = RefIdxLX[ xPCol ][ yPCol ]

      - If PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than PicOrderCnt( currPic ), the variable MvXCross is derived as follows.

        - MvXCross = 1

      - Otherwise (PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than or equal to PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than or equal to PicOrderCnt( currPic )), the variable MvXCross is derived as follows.

        - MvXCross = 0

    - If one of the following conditions is true, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvL1[ xPCol ][ yPCol ], RefIdxColL1 and L1, respectively.

      - Mv0Cross is equal to 0 and Mv1Cross is equal to 1

      - Mv0Cross is equal to Mv1Cross and reference index list is equal to L1

    - Otherwise, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvL0[ xPCol ][ yPCol ], RefIdxColL0 and L0, respectively.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol	(8-53)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-54)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-55)

mvLXCol = ClipMv( ( DistScaleFactor * mvCol + 128 ) >> 8 )	(8-56)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(8-57)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-58)