Method 2:

##### Decision process for luma block edge

…

A variable IsLCUUpperBoundary is specified as follows:

–	If ((yC+yB) % (1<< Log2MaxCUSize)) is equal to 0, the variable IsLCUUpperBoundary is set equal to 1.

–	Otherwise the variable IsLCUUpperBoundary is set equal to 0.

…

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 2,5 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-1)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-2)

- The variable d is derived as follows:

If IsLCUUpperBoundary is equal to 1

d = | q2,2 – 2*q1,2 + q0,2 | +  | q2,5 – 2*q1,5 + q0,5 |		 (8-3)

and β is set to β/2

Othewise

d = | p2,2 – 2*p1,2 + p0,2 | + | q2,2 – 2*q1,2 + q0,2 | + | p2,5 – 2*p1,5 + p0,5 | + | q2,5 – 2*q1,5 + q0,5 |	(8-4)

- The variable dE is set equal to 0.

- If bS is not equal to 0 and d is less than β, the following ordered steps apply:

- For each sample location ( xC + xB + k, yC + yB ), k = 0..7, the following ordered steps apply:

  - The decision process for a luma sample specified in subclause 8.6.1.4.2 is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables IsLCUUpperBoundary, d, β and tC as inputs and a decision dSam as output.

  - The variable dS[k] is set equal to dSam.

- The variable dE is set equal to 1.

##### Filtering process for luma block edge

Inputs of this process are:

…

Output of this process is:

...

Let s’ represent the luma sample array recPictureL of the current picture.

Let s” represent the luma sample array recPictureL of the current picture before the deblocking filter process.

…

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- ....

- The sample values pi,k and qi,k with i = 0..3 and k = ks..ke are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-5)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-6)

- The sample values p”0,k and k = ks..ke are derived as follows:

p”0,k = s”[ xC + xB +k, yC + yB  – 1 ]		(8-7)

- If xPOS is less than 0 and dEL is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = -3..-1, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause 8.1.4.1.5 is invoked with sample values pi,k, qi,k with i = 0..3, p”0,k, decision dSL[k+3], the boundary filtering strength bSL and the variable tCL as inputs and the number of filtered samples nD and the filtered sample values pi’ and qi’ as outputs.

- The filtered sample values pi’ and qi’ with i = 0..nD – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + i ] = qi’		(8-8)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’		(8-9)

- …

##### Decision process for a luma sample

[Ed: (WJ) no filtering when bS is equal to 0]

Inputs of this process are:

–	sample values, pi and qi with i = 0..2,

–	a variable bS specifying the boundary filtering strength,

–	variables IsLCUUpperBoundary, d, β and tC.

A variable adiff is specified as follows:

–	If IsLCUUpperBoundary is equal to 1, the variable adiff is set equal to  | q0 – q3 |.

–	Otherwise the variable adiff is set equal to | p3 – p0 | + | q0 – q3 |.

Output of this process is:

–	a variable dSam containing a decision

When the variable bS is not equal to 0, the following applies:

–	If d is less than ( β >> 2 ), adiff is less than ( β >> 3 ) and | p0 – q0 | is less than ( 5*tC + 1 ) >> 1, dSam is set equal to 1.

–	Otherwise, dSam is set equal to 0.

##### Filtering process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	sample value, p”0,

–	a variable dSam containing a decision,

–	variables verticalEdgeFlag, IsLCUUpperBoundary, tC.

Output of this process is:

–	number of filtered samples nD,

–	filtered sample values, pi’ and qi’ with i = 0..nD – 1,

Depending on dSam, the following applies:

–	When the variable dSam is equal to 1, the following strong filtering applies while nD is set equal to 3:

If verticalEdgeFlag is equal to 0 and IsLCUUpperBoundary is equal to 1

q0’ = Clip1Y( ( p”0 + 2*p”0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-10)

q1’ = Clip1Y( ( p”0 + q0 + q1 + q2 + 2 ) >> 2 )		(8-11)

q2’ = Clip1Y( ( p”0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 ) 	(8-12)

Otherwise

p0’ = Clip1Y( ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(8-13)

p1’ = Clip1Y( ( p2 + p1 + p0 + q0 + 2 ) >> 2 )		(8-14)

p2’ = Clip1Y( ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )		(8-15)

q0’ = Clip1Y( ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-16)

q1’ = Clip1Y( ( p0 + q0 + q1 + q2 + 2 ) >> 2 )		(8-17)

q2’ = Clip1Y( ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )		(8-18)

–	Otherwise, the following weak filtering applies while nD is set equal to 2:

If verticalEdgeFlag is equal to 0 and IsLCUUpperBoundary is equal to 1

 = Clip3( -tC, tC, ( 13*( q0 – p”0 ) + 4*( q1 – p”0 ) - 5*( q2 – p”0 ) + 16 ) >> 5 )	(8-19)

q0’ = Clip1Y( q0 – 3*/2 )		(8-20)

q1’ = Clip1Y( q1 – /2 )		(8-21)

Otherwise

 = Clip3( -tC, tC, ( 13*( q0 – p0 ) + 4*( q1 – p1 ) - 5*( q2 – p2 ) + 16 ) >> 5 )	(8-22)

p0’ = Clip1Y( p0 +  )		(8-23)

q0’ = Clip1Y( q0 -  )		(8-24)

p1’ = Clip1Y( p1 + /2 )		(8-25)

q1’ = Clip1Y( q1 – /2 )		(8-26)

##### Filtering process for a chroma sample

[Ed: (WJ) no filtering when bS is equal or less than 2]

Inputs of this process are:

–	sample values, pi and qi with i = 0..1,

–	sample value, p”0,

–	a variable bS specifying the boundary filtering strength.

–	variables verticalEdgeFlag,IsLCUUpperBoundary, tC.

Output of this process is:

–	The filtered sample values, p0’ and q0’.

When the variable bS is greater than 2, the filtered sample values p0’ and q0’ are derived by

If verticalEdgeFlag is equal to 0 and IsLCUUpperBoundary is equal to 1

 = Clip3( -tC, tC, ( ( ( ( q0 – p”0 ) << 2 ) + p”0 – q1 + 4 ) >> 3 ) )	(8-27)

q0’ = Clip1C( q0 -  )		(8-28)

Otherwise

 = Clip3( -tC, tC, ( ( ( ( q0 – p0 ) << 2 ) + p1 – q1 + 4 ) >> 3 ) )	(8-29)

p0’ = Clip1C( p0 +  )		(8-30)

q0’ = Clip1C( q0 -  )		(8-31)