Method 1:

#### Filtering process for coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize – 3 ).

- All elements of the three-dimensional array of size (2)x(nD)x(nD), dEdge are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(1<<log2CUSize), dSample are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(nD), bStrength ae initialized to zero.

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to yC+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - Boundary filtering strength bSVer is derived as follows:

bSVer = Max( bS[ 0 ][ xDk ][ yDm + i ] ) for i = 0..7 	(8-1)

    - bStrength[1][k][m] is set equal to bSVer.

    - Decision process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, and the boundary filtering strength bSVer as inputs and the decision dEdge[1][k][m] and an array dS of size (8) as outputs.

    - dSample[1][k][(m<<3)+i] is set equal to dS[i] for i=0..7.

    - Boundary filtering strength bSHor is derived as follows:

bSHor = Max( bS[ 1 ][ xDk + i ][ yDm ] ) for i = 0..7 	(8-2)

    - bStrength[0][k][m] is set equal to bSHor.

    - If ((yC+ yDm) % (1<< Log2MaxCUSize)) is equal to 0, i.e., the luma block horizontal edge is not an edge of maximum coding unit, decision process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bSHor as inputs, the decision dEdge[0][k][m] and an array dS of size (8) as outputs.

    - If ((yC+ yDm) % (1<< Log2MaxCUSize)) is equal to 0, i.e., the luma block horizontal edge is not an edge of maximum coding unit, dSample[0][m][(k<<3)+i] is set equal to dS[i] for i=0..7.

- For xDk set equal to xC+( k << 3 ), k=0..nD - 1, the following applies:

- For yDm set equal to yB+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - dS[i] is set equal to dSample[1][k][(m<<3)+i] for i=0..7.

    - Filtering process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, the boundary filtering strength bStrength[1][k][m], the decision dEdge[1][k][m], and the array of size (8), dS as inputs and the modified luma picture buffer as outputs.

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to yC+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - If ((yC+ yDm) % (1<< Log2MaxCUSize)) is equal to 1, i.e., the luma block horizontal edge is an edge of maximum coding unit, decision process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bSHor as inputs, the decision dEdge[0][k][m] and an array dS of size (8) as outputs.

    - If ((yC+ yDm) % (1<< Log2MaxCUSize)) is equal to 1, i.e., the luma block horizontal edge is an edge of maximum coding unit, dSample[0][m][(k<<3)+i] is set equal to dS[i] for i=0..7.

- For yDm set equal to yC+( m << 3 ), m=0..nD - 1, the following applies:

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following ordered steps apply:

    - If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xB+( ( nD – 1) << 3 ) xPOS is set equal to 2. Otherwise xPOS is set to 0.

    - dS[i] is set equal to dSample[0][m][ (k << 3) + i ] for i = 0..7.

    - Filtering process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bStrength[0][k][m], the decision dEdge[0][k][m], and the array of size (8), dS, xPOS, dSL[m][], dEL[m], bSL[m], and tCL[m], as inputs and the modified luma picture buffer as output.

- The elements of the two dimensional array of size (3)x(nD), dSL are set as follows. dSL[m][0], dSL[m][1], and dSL[m][2], are set equal to dS[5], dS[6] and dS[7].

- The elements of the array of size (nD), dEL are set as follows. dEL[m] is set equal to dEdge[0][k][m].

- The elements of the array of size (nD), bSL are set as follows. bSL[m] is set equal to bStrength[0][k][m].

- The elements of the array of size (nD), tC are set as follows. tCL[m] is set equal to tc.

...