| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F057 |
| --- | --- |

| Title: | Sample Adaptive Offset for Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Ming Fu, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Seungwook Park and Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea Alexander Alshin and Elena Alshina #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chihming.fu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com Byeongmoon Jeon +82-10-3922-5896 {seungwook.park, bm.jeon}lge.com Elena Alshina +82 10 3026 1305 {alexander_b.alshin, elena_a.alshina}@samsung.com |
| Source: | MediaTek Inc., LG Electronics, and Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

Sample adaptive offset (SAO) for luma was adopted in HM-3.0. In this contribution, the same techniques of band offset (BO) and edge offset (EO) used for luma were applied for chroma. The three components were independent, and each component had its own partitions and offsets. It was reported that SAO reduced Cb bitrates by 2.0%, 3.3%, 4.6%, 1.8%, 2.7%, and 6.3%, and Cr bitrates by 2.9%, 3.9%, 5.9%, 2.4%, 2.8%, and 7.6% for HE-AI, HE-RA, HE-LD, LC-AI, LC-RA, and LC-LD, respectively, while encoding and decoding times were almost unchanged.

- Algorithm description

SAO in JCTVC-E049 [1] was adopted in HM-3.0. SAO can divide a picture into LCU-aligned regions. In each region, BO or EO can be selected, pixel classification is performed, and one offset is used for each group. SAO can bring descent gains with low complexity but was applied only for luma. Therefore, we applied the same techniques of SAO used for luma to chroma in a straightforward way. Each component is independent and has its own partitions and offsets.

# Syntax design

The proposed SAO syntax is shown in Table 1 – Table 4. Modifications are highlighted with yellow backgrounds.

Table 1.  SAO syntax in SPS.

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| sao_used_flag | 1 | u(1) |
| … |  |  |
| } |  |  |

sao_used_flag equal to 0 denotes SAO is disabled for the current sequence; equal to 1 denotes the opposite.

Table 2.  SAO syntax of parameters in PPS or slice header.

| sao_param( ) { | C | Descriptor |
| --- | --- | --- |
| sao_flag | 2 | u(1)¦ae(v) |
| if( sao_flag ) { |  |  |
| sao_split_param( 0, 0, 0, 0 ) |  |  |
| sao_offset_param( 0, 0, 0, 0 ) |  |  |
| sao_flag_cb | 2 | u(1)¦ae(v) |
| if( sao_flag_cb ) { |  |  |
| sao_split_param( 0, 0, 0, 1 ) |  |  |
| sao_offset_param( 0, 0, 0, 1 ) |  |  |
| } |  |  |
| sao_flag_cr | 2 | u(1)¦ae(v) |
| if( sao_flag_cr ) { |  |  |
| sao_split_param( 0, 0, 0, 2 ) |  |  |
| sao_offset_param( 0, 0, 0, 2 ) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

sao_flag equal to 0 denotes SAO for luma is disabled for the current picture; equal to 1 denotes the opposite.

sao_flag_cb equal to 0 denotes SAO for Cb is disabled for the current picture; equal to 1 denotes the opposite.

sao_flag_cr equal to 0 denotes SAO for Cr is disabled for the current picture; equal to 1 denotes the opposite.

Table 3.  SAO syntax of partition parameters.

| sao_split_param( rx, ry, Depth, component ) { | C | Descriptor |
| --- | --- | --- |
| if( Depth < MaxSplitLevel ) |  |  |
| split_flag[ component ] [ Depth ][ ry ][ rx ] = sao_split_flag | 2 | u(1)¦ae(v) |
| Else |  |  |
| split_flag[ component ] [ Depth ][ ry ][ rx ] = 0 |  |  |
| if( split_flag[ component ] [ Depth ][ ry ][ rx ] ) { |  |  |
| sao_split_param(2* rx + 0, 2*ry + 0, Depth + 1, component ) |  |  |
| sao_split_param(2*rx + 1, 2* ry + 0, Depth + 1, component ) |  |  |
| sao_split_param(2*rx + 0, 2*ry + 1, Depth + 1, component ) |  |  |
| sao_split_param(2*rx + 1, 2*ry + 1, Depth + 1, component ) |  |  |
| } |  |  |
| } |  |  |

sao_split_flag equal to 0 denotes that current region is a leaf; equal to 1 denotes that the current region is further spitted into four children regions.

Table 4.  SAO syntax of offset parameters.

| sao_offset_param ( rx, ry, Depth, component ) { | C | Descriptor |
| --- | --- | --- |
| if( split_flag[ component ][ Depth ][ y ][ x ] ) { |  |  |
| sao_offset_param (2*rx + 0, 2*ry + 0, Depth + 1, component ) |  |  |
| sao_offset_param (2*rx + 1, 2*ry + 0, Depth + 1, component ) |  |  |
| sao_offset_param (2*rx + 0, 2*ry + 1, Depth + 1, component ) |  |  |
| sao_offset_param (2*rx + 1, 2*ry + 1, Depth + 1, component ) |  |  |
| } else { |  |  |
| type_idx[ component ] [ Depth ][ ry ][ rx ] = sao_type_idx | 2 | ue(v)¦ae(v) |
| if( sao_type_idx != 0 ) { |  |  |
| for( i = 0; i < SaoOffsetNum[ sao_type_idx ]; i++ ) |  |  |
| offset[ component ] [ Depth ][ ry ][ rx ][ i ] = sao_offset | 2 | se(v)¦ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

sao_type_idx indicates the offset type to be applied for the current region.

sao_offset indicates the offset of a luma pixel group or the offset of a Cb/Cr pixel group.

# Simulation results

The common test conditions defined in JCTVC-E700 [2] were used in the experiment, and the software is based on HM-3.0. Table 6 shows the simulation results of applying SAO for both luma and chroma where the anchor applies SAO for luma only. It can be seen that SAO can reduce Cb bitrates by 2.0%, 3.3%, 4.6%, 1.8%, 2.7%, and 6.3%, and Cr bitrates by 2.9%, 3.9%, 5.9%, 2.4%, 2.8%, and 7.6% for HE-AI, HE-RA, HE-LD, LC-AI, LC-RA, and LC-LD, respectively, while encoding and decoding times are almost unchanged.

Table 6.  Results of applying SAO for both luma and chroma.

# Conclusions

MediaTek, LGE, and Samsung applied the same sample adaptive offset (SAO) techniques used for luma on the two chroma components. Each component was independent and had its own partitions and offsets. Simulation results reportedly showed that SAO reduced Cb bitrates by 1.8-6.3% and Cr bitrates by 2.4-7.6% under the JCTVC-E700 common test conditions. Almost no change was observed for encoding times and decoding times.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-M. Fu, C.-Y. Chen, C-Y Tsai, Y.-W. Huang, and S. Lei, “CE13: Sample Adaptive Offset with LCU-Independent Decoding,” Document of Joint Collaborative Team on Video Coding, JCTVC-E049, Mar. 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-E700, Mar. 2011.