| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F058 |
| --- | --- |

| Title: | Sample Adaptive Offset with PPS-level Syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Ming Fu, Chia-Yang Tsai, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chihming.fu, chiayang.tsai, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-3.0, the sample adaptive offset (SAO) parameters are adapted at picture level. However, when a picture contains multiple slices, the entire SAO information is always sent in the first slice header, which is undesirable for parallel processing of multiple slices. In JCTVC-E045, a solution to a similar problem for adaptive loop filter (ALF) was adopted into HM-3.2. Therefore, by analogy with JCTVC-E045, this contribution proposed to add an option of moving SAO parameters to the picture parameter set (PPS). The option could be enabled for multiple slices per picture or disabled for single slice per picture by using a flag in PPS, and the flag could be shared by SAO and ALF. Compared with the SAO syntax coded in the first slice header, the proposed PPS-level SAO syntax reportedly showed no difference in BD-rate and run time under JCTVC-E700 common test conditions with 1500-byte slices per picture.

- Algorithm description

In HM-3.0, one picture can be partitioned into multiple slices with SAO parameters that are adapted at picture level. Nevertheless, the entire SAO information is always sent in the first slice header even when a picture contains multiple slices. This syntax design may cause huge decoding delay and problems for parallel slice decoding if out-of-order slices are received on the decoder side. For example, if the first slice header is received at last, SAO processes for all other slices have to wait. In the 5th JCT-VC meeting, JCTVC-E045 [1] proposed a solution to a similar problem for ALF. Therefore, a similar solution can be used for SAO as follows. When one slice per picture is used, a send_in_loop_filter_info_in_pps flag in PPS is set to zero, and the SAO parameters of a picture are transmitted in the corresponding slice header. When multiple slices per picture are used, the send_in_loop_filter_info_in_pps flag in PPS is set to one, and one PPS is transmitted for each picture. Besides, the SAO parameters of one picture are transmitted in the corresponding PPS. It is obvious that the send_in_loop_filter_info_in_pps flag in PPS can be shared by SAO and ALF.

# Simulation results

HM-3.2 was used for experiments. The anchor followed the JCTVC-E700 common test conditions [2] with one slice per picture. The proposed PPS-level SAO syntax was implemented on top of HM-3.2. Table 1 shows two to-be-tested cases with the same slice parameter settings to obtain multiple slices per picture. However, the case 1 transmits SAO parameters in the first slice header of each picture while the case 2 transmits SAO parameters in the PPS of each picture. Table 2 and Table 3 show results of case 1 and case 2, respectively. It is reported that the differences between case 1 and case 2 are almost zero, which means the PPS-level SAO syntax can solve the problem without coding efficiency decrease or run time increase.

Table 1.  Conditions of conducted experiments.

| Case | Condition |
| --- | --- |
| Anchor | One slice per picture, as defined in JCTVC-E700 |
| 1 | 1500-byte LCU-aligned slices, SAO parameters in the first slice header of each picture |
| 2 | 1500-byte LCU-aligned slices, SAO parameters in the PPS of each picture |

Table 2.  Results of SAO using slice-level syntax.

Table 3.  Results of SAO using PPS-level syntax.

# Conclusions

This contribution proposed to add an option of moving SAO parameters to the PPS. Compared with the current slice-level SAO syntax, the proposed PPS-level SAO syntax reportedly achieved unchanged coding efficiency and processing time. Moreover, the proposed method could solve the problem of parallel slice decoding encountered by the HM-3.0 when multi-slice pictures are used.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, and S. Lei, “CE4 Subset3: Slice Common Information Sharing,” Document of Joint Collaborative Team on Video Coding, JCTVC-E045, Mar. 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-E700, Mar. 2011.