| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F059 |
| --- | --- |

| Title: | CABAC with Constrained Outstanding Bits |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-3.0, there is no constraint on continuous outstanding bits in CABAC. Theoretically, the number of continuous outstanding bits could be infinite. Practically, the number was up to 189 in our analysis. When a lot of continuous outstanding bits are accumulated, a sudden increase of output bits will occur, which complicates hardware design significantly because of the unpredictable largest number of continuous outstanding bits. In this contribution, a coding interval adjustment procedure was proposed to constrain the number of continuous outstanding bits. It was reported that the proposed method achieved no differences in coding efficiency and run time in comparison with the JCTVC-E700 anchor while at most 18 bits could be outputted simultaneously.

# Introduction

Binary arithmetic coding is a recursive interval-subdividing procedure. An output bitstream is a pointer to a probability of a final coding interval. The probability of a coding interval is specified by range and low. Low is the lower bound of the coding interval, and range is the possible scope of the coding interval. Depending on that the coding symbol is a most probable symbol (MPS) or a least probable symbol (LPS), the next coding interval is updated as one of the two sub-intervals. In HM-3.0, in order to maintain the precision of arithmetic coding, a renormalization process after interval subdivision will be performed as long as the range of a new interval is smaller than 256. The flowchart of recursive renormalization is shown in Figure 1. When one loop of the recursive renormalization process is performed, it is intended to output one bit, but sometimes the bit value cannot be determined in time. Figure 2 shows more details of outputting one bit in the renormalization process. In the renormalization process, if low is smaller than 256, which means the coding interval specified by range and low is fully placed in the lower half, one bit of zero can be outputted. If low is larger than 512, which means the coding interval is fully placed in the upper half, one bit of one can be outputted. However, if low is in the range of [256,512), which means the interval is placed in the middle and across 512, the output bit cannot be determined as zero or one because a carry may be propagated into coded bitstream, and then bits-outstanding (BO) is introduced to handle this case. BO means the bit count of the outstanding bits. In CABAC, coded bits are not outputted until further carry propagation will not change these bits. When an output bit is uncertain, BO will increase by one. After the interval is fully placed in the lower half or the upper half, one bit equal to zero with a series of BO bits equal to one or one bit equal to one with a series of BO bits equal to zero can be outputted.

Figure 1.  Flowchart of the renormalization process in CABAC.

Figure 2.  Illustration of outputting one bit in the renormalization process.

In HM-3.0, there is no constraint on the BO value, which means if the coding interval is always across 512, BO can be infinite. Unlimited BO will increase the difficulty of hardware implementation because there may be a sudden burst of many outputted bits to the bitstream buffer. In this contribution, a coding interval adjustment procedure is proposed to constrain the number of BO value, which means to constrain the bit count of the outstanding bits. The rest of this contribution is organized as follows. The proposed algorithm will be described in Section 2, simulation results will be discussed in Section 3, and conclusions will be given in Section 4.

# Algorithm description

The BO increases when the coding interval crosses 512. In this contribution, we propose a coding interval adjustment procedure to avoid the coding interval crossing 512. First, a maximum constraint on the BO value is defined and called maxBO. When the renormalization operation is done and the BO reaches or exceeds maxBO, a coding interval adjustment procedure is invoked.

Figure 3 shows the concept of proposed coding interval adjustment procedure. If BO is equal to or larger than maxBO after renormalization and the coding interval is not fully placed in the upper or lower half, the coding interval is divided into two parts. The interval of the lower part is [low, 512), and the interval of the upper part is [512, low+range). The binary arithmetic coder selects the larger part of these two intervals as the new coding interval. If the upper part is selected, one bit equal to one with BO bits equal to zero can be outputted; otherwise, one bit equal to zero with BO bits equal to one can be outputted. Selecting either of them can reset BO to zero.

Figure 3.  Concept of proposed coding interval adjustment procedure.

# Simulation results

The proposed approach was implemented on HM-3.0. High efficiency all intra (HE-AI), high efficiency random access (HE-RA), and high efficiency low delay (HE-LD) configurations with different maxBO settings were simulated according to the common test conditions in JCTVC-E700 [1]. Table 1, Table 2, and Table 3 report the experimental results of the proposed method with different maxBO constraints in HE-AI, HE-RA, and HE-LD, respectively.

Table 2.  Results of the proposed method with different maxBO constraints for HE-AI.

Table 3.  Results of the proposed method with different maxBO constraints for HE-RA.

Table 4.  Results of the proposed method with different maxBO constraints for HE-LD.

The results reportedly showed that there was almost no bit rate increase when maxBO was set to 12 or higher in all cases. Therefore, we suggest to set maxBO as 12, which means at most 18 bits can be outputted together by the renormalization process.

# Conclusions

After the renormalization process, CABAC could suddenly output an infinite number of bits in theory and hundreds of bits in practical due to continuous outstanding bits. MediaTek proposed a coding interval adjustment procedure to constrain the bit count of continuous outstanding bits. Simulations for the proposed method with different maximum bit count constrains on the outstanding bits were conducted. The results reported showed no change in coding efficiency and run time while at most 18 bits could be outputted at the same time.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-E700, Mar. 2011.