| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F060 |
| --- | --- |

| Title: | Reducing Line Buffers for Motion Data and CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Line buffers of 10.7K bytes are required to store motion and CABAC context data of upper largest coding units (LCUs) for real-time decoding of 4Kx2K videos in hardware when HM-3.0 is considered. In this contribution, for reducing motion line buffer size, a motion data compression method was proposed. For reducing CABAC line buffer size, CABAC context formation processes of several syntax elements were modified to avoid using upper block data and to only use left block data when the upper block belongs to the upper LCU. Simulation results reportedly showed that the motion data compression and the modified CABAC could achieve 56% and 74% reductions of motion and CABAC line buffers with 0.0% and 0.1% bit rate increases when the motion data compression ratio is 2:1 and 4:1, respectively. No encoding or decoding time increase was observed.

# Introduction

In HM-3.0, since LCUs are processed in a raster scan order, the coded data of bottom 4x4 blocks of an LCU row are required to be stored in line buffers for the coding of a next LCU row. The data includes motion data for inter mode coding and syntax elements for entropy coding. Table 1 shows the data to be stored in line buffers for inter mode coding and CABAC context formation. Some data are only required by CABAC context formation, and some are required by both inter mode coding and CABAC context formation. The size of the line buffers is proportional to the picture width and can be calculated as 1336 bits times the number of LCUs of one row. For 4Kx2K videos, 10.7K bytes memory is needed wherein 2.2K bytes are used for CABAC context formation, 7.7K bytes are used for motion data, and 0.8K bytes are used for the rest data. In this contribution, we propose two methods to reduce the line buffer memory size. One is a motion data compression method for reducing motion data size. The other is to modify the context formation to only refer the data from the left block when the upper block belongs to the upper LCU. The rest of this contribution is organized as follows. The algorithm will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

Table 1.  Data to be stored in line buffers for inter mode coding and CABAC context formation.

| Data | Usage | CABAC Only | Required Bits in a LCU |
| --- | --- | --- | --- |
| inter prediction direction | MVP candidates, merge candidates CABAC interDir coding CABAC total MVP/merge candidates | No | 32 |
| reference picture index | MVP candidates, merge candidates CABAC refIdx coding CABAC total MVP/merge candidates | No | 96 |
| MV | MVP candidates, merge candidates CABAC total MVP/merge candidates | No | 896 |
| depth | CABAC split flag coding | Yes | 16 |
| skip flag | CABAC skip flag coding | Yes | 8 |
| merge flag | CABAC merge flag coding | Yes | 16 |
| luma intra mode | CABAC luma intra mode coding | No | 96 |
| chroma intra mode | CABAC chroma intra mode coding | Yes | 48 |
| ¦MVD¦>16 ? | CABAC MVD coding | Yes | 32 |
| CBF | CABAC cbf, no residual data flag coding | Yes | 96 |
| trafoDepth | CABAC cbf coding | Yes | 64 |
| Total bits in a 64x64 LCU |  |  | 1336 |

# Algorithm description

## Motion data compression

The motion data includes the inter prediction direction, reference picture index, and motion vector (MV). In order to reduce motion data line buffer size, it is not necessary to store the motion data of every block. For each PU that needs to access the motion data of the upper LCU row, only the compressed motion data in the line buffer can be accessed. Figure 1 shows a proposed 2:1 motion data compression of one line. For every four 4x4 blocks, the motion data of the first and the last 4x4 blocks are stored in the line buffer. The motion data of the first block represents the motion data of the first two blocks. The motion data of the last block represents the motion data of the last two blocks. For example, the motion data of block 0 represents the motion data of block 0 and 1; the motion data of block 3 represents the motion data of block 2 and 3. In this way, only half memory is required. The memory size of motion data line buffer for 4Kx2K videos is reduced from 7.7K bytes to 3.8K bytes.

Figure 1.  The proposed 2:1 motion data compression of one line.

Figure 2 shows a proposed 4:1 motion data compression of one line. For every eight 4x4 blocks, the motion data of the first and the last 4x4 blocks are stored in the line buffer. The motion data of the first block represents the motion data of the first four blocks, and the motion data of the last block represents the motion data of the last four blocks. For example, the motion data of block 0 represents the motion data of blocks 0-3; the motion data of block 7 represents the motion data of blocks 4-7. In this way, only quarter memory is required. The memory size of motion data line buffer for 4Kx2K videos is reduced from 7.7K bytes to 1.9K bytes.

Figure 2.  The proposed 4:1 motion data compression of one line.

## Modified CABAC context formation

In order to reduce the CABAC line buffer size, it is proposed to change the context formation. When the upper block is not in the current LCU and belongs to the upper LCU, the data of the left block is used to replace the data of the upper block for context formation. For example, when the skip flag is being coded, the skip flag of the left and upper blocks are required in the original context formation process. If the upper block does not belong to the current LCU, the skip flag of upper block will be replaced by the skip flag of the left block in the modified context formation process. In this way, no line buffer is needed to store skip flags of upper LCUs. In this contribution, we apply this method on skip flag, merge flag, chroma intra mode, motion vector difference (MVD), coded block flag (CBF), and no residual data flag coding, as shown in Table 2. The line buffer size for CABAC context formation is reduced from 2.2K bytes to 0.128K bytes, which is a 94% reduction.

Table 2.  Syntax elements with modified CABAC context formation.

| Data | Usage | Required Bits in a 64x64 LCU |
| --- | --- | --- |
| skip flag | CABAC skip flag coding | 8 |
| merge flag | CABAC merge flag coding | 16 |
| chroma intra mode | CABAC chroma intra mode coding | 48 |
| ¦MVD¦>16 ? | CABAC MVD coding | 32 |
| CBF | CABAC cbf, no residual data flag coding | 96 |
| trafoDepth | CABAC cbf coding | 64 |
| Total bits in a 64x64 LCU |  | 264 |

# Simulation results

The proposed approach was implemented on HM-3.0. All the results were simulated according to the common test conditions defined in JCTVC-E700 [1]. The tested conditions are high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD). In the proposed implementation, the bug-fix of ticket no.146 is included.

Table 3 and Table 4 show the simulation results of the proposed 2:1 and 4:1 motion data compression methods, respectively. Almost no bit rate increase is observed in the 2:1 motion data compression, and only 0.1% bit rate increase is caused by the 4:1 motion data compression. For the 2:1 motion data compression, memory size is reduced from 7.7K bytes to 3.8K bytes. For the 4:1 motion data compression, memory size is reduced from 7.7K bytes to 1.9K bytes. All encoding and decoding times are roughly unchanged.

Table 3.  Results of the 2:1 motion data compression.

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | 0.1 | 0.0 | 0.0 | 0.3 |
| Class B | 0.0 | -0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.0 | -0.2 | 0.0 | 0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | -0.2 | 0.0 | 0.1 | -0.2 |
| Class D | 0.0 | 0.0 | 0.3 | 0.0 | -0.3 | -0.3 |
| Class E | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 | -0.1 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 4.  Results of the 4:1 motion data compression.

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.2 | -0.1 | 0.1 | 0.2 | 0.3 |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.1 |
| Class C | 0.1 | 0.2 | 0.2 | 0.1 | 0.1 | 0.2 |
| Class D | 0.1 | 0.1 | 0.2 | 0.1 | 0.3 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 | 0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | 0.1 | 0.0 | 0.0 | -0.2 |
| Class C | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class D | 0.1 | 0.3 | 0.3 | 0.1 | -0.1 | -0.1 |
| Class E | 0.1 | -0.3 | -0.3 | 0.1 | 0.3 | 0.1 |
| All | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | -0.1 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 5 shows the simulation results of the proposed CABAC context formation. Almost no bit rate increase is observed. All encoding and decoding times are not changed.

Table 6 and Table 7 show the simulation results of the 2:1 and 4:1 motion data compression with the modified CABAC context formation, respectively. For the 2:1 motion data compression with the modified CABAC context formation, almost no bit rate increase is found in the HE-RA condition. In the HE-LD condition, there is no bit rate increase for luma, but 0.4% and 0.1% bit rate increases are observed for the two chroma components respectively. All encoding and decoding times are roughly unchanged. The line buffer size is from 10.7K bytes to 4.7K bytes, which is to a 56% decrease. For the 4:1 motion data compression with the modified CABAC context formation, the BD-rates are increased by 0.1% for the three components in the HE-RA condition and increased by 0.1%, 0.4%, and 0.2% for the three components respectively. The line buffer size is reduced from 10.7K bytes to 2.8K bytes, which is a 74% reduction.

Table 5.  Results of the modified CABAC context formation.

|  | HE-AI |  |  | HE-RA |  |  | HE-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.2 | -0.1 | -0.1 | 0.1 | 0.0 |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class E | 0.0 | -0.2 | 0.0 |  |  |  | 0.0 | 0.6 | 0.0 |
| All | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |

Table 6.  Results of the 2:1 motion data compression and the modified CABAC context formation.

|  | HE-RA |  |  | HE-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.1 | 0.0 |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | 0.1 | 0.4 | 0.1 |
| Class C | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | -0.2 |
| Class D | 0.0 | -0.2 | 0.0 | 0.0 | 0.2 | 0.4 |
| Class E |  |  |  | 0.1 | 0.9 | 0.2 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.4 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 7.  Results of the 4:1 motion data compression and the modified CABAC context formation.

|  | HE-RA |  |  | HE-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.0 |  |  |  |
| Class B | 0.1 | 0.2 | 0.1 | 0.1 | 0.2 | 0.1 |
| Class C | 0.2 | 0.2 | 0.2 | 0.1 | 0.3 | 0.1 |
| Class D | 0.2 | -0.1 | 0.1 | 0.1 | 0.0 | 0.8 |
| Class E |  |  |  | 0.2 | 1.2 | 0.0 |
| All | 0.1 | 0.1 | 0.1 | 0.1 | 0.4 | 0.2 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Conclusions

A motion data compression method and a modified CABAC context formation were proposed to reduce the motion data line buffer size and the CABAC line buffer size, respectively. Simulation results reportedly showed that the 2:1 motion data compression could reduce 50% motion data line buffer size without coding efficiency loss, and the 4:1 motion data compression could reduce 75% motion data line buffer size with 0.1% bit rate increase. As for the modified CABAC context formation, 94% CABAC line buffer size could be reduced with almost no bit rate increase. When the modified CABAC context formation was combined with the 2:1 motion data compression, 56% line buffer memory could be saved with no coding efficiency loss. When the modified CABAC context formation was combined with the 4:1 motion data compression, 74% line buffer memory could be saved with 0.1% bit rate increase. All encoding and decoding time were roughly unchanged.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-E700, Mar. 2011.