# Draft Changes

## Change for Motion data compression

##### Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- the luma location ( xLC, yLC ) specifying the top-left luma sample of the current largest coding unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A, B, C, or D and with X being replaced by 0 or 1)

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

Figure 8-1 – Spatial neighbours that can be used as merging candidates (informative) illustrates the position of the spatial neighbours A, B, C and D relative to the current prediction unit.

For the derivation of availableFlagN, with N being A, B, C or D and ( xN, yN ) being ( xP – 1,  yP ), ( xP,  yP – 1 ), ( xP + nPSW,  yP – 1 ) or ( xP – 1,  yP+nPSH ) the following applies.

–	If yP – 1 is smaller than yLC, the value of ((xB>>4)<<4)+(( xB >>3)&1)*(MinPuSize*4-1) is assigned to xB and the value of ((xC>>4)<<4)+(( xC >>3)&1) *(MinPuSize*4-1) is assigned to xC.

–	If one of the following conditions is true, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

  - The prediction unit covering luma location ( xN, yN ) is not available or PredMode is MODE_INTRA.

- PartMode of the current prediction unit is PART_2NxN and PartIdx is equal to 1 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

    - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_Nx2N and PartIdx is equal to 1 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 )  have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 ) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

##### Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- the luma location ( xLC, yLC ) specifying the top-left luma sample of the current largest coding unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A,or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-2 – Spatial motion vector neighbours

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0 .. 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- When the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] and ListA is set equal to LY.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, the following applies.

- If RefPicOrderCnt( currPic , refIdxA, ListA ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), mvLXA is set equal to mvLXA

- Otherwise, mvLXA is derived as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-1)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-2)

mvLXA = ClipMv( ( DistScaleFactor * mvLXA + 128 ) >> 8 )	(8-3)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxA, ListA ) )	(8-4)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-5)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 - MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- If yP – 1 is smaller than yLC, the value of ((xB0>>4)<<4)+(( xB0>>3)&1)*(MinPuSize*4-1) is assigned to xB0, the value of ((xB1  >>4)<<4)+(( xB1  >>3)&1)*(MinPuSize*4-1) is assigned to xB1 , and the value of ((xB2  >>4)<<4)+(( xB2  >>3)&1) *(MinPuSize*4-1) is assigned to xB2 .

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- When the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX and mvLX[ xBk ][ yBk ] is not identical to mvLXA, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- When availableFlagLXB is equal to 0, for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ](with Y = !X) is equal to 1 and RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ] and ListB is set equal to LY.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- If availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), mvLXB is set equal to mvLXB.

- Otherwise if availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is not equal to RefPicOrderCnt( currPic, refIdxLX, LX ),, mvLXB is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-6)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-7)

mvLXB = ClipMv( ( DistScaleFactor * mvLXB + 128 ) >> 8 )	(8-8)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxB, ListB ) )	(8-9)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-10)

- When availableFlagLXB is equal to 1 and mvLXB is identical to mvLXA, availableFlagLXB is set equal to 0.

#### Derivation process of boundary filtering strength

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xLC, yLC ) specifying the top-left luma sample of the current largest coding unit relative to the top-left sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process is an array of size (2)x(nS)x(nS), bS specifying the boundary filtering strength.

Let ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CUSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 4 and yEj+1 = yEj + 4.

For ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1, the following applies.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk ][ yC + yEj – 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 1.

- Otherwise, if verEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk – 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 0.

- Depending on the value of filterDir, the variable bS[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the block edge is also a coding unit edge and the following condition is true, the variable bS[ 0 ][ xEk ][ yEj ] is set equal to 4.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 3.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

NOTE – The determination of whether the reference pictures used for the two prediction are the same or different is based on which pictures are referenced, without regard to whether a prediction is formed using an index into list 0 or an index into list 1, and also without regard to whether or not the index position within a reference picture list is different or not.

- One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1 and yC + yEj – 1 is smaller than yLC, the following applies.

    - One motion vector is used to predict the prediction unit containing sample recPicture[(((xC+xEk)>>4)<<4)+(((xC+xEk)>>3)&1)*(MinPuSize*4-1)][ yC+yEj –1], one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples.

- Otherwise, the following applies.

    - One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples.

- [Ed.: (WJ) needs to be checked again whether this condition covers all 2-motion cases] Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition:

- The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to 4 in units of quarter luma samples.

- Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.

#### Derivation process for ctxIdx

Inputs to this process are binIdx, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset.

Output of this process is ctxIdx.

Table 9-2 shows the assignment of ctxIdx increments (ctxIdxInc) to binIdx for all ctxIdxTable and ctxIdxOffset.

The ctxIdx to be used with a specific binIdx is specified by first determining the ctxIdxTable and ctxIdxOffset associated with the given bin string or part thereof. The ctxIdxOffset is listed in Table 9-2, the ctxIdx for a binIdx is the sum of ctxIdxOffset and ctxIdxInc, which is found in Table 9-2. When more than one value is listed in Table 9-2 for a binIdx, the assignment process for ctxIdxInc for that binIdx is further specified in the subclauses given in parenthesis of the corresponding table entry.

All bins with binIdx greater than maxBinIdxCtx are parsed using the value of ctxIdx being assigned to binIdx equal to maxBinIdxCtx.

All entries in Table 9-2 labelled with "na" correspond to values of binIdx that do not occur for the corresponding ctxIdxOffset.

| Table 9-1 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| alf_cu_flag | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 6 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
| split_coding_unit_flag | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 6 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
| skip_flag | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
| cu_qp_delta | Error: Reference source not found | 0 | 0 | 2 | 3 | 3 | 3 |
|  |  | 4 | 0 | 2 | 3 | 3 | 3 |
|  |  | 8 | 0 | 2 | 3 | 3 | 3 |
| pred_type | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | 1 | 2 | 3 | na |
|  |  | 5 | 0 | 1 | 2 | 3 | 4 |
| prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| rem_intra_luma_pred_mode | Error: Reference source not found | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | 1 | 0 | 0 | 0 | 0 | 0 |
|  |  | 2 | 0 | 0 | 0 | 0 | 0 |
| intra_chroma_pred_mode | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) | 3 | 3 | 3 | na |
|  |  | 4 | 0,1,2 (subclause 9.3.3.1.2.1) | 3 | 3 | 3 | na |
|  |  | 8 | 0,1,2 (subclause 9.3.3.1.2.1) | 3 | 3 | 3 | na |
| merge_flag | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
| merge_idx | Error: Reference source not found | 0 | 0,1,2,3 (subclause Error: Reference source not found) | 0,1,2,3 (subclause Error: Reference source not found) | 0,2,3 (subclause Error: Reference source not found) | 3 | na |
|  |  | 4 | 0,1,2,3 (subclause Error: Reference source not found) | 0,1,2,3 (subclause Error: Reference source not found) | 0,2,3 (subclause Error: Reference source not found) | 3 | na |
| inter_pred_flag | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
| ref_idx_l0 | Error: Reference source not found | 0 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 4 | 4 | 4 | 4 |
| ref_idx_l0, ref_idx_l1, ref_idx_lc | Error: Reference source not found | 6 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 4 | 4 | 4 | 4 |
| mvd_l0[ ][ ][ 0 ] | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 3 | 4 | 5 | 6 |
| mvd_l0[ ][ ][ 1 ] | Error: Reference source not found | 7 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 3 | 4 | 5 | 6 |
| mvd_l0[ ][ ][ 0 ], mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | Error: Reference source not found | 15 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 3 | 4 | 5 | 6 |
| mvd_l0[ ][ ][ 1 ], mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] | Error: Reference source not found | 21 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 3 | 4 | 5 | 6 |
| mvp_idx_l0 | Error: Reference source not found | 0 | 0 | 1 | 1 | 1 | 1 |
| mvp_idx_l0, mvp_idx_l1, mvp_idx_lc | Error: Reference source not found | 2 | 0 | 1 | 1 | 1 | 1 |
| no_residual_data_flag | Error: Reference source not found | 0 | 0,1,2,3 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause 9.3.3.1.2.1) | na | na | na | na |
| split_transform_flag | Error: Reference source not found | 0 | cuDepth + trafoDepth | na | na | na | na |
|  |  | 4 | cuDepth + trafoDepth | na | na | na | na |
|  |  | 8 | cuDepth + trafoDepth | na | na | na | na |
| cbf_luma | Error: Reference source not found | 0 | 0,1,2,3 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 8 | 0,1,2,3 (subclause 9.3.3.1.2.1) | na | na | na | na |
| cbf_cb ( PredMode = = MODE_INTRA ) | Error: Reference source not found | 0 | 0,1,2,3 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 8 | 0,1,2,3 (subclause 9.3.3.1.2.1) | na | na | na | na |
| cbf_cb ( PredMode != MODE_INTRA ) | Error: Reference source not found | 0 | trafoDepth | na | na | na | na |
|  |  | 4 | trafoDepth | na | na | na | na |
|  |  | 8 | trafoDepth | na | na | na | na |
| cbf_cr ( PredMode = = MODE_INTRA ) | Error: Reference source not found | 0 | 0,1,2,3 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 8 | 0,1,2,3 (subclause 9.3.3.1.2.1) | na | na | na | na |
| cbf_cr ( PredMode != MODE_INTRA ) | Error: Reference source not found | 0 | trafoDepth | na | na | na | na |
|  |  | 4 | trafoDepth | na | na | na | na |
|  |  | 8 | trafoDepth | na | na | na | na |
| last_significant_coeff_x | Error: Reference source not found | 0 | 0..40 (subclause Error: Reference source not found) |  |  |  |  |
|  |  | 41 | 0..40 (subclause Error: Reference source not found) |  |  |  |  |
|  |  | 82 | 0..40 (subclause Error: Reference source not found) |  |  |  |  |
| last_significant_coeff_y | Error: Reference source not found | 0 | 0..40 (subclause Error: Reference source not found) |  |  |  |  |
|  |  | 41 | 0..40 (subclause Error: Reference source not found) |  |  |  |  |
|  |  | 82 | 0..40 (subclause Error: Reference source not found) |  |  |  |  |
| significant_coeff_flag | Error: Reference source not found | 0 | 0..111 (subclause Error: Reference source not found) | na | na | na | na |
|  | Error: Reference source not found | 0 | 0..111 (subclause Error: Reference source not found) | na | na | na | na |
|  | Error: Reference source not found | 0 | 0..111 (subclause Error: Reference source not found) | na | na | na | na |
| coeff_abs_level_greater1_flag | Error: Reference source not found | 0 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 60 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 120 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
| coeff_abs_level_greater2_flag | Error: Reference source not found | 0 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 60 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 120 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |

##### Assignment process of ctxIdxInc using neighbouring syntax elements

Subclause 9.3.3.1.2.1 specifies the derivation process of ctxIdxInc for the syntax elements alf_cu_flag, split_coding_unit_flag, skip_flag, merge_flag, intra_chroma_pred_mode, inter_pred_flag, ref_idx_lc, ref_idx_l0, ref_idx_l1, mvd_l0, mvd_l1, mvd_lc, no_residual_data_flag, cbf_luma, cbf_cb and cbf_cr.

Subclause 9.3.3.1.2.17 specifies the derivation process of ctxIdxInc for the syntax elements inter_pred_flag, ref_idx_lc, ref_idx_l0, ref_idx_l1, mvd_l0, mvd_l1, and mvd_lc.

###### Derivation process of ctxIdxInc using left and above syntax elements

Input to this process is the luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture.

Output of this process is ctxIdxInc.

Let the luma location ( xL, yL ) specify a location covered by the prediction unit to the left of the top-left luma sample of the current prediction unit with xL = xP − MinPuSize and yL = yP, the variable availableL is derived as follows. [Ed.: (BB) MinPuSize should be defined somewhere]

- If the prediction unit covering location ( xL, yL ) is available, availableL is set equal to 1.

- Otherwise (the prediction unit covering location ( xL, yL ) is not available), availableL is set equal to 0.

Let the luma location ( xA, yA ) specify a location covered by the prediction unit above the top-left luma sample of the current prediction unit with xA = xP and yA = yP − MinPuSize, the variable availableA is derived as follows. [Ed.: (BB) MinPuSize should be defined somewhere]

- If the prediction unit covering location ( xA, yA ) is available, availableA is set equal to 1.

- Otherwise (the prediction unit covering location ( xA, yA ) is not available), availableA is set equal to 0

The assignment of ctxIdxInc for the syntax elements alf_cu_flag, split_coding_unit_flag, skip_flag, merge_flag, intra_chroma_pred_mode, inter_pred_flag, ref_idx_lc, ref_idx_l0, ref_idx_l1, mvd_l0, mvd_l1, mvd_lc, no_residual_data_flag, cbf_luma, cbf_cb and cbf_cr is specified in Error: Reference source not found50.

##### Derivation process of ctxIdxInc using left and above syntax elements within current LCU row

Inputs to this process are

- the luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- the luma location ( xLC, yLC ) specifying the top-left luma sample of the current largest coding unit relative to the top-left sample of the current picture.

Output of this process is ctxIdxInc.

Let the luma location ( xL, yL ) specify a location covered by the prediction unit to the left of the top-left luma sample of the current prediction unit with xL = xP − MinPuSize and yL = yP, the variable availableL is derived as follows. [Ed.: (BB) MinPuSize should be defined somewhere]

- If the prediction unit covering location ( xL, yL ) is available, availableL is set equal to 1.

- Otherwise (the prediction unit covering location ( xL, yL ) is not available), availableL is set equal to 0.

Let the luma location ( xA, yA ) specify a location covered by the prediction unit above the top-left luma sample of the current prediction unit with xA = xP and yA = yP – 1. The variable availableA is derived as follows. [Ed.: (BB) MinPuSize should be defined somewhere]

- If yA is smaller than yLC, the variable availableA is derived as follows.

  - If the prediction unit covering location ( ((xA  >>4)<<4)+(( xA  >>3)&1)*(MinPuSize*4-1), yA) is available, availableA is set equal to 1.

  - Otherwise (the prediction unit covering location ( ((xA  >>4)<<4)+(( xA  >>3)&1)*(MinPuSize*4-1), yA)is not available), availableA is set equal to 0

- Otherwise, the variable availableA is derived as follows.

  - If the prediction unit covering location ( xA, yA ) is available, availableA is set equal to 1.

  - Otherwise (the prediction unit covering location ( xA, yA ) is not available), availableA is set equal to 0

The assignment of ctxIdxInc for the syntax elements inter_pred_flag, ref_idx_lc, ref_idx_l0, ref_idx_l1, mvd_l0, mvd_l1, and mvd_lc is specified in Error: Reference source not found50.

## Modified CABAC context formation

#### Derivation process for ctxIdx

Inputs to this process are binIdx, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset.

Output of this process is ctxIdx.

Table 9-2 shows the assignment of ctxIdx increments (ctxIdxInc) to binIdx for all ctxIdxTable and ctxIdxOffset.

The ctxIdx to be used with a specific binIdx is specified by first determining the ctxIdxTable and ctxIdxOffset associated with the given bin string or part thereof. The ctxIdxOffset is listed in Table 9-2, the ctxIdx for a binIdx is the sum of ctxIdxOffset and ctxIdxInc, which is found in Table 9-2. When more than one value is listed in Table 9-2 for a binIdx, the assignment process for ctxIdxInc for that binIdx is further specified in the subclauses given in parenthesis of the corresponding table entry.

All bins with binIdx greater than maxBinIdxCtx are parsed using the value of ctxIdx being assigned to binIdx equal to maxBinIdxCtx.

All entries in Table 9-2 labelled with "na" correspond to values of binIdx that do not occur for the corresponding ctxIdxOffset.

| Table 9-2 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| alf_cu_flag | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 6 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
| split_coding_unit_flag | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
|  |  | 6 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
| skip_flag | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
| cu_qp_delta | Error: Reference source not found | 0 | 0 | 2 | 3 | 3 | 3 |
|  |  | 4 | 0 | 2 | 3 | 3 | 3 |
|  |  | 8 | 0 | 2 | 3 | 3 | 3 |
| pred_type | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | 1 | 2 | 3 | na |
|  |  | 5 | 0 | 1 | 2 | 3 | 4 |
| prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| rem_intra_luma_pred_mode | Error: Reference source not found | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | 1 | 0 | 0 | 0 | 0 | 0 |
|  |  | 2 | 0 | 0 | 0 | 0 | 0 |
| intra_chroma_pred_mode | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 3 | 3 | 3 | na |
|  |  | 4 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 3 | 3 | 3 | na |
|  |  | 8 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 3 | 3 | 3 | na |
| merge_flag | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
| merge_idx | Error: Reference source not found | 0 | 0,1,2,3 (subclause Error: Reference source not found) | 0,1,2,3 (subclause Error: Reference source not found) | 0,2,3 (subclause Error: Reference source not found) | 3 | na |
|  |  | 4 | 0,1,2,3 (subclause Error: Reference source not found) | 0,1,2,3 (subclause Error: Reference source not found) | 0,2,3 (subclause Error: Reference source not found) | 3 | na |
| inter_pred_flag | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) | na | na | na | na |
| ref_idx_l0 | Error: Reference source not found | 0 | 0,1,2,3 (subclause 9.3.3.1.2.1) | 4 | 4 | 4 | 4 |
| ref_idx_l0, ref_idx_l1, ref_idx_lc | Error: Reference source not found | 6 | 0,1,2,3 (subclause 9.3.3.1.2.1) | 4 | 4 | 4 | 4 |
| mvd_l0[ ][ ][ 0 ] | Error: Reference source not found | 0 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 3 | 4 | 5 | 6 |
| mvd_l0[ ][ ][ 1 ] | Error: Reference source not found | 7 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 3 | 4 | 5 | 6 |
| mvd_l0[ ][ ][ 0 ], mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | Error: Reference source not found | 15 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 3 | 4 | 5 | 6 |
| mvd_l0[ ][ ][ 1 ], mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] | Error: Reference source not found | 21 | 0,1,2 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | 3 | 4 | 5 | 6 |
| mvp_idx_l0 | Error: Reference source not found | 0 | 0 | 1 | 1 | 1 | 1 |
| mvp_idx_l0, mvp_idx_l1, mvp_idx_lc | Error: Reference source not found | 2 | 0 | 1 | 1 | 1 | 1 |
| no_residual_data_flag | Error: Reference source not found | 0 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
| split_transform_flag | Error: Reference source not found | 0 | cuDepth + trafoDepth | na | na | na | na |
|  |  | 4 | cuDepth + trafoDepth | na | na | na | na |
|  |  | 8 | cuDepth + trafoDepth | na | na | na | na |
| cbf_luma | Error: Reference source not found | 0 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
|  |  | 8 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
| cbf_cb ( PredMode = = MODE_INTRA ) | Error: Reference source not found | 0 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
|  |  | 8 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
| cbf_cb ( PredMode != MODE_INTRA ) | Error: Reference source not found | 0 | trafoDepth | na | na | na | na |
|  |  | 4 | trafoDepth | na | na | na | na |
|  |  | 8 | trafoDepth | na | na | na | na |
| cbf_cr ( PredMode = = MODE_INTRA ) | Error: Reference source not found | 0 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
|  |  | 8 | 0,1,2,3 (subclause 9.3.3.1.2.1) (subclause 9.3.3.1.2.17) | na | na | na | na |
| cbf_cr ( PredMode != MODE_INTRA ) | Error: Reference source not found | 0 | trafoDepth | na | na | na | na |
|  |  | 4 | trafoDepth | na | na | na | na |
|  |  | 8 | trafoDepth | na | na | na | na |
| last_significant_coeff_x | Error: Reference source not found | 0 | 0..40 (subclause Error: Reference source not found) |  |  |  |  |
|  |  | 41 | 0..40 (subclause Error: Reference source not found) |  |  |  |  |
|  |  | 82 | 0..40 (subclause Error: Reference source not found) |  |  |  |  |
| last_significant_coeff_y | Error: Reference source not found | 0 | 0..40 (subclause Error: Reference source not found) |  |  |  |  |
|  |  | 41 | 0..40 (subclause Error: Reference source not found) |  |  |  |  |
|  |  | 82 | 0..40 (subclause Error: Reference source not found) |  |  |  |  |
| significant_coeff_flag | Error: Reference source not found | 0 | 0..111 (subclause Error: Reference source not found) | na | na | na | na |
|  | Error: Reference source not found | 0 | 0..111 (subclause Error: Reference source not found) | na | na | na | na |
|  | Error: Reference source not found | 0 | 0..111 (subclause Error: Reference source not found) | na | na | na | na |
| coeff_abs_level_greater1_flag | Error: Reference source not found | 0 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 60 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 120 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
| coeff_abs_level_greater2_flag | Error: Reference source not found | 0 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 60 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 120 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |

##### Assignment process of ctxIdxInc using neighbouring syntax elements

Subclause 9.3.3.1.2.1 specifies the derivation process of ctxIdxInc for the syntax elements alf_cu_flag, split_coding_unit_flag, skip_flag, merge_flag, intra_chroma_pred_mode, inter_pred_flag, ref_idx_lc, ref_idx_l0, ref_idx_l1., mvd_l0, mvd_l1, mvd_lc, no_residual_data_flag, cbf_luma, cbf_cb and cbf_cr.

Subclause 9.3.3.1.2.17 specifies the derivation process of ctxIdxInc for the syntax elements skip_flag, merge_flag, intra_chroma_pred_mode, mvd_l0, mvd_l1, mvd_lc, no_residual_data_flag, cbf_luma, cbf_cb, and cbf_cr.

###### Derivation process of ctxIdxInc using left and above syntax elements

Input to this process is the luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture.

Output of this process is ctxIdxInc.

Let the luma location ( xL, yL ) specify a location covered by the prediction unit to the left of the top-left luma sample of the current prediction unit with xL = xP − MinPuSize and yL = yP, the variable availableL is derived as follows. [Ed.: (BB) MinPuSize should be defined somewhere]

- If the prediction unit covering location ( xL, yL ) is available, availableL is set equal to 1.

- Otherwise (the prediction unit covering location ( xL, yL ) is not available), availableL is set equal to 0.

Let the luma location ( xA, yA ) specify a location covered by the prediction unit above the top-left luma sample of the current prediction unit with xA = xP and yA = yP − MinPuSize, the variable availableA is derived as follows. [Ed.: (BB) MinPuSize should be defined somewhere]

- If the prediction unit covering location ( xA, yA ) is available, availableA is set equal to 1.

- Otherwise (the prediction unit covering location ( xA, yA ) is not available), availableA is set equal to 0

The assignment of ctxIdxInc for the syntax elements alf_cu_flag, split_coding_unit_flag, skip_flag, merge_flag, intra_chroma_pred_mode, inter_pred_flag, ref_idx_lc, ref_idx_l0, ref_idx_l1., mvd_l0, mvd_l1, mvd_lc, no_residual_data_flag, cbf_luma, cbf_cb and cbf_cr is specified in Error: Reference source not found50.

###### Derivation process of ctxIdxInc using left and above syntax elements within current LCU row

Inputs to this process are

- the luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- the luma location ( xLC, yLC ) specifying the top-left luma sample of the current largest coding unit relative to the top-left sample of the current picture.

Output of this process is ctxIdxInc.

Let the luma location ( xL, yL ) specify a location covered by the prediction unit to the left of the top-left luma sample of the current prediction unit with xL = xP − MinPuSize and yL = yP, the variable availableL is derived as follows. [Ed.: (BB) MinPuSize should be defined somewhere]

- If the prediction unit covering location ( xL, yL ) is available, availableL is set equal to 1.

- Otherwise (the prediction unit covering location ( xL, yL ) is not available), availableL is set equal to 0.

Let the luma location ( xA, yA ) specify a location covered by the prediction unit above the top-left luma sample of the current prediction unit with xA = xP and yA = yP – MinPuSize. Let the luma location ( xL, yL ) specify a location covered by the prediction unit to the left of the top-left luma sample of the current prediction unit with xL = xP − MinPuSize and yL = yP. The variable availableA is derived as follows. [Ed.: (BB) MinPuSize should be defined somewhere]

- If yA is smaller than yLC, the variable availableA is derived as follows.

  - If the prediction unit covering location ( xL, yL) is available, availableA is set equal to 1.

  - Otherwise (the prediction unit covering location ( xL, yL ) is not available), availableA is set equal to 0

- Otherwise, the variable availableA is derived as follows.

  - If the prediction unit covering location ( xA, yA ) is available, availableA is set equal to 1.

  - Otherwise (the prediction unit covering location ( xA, yA ) is not available), availableA is set equal to 0

The assignment of ctxIdxInc for the syntax elements skip_flag, merge_flag, intra_chroma_pred_mode, mvd_l0, mvd_l1, mvd_lc, no_residual_data_flag, cbf_luma, cbf_cb, and cbf_cr is specified in Error: Reference source not found50.