| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F064 |
| --- | --- |

| Title: | CE1: Results of Experiments A.6, A.8, and A.10 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Xun Guo, and Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {yiwen.chen, jl.lin, xun.guo yuwen.huang }@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reported results of experiments A.6, A.8 and A.10 in CE1. The experiments were conducted to see the performance analysis of combinations of motion data storage reduction (MDSR) positions and temporal motion vector predictor (TMVP) positions. In the HM-3.0 anchor, TL is used for MDSR and H or C0 when H is not available is used for TMVP. In experiment A.6, C0 was used for MDSR, and C0 was used for TMVP. The average bit rates for high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD) were increased by 0.9%, 0.9%, 1.1%, and 1.1%, respectively. In experiment A.8, TL was used for MDSR, and C3 was used for TMVP. The corresponding bit rates were increased by 0.7%, 0.7%, 1.0%, and 1.0%. In experiment A.10, C0 was used for MDSR, and C3 was used for TMVP. The corresponding bit rates were increased by 0.7%, 0.7%, 0.9%, and 0.9%.

- Introduction

The category-A experiments in CE1 [1] are to test the performance analysis of combinations of motion data storage reduction (MDSR) positions and temporal motion vector predictor (TMVP) positions. As depicted in Figure 1 and Figure 2, four positions (TL, C0, C3 and BR) are tested for MDSR and five positions (TL, C0, C3, BR, and H) are tested for TMVP. The related techniques are from JCTV-E092 [2].

Figure 1.  Illustration of tested positions for MDSR.

Figure 2.  Illustration of tested positions for TMVP.

# Simulation results

This contribution reports the results of A.6, A.8 and A.10 experiments using a software implementation based on HM-3.0 with the fix of bug #146, and the anchor is HM-3.0 with the fix of bug #146 as required in CE1. All the simulations are done following the common test condition as described in JCTVC-E700 [3]. The results are shown in the following subsections.

## Experiment A.6: C0 for MDSR and C0 for TMVP

In experiment A.6, the MDSR position is C0 and the TMVP position is C0. The results are shown in Table 1. The average bit rates for high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD) are increased by 0.9%, 0.9%, 1.1%, and 1.1%, respectively.

Table 1.  Results of experiment A.6: C0 for MDSR and C0 for TMVP

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.0 | 1.0 | 1.1 | 1.0 | 0.9 | 0.9 |
| Class B | 0.6 | 0.5 | 0.6 | 0.7 | 0.5 | 0.6 |
| Class C | 0.9 | 0.9 | 0.9 | 0.9 | 0.8 | 0.9 |
| Class D | 1.0 | 1.1 | 1.0 | 0.9 | 0.7 | 0.7 |
| Class E |  |  |  |  |  |  |
| All | 0.9 | 0.8 | 0.9 | 0.9 | 0.7 | 0.8 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.9 | 0.8 | 0.5 | 1.0 | 0.7 | 0.6 |
| Class C | 1.2 | 1.1 | 1.1 | 1.2 | 1.1 | 0.8 |
| Class D | 1.0 | 0.8 | 1.4 | 1.0 | 0.7 | 0.6 |
| Class E | 1.4 | 1.2 | 1.5 | 1.0 | 0.3 | 0.9 |
| All | 1.1 | 0.9 | 1.1 | 1.1 | 0.7 | 0.7 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

## Experiment A.8: TL for MDSR and C3 for TMVP

In experiment A.8, the MDSR position is TL and the TMVP position is C3. The results are shown in Table 2. The average bit rates for HE-RA, LC-RA, HE-LD, and LC-LD are increased by 0.7%, 0.7%, 1.0%, and 1.0%, respectively.

Table 2.  Results of experiment A.8: TL for MDSR and C3 for TMVP

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.8 | 0.7 | 0.7 | 0.9 | 0.8 | 0.7 |
| Class B | 0.5 | 0.3 | 0.4 | 0.5 | 0.4 | 0.4 |
| Class C | 0.7 | 0.8 | 0.7 | 0.8 | 0.7 | 0.9 |
| Class D | 0.9 | 0.7 | 0.7 | 0.9 | 0.7 | 0.7 |
| Class E |  |  |  |  |  |  |
| All | 0.7 | 0.6 | 0.6 | 0.7 | 0.6 | 0.7 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.8 | 0.5 | 0.4 | 0.9 | 0.6 | 0.6 |
| Class C | 1.1 | 1.1 | 1.1 | 1.2 | 1.1 | 1.0 |
| Class D | 1.0 | 0.8 | 1.3 | 1.1 | 0.5 | 0.5 |
| Class E | 1.0 | 0.4 | 0.9 | 0.7 | -0.2 | 1.0 |
| All | 1.0 | 0.7 | 0.9 | 1.0 | 0.6 | 0.7 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

## Experiment A.10: C0 for MDSR and C3 for TMVP

In experiment A.10, the MDSR position is C0 and the TMVP position is C3. The results are shown in Table 3. The average bit rates for HE-RA, LC-RA, HE-LD, and LC-LD are increased by 0.7%, 0.7%, 0.9%, and 0.9%, respectively.

Table 3.  Results of experiment A.10: C0 for MDSR and C3 for TMVP

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.8 | 0.7 | 0.7 | 0.8 | 0.7 | 0.7 |
| Class B | 0.4 | 0.3 | 0.4 | 0.5 | 0.4 | 0.5 |
| Class C | 0.7 | 0.7 | 0.8 | 0.7 | 0.6 | 0.8 |
| Class D | 0.8 | 0.7 | 0.8 | 0.7 | 0.5 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | 0.7 | 0.6 | 0.7 | 0.7 | 0.6 | 0.6 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.8 | 0.7 | 0.4 | 0.9 | 0.5 | 0.7 |
| Class C | 1.1 | 0.9 | 0.9 | 1.1 | 1.0 | 0.8 |
| Class D | 0.9 | 1.0 | 1.5 | 1.0 | 0.5 | 0.3 |
| Class E | 1.0 | 0.7 | 0.8 | 0.7 | 0.4 | 0.8 |
| All | 0.9 | 0.8 | 0.9 | 0.9 | 0.6 | 0.6 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Conclusion

In CE1, MediaTek conducted experiments A.6, A.8 and A.10. The results reportedly showed that by changing the MDSR position and the TMVP position (C0, C0), (TL, C3) and (C0, C3) could cause roughly 1.0%, 0.9% and 0.8% bit rate increases, respectively.

# References

- J. Jung, P. Onno, and Y.-W. Huang, “Description of Core Experiment CE 1: Motion Data Storage Reduction,” Document of Joint Collaborative Team on Video Coding, JCTVC-E701, Mar. 2011.

- X. Guo, J.-L. Lin, Y.-W. Huang, and S. Lei, “Motion Vector Decimation for Temporal Prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-E092, Mar. 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-E700, Mar. 2011.