| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F068 M20480 |
| --- | --- |

| Title: | A study on HM3.0 parsing throughput issue |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Vivienne Sze Yasutomo Matsuba | Tel: Email:Tel: Email:Tel: Email: | +1 214.480.3816 zhou@ti.com 214-567-4005 sze@ti.com 214-480-3246 matsuba@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

# It has been reported that the current HM3.0 design has parsing throughput issue due to interdependency between the motion data reconstruction and entropy decoding. This contribution reports throughput study results and proposes a solution to the reported issue. Throughput study reveals that the entropy decoding throughput would need to be improved by roughly 2x to achieve the same decoding throughput as AVC if such interdependency remains unchanged. It is proposed to use truncated unary coding with fixed maximum code length signaled in the slice header for the MVP index coding to completely remove the interdependency between the motion data reconstruction and entropy decoding. Compared to HM3.0 the proposed algorithm leads to an average loss of 0.6% in RA-HE, 0.7% in RA-LC, 1.0% in LB-HE and 1.1% in LB-LC, respectively.  A study on combined solution with JCTVC-F470 shows that average gain of 0.0% in RA-HE, 0.1% in RA-LC, 0.1% in LB-HE and 0.5% in LB-LC can be achieved. The combined solution has considered practical implementation by limiting the maximum number of additional scales to ONE and maximum number of MVP candidates for the final pruning process to 10.

# Introduction

The current HM3.0 design is reported to have parsing throughput issue due to interdependency between the motion data reconstruction and entropy decoding. This contribution reports throughput study results and recommends solutions.

# Problem statement

Figure 1 illustrates the data flow of the merge flag decoding, merge/skip and AMVP MVP list construction, and MVP index decoding on the decoder side in HM3.0. The MVP index is needed if the MVP list size is larger than one because decoder needs to know which of MVP candidates in the MVP list is actually used in the merge/skip mode and in motion vector coding in order to reconstruct the motion vectors, prediction direction (forward/backward/bi-directional) and reference index on the decoder side.

Next CU

CU data

n

y

n

y

Skip

mode?

Merge/skip MVP list construction

Skip mode MVP index decoding

Motion compensation & CU reconstruction

merge flag decoding

Merge mode ?

AMVP list construction

Ref_idx, MVDs & MVP_idx decoding

Merge/skip MVP list construction

merge mode MVP index decoding

Repeat for PUs in the CU

Figure 1. Diagram of the merge flag decoding, merge /skip MVP list, and AMVP list construction and MVP index decoding

The interdependency between the motion data reconstruction and entropy decoding is caused by:

- MVP index coding for merge/skip mode depends on the merge/skip mode MVP list size. If the merge/skip mode MVP list size is one, the MVP index is not present in the bitsteam. Otherwise, the binarization and CAVLC coding of the MVP index depends on the merge/skip mode MVP list size. The determination of the MVP list size requires full reconstruction of motion data.

- MVP index coding for motion vector coding depends on the AMVP MVP list size. It operates in exactly the same manner as the merge/skip mode MVP index

This MVP list size dependent MVP index (merge/skip/AMVPMVP index) coding causes several:

- decoder is unlikely be able to meet throughput requirements, because AMVP and merge/skip MVP list construction consumes a significant amount of cycles. Unless the entropy decoding throughput can be dramatically improved, this interdependency between the motion data reconstruction and entropy decoding would make practical real-time HEVC decoder implementation very difficult if not impossible.

- for bitstream parsing in which video reconstruction is not required a decoder would still need to fully reconstruct motion data (motion vector, prediction direction, reference frame index, intra/inter flag and part size) to be able to parse the bitstream. This is a huge overhead for the bitstream parsing in terms of processing power and memory bandwidth (motion data needs to be transferred between on/off chip memory)

- bitstream is very sensitive to errors, any corruption in the motion data (e.g. motion vectors) will lead to incorrect construction of the merge/skip mode and AMVP MVP list, which then causes incorrect decoding of the MVP index and eventually leads to bitstream parsing stop.

# Throughput analysis

As shown in Figure 1, in HM3.0 either the merge/skip MVP list or the AMVP list needs to be fully constructed for the entropy decoding process of each PU. Therefore, it is important to understand additional overhead added by the MVP list construction operated sequentially to the entropy decoding processing.   As the merge/skip mode can directly utilize the motion vectors from neighbors, while the regular inter prediction mode cannot, the AMVP list construction and motion vector reconstruction becomes a longer thread.

Current PU

A0

E

B0

C

D

Co-located PU

Co-located PU

CR

1

2

H

Figure 2. Illustration of the HM3.0 AMVP list construction

Figure 2. depicts the AMVP list construction in HM3.0. The AMVP list construction uses five spatial neighboring MVPs and two co-located temporal MVPs  to produce a final MVP list of up to 3 MVP candidates.  Figure 3 illustrates the architecture diagram of one of hardware implementations for HM3.0 AMVP list construction and motion vector reconstruction. It is estimated that such a process consumes a total of 6 cycles (266 MHz frequency, 45 nm technology), in which 1 cycle for MVP availability check (decision), 3 cycles for spatial/temporal MVP scale, 1 cycle each for AMVP list pruning and motion vector reconstruction. The estimate does not count cycles needed for neighboring motion data store and fetch of motion data reconstruction, and for interleaved merge/skip flag and MVP index entropy decoding. A more detailed architecture diagram for scale implementation is provided in Figure 4.

Figure 3. Architecture diagram of HM3.0 AMVP list construction and motion vector reconstruction on decoder side

Legend

DistScaleFactor

mvCol

128

>>

8

clipMv

mvLXCol

FlipFlops

Combinational Block

clip3

32

>>

6

tx

tb

Tx table

128:1

refIdxCol

ListCol

3 cycles / 1 MVscale

3.5Kgates per 1 MV component

1Kgates

GS70, 266MHz

Td

calc

Figure 4. Architecture diagram of spatial/temporal MVP scale

# Because the minimum PU size is 4x4 in HM3.0, the overhead cycle count will be 96 cycles per 16x16 block(16 x 6 cyles/4x4). Adding other overheads, for example memory access, this cycle overhead can be well over 100 cycles per 16x16 block. For video cores which support 1080p@120 AVC video today, the cycle budget is around 200 cycles per 16x16 block. This implies that the HEVC entropy decoding throughput would need to be improved by at least 2x in order to accommodate the cycle overhead caused by the interdependency between the motion data reconstruction and entropy decoding.

# It is worth to point out that temporal/spatial MVP scaling is expensive not only in terms of cycles but also in terms of area. As shown in Figure 4, one single scale already costs roughly 3.5 K gates. For the HM3.0 AMVP list construction in both directions (list 0 and list 1), the total gate count for temporal/spatial MVP scaling only amounts to about 71 Kgates (3.5[Kgates] x 5[MVs] x 2[components] x 2[directions] = 70[Kgates]  plus 1Kgates Tx table). It is estimated that the area of the current HM3.0 AMVP design is about 10x of the AVC median filter based design.

# Proposed algorithm

To resolve the parsing throughput issue discussed about, the proposed algorithm uses truncated unary code for the MVP index coding to completely remove the interdependency between the motion data reconstruction and entropy decoding. The key features of proposed algorithms are as follows:

- the MVP index is still transmitted at PU level using truncated unary coding, but code table size (or maximum MVP list size allowed) is signaled in the slice header.

- If the MVP list size (for merge/skip and for AMVP) is larger than selected table size signaled in slice header, the low-priority MVP candidates in the MVP list are dropped so that the final MVP list size is always less or equal to the MVP list size defined in slice header.

- for merge/skip MVP list if its size is smaller than selected table size signaled in slice header and the MVP list is not empty,  zero motion vectors is appended to the merge/skip list to improve coding efficiency.

## Straightforward approach for removing interdependency

As mentioned earlier, in HM-3.0 the AMVP mvp index (mvp_idx) and merge/skip MVP index (merge_idx) are coded using truncated unary where the max codeword size is dictated by the MVP list size (i.e. 1, 2, 3, 4, or 5 for merge/skip MVP list, and 1, 2, or 3 for AMVP list). Furthermore, for CABAC encoding, the context selection for bins in merge_idx depend on the list size and which candidates were in the list.

The straightforward method involves using the max list size for merge_idx and mvp_idx to determine the max codeword size in truncated unary (i.e. the highlighted column to the most right in the table below).  As compared to HM-3.0, an additional bit could potentially be sent for merge_idx<4 and mvp_idx<2. Furthermore, the merge_idx context selection was modified to only depend on binIdx. The total number of contexts used for merge_idx (i.e. 4) remains the same.

| Merge/skip MVP index | Merge candidate list size |
| --- | --- |
|  | 5 |
| 0 | 1 |
| 1 | 01 |
| 2 | 001 |
| 3 | 0001 |
| 4 | 0000 |

| AMVP index | AMVP candidate list size |
| --- | --- |
|  | 3 |
| 0 | 1 |
| 1 | 01 |
| 2 | 00 |

## MVP list size signaling in slice header

Two high-level syntax elements, namely max_active_merge_candis and max_active_amvp_cands, are defined in slice header to signal the maximum merge/skip MVP list size and AMVP (motion vector coding) list size used in the current slice. Based on the values of  max_active_merge_cands and max_active_amvp_cands  the following truncated unary tables are specified.

| Merge/skip MVP index | max_active_merge_cands |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| 0 | N/A | 1 | 1 | 1 | 1 |
| 1 |  | 0 | 01 | 01 | 01 |
| 2 |  |  | 00 | 001 | 001 |
| 3 |  |  |  | 000 | 0001 |
| 4 |  |  |  |  | 0000 |

Table 1. truncated unary tables for CABAC binarization or variable length coding of MVP index for merge/skip mode

| AMVP MVP index | max_active_AMVP_cands |  |  |
| --- | --- | --- | --- |
|  | 1 | 2 | 3 |
| 0 | N/A | 1 | 1 |
| 1 |  | 0 | 01 |
| 2 |  |  | 00 |

Table 2, truncated unary tables for CABAC binarization or variable length coding of MVP index for motion vector coding (AMVP)

For merge/skip mode, max_active_merge_cands can have value of 1, 2, 3, 4, or 5. While for motion vector coding, the value of max_active_amvp_cands is 1, 2 or 3. In  Table 1 and 2, the coding of the MVP index is independent of the MVP list size of PUs. Instead, it just depends on the max_active_merge_cands and max_active_amvp_cands pre-defined in the slice header, the value setting for  max_active_merge_cands and max_active_amvp_candidate is an encoder choice.  Note that the MVP index is not present in the bitstream if the maximum number of active MVP candidates (max_active_merge_cands and /or max_active_amvp_cands) is set to one.

## Modified merge/skip MVP list construction

Final list

Final list

MVP candidates

n

y

Valid MVP candidates

≥ max_active_merge_cands

HM3.0 Merge/skip MVP list construction

Discard low-priority MVP candidates to reduce the MVP list size to  max_active_merge_cands

If the MVP list is not empty, add zero MVP candidate to the list

2

3

1

4

5×

1

2

Final MVP list

Valid MVP candidates        = 4

max_active_merge_cands = 3

(case A)

(case a)

MVP candidates

2

3

1

4

5

1

2

Final MVP list

Valid MVP candidates        = 2

max_active_merge_cands = 3

(case B)

(case b)

Zero MVP candidate

0

0

Figure 5. Diagram of modified merge/skip MVP list construction

To use tables defined in Table 1 and Table 2, the HM3.0 merge/skip and AMVP list construction need to be modified so that the number of valid MVP candidates in the final MVP list will be less than or equal to the maximum number of MVP candidates specified by max_active_merge_cands and max_active_amvp_cands.  As shown in Figure 5, after the HM3.0 merge/skip MVP list construction, the number of valid MVP candidates is compared with max_active_merge_cands. If the number of valid MVP candidates exceeds the maximum number allowed, the low-priority MVP candidates are discarded (see case A in Figure 2, MVP candidate 5 is discarded, here high priority MVP candidates have lower MVP indices). On the contrary, if the number of valid MVP candidates is less than maximum number of candidates allowed, zero MVP candidate is added to the MVP list to improve the coding efficiency (see case B in Figure 2, a zero MVP candidate is added to the final list because the valid number of MVP candidates is less than maximum number of MVP candidates allowed and the MVP list is not empty).

## Modified AMVP list construction

The modified AMVP list construction is depicted in Figure 6, the modification is essentially the same as that of the modified merge/skip MVP list construction depicted in Figure 5, except that zero vectors are not appended to the final AMVP list for case in which the MVP list size is less than max_active_amvp_cands.

Final list

Final list

MVP candidates

n

n

y

Valid MVP candidates

≥ max_active_AMVP_cands

HM3.0 AMVP MVP list formation

Discard low-priority MVP candidates to reduce the MVP list size to  max_active_AMVP_cands

2

3

1

1

Final MVP list

Valid MVP candidates        = 3

max_active_amvp_cands  = 2

(case A)

MVP candidates

2

3

1

Final MVP list

Valid MVP candidates        = 1

max_active_amvp_cands =  2

(case B)

0

0

Figure 6. Diagram of modified AMVP (motion vector coding) MVP list construction

# Test Settings and Conditions

The simulations of this document have used HM3.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed. The test settings are summarized in the table below, among them the straightforward method uses truncated unary coding for merge/skip MVP index and for AMVP MVP index with fixed maximum code length of 5 and 3, respectively. In the straightforward method no zero vectors are appended if the MVP list size is less than max_active_merge_cands and MVP list is not empty.

| Method | max_active_merge_cands | max_active_amvp_cands | Adding zero vector to the merge/skip MVP list if list size is less than max_active_merge_cands and the list is not empty |
| --- | --- | --- | --- |
| straightforward | 5 | 3 | NO |
| Proposed setting 1 | 5 | 3 | Yes |
| Proposed setting 2 | 4 | 3 | Yes |
| Proposed setting 3 | 3 | 3 | Yes |
| Proposed setting 4 | 3 | 2 | Yes |

Table 3, test settings for the proposed algorithm

# Experimental results

# The simulation results using HM3.0 are summarized in Table 4. For setting 4 in which truncated unary codes with fixed maximum size of 3 and 2 are used for merge/skip and AMVP MVP index coding, respectively, an average loss of loss of 0.6% in RA-HE, 0.7% in RA-LC, 1.0% in LB-HE and 1.1% in LB-LC are observed. Compared to the straightforward method, the proposed algorithm cuts loss by 0.3% in RA-HE, 0.5% in RA-LC, 0.4% in LB-HE and 0.8% in LB-LC, respectively. Adding zero vectors to merge/skip MVP list can reduce loss (i.e. difference between “straightforward” and “proposed setting 1” in Table 4) by 0.2% in RA-HE, 0.3% in RA-LC, 0.2% in LB-HE and 0.4% in LB-LC, respectively.

| Method | max_active_merge_cands | max_active_AMVP_cands | RA-HE (%) | RA_LC(%) | LB-HE(%) | LB-LC(%) |
| --- | --- | --- | --- | --- | --- | --- |
| straightforward | 5 | 3 | 0.9 | 1.2 | 1.4 | 1.9 |
| Proposed setting 1 | 5 | 3 | 0.7 | 0.9 | 1.2 | 1.5 |
| Proposed setting 2 | 4 | 3 | 0.7 | 0.9 | 1.1 | 1.5 |
| Proposed setting 3 | 3 | 3 | 0.7 | 0.9 | 1.1 | 1.3 |
| Proposed setting 4 | 3 | 2 | 0.6 | 0.7 | 1.0 | 1.1 |

Table 4, Average BD-Rate increase (%) relative to HM3.0 anchor

Table 5 summarizes the results for best performing setting of max_active_merge_cands =3 and max_active_amvp_cands = 2 of proposed algorithm.

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 1.7 | 1.7 | 0.6 | 0.7 | 0.6 |
| Class B | 0.7 | 0.4 | 0.4 | 0.8 | 0.7 | 0.7 |
| Class C | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.8 |
| Class D | 0.6 | 0.7 | 0.7 | 0.6 | 0.4 | 0.4 |
| Class E |  |  |  |  |  |  |
| All | 0.6 | 0.8 | 0.8 | 0.7 | 0.6 | 0.7 |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.8 | 0.4 | 0.1 | 0.9 | 0.7 | 0.5 |
| Class C | 0.7 | 0.6 | 0.5 | 0.7 | 0.7 | 0.5 |
| Class D | 0.7 | 0.3 | 0.7 | 0.6 | 0.6 | 0.2 |
| Class E | 2.3 | 1.7 | 2.0 | 2.7 | 2.2 | 2.4 |
| All | 1.0 | 0.7 | 0.7 | 1.1 | 0.9 | 0.8 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

Table 5, Average BD-Rate increase (%) relative to HM3.0 anchor for max_active_merge_cands =3 and max_active_amvp_cands =2 of proposed algorithm

# Study on combined solution with JCTVC-F470

A study has been conducted to investigate the potential of quality loss reduction by adding more additional bi-directional MVPs generated by MVP combinations and scaling as proposed in [4]. The diagram of combined solution is depicted in Figure 7.  To address complexity and latency concerns raised by [5], the combined solution has changed following aspects:

- Generation of additional bi-directional MVP candidates with MVP scaling  (i.e. scaled MVP candidate derivation process)

  - If the temporal MVP is selected for scaling, co-located temporal MVP, but not derived temporal MVP, is used for generating scaled MVP in order to avoid cascaded MVP scaling.

  - Regardless of reference index of the original MVP of one direction (list 0 or list 1), reference index for the scaled MVP of other direction (list 1 or list0) is fixed to ZERO.

  - Maximum number of additional scales is limited to ONE, i.e. only the first available MVP in priority order is scaled, the rest of  available MVPs are discarded from the scaled MVP candidate derivation process.

- Generation of additional bi-directional MVP candidates with MVP combinations (i.e. combined MVP candidate derivation process)

  - Only up to FIVE first available combined MVP candidates in priority order are allowed to participate in the final MVP pruning process (see Figure 7), the rest of combinations are discarded. For example in Table 6 and Table 7, highlighted MVPs are available ones, but the last two combinations (priority order 9 and 11) are dropped.

With those modifications, the complexity can be significantly reduced:

- The worst case number of additional scales is educed from 8 to 1

- Number of MVP candidates for final MVP pruning is reduced from 24 (i.e. 4 MVPs after the 1st stage pruning + 12 combined MVPs + 8 scaled MVPs) to 10 (i.e. 4 MVPs after the 1st stage pruning + 5 combined MVPs + 1 scaled MVP). This reduces the number of compares of the final pruning process from 12*23 to 5*9, a roughly 6x of complexity reduction.

1

2

3

4

5

Scale

1

2

3

4

5

Final MVP pruning

Scale

3

1

3

1

2

1

3

2

3

1

3

2

3

0

1st MVP pruning

1

2

3

4

5

Scale

Scale

11

Scale

MVP combinations

1

2

1

3

Legend:

MVP positions     
 MVP availability check              MVP pruning

MVP candidates                         additional candidates              zero candidate

MVP list after pruning

Final MVP list

Figure 7. Diagram of combined solution with JCTVC-F470

| MVP Index | MVP (list 0) | MVP (list 1) |
| --- | --- | --- |
| 1 | MVf1 | MVb1 |
| 2 | MVf2 | MVb2 |
| 3 | MVf3 | MVb3 |
| 4 | MVf4 | MVb4 |

Table 6,  MVP candidates for merge/skip MVP list in B-slices after the 1st pruning process in Figure 7.

| Priority index | 0 | 1 | 2 | 4 |
| --- | --- | --- | --- | --- |
| Combination | (MVf1, MVb2) | (MVf2, MVb1) | (MVf1, MVb3) | (MVf3, MVb1) |
| Priority index | 4 | 5 | 6 | 7 |
| Combination | (MVf2, MVb3) | (MVf3, MVb2) | (MVf1, MVb4) | (MVf4, MVb1) |
| Priority index | 8 | 9 | 10 | 11 |
| Combination | (MVf2, MVb4) | (MVf4, MVb2) | (MVf3, MVb4) | (MVf4, MVb3) |
| Priority index |  |  |  |  |

Table 7, Priority table for combined additional bi-directional MVP candidates in B-slices

Experimental results with HM3.0 are summarized in Table below. Among the settings, the best setting is setting 8 in which an average gain of 0.0% in RA-HE, 0.1% in RA-LC, 0.1% in LB-HE and 0.5% in LB-LC can be achieved by adding up to one scaled MVP and five combined MVPs to the final merge/skip MVP list pruning process. Note that the AMVP list size is set to 2 in setting 8. The average results for setting 8 are provided in Table 9. for detailed results please refer to spreadsheets in combined_study_results sub-directory.

| Method | max_active_merge_cands | max_active_amvp_cands | No. of scales (max scaled MVPs) | Max. of combined MVPs | RA-HE (%) | RA_LC (%) | LB-HE (%) | LB-HE (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F068-Table 5 | 3 | 2 | 0 | 0 | 0.6 | 0.7 | 1.0 | 1.1 |
| Setting 1 | 5 | 3 | 8 | 12 | 0.0 | -0.1 | 0.0 | -0.4 |
| Setting 2 | 5 | 3 | 2 | 12 | 0.1 | 0.0 | 0.0 | -0.4 |
| Setting 3 | 5 | 3 | 1 | 12 | 0.1 | 0.0 | 0.0 | -0.4 |
| Setting 4 | 5 | 3 | 0 | 12 | 0.2 | 0.2 | 0.1 | -0.3 |
| Setting 5 | 5 | 3 | 2 | 5 | 0.1 | 0.0 | 0.0 | -0.4 |
| Setting 6 | 5 | 3 | 1 | 5 | 0.1 | 0.0 | 0.0 | -0.4 |
| Setting 7 | 5 | 2 | 2 | 5 | 0.0 | -0.2 | -0.1 | -0.5 |
| Setting 8 | 5 | 2 | 1 | 5 | 0.0 | -0.1 | -0.1 | -0.5 |

Table 8, Summary results of combined study

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.2 | -0.1 | -0.2 | 0.0 | -0.2 |
| Class B | 0.1 | -0.2 | -0.3 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | -0.1 | -0.1 | -0.2 | -0.2 |
| Class D | -0.1 | -0.1 | -0.2 | -0.2 | -0.3 | -0.4 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | -0.2 | -0.1 | -0.1 | -0.2 |
| Enc Time[%] | 120% |  |  | 112% |  |  |
| Dec Time[%] | 107% |  |  | 105% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.9 | -1.5 | -1.1 | -1.1 | -1.0 |
| Class C | -0.4 | -0.7 | -0.9 | -0.7 | -0.8 | -0.9 |
| Class D | -0.3 | -0.3 | 0.3 | -0.4 | -0.3 | -0.7 |
| Class E | 1.0 | 0.7 | 0.5 | 0.5 | 0.1 | 0.2 |
| All | -0.1 | -0.4 | -0.5 | -0.5 | -0.6 | -0.7 |
| Enc Time[%] | 115% |  |  | 109% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |

Table 9,  Average BD-rate decrease of setting 8

# Conclusion and recommendations

Throughput study presented in this contribution has demonstrated the need of decoupling the motion data reconstruction from entropy decoding for the real-time HEVC decoder implementation. An algorithm has been proposed to resolve the reported parsing throughput issue. The proposed algorithm introduced minimum amount of changes to the current HM3.0 design. It is recommended to adopt this algorithm into the HEVC test model to address parsing throughput concerns and improve parsing robustness.  A combined solution with JCTVC-F470 has also been presented in this contribution, it is recommended to include this part of algorithm into CE for further study.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

[2]	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-E603, Geneva, April 2011.

[3] M. Zhou, V. Sze, “A study on HM2.0 bitstream parsing and error resiliency issue”. JCT-VC Document, JCTVC-E603, Geneva, April 2011.

[4] T. Sugio, T. Nishi, “Parsing Robustness for Merge/AMVP”, JCTVC-F470, 6th Meeting: Torino, IT, 14-22 July, 2011

[5] M. Zhou, “Cross-verification of Panasonic's proposal JCTVC-F470 on parsing robustness for merge/AMVP”, JCTVC-F090, 6th Meeting: Torino, IT, 14-22 July, 2011

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

WD texts

Related sections

### 7. 3. 3 Slice header syntax

### 7.4.3 Slice header semantics

### 7.3.7 Prediction unit syntax

##### 8.4.2.1.1 Derivation process for luma motion vectors for merge mode

##### 8.4.2.1.4 Derivation process for luma motion vector prediction

Table 9-1 and Table 9-2

### 3. 3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_tb_in_slice | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if (slice_type ! = I) { |  |
| max_active_merge_cands | u(3) |
| max_active_amvp_cands | u(2) |
| } |  |
| } else |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| } |  |

### 7.4.3 Slice header semantics

Add :

max_active_merge_cands specifies maximum code length of truncated unary table size used for merge_idx coding of the slice. The value of max_active_merge_cands is in the range of 1 to 5, inclusive.

max_active_amvp_cands specifies maximum code length of truncated unary table size used for mvp_idx coding of the slice. The value of max_active_amvp_cands is in the range of 1 to 3, inclusive.

### Syntax

### 7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| If ( max_active_merge_cands > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2PUWidth >= Log2IPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| if( IntraPredMode[ x0 ][ y0 ] == 2 ) |  |
| planar_flag_luma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| if( IntraPredMode[ x0 ][ y0 ] != 2 && IntraPredMode[ x0 ][ y0 ]!=34 && SignaledAsChromaDC ) |  |
| planar_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( !InferredMergeFlag ) |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && max_active_merge_cands > 1) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if(!entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( LcToLx ) > 1 ) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| If ( max_active_amvp_cands > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| If ( max_active_amvp_cands > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

##### 8.4.2.1.1 Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N and the prediction list utilization flags predFlagL0N and predFlagL1N with N being replaced by A, B, C or D.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-1)

predFlagLXCol = availableFlagLXCol	(8-2)

- The merging candidate list, mergeCandList, is constructed of which elements are given as specified order:

  - A, if availableFlagA is equal to 1

  - B, if availableFlagB is equal to 1

  - Col, if availableFlagCol is equal to 1

  - C, if availableFlagC is equal to 1

  - D, if availableFlagD is equal to 1

  - Z,  zero merging candidate (always available)

- If several merging candidates (excluding Z merging candidate) have the motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- If the number of elements NumMergeCand max_active_merge_cands within the mergeCandList is equal to 1, mergeIdx is set equal to 0, otherwise, mergeIdx is set equal to merge_idx[ xP][ yP ].

- The following assignments are made with N being the candidate at position mergeIdx in the merging candidate list mergeCandList ( N = mergeCandList[ mergeIdx ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-3)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-4)

refIdxLX = refIdxLXN	(8-5)

predFlagLX = predFlagLXN	(8-6)

- If all availability flags availableFlagN (with N being replaced by A, B, Col, C, or D) are equal to 0, mergeIdx is set equal to 0 and The derivation process of  zero merging candidate mvLXZ, refIdxLXZ and predFlagLXZ (with X being replaced by 0 or 1) is are inferred derivated  as follows.

If slice_type is equal to P, the following applies.

mvLXZ[ 0 ] = 0	(8-7)

mvLXZ[ 1 ] = 0	(8-8)

refIdxL0Z = 0		  (8-9)

refIdxL1Z = -1	(8-10)

predFlagL0Z = 1	(8-11)

prefFlagL1Z = 0	(8-12)

Otherwise ( slice_type is equal to B ), the following applies.

mvLXZ[ 0 ] = 0	(8-13)

mvLXZ[ 1 ] = 0	(8-14)

refIdxL0Z = 0		(8-15)

refIdxL1Z = 0		(8-16)

predFlagL0Z = 1	(8-17)

prefFlagL1Z = 1	(8-18)

##### 8.4.2.1.4 Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, B as the output.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The motion vector predictor list, mvpListLX, is constructed of which elements are given as specified order:

- mvLXCol, if availableFlagLXCol is equal to 1

- mvLXA, if availableFlagLXA is equal to 1

- mvLXB, if availableFlagLXB is equal to 1

- If several motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- If the number of elements NumMVPCand( LX ) within the mvpListLX max_active_amvp_cands is equal to 1, mvpIdx is set equal to 0, otherwise, mvpIdx is set equal to mvp_idx_lX[ xP, yP ].

- The motion vector of mvListLX[ mvpIdx ] is assigned to mvpLX.

##### 8.4.2.1.4 Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, B as the output.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The motion vector predictor list, mvpListLX, is constructed of which elements are given as specified order:

- mvLXCol, if availableFlagLXCol is equal to 1

- mvLXA, if availableFlagLXA is equal to 1

- mvLXB, if availableFlagLXB is equal to 1

- If several motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- If the number of elements NumMVPCand( LX ) within the mvpListLX max_active_amvp_cands is equal to 1, mvpIdx is set equal to 0, otherwise, mvpIdx is set equal to mvp_idx_lX[ xP, yP ].

- The motion vector of mvListLX[ mvpIdx ] is assigned to mvpLX.

| Table 9-1 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| merge_idx | P | TU, cMax = max_active_merge_cands | 3 | Error: Reference source not found | 0 |
|  | B |  | 3 | Error: Reference source not found | 4 |
| mvp_idx_lc | B | TU, cMax = max_active_amvp_cands | 1 | Error: Reference source not found | 2 |
| mvp_idx_l0 | P | TU, cMax = max_active_amvp_cands | 1 | Error: Reference source not found | 0 |
|  | B |  | 1 | Error: Reference source not found | 2 |
| mvp_idx_l1 | B | TU, cMax = max_active_amvp_cands | 1 | Error: Reference source not found | 2 |

| Table 9-2 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| merge_idx | Error: Reference source not found | 0 | 0 | 1 | 2 | 3 | na |
|  |  | 4 | 0 | 1 | 2 | 3 | na |